3GPP TSG-RAN WG4 Meeting # 96-e 	R4-2011079

# Electronic Meeting, 17 August – 28 August, 2020

Agenda item:	7.1.5.9

Source: 	Huawei, HiSilicon

Title: 	Discussion on beam management requirements for NR-U

Document for:	Discussion

- Introduction

In last RAN#95e meeting the RRM requirements for beam management requirements in NR-U were discussed with agreements captured in [1]. In this paper, we further present our views on the remaining issues.

- Discussion

Based on the discussion in the last RAN4#95e meeting, the agreements and remaining issues are listed as follows:

- CSI-RS based CBD requirement - RAN4 start to discuss CSI-RS based BFD requirement after RAN1 conclude on CSI-RS validation - CSI-RS based RLM requirement - Continue discussion in #96e meeting - SSB based BFD requirement - Wait for the conclusion of SSB based OOS requirement - CSI-RS based BFD requirement - RAN4 start to discuss CSI-RS based BFD requirement after RAN1 conclude on CSI-RS validation. - CSI-RS based CBD requirement - RAN4 start to discuss CSI-RS based BFD requirement after RAN1 conclude on CSI-RS validation.

Following the conclusion from RP#88e [2], only SSB-based requirements are considered in this paper. As analysis in the RLM part, the SSB-based BFD shall follow the same principles to define the evaluation period.

Observation 1: The SSB-based BFD requirements shall follow the same principles as RLM.

We present the similar analysis in the paper for RLM [3]. Thus, we present the following text proposals for the evaluation period for SSB-based BFD:

| Configuration | TEvaluate_BFD_SSB_CCA (ms) |
| --- | --- |
| no DRX | Max(50, Ceil((5-LBFD,available) K P) TSSB) |
| DRX cycle ≤ 320ms | Max(50, Ceil(1.5 (5- LBFD,available) K P) Max(TDRX,TSSB)) |
| DRX cycle > 320ms | Ceil((5- LBFD,available ) K P) TDRX |
| Note 1: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. Note 2: LBFD,avaiable is the number of available SSB RLM-RS at UE, where Es/Iot > [-7] dB Note 3: K = [2] for Max(TDRX,TSSB) ≤ 320 assuming TDRX=0 for non-DRX case, K = [1.5] for TDRX>320. |  |

Proposal 2:

| Configuration | TEvaluate_BFD_SSB_CCA (ms) |
| --- | --- |
| no DRX | Max(50, Ceil((5-LBFD,available) K P) TSSB) |
| DRX cycle ≤ 320ms | Max(50, Ceil(1.5 (5- LBFD,available) K P) Max(TDRX,TSSB)) |
| DRX cycle > 320ms | Ceil((5- LBFD,available ) K P) TDRX |
| Note 1: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. Note 2: LBFD,avaiable is the number of available SSB RLM-RS at UE, where Es/Iot > [-7] dB Note 3: K = [2] for Max(TDRX,TSSB) ≤ 320 assuming TDRX=0 for non-DRX case, K = [1.5] for TDRX>320. |  |

- Conclusions

Observation 1: The SSB-based BFD requirements shall follow the same principles as RLM.

Proposal 1:

| Configuration | TEvaluate_BFD_SSB_CCA (ms) |
| --- | --- |
| no DRX | Max(50, Ceil((5-LBFD,available) K P) TSSB) |
| DRX cycle ≤ 320ms | Max(50, Ceil(1.5 (5- LBFD,available) K P) Max(TDRX,TSSB)) |
| DRX cycle > 320ms | Ceil((5- LBFD,available ) K P) TDRX |
| Note 1: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. Note 2: LBFD,avaiable is the number of available SSB RLM-RS at UE, where Es/Iot > [-7] dB Note 3: K = [2] for Max(TDRX,TSSB) ≤ 320 assuming TDRX=0 for non-DRX case, K = [1.5] for TDRX>320. |  |

References

[1] R4-2008567 “WF on NR-U RRM Requirements (Part 2)”, MediaTek inc.

[2] R4-2005367 “WF on NR-U RRM Requirements (Part 2)”, MediaTek inc.

[3] R4-20xxxxx Discussion on RLM requirements for NR-U