3GPP TSG-RAN4 Meeting #96-e	R4-2011109

# Electronic Meeting, 17th – 28th August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1061 | rev | - | Current version: | 15.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to inter-RAT measurement on NR serving carrier |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed e?planations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In TS 36.133 clause 8.17.4.1 it is specified that the when UE is configured with EN-DC the intra-RAT NR measurement on NR serving carrier should obey requirements for NR intra-frequency measurements. On the other hand. Intra-frequency measurement shall be performed without MG if SSB Is completely contained by active BWP. As a result, it implies that intra-RAT measurement on NR serving carrier shall also be performed without MG in some cases. However, It conflicts with the calculation of CSSFoutside_gap given in 38.133. cl. 9.1.5.1. One can observe that in RAN4’s understanding only intra-frequency meansurements are considered in CSSFoutside_gap in Rel-15. Then UE don’t know how to calculate CSSF for inter-RAT NR measurments on serving carriers. Measurement delay requirement for inter-RAT measurement on serving carrier is unclear. The carrier-specific scaling factor CSSFoutside_gap,i for measurement object i derived in this chapter is applied to following measurement types: - Intra-frequency measurement with no measurement gap in clause 9.2.5, when none of the SMTC occasions of this intra-frequency measurement object are overlapped by the measurement gap. - Intra-frequency measurement with no measurement gap in clause 9.2.5, when part of the SMTC occasions of this intra-frequency measurement object are overlapped by the measurement gap. UE is expected to conduct the measurement of this measurement object i only outside the measurement gaps. For UE configured with the E-UTRA-NR dual connectivity operation, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency SSB-based measurements performed outside measurements gaps will be as specified in Table 9.1.5.1.1-1. Table 9.1.5.1.1-1: CSSFoutside_gap,i scaling factor for EN-DC mode Scenario; CSSFoutside_gap,i for FR1 PSCC; CSSFoutside_gap,i for FR1 SCC; CSSFoutside_gap,i for FR2 PSCC; CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required Note 2; CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required. EN-DC with FR1 only CA; 1; Number of configured FR1 SCell(s); N/A; N/A; N/A. EN-DC with FR2 only intra band CA; N/A; N/A; 1; N/A; Number of configured FR2 SCells. EN-DC with FR1 +FR2 CA (FR1 PSCell) Note 1; 1; 2×(Number of configured SCell(s)-1); N/A; 2; 2×(Number of configured SCell(s)-1). EN-DC with FR1 +FR2 CA (FR2 PSCell) Note 1; N/A; Number of configured SCell(s); 1; N/A; Number of configured SCell(s). Note 1: Only one NR FR1 operating band and one NR FR2 operating band are included for FR1+FR2 inter-band EN-DC. Note 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2... So we purpose to take inter-RAT measurement on serving carrier into account in the calculation of CSSFoutside_gap. To be more specific, inter-RAT measurement shall be treated as intra-frequency measurement on SCC. Then the baseline assumption for CSSFoutside_gap calculation is: - UE equips two searchers; - One searcher is dedicated for PSCC; - If a FR2 SCC is configured to UE and it is the first activated serving carrier in that band, it will use half the measurement capability of the second searcher. - All the intra-frequency measurements on other SCells and inter-RAT measurements on NR serving carriers equally share the rest measurement capability of the second searcher. |
| Summary of change: |  | Take inter-RAT measurement on serving carrier into account in the calculation of CSSFoutside_gap. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE can not satisfiy the measurement requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

9.1.5.1	Monitoring of multiple layers outside gaps

The carrier-specific scaling factor CSSFoutside_gap,i for measurement object i derived in this chapter is applied to following measurement types:

-	Intra-frequency measurement with no measurement gap in clause 9.2.5, when none of the SMTC occasions of this intra-frequency measurement object are overlapped by the measurement gap.

-	Intra-frequency measurement with no measurement gap in clause 9.2.5, when part of the SMTC occasions of this intra-frequency measurement object are overlapped by the measurement gap.

-	Inter-RAT measurement in TS 36.133 [15] clause 8.17.4, when

-	this inter-RAT measurement object is configured on serving carrier, and

-	the SSB is completely contained in the active BWP of the UE, and

-	none or part of the SMTC occasions of this inter-RAT measurement object are overlapped by the measurement gap;

UE is expected to conduct the measurement of this measurement object i only outside the measurement gaps.

If the higher layer signaling in TS 38.331 [2] signaling of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, CSSFoutside_gap,i and requirements derivied from CSSFoutside_gap,i are not specified.

The UE cell identification and measurement periods derived based on CSSFoutside_gap,i in clauses 9.2.5.1, 9.2.5.2 may be extended for measurement objects of which the cell identification and measurement periods are overlapped with Tmeasure_SFTD1 specified in clause 9.3.8 when no measurement gaps are provided.

9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps

For UE configured with the E-UTRA-NR dual connectivity operation, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency SSB-based measurements performed outside measurements gaps will be as specified in Table 9.1.5.1.1-1.

Table 9.1.5.1.1-1: CSSFoutside_gap,i scaling factor for EN-DC mode

| Scenario | CSSFoutside_gap,i for FR1 PSCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PSCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required Note 2 | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,I for inter-RAT measurement on serving carriers |
| --- | --- | --- | --- | --- | --- | --- |
| EN-DC with FR1 only CA | 1 | Number of configured FR1 SCell(s) +number of inter-RAT measurement on serving carriers | N/A | N/A | N/A | Number of configured FR1 SCell(s)+number of inter-RAT measurement on serving carriers |
| EN-DC with FR2 only intra band CA | N/A | N/A | 1 | N/A | Number of configured FR2 SCells+number of inter-RAT measurement on serving carriers | Number of configured FR2 SCells +number of inter-RAT measurement on serving carriers |
| EN-DC with FR1 +FR2 CA (FR1 PSCell) Note 1 | 1 | 2×(Number of configured SCell(s) + number of inter-RAT measurement on serving carriers -1) | N/A | 2 | 2×(Number of configured SCell(s) + number of inter-RAT measurement on serving carriers -1) | 2×(Number of configured SCell(s)+ number of inter-RAT measurement on serving carriers-1) |
| EN-DC with FR1 +FR2 CA (FR2 PSCell) Note 1 | N/A | Number of configured SCell(s) +number of inter-RAT measurement on serving carriers | 1 | N/A | Number of configured SCell(s) +number of inter-RAT measurement on serving carriers | Number of configured FR1 SCell(s)+number of inter-RAT measurement on serving carriers |
| Note 1: Only one NR FR1 operating band and one NR FR2 operating band are included for FR1+FR2 inter-band EN-DC. Note 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2. |  |  |  |  |  |  |

<End of modified section 1>