3GPP TSG-RAN WG4 Meeting # 96-e	R4-2011217

# Electronic Meeting, 17 August - 28 August 2020

Agenda item:	13.1.2

Source:	Keysight Technologies

Title:	On minimizing the impact of polarization basis mismatch between the TE and DUT on the RF testing

Document for:	Approval

# Introduction

This contribution outlines our view on topic #2 of this SI [1] to define solutions to minimize the impact of polarization basis mismatch between the TE and DUT on the RF testing.

# Discussion

Various test or augmented test system approaches to minimize the impact of polarization basis mismatch were presented in the past, e.g., [2][3].

In [2], it was proposed to leverage the gNB feedback precoder, i.e., TPMI, to force the UE to transmit simultaneously on both polarizations

Proposals; Solve polarization basis mismatch issue?; Support advanced beam management? (# from hardware capability view). Current Test Method; No; No. Proposal 1; Yes; (1) gNB feedback precoder (i.e. TPMI) (2) 2-port CSI-RS beam management. Proposal 2; Yes; 2-port CSI-RS beam management. Proposal 3; Yes; No. Table 2. Test facility hardware capability [from [2]]

In a subsequent email discussion held on this topic, the use of TPMI was discussed further [4] with varying views on the applicability of TPMI on minimizing the impact of polarization basis mismatch.

## 3.2 Objective 2 aspects ### 3.2.1 EIRP measurement Qualcomm: discussion has focused on UE behavior, which is closer to a core requirement; responsibility of testability ends when the test system can correctly capture the radiated power Samsung: we agree that current test procedure captures the power from the UE; the problem is the test configuration should make sure the UE transmit with 2 Tx to align with the core requirement Sony: we believe the UE should always transmit with 2 Tx regardless of the polarization of the downlink signal Apple: we agree with Samsung LGE: same view as Samsung and Apple; if UE transmits with 1 pol, this could be an issue with core requirement MediaTek: we agree with Samsung; from the email discussion, maybe only 2 companies think that this is not needed to be changed; 2 other companies think how to trigger 2 Tx and they recognize the issue Qualcomm: more than 2 companies mentioned that a change in EIRP procedure is not needed MediaTek: we see three camps: one camp sees the need for a change; second camp sees current test procedure can capture UE EIRP correctly, but based on this procedure in some cases the UE can only be triggered with 1 TX, and they are open to correct this; third camp sees no need for change Rapporteur: can we discuss camp #2 and how to trigger the UE to 2 Tx? Qualcomm: we have a concern that the gNB does not have this ability to always trigger 2 Tx Sony: we share the same view as Qualcomm Keysight: it would help if we knew what the BS actually does Samsung: we think that the gNB can decide the UE transmission mode (decided by precoding) Sony: if Samsung’s comment is valid, then we need to introduce new signaling Qualcomm: it is correct the TPMI matrix refers to 1 port, but this is a logical port; it is up to UE implementation how to map Tx chains to logical ports Samsung: in the RAN5 MOP test it is a common rule that the test configuration should clarify the detailed mode; it is not new signaling Apple: we had a paper that showed in a real deployment gNB signal polarization modes are not constrained and can vary over time; this is not the case in the test system Keysight: what can vary? Apple: we are saying there are no constraints on the polarization modes of the gNB signal Qualcomm: are we discussing non-LOS testing? [Rapporteur] Does it help to focus on the methods which are reliable and easy to trigger 2TX transmission? MediaTek: we think this is helpful Samsung: agree R&S: since it is not described in the specification, how can 2TX be triggered? Could this be implementation dependent? Keysight: the trigger is implementation dependent; is this a must-have feature for the test system? LGE: we need to differentiate between 1TX and 2TX; first we need to find how to trigger 2TX during test; if we consider a new method to do this, then we prefer to consider test time also Anritsu: we have a concern about actual UE behavior in the field; there must be a case when the UE cannot transmit minimum output power R&S: can TPMI be the solution? Samsung: we think the R&S suggestion is good Qualcomm: we also think that the TPMI method is good, but it is only available to Rel-16 UEs Keysight: if TPMI solves this problem for Rel-16 and onwards, then we could use Option 7 (test mode) for Rel-15 UEs Qualcomm: we are not OK with the test mode Samsung: we understand that Rel-15 UEs have limitations on TPMI; but Rel-15 core requirement and test method have a conflict; as Keysight mentioned, test mode may be able to resolve this Qualcomm: if compliance testing is release agnostic, is this something that can be done? R&S: for testing this would not be an issue; there can be different test configurations for different releases Qualcomm: but this is still not a solution for UEs which have polarization specific beam correspondence MediaTek: we are fine with the TPMI solution Apple: we can accept the majority view related to the TPMI solution Sony: we are OK with the TPMI solution Qualcomm: so Rel-15 UEs will not have TPMI solution; Rel-16+ UEs can use it; is this the right understanding? LGE: we are fine with the TPMI solution; we consider both Rel-15 and Rel-16 for this Qualcomm: in our understanding, this solution is not available to both releases Rapporteur: it looks like we are converging on a solution for Rel-16+, but there is still an open issue related to Rel-15 UEs R&S: in testing we can always use TPMI regardless of the release; Rel-15 UEs can ignore this; so there may only be a subset of devices which don’t do anything with this; we consider this a good step forward Keysight: does TPMI really force the UE to transmit on both polarizations? We think we are still far away from convergence on this; we don’t need to worry about Rel-15 UEs, since they must pass certification with test systems available today Samsung: we think that the TPMI can really force the UE to transmit on both polarizations Qualcomm: we think that it still up to UE implementation; this is not automatic

Here, we present our views and subsequent comments/questions to industry.

The Transmitted Matrix Precoding Indicator (TPMI) for NR FR2 was introduced in RAN1 led TS 38.212 for Rel-15 [5] but important and potentially relevant changes were made in Rel-16 [6].

The precoding matrix with indices 0-5 are defined in [7], specifically Table 6.3.1.5-1

Table 6.3.1.5-1: Precoding matrix for single-layer transmission using two antenna ports. TPMI index; (ordered from left to right in increasing order of TPMI index). 0 – 5; -; -.

While indices 0 and 1 (highlighted in yellow) force single-layer transmission using a single antenna port, indices 2-5 (highlighted in green) force single-layer transmission using two antenna ports.

## TPMI for Fully or Partially Coherent UEs

For coherent UEs, both Rel-15 and Rel-16 allow a TPMI to be used that forces single-layer transmission using two antenna ports, as highlighted in Table 7.3.1.1.2-4 of the Rel-15 spec [5]

Table 7.3.1.1.2-4: Precoding information and number of layers, for 2 antenna ports, if transform precoder is disabled and maxRank = 2 Bit field mapped to index; codebookSubset = fullyAndPartialAndNonCoherent; Bit field mapped to index; codebookSubset = nonCoherent. 0; 1 layer: TPMI=0; 0; 1 layer: TPMI=0. 1; 1 layer: TPMI=1; 1; 1 layer: TPMI=1. 2; 2 layers: TPMI=0; 2; 2 layers: TPMI=0. 3; 1 layer: TPMI=2; 3; reserved. 4; 1 layer: TPMI=3. 5; 1 layer: TPMI=4. 6; 1 layer: TPMI=5. 7; 2 layers: TPMI=1. 8; 2 layers: TPMI=2. 9-15; reserved.

and the same table of the Rel-16 spec [6]

Table 7.3.1.1.2-4: Precoding information and number of layers, for 2 antenna ports, if transform precoder is disabled, maxRank = 2, and ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower Bit field mapped to index; codebookSubset = fullyAndPartialAndNonCoherent; Bit field mapped to index; codebookSubset = nonCoherent. 0; 1 layer: TPMI=0; 0; 1 layer: TPMI=0. 1; 1 layer: TPMI=1; 1; 1 layer: TPMI=1. 2; 2 layers: TPMI=0; 2; 2 layers: TPMI=0. 3; 1 layer: TPMI=2; 3; reserved. 4; 1 layer: TPMI=3. 5; 1 layer: TPMI=4. 6; 1 layer: TPMI=5. 7; 2 layers: TPMI=1. 8; 2 layers: TPMI=2. 9-15; reserved.

Hence, for coherent UEs, both Rel-15 and Rel-16 allow a TPMI to be used that forces single-layer transmission using two antenna ports.

Observation 1: For coherent UEs, both Rel-15 and Rel-16 allow a TPMI to be used that forces single-layer transmission using two antenna ports.

## TPMI for Non-Coherent UEs

For non-coherent UEs, however, Rel-15 does not allow a TPMI to be used that forces single-layer transmission using two antenna ports as highlighted in Table 7.3.1.1.2-4 of the Rel-15 spec [5]

Table 7.3.1.1.2-4: Precoding information and number of layers, for 2 antenna ports, if transform precoder is disabled and maxRank = 2 Bit field mapped to index; codebookSubset = fullyAndPartialAndNonCoherent; Bit field mapped to index; codebookSubset = nonCoherent. 0; 1 layer: TPMI=0; 0; 1 layer: TPMI=0. 1; 1 layer: TPMI=1; 1; 1 layer: TPMI=1. 2; 2 layers: TPMI=0; 2; 2 layers: TPMI=0. 3; 1 layer: TPMI=2; 3; reserved. 4; 1 layer: TPMI=3. 5; 1 layer: TPMI=4. 6; 1 layer: TPMI=5. 7; 2 layers: TPMI=1. 8; 2 layers: TPMI=2. 9-15; reserved.

Rel-16, on the other hand, introduced a new transmission mode, i.e., ul-FullPowerTransmission = fullpowerMode1, which allows even non-coherent UE to force single-layer transmission using two antenna ports as highlighted in Table 7.3.1.1.2-4A of [6]

Table 7.3.1.1.2-4A: Precoding information and number of layers, for 2 antenna ports, if transform precoder is disabled, maxRank = 2, and ul-FullPowerTransmission = fullpowerMode1 Bit field mapped to index; codebookSubset= nonCoherent. 0; 1 layer: TPMI=0. 1; 1 layer: TPMI=1. 2; 2 layers: TPMI=0. 3; 1 layer: TPMI=2.

Observation 2: For non-coherent UEs, Rel-15 does not allow a TPMI to be used that forces single-layer transmission using two antenna ports.

Observation 3: For non-coherent UEs, Rel-16 allows a TPMI to be used that forces single-layer transmission using two antenna ports with the newly introduced ul-FullPowerTransmission = fullpowerMode1 mode.

In summary, TPMI does not allow all UEs to force single-layer transmission using two antenna ports. TPMI therefore does not seem to be a reliable approach to minimize the impact of polarization basis mismatch.

On the other hand, OEMs should comment whether RAN1’s definition of TPMI indices 2-5 forcing single-layer transmission using two antenna ports corresponds to the UE enabling two transmit chains at all times.

Observation 4:  OEMs to comment whether RAN1’s definition of TPMI indices 2-5 forcing single-layer transmission using two antenna ports corresponds to the UE enabling two transmit chains at all times

Since the TPMI cannot guarantee that all UEs, specifically Rel-15 non-coherent UEs, simultaneously transmit on both transmit chain/antennas, it is proposed to focus on the polarization scan which can reliably force the UE to simultaneously transmit on both transmit chain/antennas [3] in order to minimize the impact of polarization basis mismatch between the TE and DUT on the RF testing..

Proposal 1: RAN4 to focus on the polarization scan which can reliably force the UE to simultaneously transmit on both transmit chain/antennas in order to minimize the impact of polarization basis mismatch between the TE and DUT.

# Conclusion

The following observations and proposals were made in this contribution

Observation 1: For coherent UEs, both Rel-15 and Rel-16 allow a TPMI to be used that forces single-layer transmission using two antenna ports.

Observation 2: For non-coherent UEs, Rel-15 does not allow a TPMI to be used that forces single-layer transmission using two antenna ports.

Observation 3: For non-coherent UEs, Rel-16 allows a TPMI to be used that forces single-layer transmission using two antenna ports with the newly introduced ul-FullPowerTransmission = fullpowerMode1 mode.

Observation 4:  OEMs to comment whether RAN1’s definition of TPMI indices 2-5 forcing single-layer transmission using two antenna ports corresponds to the UE enabling two transmit chains at all times

Proposal 1: RAN4 to focus on the polarization scan which can reliably force the UE to simultaneously transmit on both transmit chain/antennas in order to minimize the impact of polarization basis mismatch between the TE and DUT.

# References

- RP-192322, New SID: Study on enhanced test methods for FR2, Apple Inc., CAICT, 3GPP TSG-RAN Meeting #85, September 2019

- R4-1913674, FR2 SISO enhancement for polarization mismatch, MediaTek Inc., 3GPP RAN TSG RAN4 Meeting #93, November 2019

- R4-1915082, On Minimizing Impact of Polarization Basis Mismatch, Keysight Technologies, 3GPP RAN TSG RAN4 Meeting #93, November 2019

- R4-2009959, FR2 test method enhancement informal email discussion summary, Apple, 3GPP RAN TSG RAN4 Meeting #96-e, August 2020

- TS 38.212, Multiplexing and channel coding, V15.9.0 (2020-06)

- TS 38.212, Multiplexing and channel coding, V16.2.0 (2020-06)

- TS 38.211, Physical channels and modulation, V15.8.0 (2019-12)