3GPP TSG-RAN WG4 Meeting#96-e	R4-2011395

Online, 17th Aug – 28th Aug, 2020

Source: 	Qualcomm Incorporated

Title: 	Views on CA CQI Reporting Tests

Agenda item:	7.16.1.5

Document for:	Discussion

# Introduction

In WF [1], there were many open issues listed for CA CQI reporting requirements. In this paper, we discuss our views on test cases for CA CQI reporting.

# Duplex Mode and SCS

In WF [1], there were two options listed for duplex mode and SCS. As we have already covered all the CA combinations for NR CA normal demodulation test cases, we do not need to cover all of them again for CA CQI reporting requirements. Therefore, our preference is Option 2 and we propose the following.

Proposal 1: Define the CA CQI requirements for following cases:

- FR1: FDD + FDD with 15 kHz SCS and TDD + TDD with 30 kHz SCS

- FR2: TDD + TDD with 120 kHz SCS

# FR2 TDD Pattern

In 38.101-4, all the existing FR2 CQI requirements are defined with TDD pattern DDSU. So, we would like to reuse the same pattern for defining CA CQI requirements for FR2. Therefore, we propose the following.

Proposal 2: For defining FR2 CA CQI requirements, use DDSU (S = 11D+3G) TDD Pattern and CSI reporting periodicity of 8 slots.

# Test Requirements

Based on our simulation results, we are ok with Option 1 in WF [1] for SNR configurations and threshold for CA CQI reporting tests. Therefore, we propose the following.

Proposal 3: Define CA CQI reporting requirements with the following configuration:

- SNR configuration for 2DL CA CQI test: SNRPcell = 10dB and SNRScell = 4dB

- SNR configuration for 3 or more DL CA CQI test: SNRPcell = 12dB, SNRScell1 = 6dB, SNRScell2, 3,… = 0dB

- Delta CQI threshold for CA CQI test = 2 for 2 or more DL CA

Proposal 4: For 4Rx requirements, reduce the SNRs by 3dB compared to that for 2Rx.

# Conclusions

This paper discusses open issues for power imbalance test cases. Following has been proposed.

Proposal 1: Define the CA CQI requirements for following cases:

- FR1: FDD + FDD with 15 kHz SCS and TDD + TDD with 30 kHz SCS

- FR2: TDD + TDD with 120 kHz SCS

Proposal 2: For defining FR2 CA CQI requirements, use DDSU (S = 11D+3G) TDD Pattern and CSI reporting periodicity of 8 slots.

Proposal 3: Define CA CQI reporting requirements with the following configuration:

- SNR configuration for 2DL CA CQI test: SNRPcell = 10dB and SNRScell = 4dB.

- SNR configuration for 3 or more DL CA CQI test: SNRPcell = 12dB, SNRScell1 = 6dB, SNRScell2, 3,… = 0dB.

- Delta CQI threshold for CA CQI test = 2 for 2 or more DL CA.

Proposal 4: For 4Rx requirements, reduce the SNRs by 3dB compared to that for 2Rx.

# References

[1] R4-2008849, “Way forward on CA CQI reporting requirements”, 3GPP TSG RAN WG4 Meeting #95-e, Online, May 25 - Jun 5, 2020.