3GPP TSG-RAN WG4 Meeting # 96-e 	R4-2011400

# Electronic Meeting, 17-28 Aug., 2020

Source:	Sony

Title:	Test frequencies for NB-IOT UE in standalone operation

Agenda item:	5.2

Document for:	Discussion

- Background

Band-edge emission at stand-alone and guard-band NB-IoT conditions is regulated by FCC OOB emission requirements where there are quite stringent requirements. Below we discuss a testing requirement and a possible inconsistency between the UE test specification TS 36.508 and the base station specification TS 36.104.

- Discussion

The issue is related to band-edge emission at stand-alone and guard-band NB-IoT conditions that is regulated by FCC OOB emission requirements.

According to 3GPP TS 36.508 [1] the NB-IoT channel in stand-alone and guard-band operating modes may be allocated just at 100kHz offset from the band edges; see for example band 12.

Table 8.1.3.1.1.12-1: NB-IoT standalone Test frequencies for operating band 12

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 23011 | 0 | 699.1 | 5011 | -0.5 | 729.1 |
| Mid Range | 23095 | 0 | 707.5 | 5095 | -0.5 | 737.5 |
| High Range | 23179 | 0 | 715.9 | 5179 | -0.5 | 745.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.12-3: NB-IoT guard-band Test frequencies for operating band 12

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 23011 | 0 | 699.1000 | 5011 | 1 | 729.1075 |
| Mid Range | 5 | 23071 | 0 | 705.1000 | 5071 | 1 | 735.1075 |
|  | 10 | 23141 | 0 | 712.1000 | 5141 | 1 | 742.1075 |
| High Range | Any | 23179 | 0 | 715.9000 | 5179 | -2 | 745.8925 |
| NOTE 1: Void NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

The table refers to a NB-IoT channel of 180kHz, so the first NB-IoT tone (15kHz or 3.75kHz) will be only about 10kHz from the band edges where the FCC regulation applies.

One way to avoid the violation of this tough FCC regulation would be to omit operation at the first and last NB-IoT EARFCNs in stand-alone and guard-band modes (in the above example NUL=23011 and NUL=23179).

This could somehow be justified by the Foffset specified in Table 5.6-3A in TS 36.104 [2] defining 200kHz offset from band edges for NB-IoT Rx and Tx requirements in stand-alone operation for base stations. However, applying this for UEs would formally conflict with the above TS 36.508 test requirements defining 100kHz offset for UE.

Table 5.6-3A: Foffset for NB-IoT standalone operation

| Lowest or Highest Carrier | Foffset |
| --- | --- |
| Standalone NB-IoT | 200 kHz |

Given the above reasoning we can see an inconsistency between TS 36.508 and FCC regulation and also between TS 36.508 and TS 36.104. This inconsistency applies to all (NB-IOT) frequency bands where the FCC regulation applies.

Observation 1:	TS 36.104 test conditions (test frequencies) for both stand-alone and guard-band NB-IoT operation are conflicting with FCC band-edge spectrum emission requirements.

Observation 2:	There is also an inconsistency between TS 36.508 and TS 36.104 regarding testing of NB-IOT UE in standalone operation.

Proposal 1:	RAN4 to confirm there is inconsistency between TS 36.508 and FCC regulation and between TS 36.104 and TS 36.508.

Proposal 2:	If so, send an LS to RAN5 with proposal to exclude the first and last EARFCNs in TS 36.104 test frequencies for both stand-alone and guard-band IoT operation modes for all frequency bands were FCC regulation applies.

- Conclusion

In this contribution, we have discussed testing of NB-IOT and found a potential mismatch in the specifications. The following observations and proposals are made.

Observation 1:	TS 36.104 test conditions (test frequencies) for both stand-alone and guard-band NB-IoT operation are conflicting with FCC band-edge spectrum emission requirements.

Observation 2:	There is also an inconsistency between TS 36.508 and TS 36.104 regarding testing of NB-IOT UE in standalone operation.

Proposal 1:	RAN4 to confirm there is inconsistency between TS 36.508 and FCC regulation and between TS 36.104 and TS 36.508.

Proposal 2:	If so, send an LS to RAN5 with proposal to exclude the first and last EARFCNs in TS 36.104 test frequencies for both stand-alone and guard-band IoT operation modes for all frequency bands were FCC regulation applies.

- References

TS 36.508 “Common test environments for User Equipment (UE) conformance testing”, Section 8.1.3

TS 36.104 “Base Station (BS) radio transmission and reception”, Section 5.6