3GPP TSG-RAN WG4 Meeting #96-e	R4-2011510

Electronic Meeting, 17 – 28 August, 2020

Agenda item:	7.12.1.5

Source: 		NTT DOCOMO, INC.

Title: 	PSD for FR2 inter-band CA with IBM

Document for:	Approval

- Introduction

RAN4#93 agreed that PSD difference shall be considered in the conformance test configuration for 28GHz + 40GHz FR2 inter band DL CA [1]. RAN4#94-e discussed further the feasibility and testability of PSD difference. In RAN4#94-e-bis, approved WF [2] contains remaining issues about how to decide CBM (Common Beam Management) and IBM (independent Bean management), and what values of PSD difference for both CBM and IBM. This paper shows our views about the PSD imbalance requirements for IBM, and how to specify the requirements.

The content of approved WF is shown as below:

- Discussion

2.1 PSD difference for IBM band pairs

In [3], we summarized our view about PSD difference for IBM band pairs from the point view of “Necessity”, “Feasibility”, and “Testability”, and proposed to specify 25-30 dB PSD difference requirements for non-co-located BS deployment scenario. However, different companies have different views on this PSD related requirements. For 25-30dB power difference, there was a concern that it seems difficult to address such a PSD level in real products. In addition, another issue is the Rx performance such as peak EIS and spherical coverage EIS are different between bands, and it may difficult to apply a fixed PSD value when designing test condition. Now, this is the time Rel-16 WI should be finalized, we would like to seek a middle way between companies.

To address the concerns above mentioned, we would like to discuss a proposal from [4] and propose a modified proposal. Our proposal for DL power condition for PSD difference for IBM inter-band CA is shown in Table 2.1-1. The power condition is based on Peak EIS and spherical coverage EIS for each band. Power imbalance is equal to “Delta” which is power difference between peak EIS of one band and spherical coverage of the other band. One example is shown in Figure 2.1-1. For test case 1, if Delta B-A is larger than Delta A-B like band A=28GHz band and band B=40GHz band, DL power for band A is “Peak EIS of band A + Multi-band relaxation” and DL power for band B is “Spherical coverage EIS of Band B + Multi-band relaxation + inter-band CA relaxation”. Note that this proposal is based on the assumption that relaxation for inter-band CA applies to spherical coverage EIS but not to peak EIS.

This proposal can accommodate the Rx performance difference among different bands. If both Band A and Band B are 28GHz bands, then “Delta” would be smaller. On the other hand, if Band A is 28GHz band and band B is 40GHz band, then “Delta” would be larger. This can take into account the fact the PSD difference of 28GHz + 40GHz could be larger than 28GHz + 28GHz or 40GHz + 40GHz due to the difference of propagation loss and beam direction. In this method, PSD difference is about 10-15dB for 28GHz + 28GHz or 40GHz + 40GHz, and about 15-20dB for 28GHz + 40GHz. But we would like to note the value is depending on which bands we use and how relaxation we apply.

Proposal: DL power condition for PSD imbalance requirements for  FR2 inter-band CA with IBM capability (Bands A+B) should be specified as following:

- Test case 1:

  - DL power of band A = Peak EIS of band A + Multi-band relaxation

  - DL power of band B = Peak EIS of band A + Multi-band relaxation + Delta

- Test case 2:

  - DL power of band A = Peak EIS of band B + Multi-band relaxation

  - DL power of band B = Peak EIS of band B + Multi-band relaxation + Delta

- Delta = MAX (Delta A-B, Delta B-A)

  - Delta A-B = (Spherical coverage EIS of Band A + Multi-band relaxation + inter-band CA relaxation)-(Peak EIS of Band B + Multi-band relaxation)

  - Delta B-A = (Spherical coverage EIS of Band B + Multi-band relaxation + inter-band CA relaxation)-(Peak EIS of Band A + Multi-band relaxation)

Table 2.1-1: DL power condition for PSD difference for IBM inter-band CA

Figure 2.1-1: Example for Test case 1 with Delta=Delta B-A

- Conclusion

In this contribution, we proposed a compromised method for PSD imbalance requirement for IBM FR2 DL inter-band CA.

References

[1] R4-1916024, “WF on FR2 inter-band DL CA”, Nokia, Nokia Shanghai Bell

[2] R4-2005676, “WF on inter-band DL CA”, Qualcomm

[3] R4-2004814, “PSD difference on FR2 inter-band CA 28GHz+40GHz”, NTT DOCOMO, INC.

[4] R4-2008051, “Inter-band CA remaining open requirements”, Qualcomm Incorporated