3GPP TSG-RAN WG4 Meeting #96-e                                                               R4-2011583

Electronic Meeting, 17th August – 28th August, 2020

Source:	Huawei, HiSilicon

Title:	TP for TR 36.717-03-01: CA_8A-20A-38A

Agenda item:	14.2.3

Document for:	Approval

Background

This contribution provides text proposal on the LTE CA configuration CA_8A-20A-38A as defined in the revised WID [1].

Reference

[1]	RP-201023, “New WID: LTE Advanced inter-band CA Rel-17 for 3 bands DL with 1 band UL”, Huawei, HiSilicon

Text Proposal

---Start of changes---

5.x	CA_8-20-38

5.x.1	Channel bandwidths per operating band for CA

Table 5.x.1-2: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_8A-20A-38A | - | 8 |  |  | Yes | Yes |  |  | 50 | 0 |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

5.x.2	∆TIB and ∆RIB values

Table 5.x.2-1: IB,c

| CA_8-20-38 | 8 | 0.4 |
| --- | --- | --- |
|  | 20 | 0.4 |
|  | 38 | 0.3 |

Table 5.x.2-2: R IB,c

| CA_8-20-38 | 8 | 0 |
| --- | --- | --- |
|  | 20 | 0 |
|  | 38 | 0 |

5.x.3 	REFSENS requirements

The exception due to band n20 harmonic issue is specified as below for CA_8A-20A-38A.

Table 5.x.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_8A-20A-38A8 | 38 |  |  | N/A | N/A | N/A | N/A | TDD |
| NOTE 8: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). |  |  |  |  |  |  |  |  |

---End of changes---