3GPP TSG-RAN WG4 Meeting # 97-e 	R4-2014241

# Electronic Meeting, 2 – 13 Nov, 2020

Agenda item:	7.8.1.1.1

Source:	Apple Inc.

Title:	UE demodulation requirements for Ultra low BLER

Release:	Rel-16

Document for:	Discussion

# 1. Introduction

In RAN4#96e UE demodulation requirements for URLLC with ultra-low BLER was discussed and way forward [1] was agreed. In this contribution we present simulation results for PDSCH demodulation for ultra-low BLER requirements.

# 2. Discussion

The agreements in [1] related to UE demodulation requirements for ultra-low BLER:

- No ultra-low BLER requirements for the following features within this WI: - PDCCH enhancements - Multi-TRP for URLLC - FR2: Not introduce FR2 requirements in Rel-16 for URLLC with ultra-BLER - No requirement for 10^-6 BLER in rel-16

The agreed simulation assumption for PDSCH demodulation for ultra-low BLER is given in table below.

Table 1: Agreed UE Simulation Assumptions

| Parameter |  | Value |
| --- | --- | --- |
| Frequency range |  | FR1 |
| Transform precoding |  | Disabled |
| Duplex mode |  | FDD/TDD |
| Antenna configuration |  | 1x2, ULA low 1x4, ULA low |
| PDSCH configuration | Mapping type | Type A |
|  | Starting symbol (S) | 2 |
|  | Length (L) | 12 |
|  | PUSCH aggregation factor | 1 |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | DM-RS duration | Single-symbol DM-RS |
|  | Number of additional DMRS | 1 |
| Number of HARQ Transmissions |  | 1 |
| PT-RS |  | Disabled |
| Propagation condition |  | AWGN |
| MCS Table |  | Table 3, MCS14 |
| SCS and BW |  | FDD:15KHz, 10MHz TDD:30KHz, 40MHz |
| Frequency domain resource |  | Full BW |
| TDD pattern |  | 7D1S2U (S=6:4:4) |
| Testing metric |  | Target BLER: 10-5 |

Based on the simulation assumptions in Table 1 for UE demodulation requirements for ultra-low BLER, we provide alignment and impairment results below.

Table 2: Alignment Results for Ultra-low BLER

| SNR @1e-5 BLER | 1x2 | 1x4 |
| --- | --- | --- |
| FDD | 0.1 | -2.7 |
| TDD | 0.2 | -2.5 |

Table 2: Impairment Results for Ultra-low BLER

| SNR @1e-5 BLER | 1x2 | 1x4 |
| --- | --- | --- |
| FDD | 2.1 | -0.7 |
| TDD | 2.2 | -0.5 |

# 3. Conclusion

In this paper, we provide alignment and impairment results for ultra-low BLER to define PDSCH demodulation requirements.

# Reference

- R4-2012646, “Way Forward on ultra-low BLER requirement”, Ericsson.