3GPP TSG-RAN WG4 Meeting # 97-e 	R4-2014249

# Electronic Meeting, 2 – 13 Nov, 2020

Agenda item:	7.9.4.3

Source:	Apple Inc.

Title:	On PMI reporting requirements with eType II codebook

Release:	Rel-16

Document for:	Discussion

# 1. Introduction

In RAN4#96e PMI reporting with eType II codebook was discussed. In this contribution we present our views on the open issues and provide simulation results with enhanced Type II codebook.

# 2. Discussion

## Test Setup

In RAN4#96e, there was an extensive discussion on the test setup for test cases with Type II codebook. Some of the agreements regarding the setup captured in [1] were:

- The baseline receiver assumption is UE without interference cancellation capability with/without co-scheduled UE.

- Under the baseline UE receiver assumption, the PMI calculation processing will not change with and without co-scheduled UE.

- The test purpose of such requirements is to verify UE PMI reporting accuracy following NW configuration with RAN1 feature: enhanced type II codebook

  - There is no restriction for gNB scheduling with such requirements.

  - RAN4 need to ensure UE reporting PMI follow Type II codebook other than Type I codebook under proper test set-up either with MU-MIMO set-up or SU-MIMO set-up.

  - We need to ensure the performance requirements with proper test set-up as receiver implementation agonistic manner i.e. no punishment for advanced receiver with inference cancellation capability.

The purpose of introducing PMI reporting requirements with Type II codebook is to verify PMI reporting by the UE with Type II codebook. Type II codebook was designed to be used in MU-MIMO scenario. But the gains from Type II codebook in a MU-MIMO setup would only be realized from a system level when we have multiple UEs and the overall system throughput is improved. For a link level assessment, we don’t believe that MU-MIMO test setup would have significant improvement in performance over SU-MIMO setup.

For the purpose of verifying UE reporting of Type II PMI with SU-MIMO setup, we compare performance of UE reporting of Type II PMI, UE reporting of incorrect Type II PMI and reporting of Type I PMI for the same propagation conditions and PMI format.

Simulation Assumptions:

MCS: 20; Rank: 2

Duplex Mode: FDD

Channel Model: TDLA30-5Hz

Antenna Correlation: XP-Medium

Antenna config: 16x2

L (Number of beams): 2

Npsk (phaseAlphabetSize): 8

SubbandAmplitude: True

PMI-FormatIndicator: Subband

Subband Size: 8

Table 1: PMI Reporting with SU-MIMO setup

| 16x2; SB PMI | Type II PMI | Incorrect Type II PMI | Type I PMI |
| --- | --- | --- | --- |
| SNR @ 90% Max TP | 12.2 | 23.0 | 14.5 |
| TP Gain Vs Random PMI | 3.6 | 1.25 | 2.4 |

The results for correctly reporting Type II PMI is significantly better than incorrect Type II PMI and also better than Type I PMI with SU-MIMO setup.

Observation #1: With SU-MIMO setup performance with correctly reported Type II PMI is significantly better than incorrect Type II PMI reporting.

Observation #2: With SU-MIMO setup performance of Type II PMI is better than Type I PMI.

Based on the observations above, we propose to use SU-MIMO test setup for defining requirements for PMI reporting with Type II codebook and enhanced Type II codebook in Rel-16.

Proposal #1: Define PMI reporting requirements in Rel-16 with SU-MIMO test setup for Type II and enhanced Type II codebook.

## Simulation Results

For PMI reporting with Type II codebook with SU-MIMO test setup, the following agreements were made in RAN4#96e.

- Number of CSI-RS ports:

  - 16 ports with (N1,N2) = (4,2) and (O1,O2)=(4,4)

- Number of PMI Sub-bands per CQI Sub-band

  - R = 1

- Codebook parameter configuration

  - paramCombination-r16: 6, with L =4, pν =1/2, β=1/2

- Sub-band Size:

  - 4 for FDD with 15kHz SCS, 10MHz CBW

  - 8 for TDD with 30kHz SCS, 40MHz CBW

- Channel Model

  - TDLA30-5

- MIMO Correlation

  - XP Medium

- MCS and Rank

  - MCS 20 (64QAM Table), Rank 2

- Beam-Steering Model

  - How to specify beam steering model into specification.

    - Same as specified in B.2.3B.4A of TS 36.101

For the simulation assumptions agreed in and listed above, the simulation results are provided below for 2Rx in FDD mode using SU-MIMO test set-up.

Table 2: Simulation results with e Type II codebook with SU-MIMO test setup

|  | SNR @ 90% | TP Gain |
| --- | --- | --- |
| 16x2 FDD | 11.28 | 4.69 |

# 3. Conclusion

In this contribution we present our views on the test setup for PMI reporting Type II codebook and simulation results for the agreed parameters. Our observations and proposals are captured below.

Observation #1: With SU-MIMO setup performance with correctly reported Type II PMI is significantly better than incorrect Type II PMI reporting.

Observation #2: With SU-MIMO setup performance of Type II PMI is better than Type I PMI.

Proposal #1: Define PMI reporting requirements in Rel-16 with SU-MIMO test setup for Type II and enhanced Type II codebook.

# Reference

- R4-2012761, “Way Forward on PMI Reporting Requirement for NR eMIMO”, Qualcomm, Ericsson.