3GPP TSG-RAN4 Meeting #97-e	R4-2014433

# Electronic meeting, Nov. 2-13, 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on CSI-RS configuration for mobility |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CSIRS_L3meas-Core |  |  |  |  |  | Date: |  |  | 2020-10-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | CSI-RS based L3 measurement was introduced in 38.133. The CSI-RS configuration for mobility needs to be specified when defining test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the CSI-RS configuration for mobility in 38.133. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CSI-RS configuration for mobility is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.19, A.3.19.1, A.3.19.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.3.19	Configurations of CSI-RS for mobility

A.3.19.1	Configuration of CSI-RS for mobility for FR1

A.3.19.1.1	FDD

Table A.3.19.1.1-1: CSI-RS for mobility for SCS=15kHz

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | Set 1 | Set 2 |
| Reference channel |  | CSI-RS-L3 1.1-1 FDD | CSI-RS-L3 1.1-2 FDD |
| Bandwidth |  | BW of Active BWPNote 1 |  |
| SCS | kHz | 15 |  |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2 |  |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 l0 = 9 for CSI-RS resource 2 |  |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2 |  |
| CDM Type |  | ‘No CDM’ for all CSI-RS resources |  |
| Density (ρ) |  | 3 for CSI-RS resource 1,2 |  |
| CSI-RS periodicity | slots | 40 for CSI-RS resource 1,2 |  |
| CSI-RS offset | slots | 2 for CSI-RS resource 1,2 | 12 CSI-RS resource 1,2 |
| EPRE ratio to SSS | dB | 0Note 2 |  |
| Associated with SSB |  | Yes |  |
| QCLed with SSB |  | Yes |  |
| Note 1: BW of CSI-RS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case |  |  |  |

A.3.19.1.2	TDD

Table A.3.19.1.2-1: CSI-RS for mobility for SCS=15kHz

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | Set 1 | Set 2 |
| Reference channel |  | CSI-RS-L3 1.1-1 TDD | CSI-RS-L3 1.1-2 TDD |
| Bandwidth |  | BW of Active BWPNote 1 |  |
| SCS | kHz | 15 |  |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2 |  |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 l0 = 9 for CSI-RS resource 2 |  |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2 |  |
| CDM Type |  | ‘No CDM’ for all CSI-RS resources |  |
| Density (ρ) |  | 3 for CSI-RS resource 1,2 |  |
| CSI-RS periodicity | slots | 40 for CSI-RS resource 1,2 |  |
| CSI-RS offset | slots | 2 for CSI-RS resource 1,2 | 12 for CSI-RS resource 1,2 |
| EPRE ratio to SSS | dB | 0Note 2 |  |
| Associated with SSB |  | Yes |  |
| QCLed with SSB |  | Yes |  |
| Note 1: BW of CSI-RS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case |  |  |  |

Table A.3.19.1.2-2: CSI-RS for mobility for SCS=30kHz

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | Set 1 | Set 2 |
| Reference channel |  | CSI-RS-L3 1.2-1 TDD | CSI-RS-L3 1.2-2 TDD |
| Bandwidth |  | BW of Active BWPNote 1 |  |
| SCS | kHz | 30 |  |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2 |  |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 l0 = 9 for CSI-RS resource 2 |  |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2 |  |
| CDM Type |  | ‘No CDM’ for all CSI-RS resources |  |
| Density (ρ) |  | 3 for CSI-RS resource 1,2 |  |
| CSI-RS periodicity | slots | 80 for CSI-RS resource 1,2 |  |
| CSI-RS offset | slots | 2 for CSI-RS resource 1,2 | 22 for CSI-RS resource 1,2 |
| EPRE ratio to SSS | dB | 0Note 2 |  |
| Associated with SSB |  | Yes |  |
| QCLed with SSB |  | Yes |  |
| Note 1: BW of CSI-RS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case |  |  |  |

A.3.19.2	Configuration of CSI-RS for mobility for FR2

A.3.19.2.1	TDD

Table A.3.19.2.1-1: CSI-RS for mobility for SCS=120kHz

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | Set 1 | Set 2 |
| Reference channel |  | CSI-RS-L3 2.1-1 TDD | CSI-RS-L3 2.1-2 TDD |
| Bandwidth |  | BW of Active BWPNote 1,3 |  |
| SCS | kHz | 120 |  |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2 |  |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 l0 = 9 for CSI-RS resource 2 |  |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2 |  |
| CDM Type |  | ‘No CDM’ for all CSI-RS resources |  |
| Density (ρ) |  | 3 for CSI-RS resource 1,2 |  |
| CSI-RS periodicity | slots | 320 for CSI-RS resource 1,2 |  |
| CSI-RS offset | slots | 2 for CSI-RS resource 1,2 | 82 for CSI-RS resource 1,2 |
| EPRE ratio to SSS | dB | 0Note 2 |  |
| Associated with SSB |  | Yes |  |
| QCLed with SSB |  | Yes |  |
| Note 1: BW of CSI-RS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case |  |  |  |

<End of Change 1>