3GPP TSG-WG RAN4 Meeting #97-e	R4-2015083

# Online, 2nd – 13th November, 2020

Source: 				Nokia, Nokia Shanghai Bell

Title: 				TP to TR 38.847 on regulatory background and system parameters.

Agenda Item: 	10.28.4 [NR_47GHz_Band]

Document for:	Approval

# Introduction

The work item to specify a new band (47.2-48.2 GHz) was approved in RAN#88e meeting [1]. RAN4 has started the technical work in RAN4#89-e and made several agreements [2, 3]. In this document, the text proposal on the system parameters and regulatory requirements are proposed according to these agreements. The text proposal is mostly based our earlier contributions [4, 5]

# Reference

[1] RP-201232 New WID: introduction of NR 47 GHz band, T-Mobile USA, Dish Network, RAN#88-e

[2] RP-201560 Status report for WI Introduction of NR 47GHz band; rapporteur: Nokia, RAN#89-e

[3] R4-2011873 Email discussion summary for [96e][135] NR_47GHz_Band, Nokia, RAN4#96-e

[4] Regulatory Background of 47 GHz band, Nokia, RAN4#96-e

[5] R4-2010522 Band number and System parameters of 47 GHz band, Nokia, RAN4#96-e

TP to TR 38.847

<START of Changes>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.141-2: "NR; Base Station (BS) conformance testing; Part 2: Radiated conformance testing".

[3]	3GPP TS 38.133: "NR; Radio Resource Control (RRC); Protocol specification".

[4]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone"

[5]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[6]	World Radiocommunication Conference 2019 (WRC-19) Final Acts, ITU-R

[7]		Title 47 of the Code of Federal Regulations (CFR) Part 30, FCC

<Next Changes>

# 4	Background

As part of facilitating the development of 5G mobile networks, WRC-19 identified the 47 GHz frequency band (47.2-48.2 GHz) for International Mobile Telecommunications (IMT) under Footnote 5.553B  of the ITU Radio Regulations for use in 71 nations across Africa, Europe, the Middle East and Asia Pacific (ITU Regions 1 and 3 respectively), in addition to the entire Americas Region (ITU Region 2, which consists of 35 nations) [6].

Table 1: Allocation Table and footnote in Radio Regulations (RR)

| Allocation to services |  |  |  |
| --- | --- | --- | --- |
| Region 1 |  | Region 2 | Region 3 |
| 47.2-47.5 | FIXED FIXED-SATELLITE (Earth-to-space) 5.550C 5.552 MOBILE 5.553B 5.552A |  |  |
| 47.5-47.9 |  | 47.5-47.9 |  |
| FIXED |  | FIXED |  |
| FIXED-SATELLITE (Earth-to-space) 5.550C 5.552 (space-to-Earth) 5.516B 5.554A |  | FIXED-SATELLITE (Earth-to-space) 5.550C 5.552 MOBILE 5.553B |  |
| MOBILE 5.553B |  |  |  |
| 47.9-48.2 | FIXED FIXED-SATELLITE (Earth-to-space) 5.550C 5.552 MOBILE 5.553B 5.552A |  |  |

5.553B	In Region 2 and Algeria, Angola, Saudi Arabia, Australia, Bahrain, Benin, Botswana, Burkina Faso, Burundi, Cameroon, Central African Rep., Comoros, Congo (Rep. of the), Korea (Rep. of), Côte d’Ivoire, Djibouti, Egypt, United Arab Emirates, Eswatini, Ethiopia, Gabon, Gambia, Ghana, Guinea, Guinea-Bissau, Equatorial Guinea, India, Iran (Islamic Republic of), Iraq, Japan, Jordan, Kenya, Kuwait, Lesotho, Liberia, Libya, Lithuania, Madagascar, Malaysia, Malawi, Mali, Morocco, Mauritius, Mauritania, Mozambique, Namibia, Niger, Nigeria, Oman, Uganda, Qatar, the Syrian Arab Republic, the Dem. Rep. of the Congo, Rwanda, Sao Tome and Principe, Senegal, Seychelles, Sierra Leone, Singapore, Slovenia, Somalia, Sudan, South Sudan, South Africa, Sweden, Tanzania, Chad, Togo, Tunisia, Zambia and Zimbabwe, the frequency band 47.2-48.2 GHz is identified for use by administrations wishing to implement International Mobile Telecommunications (IMT). This identification does not preclude the use of this frequency band by any application of the services to which it is allocated, and does not establish any priority in the Radio Regulations. Resolution 243 (WRC-19) applies. (WRC-19)

According to Resolution 243 (WRC-19) [6], IMT in 47.2-48.2 GHz may coexist with satellite services and IMT base stations may require some measure to protect the services by bilateral agreement and possibly with site engineering solutions. Such protection measure is not a scope of 3GPP work, therefore, no specific coexistence requirement is specified in 3GPP for the mobile and base stations to protect other services for example by having additional spurious emission requirements.

However, it cannot be excluded any requirement is introduced by individual administration in nation level in the future to protect the service in the same or adjacent bands. Current 3GPP framework is flexible enough to introduce additional requirement later by Network signaling mechanism. Therefore, this band can be reused even if additional coexistence requirement is introduced in future by some administrations.

The US FCC auctioned the 47 GHz band beginning in December 2019 as part of Auction 103. The 47 GHz band (47.2-48.2 GHz) was auctioned in 10 blocks of 100 megahertz in each PEA (Partial Economic Area) license. The auction was concluded on 3/5/2020.

The radio regulatory requirements in FCC rules are specified in CFR Title 47 Part 30 [7]. The channelization is 100 MHz starting from 47.2 GHz ending at 48.2 GHz. The emission requirements for both BS and UE are aligned with other FR2 bands in FCC, i.e., bands n260, and n261 in 3GPP.

The power limits and unwanted emissions for the transportable station (intended for CPE devices) are consistent with the existing UE power class 1 for FR2 bands. The ones for the mobile stations are consistent with the existing FR2 UE power class 2, 3, and 4.

Table 1: Regulatory requirements in FCC

| Channel arrangement | 47.2-47.3 GHz; 47.3-47.4 GHz; 47.4-47.5 GHz; 47.5-47.6 GHz; 47.6-47.7 GHz; 47.7-47.8 GHz; 47.8-47.9 GHz; 47.9-48.0 GHz; 48.0-48.1 GHz; and 48.1-48.2 GHz |
| --- | --- |
| Power limit (EIRP) | Base station +75 dBm/100MHz Mobile station +43 dBm Transportable station +55 dBm |
| Unwanted emissions | -5 dBm/MHz (within 10% of channel bandwidth separation) -13 dBm/MHz (outside more than 10% of channel bandwidth apart) |

# 5	NR Frequency band definition

The new band 47.2-48.2 GHz is within the range of FR2 (24250 – 52600 MHz) and is proposed as a TDD band. The first unused FR2 band number, n262, is proposed for this new band.

Table 5-11: The new FR2 TDD operating band 47.2-48.2 GHz

| Band number | UL | DL | Duplex mode |
| --- | --- | --- | --- |
| n262 | 47.2 – 48.2 GHz | 47.2 – 48.2 GHz | TDD |

# 6	Channel numbering and channel bandwidth

Though the channelization in FCC rules is 100 MHz, it is not precluded to use 50 MHz channel bandwidth. Allocation block size is still unknown in other administrations. For maximum flexibility, the channel bandwidths for NR band n262 is proposed to be aligned with the existing FR2 bands as shown in Table 6-1.

Table 6-1: NR channel bandwidth for NR band n262

| NR band |  | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Band number | data SCS(kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n262 | 60 | Yes | Yes | Yes |  |
|  | 120 | Yes | Yes | Yes | Yes |

The channel raster for n262 is also proposed to cover all the frequency with the 60/120 kHz channel raster aligned with the other F2 band as presented in Table 6-2.

Table 6-2: NR-ARFCN for NR band n262

| NR Operating Band | ΔFRaster (kHz) | Uplink and Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- |
| n262 | 60 | 2399166 – <1> – 2415832 |
|  | 120 | 2399167 – <2> – 2415831 |

The sync raster and sync burst pattern for n262 is proposed in Table 6-3.

Table 6-3: SS raster for NR band n262

| NR Operating Band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n262 | 120 kHz | Case D | 23586 – <1> – 23641 |
|  | 240 kHz | Case E | 23588 – <2> – 23640 |
| NOTE 1: SS Block pattern is defined in subclause 4.1 in TS 38.213. |  |  |  |

<End of Changes>