3GPP TSG RAN4 Meeting #97e	R4-2015284

# Online, 2nd – 13th November, 2020

Title: 	Removing restrictions on SUL UL-MIMO in Rel-17

Source: 	Huawei, HiSilicon

Agenda item:	12.2.2.1

Document for:	Decision

# Introduction

In the last plenary meeting, a new WI was established for RF requirements enhancement for NR frequency range 1 (FR1) in [1]. Among the objectives, the 1st one is to specify UL MIMO PC3 UE support for SUL band n80. The content of the objective is copied as in the below box:

- Enable UL MIMO configuration for SUL band configurations - Specify UL MIMO requirements for example SUL configurations with SUL band n80 - Take SUL_n41A-n80A for the example SUL band configuration - Remove the RAN2 and RAN4 restriction on configuring UL MIMO for SUL band configurations

In this paper, we discuss the corresponding task in RAN4 according to the above guidance from the WID. The paper also propose to send an LS to RAN2 to trigger discussion upon removing the RAN2 restrictions on configuring UL MIMO for SUL configurations.

# Removing RAN4 restriction on configuring UL MIMO for SUL

According to the newly established WI NR_RF_FR1_enh, RAN4 tasks to specify UL MIMO support for SUL band and its configurations/combinations to enable UE UL switching between SUL band with 2Tx and NUL band 2Tx carriers. One important prerequisite for such operation is to remove the restriction in RAN4 spec that refrain the network from scheduling more than 1 layer for PUSCH on SUL bands.

In TS 38.101-1, notes that imply certain restrictions mentioned in both tables below will be removed in Rel-17.

Table 5.2C-1: Operating band combination for SUL in FR1 NR Band combination for SUL; NR Band (Table 5.2-1). SUL_n41-n80; n41, n80. SUL_n41-n81; n41, n81. SUL_n41-n95; n41, n95. SUL_n77-n802; n77, n80. SUL_n77-n842; n77, n84. SUL_n78-n802; n78, n80. SUL_n78-n812; n78, n81. SUL_n78-n822; n78, n82. SUL_n78-n832; n78, n83. SUL_n78-n842; n78, n84. SUL_n78-n862; n78, n86. SUL_n79-n802; n79, n80. SUL_n79-n812; n79, n81. SUL_n79-n84; n79, n84. SUL_n79-n95; n79, n95. NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. NOTE 3: Void. Table 5.2C-2: Operating SUL band combination with downlink CA in FR1 NR Band combination for SUL; NR Band (Table 5.2-1). CA_n78(*)_SUL_n78-n862; n78, n86. NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. NOTE 3: Void NOTE 4: The notation CA_nX(*) in this table indicates intra-band non-contiguous CA for band nX. The configurations for each band are in table 5.5C-2..

Proposal 1: Void note 3 in both table 5.2C-1 and 5.2C-2 in TS 38.101-1 to enable UL MIMO configuration on SUL carriers.

# Removing RAN2 restriction on configuring UL MIMO for SUL

Also in RAN2 specs, there are currently restrictions refraining UL MIMO configurations on SUL carriers.

In TS 38.331 descriptions for uplink configuration IE maxMIMO-Layers and maxMIMO-LayersForDCI-Format0-2 need to be modified accordingly.

maxMIMO-Layers Indicates the maximum MIMO layer to be used for PUSCH in all BWPs of the normal UL of this serving cell (see TS 38.212 [17], clause 5.4.2.1). If present, the network sets maxRank to the same value. The field maxMIMO-Layers refers to DCI format 0_1.

maxMIMO-LayersForDCI-Format0-2 Indicates the maximum MIMO layer to be used for PUSCH for DCI format 0_2 in all BWPs of the normal UL of this serving cell (see TS 38.212 [17], clause 5.4.2.1). If present, the network sets maxRankForDCI-Format0-2 to the same value.

And in TS 38.306 descriptions to the definitions for uplink feature set per CC parameters need to be corrected altogether.

| maxNumberMIMO-LayersCB-PUSCH Defines supported maximum number of MIMO layers at the UE for PUSCH transmission with codebook precoding. UE indicating support of this feature shall also indicate support of PUSCH codebook coherency subset. | FSPC | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| maxNumberMIMO-LayersNonCB-PUSCH Defines supported maximum number of MIMO layers at the UE for PUSCH transmission using non-codebook precoding. UE supporting non-codebook based PUSCH transmission shall indicate support of maxNumberMIMO-LayersNonCB-PUSCH, maxNumberSRS-ResourcePerSet and maxNumberSimultaneousSRS-ResourceTx together. | FSPC | No | N/A | N/A |
| maxNumberSimultaneousSRS-ResourceTx Defines the maximum number of simultaneous transmitted SRS resources at one symbol for non-codebook based transmission to the UE. | FSPC | No | N/A | N/A |
| maxNumberSRS-ResourcePerSet Defines the maximum number of SRS resources per SRS resource set configured for codebook or non-codebook based transmission to the UE. | FSPC | No | N/A | N/A |

Thus an LS has to be sent to RAN2 tasking corresponding removal of the restrictions against configuring UL MIMO on SUL carriers and UE indicating correct UL MIMO capabilities for SUL bands.

Proposal 2: Send an LS to RAN2 tasking corresponding removal of the restrictions against configuring UL MIMO on SUL carriers and UE indicating correct UL MIMO capabilities for SUL bands.

Below is an example of the LS draft to RAN2.

Title: LS on removing restrictions against UL MIMO operations on SUL carriers Response to: Release: Rel-17 Work Item: NR_RF_FR1_enh Source: RAN4 To: RAN2 Cc: # 1 Overall description In RAN2 specs, there are currently restrictions refraining UL MIMO configurations on SUL carriers which need to be removed to enable SUL UL MIMO configurations. - In TS 38.331 descriptions for uplink configuration IE maxMIMO-Layers and maxMIMO-LayersForDCI-Format0-2 need to be modified accordingly - In TS 38.306 descriptions to the definitions for uplink feature set per CC parameters need to be corrected altogether: maxNumberMIMO-LayersCB-PUSCH, maxNumberMIMO-LayersNonCB-PUSCH, maxNumberSimultaneousSRS-ResourceTx and maxNumberSRS-ResourcePerSet # 2 Actions To RAN2 ACTION: RAN4 asks RAN2 to kindly take the above information into consideration. # 3 Dates of next TSG-RAN WG4 meetings

# Adding n80 to the band list supporting UL MIMO

In the meanwhile, we also need to add n80 in the UL MIMO band list in TS 38.101-1.

Table 5.2D-1: NR operating bands for UL MIMO in FR1 NR operating band. n1. n2. n3. n7. n25. n301. n34. n38. n39. n40. n41. n46. n48. n66. n70. n712. n77. n78. n79. n80. n96. NOTE 1: Uplink transmission is not allowed at this band for UE with external vehicle-mounted antennas. NOTE 2: UL MIMO is targeted for FWA form factor..

Proposal 3:  Add n80 into table 5.2D-1 UL MIMO band list in TS 38.101-1.

# Basket WID for bands supporting UL MIMO

As per RAN4 working procedure, it is orthodox that we should introduce a basket WI for the potential band supporting UL MIMO in order to set a place holder handling the requests from operators in Rel-17. Thus in [2] we propose a basket WID. Note that this WI not only provide services to SUL bands but also TDD and FDD bands according to potential requests from operators.

It is also possible that we directly introduce the bands with UL MIMO in either the FR1 enhancement WI or in TEI17 however we don’t see this approach as the best one.

Proposal 4: Establish a new basket WI for introduction of bands supporting UL MIMO according to operators’ requests in Rel-17.

# Conclusions

In this paper, we discuss the corresponding task in RAN4 according to the above guidance from the WID. The paper also propose to send an LS to RAN2 to trigger discussion upon removing the RAN2 restrictions on configuring UL MIMO for SUL configurations.

Together we propose:

Proposal 1: Void note 3 in both table 5.2C-1 and 5.2C-2 in TS 38.101-1 to enable UL MIMO configuration on SUL carriers.

Proposal 2: Send an LS to RAN2 tasking corresponding removal of the restrictions against configuring UL MIMO on SUL carriers and UE indicating correct UL MIMO capabilities for SUL bands.

Proposal 3:  Add n80 into table 5.2D-1 UL MIMO band list in TS 38.101-1.

Proposal 4: Establish a new basket WI for introduction of bands supporting UL MIMO according to operators’ requests in Rel-17.

# References

[1] RP-202088, New WID proposal: RF requirements enhancement for NR frequency range 1 (FR1) in Rel-17, Huawei, HiSilicon, China Telecom

[2] R4-2015285, New basket WID on bands with UL-MIMO PC3, Huawei, HiSilicon