3GPP TSG-RAN WG4 Meeting # 97-e 	R4-2015316

Electronic Meeting, 2-13 Nov., 2020

Agenda item:		     7.16.1.5

Source:				NTT DOCOMO, INC.

Title:		Views on release independence aspect for power imbalance requirements

Document for:	Discussion

1. Introduction

At the last RAN4 meeting, we discussed release independent and RAN4 agreed a WF [1]. This contribution presents our views on release independent aspect for power imbalance requirements.

2. Discussion

Power imbalance requirements for FR1 Intra-band contiguous CA

Referring to Section 5 in TS38.307 [2] and Huawei’s contribution [3], we needs to consider the following two points.

- All the newly introduced operating bands in release 16 are release independent from release 15

- Only intra-band contiguous CA configurations for TDD are release independent from release 15

Therefore, Only TDD Power imbalance requirements for FR1 Intra-band contiguous CA can be treated as release independent from release 15.

Observation 1: In case UE is tested with CA configurations that defined as release independent from release 15, TDD power imbalance requirements for FR1 intra-band contiguous CA can be treated as release independent from release 15.

Power imbalance requirements for FR1 Intra-band contiguous and non-contiguous EN-DC

Referring to Section 8 in TS38.307 [2], we needs to consider the following two points.

- Only intra-band contiguous EN-DC configurations within the condition of the maximum number of E-UTRA CC and the maximum number of NR CCs as shown in Table 8.1.1-1 of TS38.307 are release independent from release 15

- Only intra-band non-contiguous EN-DC configurations within the condition of the maximum number of sub-blocks, the maximum number of E-UTRA CC and the maximum number of NR CCs as shown in Table 8.1.1-2 of TS38.307 are release independent from release 15

Table 8.1.1-1: EN-DC contiguous intraband configurations within FR1 [2]

| Feature | DL/UL | maximum number of E-UTRA CCs | maximum number of NR CCs | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- |
| intraband contiguous EN-DC | DL | 3 | 1 | FDD, TDD | Rel-15 | Table B.4.6-1 |

Table 8.1.1-2: EN-DC non-contiguous intraband configurations within FR1 [2]

| Feature | DL/UL | maximum number of sub-blocks | maximum number of E-UTRA CCs | maximum number of NR CCs | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| intraband non-contiguous EN-DC | DL | 2 | 3 | 1 | FDD, TDD | Rel-15 | Table B.4.6-1 |

Observation 2: In case UE is tested with EN-DC configurations that defined as release independent from release 15, FDD power imbalance requirements for FR1 intra-band contiguous EN-DC and that of TDD can be treated as release independent from release 15.

From observation 1 and 2, all test cases for CA/EN-DC power imbalance requirements cannot be treated as release independent from Rel.15 but some of the test cases can be applicable for release independent from Rel.15.

Observation 3: Only the power imbalance requirements for CA/EN-DC configurations that are defined as release independent from release 15 can be release independent from release 15.

Proposal 1: Define the power imbalance requirements for CA/EN-DC as release independent from release 15.

3. Conclusion

In this contribution, we present our views on release independent aspect for power imbalance requirements. Our proposals and observations are summarized below.

Observation 1: In case UE is tested with CA configurations that defined as release independent from release 15, TDD power imbalance requirements for FR1 intra-band contiguous CA can be treated as release independent from release 15.

Observation 2: In case UE is tested with EN-DC configurations that defined as release independent from release 15, FDD power imbalance requirements for FR1 intra-band contiguous EN-DC and that of TDD can be treated as release independent from release 15.

Observation 3: Only the power imbalance requirements for CA/EN-DC configurations that are defined as release independent from release 15 can be release independent from release 15.

Proposal 1: Define the power imbalance requirements for CA/EN-DC as release independent from release 15.

References

- R4-2012687, Huawei, Way forward on release independence aspects for NR Rel-16 Type II PMI and power imbalance, RAN4 #96e

- TS38.307, ver 16.4.0

- R4-2001445, Huawei, Discussion on release independence for NR Rel-16 UE demodulation requirements, RAN4#94e