3GPP TSG WG4 Meeting  #97-e                                                                       R4-2016151

Electronic meeting, 2 – 13 November 2020

Title:	[draft]LS on cell-grouping UE capability for synchronous NR-DC

Response to:	LS R2-2008662 on cell-grouping UE capability for synchronous NR-DC from WG2

Release:	Rel-16

Work Item:	LTE_NR_DC_CA_enh-Core

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:

Contact person:

Reihaneh Malekafzaliardakani

E-mail Address: reihaneh.malekafzaliardakani@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN4 would like to thank RAN2 for the LS on cell-grouping UE capability for synchronous NR-DC (R2-2008662). In the LS the following reasons are listed if the cell-grouping is necessary or not for synchronous DC:

Some companies think it is not necessary for the following reasons.

- In LTE, cell grouping signalling is only defined for asynchronous DC, but not for synchronous DC.

- Synchronous NR-DC is almost the same as NR CA from the UE implementation perspective.

- Supported cell grouping will be limited by NR-DC band combinations defined by RAN4.

Some companies think it is necessary for the following reasons.

- NR supports FR2 (as a key difference from LTE)

  - The UE may not support FR2 MCG and thus there might be need to indicate whether FR2 MCG is supported in NR-DC if RAN4 has such potential band combinations.

  - The UE may not support FR1-FR2 CA and thus there might be need to differentiate whether it is a FR1-FR2 CA or a FR1-FR2 DC.

- The UE may not support inter-CG power sharing for a given frequency range.

- The UE may not support PUCCH group across serving cells with different numerologies.

RAN4 has discussed the reasons above and is of  the opinion that it is beneficial to define cell-grouping UE capability also for synchronous NR-DC not to limit support of deployment options. However, RAN4 recognises the concern on signalling overhead should LTE style cell grouping be adopted and that this approach would increase signalling overhead significantly. RAN4 is hoping that methods for limiting the signalling overhead used for other features could be employed also for cell grouping.

# 2	Actions

To RAN WG2 group.

ACTION: 	RAN WG4 kindly asks RAN WG2 to take the information above into account.

# 3	Dates of next TSG RAN WG 4 meetings

TSG WG4 Meeting #98-e	                    January 25 – February 5, 2021	       Online

TSG WG4 Meeting #98-bis-e	      12-20 April, 2021	                      Online