3GPP TSG-RAN WG4 Meeting #97-e		                    R4-2016155

Online, 2nd Nov – 13th Nov, 2020

Source: 	Keysight Technologies

Title: 	47GHz band TT for NR BS RF requirement
Agenda Item:	10.28.2

Document for:	Agreement

# Introduction

In RAN4#96e, it was proposed to seek contributions about BS RF requirement [1]. In BS RF requirement, test tolerance values need to be set for setting conformance requirement. In currently defined TT values for existing band, there are some values which defined by TT=MU. With this calculation, in order to set TT, MU analysis is needed. Ideally, these MU values should be studied/experimented from actual system to set these values, however, there was no such analysis done in 3gpp RAN4 for frequency range of n262 band. Also, there is not enough time left for this WI timeline. This document proposes how to approach this issue and proposes values.

# Description

When OTA study was conducted and TR document[4][5] created, it covered up to 40GHz in FR2 frequency. For n262 (47.2GHz – 48.2GHz), values in this TR document as well as existing TT values in TS document (up to n259 band) can’t be reused because too much of frequency difference for upper frequency side in mmWave.  For example;

- There is more pathloss which degrade SNR at receiver

- Mis-match uncertainty increases with higher frequency

- Various uncertainty changes/increases for OTA related terms

- Measurement Equipment uncertainty increases with higher frequency

While ideal situation is to conduct study and experimental measurement of various chamber and OTA method, unfortunately, there is not enough time under given this WI timeframe. However, what needs to be done is to set TT values, therefore this document proposes extrapolation method to calculate “Estimated MU” for setting TT value.

Please note that this same method is used for setting TT value for UE n259 requirement with calculating “Estimated MU” values for setting TT values. Also note that this method is only for setting TT value. For setting actual MU values, it should be done through study and actual measurement of test system and method.[2][3]

Proposed Method

Linear extrapolation is as following figure 1.

Figure 1. Illustration of proposed linear extrapolation approach to estimate MU value for n262

Here is calculated “Estimated MU” values for n262.

Table 1. Estimated MU values to set TT values for n262, for Tx test cases (in dB)

| Tx test | 24.25 ~ 29.5GHz | 37 ~ 40GHz | Estimated MU 47.2~48.2GHz |
| --- | --- | --- | --- |
| EIRP | 1.7 | 2 | 2.2 |
| EIRP extreme | 3.1 | 3.3 | 3.5 |
| Output Power | 2.1 | 2.4 | 2.6 |
| Tx Off power | 2.9 | 3.3 | 3.6 |
| Relative ACLR | 2.3 | 2.6 | 2.8 |

For some Tx test cases which has same MU/TT values for both frequency range (24.25~29.5G and 37~40G), there is no difference in values for n262 with extrapolation approach. This is risky for values in this high frequency range therefore to add some margin based on above tables, 0.2dB

Table 2. Estimated MU values to set TT values for n262, for Tx test cases where two values are the same (in dB)

| Tx test | 24.25 ~ 29.5GHz | 37 ~ 40GHz | Estimated MU 47.2 ~ 48.2Ghz |
| --- | --- | --- | --- |
| Absolute ACLR | 2.7 | 2.7 | 2.9 |
| OBUE (close) | 2.7 | 2.7 | 2.9 |

# Proposal

This document proposes, “Estimated MU” values which calculated by extrapolation method for setting TT values.

Here is overall TT values table with “Estimated MU” values

Table 3. updated table of Table C.1-2 in TS38.141-2 v16.5.0[6], Derivation of test requirements (FR2 OTA transmitter tests) under Annex C Test tolerances and derivation of test requirements

| Test | Minimum requirement in TS 38.104 [2] | Test Tolerance (TTOTA) | Test requirement in the present document |
| --- | --- | --- | --- |
| 6.2 Radiated transmit power | See TS 38.104 [2], clause 9.2 | Normal conditions: 1.7 dB, 24.25GHz < f ≦ 29.5GHz 2.0 dB, 37GHz < f ≦ 43.5GHz 2.2 dB, 47.2GHz < f ≦48.2GHz Extreme conditions: 3.1 dB, 24.25GHz < f ≦ 29.5GHz 3.3 dB, 37GHz < f ≦ 43.5GHz 3.5 dB, 47.2GHz < f ≦48.2GHz | Formula: Upper limit + TT, Lower limit – TT |
| 6.3 OTA base station output power | See TS 38.104 [2], clause 9.3 | 2.1 dB, 24.25GHz < f ≦ 29.5GHz 2.4 dB, 37GHz < f ≦ 43,5GHz 2.6 dB, 47.2GHz < f ≦48.2GHz | Formula: Upper limit + TT, Lower limit – TT |
| 6.4 OTA output power dynamics | See TS 38.104 [2], clause 9.4 | 0.4 dB | Formula: Total power dynamic range – TT |
| 6.5.1 OTA transmitter OFF power | See TS 38.104 [2], clause 9.5.2 | 2.9 dB, 24.25GHz < f ≦ 29.5GHz 3.3 dB, 37GHz < f ≦ 43.5GHz 3.6 dB, 47.2GHz < f ≦48.2GHz | Formula: Minimum Requirement + TT |
| 6.6.2 OTA frequency Error | See TS 38.104 [2], clause 9.6.1 | 12 Hz | Formula: Frequency Error limit + TT |
| 6.6.3 OTA Modulation quality (EVM) | See TS 38.104 [2], clause 9.6.2 | 1 % | Formula: EVM limit + TT |
| 6.6.4 OTA time alignment error | See TS 38.104 [2], clause 9.6.3 | 25 ns |  |
| 6.7.2 OTA occupied bandwidth | See TS 38.104 [2], clause 9.7.2 | 0 Hz | Formula: Minimum Requirement + TT |
| 6.7.3 OTA Adjacent Channel Leakage Power Ratio (ACLR) | See TS 38.104 [2], clause 9.7.3 | Relative: 2.3 dB, 24.25GHz < f ≦ 29.5GHz 2.6 dB, 37GHz < f ≦43.5GHz 2.8 dB, 47.2GHz < f ≦48.2GHz Absolute: 2.7 dB, 24.25GHz < f ≦ 29.5GHz 2.7 dB, 37GHz < f ≦43.5GHz 2.9 dB, 47.2GHz < f ≦48.2GHz | Formula: Relative limit - TT Absolute limit +TT |
| 6.7.4 OTA operating band unwanted emissions | See TS 38.104 [2], clause 9.7.4 | 0 MHz f < 0.1*BWcontiguous 2.7 dB, 24.25GHz < f ≦ 29.5GHz 2.7 dB, 37GHz < f ≦43.5GHz 2.9 dB, 47.2GHz < f ≦48.2GHz 0.1*BWcontiguous f < fmax 0 dB For co-existence with Earth Exploration Satellite Service 0 dB | Formula: Minimum Requirement + TT |
| 6.7.5.2 General transmitter spurious emissions requirements Category A | See TS 38.104 [2], clause 9.7.5.3.2 | 0 dB | Formula: Minimum Requirement + TT |
| 6.7.5.2 General transmitter spurious emissions requirements Category B | See TS 38.104 [2], clause 9.7.5.3.2 | 0 dB | Formula: Minimum Requirement + TT |
| 6.7.5.4 OTA transmitter spurious emissions, additional requirements | See TS 38.104 [2], clause 9.7.5.3.3 | For co-existence with Earth Exploration Satellite Service 0 dB | Formula: Minimum Requirement + TT |
| NOTE: TT values are applicable for normal condition unless otherwise stated. |  |  |  |

# References

[1] R4-2011412	BS RF requirements for 47GHz band, Nokia, Nokia Shanghai Bell

[2] R5-200316 On AP#85.28: n259 MU Estimate, Keysight Technologies

[3] R5-200816 Estimate of Measurement Uncertainty for n259, Rohde & Schwarz

[4]	TR38.817-02 v15.7.0

[5] TR37.941 v15.1.0

[6] TS38.141-2 v16.5.0