3GPP TSG-RAN WG4 Meeting #97-e	R4-2016563

Online, 02 – 13 November, 2020

Agenda item:	7.1.6.12

Source: 	Qualcomm Incorporated

Title: 	Definition of Available Reference Cell for Timing Requirements in NR-U

Document for:	Discussion

# Introduction

In RAN4#96-e meeting, the definition of an available reference cell was for timing requirements in NR-U was discussed and the following FFS item on this topic was captured in WF [1] :

- Definition of an available reference cell

  - FFS: whether a reference cell is available at the UE provided at least one SSB is available at the UE during the last 160 ms; otherwise it is unavailable at the UE

In this paper, we provide our views on this topic.

# Definition of an available reference cell

In RAN4#96-e meeting, it was agreed to adopt following NR-U related terminologies[2]:

The term “X not available at the UE” refers to when X is configured by gNB but the first two (FFS for cell detection, see also the previous slide) successive candidate SSB positions for the same SS/PBCH block index within the discovery burst transmission window (for RRM)/the set of configured resources (for RLM/CBD/BFD) are not available at the UE due to DL CCA failures at gNB during the corresponding … period, where

X shall be replaced depending on the requirement with:

- RLM-RS SSB in RLM requirements,

- BFD-RS SSB in BFD requirements,

- CBD-RS SSB in CBD requirements,

- SSB in L1-RSRP measurement requirements,

- SMTC in measurement requirements other than RSSI requirements and L1-RSRP,

- SSB in TCI state switching requirements,

- SMTC in SCell activation, PSCell addition/release, HO, RRC re-establishment, RRC release with redirection requirements, etc.

and … shall be replaced with what is appropriate:

- evaluation,

- detection,

- identification,

- activation, etc.

We believe that the availability/unavailability of a reference cell for timing purposes should be treated the same way as in other RRM/RLM cases and similar conclusion should apply.

Proposal 1. The availability/unavailability of a reference cell for timing purposes should be treated similar to the availability/unavailability of ‘X’s as in other RRM/RLM cases.

# Conclusions

Proposal 1. The availability/unavailability of a reference cell for timing purposes should be treated similar to the availability/unavailability of ‘X’s as in other RRM/RLM cases.

# References

[1] R4-2012274

[2] R4-2012249