3GPP TSG-RAN WG4 Meeting #97-e	R4-2017546

# Electronic Meeting,  2nd - 13th November 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0113 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS38.101-4: Addition of Rel-16 HST FRCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST-perf |  |  |  |  |  | Date: |  |  | 2020-10-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Endorsed in RAN4#96-e R4-2011369 Introduction of Rel-16 HST TDD FRC without Special slot data. Addition of HST single Tap MCS17 FRC, Addition of HST-DPS FRC |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modification of FRC R.PDSCH.2-10.1 to R.PDSCH.2.10-2 with no PDSCH data scheduled on special slot. - Correction of TRS slot transport block size - Addition of HST single tap FRC with MCS17 - Addition of HST-SFN FRC with rank 2 MCS 13 - Addition of HST multipath TDD with rank 1 MCS 13 - Addition of HST-DPS FRC with rank 2 MCS17 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no FRC defined for Rel-16 HST-single tap, HST-SFN and HST multipath and HST-DPS test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R4-2016108 |  |  |  |  |  |  |  |  |

START OF 1st CHANGE

Table A.3.2.1.1-8: PDSCH Reference Channel for FDD HST scenario

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-8.1 FDD | R.PDSCH.1-8.2 FDD | R.PDSCH.1-8.3 FDD | R.PDSCH.1-8.4 FDD |  |
| Channel bandwidth | MHz | 10 | 10 | 10 | 10 |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 |  |
| Number of allocated resource blocks | PRBs | 52 | 52 | 52 | 52 |  |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 | 12 |  |
| Allocated slots per 2 frames | Slots | 19 | 19 | 19 | 19 |  |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM |  |
| MCS index |  | 13 | 17 | 13 | 17 |  |
| Modulation |  | 16QAM | 64QAM | 16QAM | 64QAM |  |
| Target Coding Rate |  | 0.48 | 0.43 | 0.48 | 0.43 |  |
| Number of MIMO layers |  | 1 | 1 | 2 | 2 |  |
| Number of DMRS REs |  | 18 | 18 | 18 | 18 |  |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A |  |
| For Slots i = 1,…, 19 | Bits | 12552 | 16896 | 25104 | 28680 |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A |  |
| For Slots i = 1,…, 19 | Bits | 24 | 24 | 24 | 24 |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A | N/A | N/A |  |
| For Slots i = 1,…, 19 | CBs | 2 | 3 | 3 | 4 |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A |  |
| For Slots i = 1,2,11,12 | Bits | 24960 | 37440 | 51168 | 76752 |  |
| For Slots i = 3,…, 10, 13, …, 19 | Bits | 26208 | 39312 | 52416 | 78624 |  |
| Max. Throughput averaged over 2 frames | Mbps | 11.924 | 16.0512 | 23.8488 | 27.246 |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

END OF 1st CHANGE

START OF 2nd CHANGE

Table A.3.2.2.2-10: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-1 and HST scenario

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-10.1 TDD | R.PDSCH.2-10.2 TDD | R.PDSCH.2-10.3 TDD | R.PDSCH.2-10.4 TDD | R.PDSCH.2-10.5 TDD |
| Channel bandwidth | MHz | 40 | 40 | 40 | 40 | 40 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 | 30 |
| Allocated resource blocks | PRBs | 106 | 106 | 106 | 106 | 106 |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 4 | N/A | 4 | N/A | 4 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per 2 frames |  | 31 | 27 | 31 | 27 | 31 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 13 | 13 | 17 | 13 | 17 |
| Modulation |  | 16QAM | 16QAM | 64QAM | 16QAM | 64QAM |
| Target Coding Rate |  | 0.48 | 0.48 | 0.43 | 0.48 | 0.43 |
| Number of MIMO layers |  | 1 | 1 | 1 | 2 | 2 |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 6 | N/A | 6 | N/A | 6 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 18 | 18 | 18 | 18 | 18 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 8456 | N/A | 11528 | N/A | 19464 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 25608 | 25608 | 33816 | 51216 | 58384 |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 24 | N/A | 24 | N/A | 24 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | 2 | N/A | 2 | N/A | 3 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | CBs | 4 | 4 | 5 | 7 | 7 |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,2,21,22 | Bits | 52176 | 50880 | 76320 | 104304 | 156456 |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 17808 | N/A | 26712 | N/A | 53424 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {3,…,20,23,…,39} | Bits | 53424 | 53424 | 80136 | 106848 | 160272 |
| Max. Throughput averaged over 2 frames | Mbps | 36.262 | 34.5708 | 47.9572 | 69.1416 | 82.7112 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

END OF 2nd CHANGE