3GPP TSG-RAN4 Meeting RAN4#98-e	Tdoc  R4-2100071

# Online, 25th January – 5th February 2021

# Title:						FR2 Reference channels for RRM Test cases with 240kHz SSB SCS

Source:			Anritsu

Agenda Item:			4.8

Document for:			Endorsement

# 1.	Introduction

Some FR2 RRM Test cases in Annex A of TS 38.133 [1] specify a configuration with 240kHz SSB SCS, often in addition to a configuration with the more common 120kHz SSB SCS. In general they also specify an RMSI CORESET Reference Channel CR.3.1 TDD, which is defined in Table A.3.1.2.2-3.

TS 38.133 [1] Table A.3.1.2.2-3 Note 7 states that the configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 0 in Table 13-8 in TS 38.213 [6], but this table applies only for 120kHz SSB SCS.

RRM test cases that specify 240kHz SSB SCS need different Reference channels which are not currently specified in TS 38.133. This Tdoc identifies the new Reference channels and proposes parameters. An accompanying CR is provided in R4-2100068 [2], and also updates selected test cases to refer to the new Reference channels.

# 2.	Configurations in TS 38.213

2.1 Sets of resource blocks and slot symbols

The relevant tables in TS 38.213 [6] are extracted below:

Table 13-8: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {120, 120} kHz

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 4 |
| 2 | 1 | 48 | 1 | 14 |
| 3 | 1 | 48 | 2 | 14 |
| 4 | 3 | 24 | 2 | -20 if -21 if |
| 5 | 3 | 24 | 2 | 24 |
| 6 | 3 | 48 | 2 | -20 if -21 if |
| 7 | 3 | 48 | 2 | 48 |
| 8 | Reserved |  |  |  |
| 9 | Reserved |  |  |  |
| 10 | Reserved |  |  |  |
| 11 | Reserved |  |  |  |
| 12 | Reserved |  |  |  |
| 13 | Reserved |  |  |  |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |

<< sections skipped >>

Table 13-10: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {240, 120} kHz

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 48 | 1 | 0 |
| 1 | 1 | 48 | 1 | 8 |
| 2 | 1 | 48 | 2 | 0 |
| 3 | 1 | 48 | 2 | 8 |
| 4 | 2 | 24 | 1 | -41 if -42 if |
| 5 | 2 | 24 | 1 | 25 |
| 6 | 2 | 48 | 1 | -41 if -42 if |
| 7 | 2 | 48 | 1 | 49 |
| 8 | Reserved |  |  |  |
| 9 | Reserved |  |  |  |
| 10 | Reserved |  |  |  |
| 11 | Reserved |  |  |  |
| 12 | Reserved |  |  |  |
| 13 | Reserved |  |  |  |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |

The strategy for RRM Test case Reference channels and CORESET was originally outlined in R4-1810156 [4] at RAN4#88, and R4-1811861 [5] was endorsed at the same meeting.

Keeping Proposal 3 in R4-1810156 [4] to apply SS/PBCH and RMSI CORESET multiplexing pattern 1, keeping Proposal 4 where RMSI CORESET starts at the same PRB as SS/PBCH, i.e., RB offset 0, and keeping number of control symbols = 2, we choose Index 2 in Table 13-10. As all the options with multiplexing pattern 1 have 48 CORESET RBs, an additional RMSI CORESET Reference Channel CR.3.2 TDD is needed. Our preference is also to specify 48RBs Control Channel RMC and 48RBs PDSCH RMC.

2.2 RMSI CORESET Reference Channel

The relevant table in TS 38.133 [1] defining RMSI CORESET Reference Channels for TDD is extracted below, with the new proposed channel CR.3.2 TDD added:

Table A.3.1.2.2-3: RMSI CORESET Reference Channel for TDD with SCS=120KHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CR.3.1 TDD | CR.3.2 TDD |  |  |  |  |  |
| Channel bandwidth | MHz | 100 | 100 |  |  |  |  |  |
| Subcarrier spacing | kHz | 120 | 120 |  |  |  |  |  |
| Allocated resource blocks for RMSI CORESET Note 7 |  | 24 Note 7 | 48 Note 9 |  |  |  |  |  |
| SSB and RMSI CORESET multiplexing configuration Note 7 |  | Pattern 1 Note 7 | Pattern 1 Note 9 |  |  |  |  |  |
| Offset between SSB and RMSI CORESET Note 3, 7 | RB | 0 (Note 8) Note 7 | 0 (Note 8) Note 9 |  |  |  |  |  |
| Configuration of PDCCH monitoring occasions for RMSI CORESET Note 4 |  | Index 4 | Index 4 |  |  |  |  |  |
| Number of transmitter antennas |  | 1 | 1 |  |  |  |  |  |
| Duration of RMSI CORESET Note 7 | symbols | 2 Note 7 | 2 Note 9 |  |  |  |  |  |
| DCI Format Note 1 |  | Note 2 | Note 2 |  |  |  |  |  |
| Aggregation level | CCE | 8 | 8 |  |  |  |  |  |
| DMRS precoder granularity |  | 6 | 6 |  |  |  |  |  |
| REG bundle size |  | 6 | 6 |  |  |  |  |  |
| Mapping from REG to CCE |  | Distributed | Distributed |  |  |  |  |  |
| Cell ID |  | Note 5 | Note 5 |  |  |  |  |  |
| Payload (without CRC) | bits | Note 6 | Note 6 |  |  |  |  |  |
| Note 1: DCI formats are defined in TS 38.212. Note 2: DCI format shall depend upon the test configuration. Note 3: The offset is defined with respect to the subcarrier spacing of the CORESET from the smallest RB index of RMSI CORESET to the smallest RB index of the common RB overlapping with the first RB of the SS/PBCH block. Note 4: The configuration of PDCCH monitoring occasions for RMSI CORESET is defined in Table 13-12 in TS 38.213 [3]. Note 5: Cell ID shall depend upon the test configuration. Note 6: Payload size shall depend upon the test configuration. Note 7: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 0 in Table 13-8 in TS 38.213 [3]. Note 8: Other values can be used to align with GSCN [13] as long as SSB does not overlap the RMC. Note 9: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 2 in Table 13-10 in TS 38.213 [3]. |  |  |  |  |  |  |  |  |

2.3 Control Channel RMC

The relevant table in TS 38.133 [1] defining Control Channel RMC for TDD is extracted below, with the new proposed channel CR.3.7 TDD added:

Table A.3.1.3.2-3: Control Channel RMC for TDD with SCS=120KHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.3.1 TDD | CCR.3.2 TDD | CCR.3.3 TDD | CCR.3.4 TDD | CCR.3.5 TDD | CCR.3.6 TDD | CCR.3.7 TDD |
| Channel bandwidth | MHz | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| Subcarrier spacing | kHz | 120 | 120 | 120 | 120 | 120 | 120 | 120 |
| Allocated resource blocks for CORESET Note 3 |  | 24 | 24 | 24 | 24 | 24 | 24 | 48 |
| Number of transmitter antennas |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| monitoringSlotPeriodicityAndOffset |  | sl160 0 | sl160 0 | sl160 80 | sl160 0 | sl160 0 | sl160 80 | sl160 0 |
| monitoringSymbolsWithinSlot |  | 1100000 0000000 | 0011000 0000000 | 1100000 0000000 | 1100000 0000000 | 0011000 0000000 | 1100000 0000000 | 1100000 0000000 |
| Duration of CORESET | slot | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| REG bundle size |  | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |
| CCE to REG mapping |  | Interleaved | Interleaved | Interleaved | Interleaved | Interleaved | Interleaved | Interleaved |
| Interleave n_shift |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Interleave size |  | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| Beamforming Pre-Coder |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Aggregation level | CCE | 4 | 4 | 4 | 8 | 8 | 8 | 4 |
| DCI formats |  | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |
| Payload size (without CRC) | bits | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration. Note 3: Allocated in the same resource blocks where the associated PDSCH RMC is scheduled. |  |  |  |  |  |  |  |  |

We note there are other 24RB Control Channel RMCs defined with different values for parameters “monitoringSlotPeriodicityAndOffset” and “monitoringSymbolsWithinSlot”. However as the test cases that use these don’t specify a 240kHz SSB SCS configuration, there is no need to define additional RMCs with 48RB.

2.4 PDSCH Reference Measurement Channel

A related Tdoc in R4-2100067 [3] explains the motivation for defining a 24RB PDSCH Reference channel SR.3.2 TDD occupying the same RBs as the CORESET, and also gives the rationale for changes to the table notes. Following the same principle, we define a 48RB PDSCH Reference channel SR.3.3 TDD:

Table A.3.1.1.12-3: PDSCH Reference Measurement Channels for SCS=120kHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | SR.3.1 TDD | SR.3.2 TDD | SR.3.3 TDD |  |  |  |  |
| Channel bandwidth | MHz | 100 | 100 | 100 |  |  |  |  |
| Number of transmitter antennas |  | 1 | 1 | 1 |  |  |  |  |
| Allocated resource blocks for PDSCH Note 1 |  | 24Note 1 | 24Note 7 | 48Note 7 |  |  |  |  |
| Allocated slots per Radio Frame |  |  |  |  |  |  |  |  |
| Radio frame containing SSB | slots | Note 5 | Note 8 | Note 8 |  |  |  |  |
| Radio frame not containing SSB | slots | 48 | 48 | 48 |  |  |  |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |  |  |  |
| MCS index |  | 4 | 4 | 4 |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK |  |  |  |  |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 |  |  |  |  |
| Number of control symbols |  | 2 | 2 | 2 |  |  |  |  |
| PDSCH mapping type |  | Type A | Type A | Type A |  |  |  |  |
| Information Bit Payload |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2 | bits | 1608 | 1608 | 3104 |  |  |  |  |
| For slots without RMSI | bits | 1864 | 1864 | 3624 |  |  |  |  |
| Number of Code Blocks per slot |  | 1 | 1 | 1 |  |  |  |  |
| Binary Channel Bits Per slot |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2, Note 4 | bits | 5184 | 5184 | 10368 |  |  |  |  |
| For slots without RMSI Note 6 | bits | 6048 | 6048 | 12096 |  |  |  |  |
| Note 1: Allocated outside the SMTC duration in time and in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block Note 2: VoidPDSCH is scheduled on the slots with RMSI. Note 3: If necessary the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in TS 38.213 [3]. Note 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditionalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 2. Note 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in clause A.3.10. Note 6: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditionalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. Note 7: Allocated in the same resource blocks as the CORESET. Note 8: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test, and is not scheduled in slots containing CORESET. |  |  |  |  |  |  |  |  |

# 3.	Way Forward

RAN4 is asked to endorse the following points and to agree the associated CR:

- Add a new 48RB RMSI CORESET Reference Channel CR.3.2 TDD for use with 240kHz SSB SCS

- Add a new 48RB Control Channel RMC CCR.3.4 TDD for use with 240kHz SSB SCS

- Add a new 48RB PDSCH Reference Measurement Channel SR.3.3 TDD for use with 240kHz SSB SCS

# References

[1] TS 38.133 v16.6.0, “Requirements for support of radio resource management”

[2] R4-2100068, Anritsu, “Update FR2 Reference channels and OCNG for FR2 RRM Test cases”

[3] R4-2100067, Anritsu, “FR2 PDSCH Reference channel and OCNG for RRM Test cases”

[4] R4-1810156, Ericsson, “Discussion on RMC for RRM performance requirements”

[5] R4-1811861, Huawei, “CR on TS38.133 for PDSCH and CORESET Reference Measurement Channel”

[6] TS 38.213 v15.7.0, “NR Physical layer procedures for control”

[7] TS 38.214 v15.7.0, “NR Physical layer procedures for data”

# Annex A: Calculation of Binary Channel Bits Per slot and Information Bit Payload

Slots without RMSI

In this case, we assume PDSCH with 12 symbols and one port DMRS 1+1+1. The number of PDSCH symbols per PRB is given by 12 subcarriers *12 symbols  – 6 (DMRS) * 3 (1+1+1) = 126. With QPSK with 48PRB, the channel bits = 126*48*2 = 12096. With MCS4, TBS = 3624bits. According to TS 38.214 5.1.3.2.

Slots with RMSI

Number of DMRS CDM groups without data is 2, according to R4-2001271. So the number of PDSCH symbols per PRB is 12*12 – 12 (DMRS) * 3 = 108. With QPSK with 48PRB, the channel bits = 108*48*2 = 10368. With MCS4, TBS = 3104bits.