3GPP TSG-RAN WG4 Meeting #98-e	R4-2100486

Electronic Meeting, 25th Jan. – 5th Feb., 2021

Title: 	Simulation assumptions for NTN co-existence

Source: 	CATT

Agenda item:	11.8.3.1

Document for:	Discussion

- Introduction

In last RAN4#97-e meeting, work plan and way forward on NTN solutions were approved [1][2]. It is agreed to further discuss scenarios for NTN coexistence studies and simulation assumptions in RAN4#98e meeting.

25 January–5 February 2021, RAN4#98-e, e-meeting

- Agree on use cases & scenarios

- Target Agreement on the exemplary band.

- Further discuss and align on NTN architecture and NTN components description

- Initial discussion on coexistence study scenarios to be considered and related simulations assumptions.

This document will discuss and share our considerations on coexistence simulation assumptions for the NTN coexistence study.

- Discussion

2.1 Co-existence simulation scenarios

In [3], the proposed scenarios for coexistence study are duplicated in the following table.

Table 2.1-1 proposed scenarios for NTN co-existence

|  |  |  | Set 1 |  |  | Set 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | GEO | LEO 1200km | HAPS | GEO | LEO 1200km | HAPS |
| NR / NB-IoT | Rural |  | X | X | X | X | X | X |
|  | Dense Urban |  | X | X | X | X | X | X |
| NTN | GEO | Set 1 | X | X | X | N/A | N/A | N/A |
|  | LEO 1200km |  | X | X | X | N/A | N/A | N/A |
|  | HAPS |  | X | X | X | N/A | N/A | N/A |
|  | GEO | Set 2 | N/A | N/A | N/A | X | X | X |
|  | LEO 1200km |  | N/A | N/A | N/A | X | X | X |
|  | HAPS |  | N/A | N/A | N/A | X | X | X |

It is proposed that only earth fixed beams is used and only FDD is used for co-existence study. So the aggressor and victim combination is list in Table 2.1-2.

Table 2.1-2  Aggressor and victim

| No. | Combination | Aggressor | Victim | Notes |
| --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL | Assuming TN BS ACLR is not impacted. |
| 2 | TN with NTN | TN UL | NTN UL | Assuming TN UE ACLR is not impacted |
| 3 | TN with NTN | NTN DL | TN DL | Assuming TN UE ACS is not impacted. |
| 4 | TN with NTN | NTN UL | TN UL | Assuming TN BS ACS is not impacted |
| 5 | TN with NTN | NTN UL | TN DL | Co-existence between NTN and adjacent TDD bands. E.g. TDD in 2010-2025MHz. |
| 6 | TN with NTN | TN UL | NTN DL | Co-existence between NTN and adjacent TDD bands. E.g. TDD in 2010-2025MHz. |
| 7 | TN with NTN | NTN DL | TN UL | Co-existence between NTN and adjacent TDD bands. E.g. TDD in 2010-2025MHz. |
| 8 | TN with NTN | TN DL | NTN UL | Co-existence between NTN and adjacent TDD bands. E.g. TDD in 2010-2025MHz. |
| 9 | NTN with NTN | NTN DL | NTN DL |  |
|  |  | NTN UL | NTN UL |  |

The proposed frequency and bandwidth are listed as table 2.1-3.

Table 2.1-3.  Proposed frequency and bandwidth for co-existence study

|  | Frequency | Bandwidth | Note |
| --- | --- | --- | --- |
| Rural | 2 GHz | 30 MHz | Include both FDD and TDD |
| Dense Urban | 2 GHz | 30 MHz | Include both FDD and TDD |
| GEO | 2 GHz | 30 MHz | FDD |
| LEO 1200km | 20 GHz | 200 MHz | FDD |
|  | 2 GHz | 30 MHz | low priority, FDD |
| HAPS | 2 GHz | 30 MHz | FDD |

2.2. Network layout model

The network layout between NTN and TN is depicted in the Figure 2.2-1. Since the satellite beam coverage is quite large, it is proposed only one satellite is considered in the co-existence study between NTN and TN.

The network layout between NTN and NTN is depicted in the Figure 2.2-2 and figure 2.2-3. In Figure 2.2-2, only one satellite carry two neighbour carriers is assumed and the footprints of the 2 carriers are the same. In Figure 2.2-3, two satellites operating on two neighbour carriers but at different height are assumed, where the GEO have larger cell coverage and LEO has smaller coverage

For NTN UE distribution, at least X = [10] UEs per beam with uniform distribution in all the cell coverage area associated to each beam.

The TN UE distribution could refer to TR38.803 [6], which also considered uniform UE distribution.

For both TN and NTN network, it is proposed frequency reuse factor 1 is considered in the coexistence study.

The wrap around mechanism should be considered in system level simulations.

Figure 2.2-1 layout for coexistence between NTN and TN system

Figure 2.2-2  layout for coexistence between NTN systems

Figure 2.2-3  layout for coexistence between NTN systems (different height satellites)

2.3. Simulation parameters assumptions

RAN4 has decided that “Use TR 38.821 as a baseline/starting point”. The simulation assumptions defined in section 6.1.1 in TR38.821 [4] can be as baseline.

Two sets of satellite parameters are considered as in Table 2.3-1 and Table 2.3-2.

Table 2.3-1 Set-1 satellite parameters for co-existence study

| Satellite orbit |  | GEO | LEO-1200 |
| --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km |
| Payload characteristics for DL transmissions |  |  |  |
| Satellite EIRP density | 2GHz | 59 dBW/MHz | 40 dBW/MHz |
| Satellite Tx max Gain |  | 51 dBi | 30 dBi |
| 3dB beamwidth |  | 0.4011 deg | 4.4127 deg |
| Satellite beam diameter |  | 250 km | 90 km |
| Satellite EIRP density | 20GHz | 40 dBW/MHz | 10 dBW/MHz |
| Satellite Tx max Gain |  | 58.5 dBi | 38.5 dBi |
| 3dB beamwidth |  | 0.1765 deg | 1.7647 deg |
| Satellite beam diameter |  | 110 km | 40 km |
| Payload characteristics for UL transmissions |  |  |  |
| G/T | 2 GHz | 19 dB K-1 | 1.1 dB K-1 |
| Satellite Rx max Gain |  | 51 dBi | 30 dBi |
| G/T | [20] GHz | 28 dB K-1 | 13 dB K-1 |
| Satellite RX max Gain |  | 58.5 dBi | 38.5 dBi |

Table 2.3-2 Set-2 satellite parameters for co-existence study

| Satellite orbit |  | GEO | LEO-1200 |
| --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km |
| Payload characteristics for DL transmissions |  |  |  |
| Satellite EIRP density | 2GHz | 53.5 dBW/MHz | 34 dBW/MHz |
| Satellite Tx max Gain |  | 45.5 dBi | 24 dBi |
| 3dB beamwidth |  | 0.7353 deg | 8.8320 deg |
| Satellite beam diameter |  | 450 km | 190 km |
| Satellite EIRP density | 20GHz | 32 dBW/MHz | 2 dBW/MHz |
| Satellite Tx max Gain |  | 50.5 dBi | 30.5 dBi |
| 3dB beamwidth |  | 0.4412 deg | 4.4127 deg |
| Satellite beam diameter |  | 280 km | 90 km |
| Payload characteristics for UL transmissions |  |  |  |
| G/T | 2 GHz | 14 dB K-1 | -4.9 dB K-1 |
| Satellite Rx max Gain |  | 45.5 dBi | 24 dBi |
| G/T | [20] GHz | 20 dB K-1 | 5 dB K-1 |
| Satellite RX max Gain |  | 50.5 dBi | 30.5 dBi |

UE characteristics are considered as in Table 2.3-3.

Table 2.3-3 UE characteristics for co-existence study

| Characteristics | VSAT | Handheld |
| --- | --- | --- |
| Frequency band | [30 GHz UL and 20 GHz DL] | 2 GHz |
| Polarisation | circular | Linear: +/-45°X-pol |
| Rx Antenna gain | 39.7 dBi | 0 dBi per element |
| Antenna temperature | 150 K | 290 K |
| Noise figure | 1.2 dB | 7 dB |
| Tx transmit power | 2 W (33 dBm) | 200 mW (23 dBm) |
| Tx antenna gain | 43.2 dBi | 0 dBi per element |

2.4. Antenna and beam forming pattern modelling

Satellite and UE Antenna and beam forming pattern modelling of satellite could be referred to section 6.4.1 in TS 38.811 [5].

Antenna and beam forming pattern modelling of TN BS and UE could be referred to TR38.803 [6].

2.3. Propagation model

Propagation model between NTN and UE could be referred to section 6.6 in TR 38.811 [5].

Propagation model between TN BS and UE could be referred to section 5.2.2 in TR 38.803 [6].

2.4. Transmission power control model

For downlink scenario, no power control scheme is applied.

For uplink scenario, TPC model specified in Section 9.1 TR 36.942 [7] could be applied with following parameters.

Where, Pmax = 24dBm, Rmin = -54dB if UE minimum power is -30dBm (or Rmin = -64dB if UE minimum power is -40dBm), CLx-ile and γ are set as following:

-	CLx-ile = 88 + 10*log10 (200/X) + 11 – Y,

where X is UL transmission BW (MHz) and Y is the BS noise figure

-	γ = 1

2.5. Received power model

The received power in downlink and uplink scenarios is defined as below:

RX_PWR = TX_PWR – Path loss + G_TX + G_RX

Where,

RX_PWR is the received power

TX_PWR is the transmitted power

G_TX is the transmitter antenna gain (directional array gain)

G_RX is the receiver antenna gain (directional array gain).

2.6. Performance metric

Same criteria as previous study, e.g. the throughput loss of victim system should be below 5%.

- Conclusion

This contribution discussed the simulation assumption for NTN co-existence study.

- Reference

[1]	R4-2017600, WF on NTN solutions, THALES

[2]	R4-2017661, NR_NTN_solutions work plan, THALES

[3]	R4-2100399, Discussion on frequency band and scenarios for NTN, CATT

[4]	3GPP TR 38.821 v16.0.0, Solutions for NR to support non-terrestrial networks (NTN)

[5]	3GPP TR 38.811 v15.4.0, Study on New Radio (NR) to support non-terrestrial networks

[6]	3GPP TR 38.803 v14.2.0, Radio Frequency (RF) and co-existence aspects

[7]	3GPP TR 36.942 v16.0.0, Radio Frequency (RF) system scenarios