3GPP TSG-RAN WG4 Meeting #98-e	R4-2101265

Electronic Meeting, 25 Jan. – 05 Feb. 2021

Source: 	Intel Corporation

Title: 					Discussion on NR-U PRACH requirements

Agenda Item: 			7.1.7.4.4

Document for:	Discussion

# 1	Introduction

During RAN#97-e a number of agreements were reached for NR-U PRACH formats and test scenario parameters. There are several issues that are still open and in this contribution we would like to discuss them. Here is a list of those open issues:

Propagation conditions - Fading channel - Option 1: TDLC300-100 - Option 2: TDLA30-10 Ncs - Option 1:127 for LRA=1151 and 63 for LRA=571 - Option 2:164 for LRA=1151 and 190 for LRA=571 - Option 3: FFS Time error estimation tolerance - Option 1: New values: PRACH preamble; PRACH SCS (kHz); Time error tolerance. AWGN; TDLA30-10. A2, B4, C2; 15; [0.065us]; FFS. 30; FFS. - Option 2: Reuse the Table 8.4.2.1-1 of TS 38.104 for the moment, and encourage companies to bring analysis on the scaled tolerance for the next meeting. - Other options are not precluded

# 2	Discussion

In the NR-U WID [2] the following scenarios are mentioned:

- Scenario A: Carrier aggregation between licensed band NR (PCell) and NR-U (SCell)

  - NR-U SCell may have both DL and UL, or DL-only.

- Scenario B: Dual connectivity between licensed band LTE (PCell) and NR-U (PSCell)

- Scenario C: Stand-alone NR-U (PCell)

- Scenario D: A stand-alone NR cell in unlicensed band and UL in licensed band

- Scenario E: Dual connectivity between licensed band NR and NR-U.

As we can see, some scenarios consider joint operation of licensed and unlicensed NR. So, it is essential to test them in the same environment. Based on that, for NR-U tests we propose to reuse test parameters for licensed NR as much as possible.

Propagation conditions

The performance requirements for Rel-15 NR PRACH are defined for TDLC300-100 channel. Even though high doppler is not very typical for NR-U, as we mentioned it is better to have similar environment in licensed and unlicensed tests i.e. have the same fading channel as defined for Rel-15 NR PRACH tests.

Proposal 1: RAN4 to define performance requirements for NR-U PRACH for TDLC300-100 channel

Ncs value

Using the same considerations, it is better for NR-U PRACH tests to use Ncs values supporting the same cell size as in case of Rel-15 NR PRACH tests. Option 2 from the WF corresponds to this.

Proposal 2: RAN4 to define performance requirements for NR-U PRACH considering Ncs = 164 for LRA=1151 and Ncs = 190 for LRA=571

Time estimation error tolerance

Option 1 mentioned in the WF propose to scale down the Rel-15 time error tolerance by 8 times for 15kHz and by 4 times for 30kHz since NR-U PRACH bandwidth is 8 and 4 times wider than Rel-15 PRACH bandwidths. However, PRACH resolution is not the main limiting factor here. The limiting factor in this case is the TA command resolution. According to TS 38.213:

For a SCS of kHz, the timing advance command for a TAG indicates the change of the uplink timing relative to the current uplink timing for the TAG in multiples of .

So, the TA command in random access response has a resolution of  what gives us 0.521 us for 15 kHz and 0.26 us for 30 kHz. That means that the existing time error tolerance requirements for AWGN channel has already minimal possible values and cannot be scaled down.

The time estimation error tolerance for fading channel depends on the maximum delay in the respective PDP and the time estimation error tolerance for the AWGN channel. Since we consider using the same fading channel and the value for time error tolerance for AWGN channels are reused from Rel-15, the values for fading channel can also be reused.

Table 2: Time error tolerance for AWGN and TDLC300-100

| PRACH | PRACH SCS | Time error tolerance |  |
| --- | --- | --- | --- |
| preamble | (kHz) | AWGN | TDLC300-100 |
| A2, B4, C2 | 15 | 0.52 us | 2.03 us |
|  | 30 | 0.26 us | 1.77 us |

Proposal 3: RAN4 to reuse Rel-15 time error tolerance requirements given in Table 8.4.2.1-1 of TS38.104

# 3	Conclusion

In this contribution, we discussed the NR-U PRACH requirements. After discussion, the following conclusions are provided:

Proposal 1: RAN4 to define performance requirements for NR-U PRACH for TDLC300-100 channel

Proposal 2:  RAN4 to define performance requirements for NR-U PRACH considering Ncs = 164 for LRA=1151 and Ncs = 190 for LRA=571

Proposal 3: RAN4 to reuse Rel-15 time error tolerance requirements given in Table 8.4.2.1-1 of TS38.104

# 4	References

- R4-2017468, Way forward on NR-U PRACH demodulation requirements, Nokia, Nokia Shanghai Bell

- RP-192926, Revised WID for Rel-16 NR-U WI