3GPP TSG-RAN WG4 Meeting #98-e	R4-2101355

# Electronic Meeting, Jan. 25th – Feb. 5th, 2021

Title: 	Discussions on PSBCH requirements

Source: 	Huawei, HiSilicon

Agenda item:	7.3.6.2.3

Document for:	Discussion

Background

At last meeting, simulation assumptions and test scope for PSBCH were discussed. In this paper, we provide our simulation results.

Simulation results

Simulation results for PSBCH are shown in Figure 2-1 and Table 2-2. Corresponding simulation assumptions are shown in Table 5-1.

Figure 2-1: Simulation results for PSBCH

Table 2-2 Summary of PSBCH simulation results

|  | SNR@1% BLER |  |
| --- | --- | --- |
|  | Ideal results | Impairment results |
| TDLA-30-180 | -3.38dB | -2.38dB |

Conclusion

In this contribution, we provide our simulation results for PSBCH test.

Reference

[1]   R4-2017687(WF on V2X single link demodulation). LG Electronics

Appendix

Table 5-1 Simulation assumption for PSBCH demodulation performance

| Parameters | Unit | Value |
| --- | --- | --- |
| Test |  | BCH_Test1 |
| SLID |  | 0 |
| Synchronization source |  | S-SSB |
| Propagation condition |  | TDLA30-180 |
| Channel bandwidth | MHz | 20 |
| Number SSB per SL period(NperiodS-SSB) |  | 1 |
| Allocated resource blocks |  | 11 |
| Subcarrier spacing | kHz | 30 |
| Timing offset |  | 0 |
| Frequency offset | Hz | 0 |
| The number of symbols (Note 1) |  | 8 |
| Modulation order |  | QPSK |
| Transport Block Size (without CRC) | Bits | 32 |
| Transport block CRC | Bits | 24 |
| Binary Channel Bits | Bits | 1782 |
| PSBCH performance metric (Note 2) |  | SNR@1%BLER of PSBCH |
| Note 1: The first symbol is for AGC. Note 2: The performance metric is used for PSBCH simulation assumption. |  |  |