3GPP TSG-RAN WG4 Meeting #98-e	R4-2102167

# Electronic meeting, January 25th – February 5th, 2021

Source:		Ericsson

Title:			Additional channel bandwidths in bands n2, n5 and n48 – REFSENS

Agenda item:		9.21.2.1

Document for:		Discussion

# Introduction

In the context of the “adding channel BW to existing NR bands basket” WI ([1]), RAN#90 agreed to add request adding 30 MHz (15, 30 and 60kHz SCS) in band n48, 25-30-40MHz (15, 30 and 60kHz SCS) in band n2 and 25 MHz (15, 30 and 60kHz SCS) in band n5.

This contribution is discussing REFSENS impacts when considering those requests.

# Discussion

## Band n2

For REFSENS, we propose to scale existing values as shown in Table 1, further discussion might be needed for 40MHz to consider potential IM5 impact.

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n2 | 15 | -98.0 | -94.8 | -93.0 | -91.8 | -90.8 | -90.0 | -88.8 |  |  |  |  |  |  | FDD |
|  | 30 |  | -95.1 | -93.1 | -92.0 | -91.0 | -90.2 | -88.8 |  |  |  |  |  |  |  |
|  | 60 |  | -95.5 | -93.4 | -92.2 | -91.1 | -90.2 | -88.9 |  |  |  |  |  |  |  |

Table 1: REFSENS values for n2 and 25, 30, 40 MHz CBW

And associated RB allocations should be as defined in Table 2.

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n2 | 15 | 25 | 501 | 501 | 501 | 501 | 501 | 501 |  |  |  |  |  |  | FDD |
|  | 30 | 101 | 24 | 241 | 241 | 241 | 241 | 241 |  |  |  |  |  |  |  |
|  | 60 |  | 101 | 101 | 101 | 101 | 101 | 101 |  |  |  |  |  |  |  |

Table 2: RB allocations for REFSENS band n2 and 25, 30, 40 MHz CBW

## Band n5

A simple scaling could be done to define REFSENS values for 25 MHz, as shown in Table 3.

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n5 | 15 | -98.0 | -94.8 | -93.0 | -86.8 | -85.8 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -95.1 | -93.1 | -88.6 | -87.5 |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 3: REFSENS values for n5 and 25 MHz CBW

And associated RB allocations should be as defined in Table 4.

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n5 | 15 | 25 | 251 | 201 | 201 | 201 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 121 | 101 | 101 | 101 |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4: RB allocations for REFSENS band n5 and 25 MHz CBW

## Band n48

A simple scaling could be done to define REFSENS values for 30 MHz, as shown in Table 5.

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n481 | 15 | -99 | -95.8 | -94.0 | -92.7 |  | -90.9 | -89.6 | -88.64 |  |  |  |  | TDD |
|  | 30 |  | -96.1 | -94.1 | -92.9 |  | -91.0 | -89.7 | -88.74 | -87.94 | -86.64 | -86.14 | -85.64 |  |
|  | 60 |  | -96.5 | -94.4 | -93.1 |  | -91.1 | -89.9 | -88.84 | -88.04 | -86.74 | -86.24 | -85.74 |  |

Table 5: REFSENS values for n48 and 30 MHz CBW

And associated RB allocations should be as defined in Table 6.

| Operating band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Duplex Mode |
| n48 | 15 | 25 | 50 | 75 | 100 |  | 160 | 216 |  |  |  |  |  | TDD |
|  | 30 |  | 24 | 36 | 50 |  | 75 | 100 |  |  |  |  |  |  |
|  | 60 |  | 10 | 18 | 24 |  | 36 | 50 |  |  |  |  |  |  |

Table 6: RB allocations for REFSENS band n48 and 30 MHz CBW

# Conclusion

In this contribution, we described UE REFSENS impacts when introducing 30 MHz (15, 30 and 60kHz SCS) in band n48, 25-30-40MHz (15, 30 and 60kHz SCS) in band n2 and 25 MHz (15, 30 and 60kHz SCS) in band n5.

Proposal: Update REFSENS and RB allocation tables according to above tables for n2, n5 and n48.

# References

- RP-202277, Revised Basket WID on adding channel bandwidth support to existing NR bands, Ericsson