3GPP TSG-RAN WG4 Meeting #97-e                                                            R4-2103049

Online, 2nd-13th Nov, 2020

Source:	Samsung, KDDI

Title:	TP for TR 38.717-02-01: CA_n41-n257

Agenda item:			9.2.3

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.717-02-01 to include CA_n41-n257 according to the request in [1].

2. Reference

- RP-202199, Revised WID on Rel-17 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2).

# 3. Text Proposal

<Start of Text Proposal>

8.x	CA_n41-n257

8.x.1	Common for 1 band UL and 2 bands UL CA

8.x.1.1	Operating bands for CA

Table 8.x.1.1-1: CA band combination of band n41+n257

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

8.x.1.2	Channel bandwidths per operating band for CA

Table 8.x.1.2-1: Supported bandwidths per CA band combination of band n41+n257

< Editor's note: see Table 5.5A.1-1 in TS38.101-3 for the table format>

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 200 | 400 |  |
| CA_n41A-n257A | CA_n41A-n257A | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  | 0 |
|  |  | n257 |  |  |  |  |  |  |  | 50 |  |  |  |  | 100 | 200 | 400 |  |
| CA_n41A-n257G | CA_n41A-n257A, CA_n41A-n257G | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  | 0 |
|  |  | n257 | See CA_n257G Bandwidth Combination Set 0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n257H | CA_n41A-n257A, CA_n41A-n257G, CA_n41A-n257H | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  | 0 |
|  |  | n257 | See CA_n257H Bandwidth Combination Set 0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n257I | CA_n41A-n257A, CA_n41A-n257G, CA_n41A-n257H, CA_n41A-n257I | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  | 0 |
|  |  | n257 | See CA_n257I Bandwidth Combination Set 0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.x.1.3	∆TIB and ∆RIB values

Assuming separate antenna architecture between FR1 and FR2, TIB,c and RIB values for this CA configuration are given in the tables below.

Table 8.x.1.3-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n41-n257 | n41 | 0 |
|  | n257 | 0 |

Table 8.x.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n41-n257 | n41 | 0 |
|  | n257 | 0 |

8.x.1.4	REFSENS requirements

No IMD issues are expected in this CA configuration as these are TDD bands. These TDD bands do not transmit and receive simultaneously.

9.X	DC_n41-n257

9.x.1	Operating bands for DC_n41-n257

Table 9.X.1-2: Inter-band NR DC operating bands for FR1+FR2

| NR DC Band | NR Band |
| --- | --- |
| DC_n41-n257 | n41, n257 |

9.x.2   Configurations for DC_n41-n257

Table 9.x.2-2: Inter-band NR DC configurations for FR1+FR2

| NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_n41A-n257A DC_n41A-n257G DC_n41A-n257H DC_n41A-n257I | DC_n41A-n257A DC_n41A-n257G DC_n41A-n257H DC_n41A-n257I |

<End of Text Proposal>