3GPP TSG-RAN WG4 Meeting # 98-bis-e	R4-2104456

Electronic Meeting, 12 – 20 April 2021

Agenda Item:	8.12.5.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Proposals on BS transmitter requirements for extending current NR operation to 71 GHz

Document for:	Approval

# 1.	Introduction

The work item for extending current NR operation to 71 GHz was approved at TSG RAN#91-e [1]. One of the objectives of this work item is to define BS RF core requirements for the band(s) in the frequency range between 52.6GHz and 71GHz.

This contribution provides proposals on BS transmitter requirements for extending current NR operation to 71 GHz according to the findings in the corresponding study item as recorded in TR 38.808 [2].

# 2.	Discussion

2.1	General

As stated in TR 38.808 clause 4.2.5.4, BS type 2-O architecture (i.e. the AAS BS architecture) will be considered as the baseline for NR operation in 52.6 – 71 GHz range, thus the radiated transmitter characteristics requirements apply to the BS type 2-O should be considered as the baseline for NR operation in this frequency range.

Proposal 1: The radiated transmitter characteristics requirements apply to the BS type 2-O should be considered as the baseline for NR operation in 52.6 – 71 GHz range.

2.2	Radiated transmit power

The technical reasons (as recorded in TR 38.817-02 [3]) to allow the radiated transmit power of BS type 2-O to be declared by the manufacturer remain valid for NR operation in 52.6 – 71 GHz range, thus the radiated transmit power for NR operation in this frequency range should be declared by the manufacturer.

Proposal 2: The radiated transmit power for NR operation in 52.6 – 71 GHz range should be declared by the manufacturer.

2.3	BS output power

Like the radiated transmit power, the BS output power for NR operation in 52.6 – 71 GHz range should be declared by the manufacturer. Additional regional requirements can be specified to align with regulatory requirements in certain regions.

Proposal 3: The BS output power for NR operation in 52.6 – 71 GHz range should be declared by the manufacturer. Additional regional requirements can be specified to align with regulatory requirements in certain regions.

2.4	Output power dynamics

The total power dynamic range for BS type 2-O is specified for each SCS and channel bandwidth combination. As new numerologies will be defined for NR operation in 52.6 – 71 GHz range, the total power dynamic range should be specified for the new numerologies. The current total power dynamic range for BS type 2-O can be used as baseline, but special consideration should be placed on the wider channel bandwidth with large SCS.

Proposal 4: The current total power dynamic range for BS type 2-O can be used as baseline for NR operation in 52.6 – 71 GHz range, but special consideration should be placed on the wider channel bandwidth with large SCS.

2.5	Transmit ON/OFF power

The transmit OFF power for BS type 2-O is specified based on calculation with a suitable set of operation parameters at 28 GHz carrier frequency (as recorded in TR 38.712-02). Similar calculation should be used to decide the transmit OFF power level for NR operation in 52.6 – 71 GHz range.

Moreover, the transient period for BS type 2-O is shorter than that for BS type 1-O considering the shorter symbol duration with the larger SCS and the target cell sizes in these frequency ranges. Similarly, the transient period for NR operation in 52.6 – 71 GHz range should be shorter than that for BS type 2-O with the same considerations.

Proposal 5: The transmit OFF power for NR operation in 52.6 – 71 GHz range should be calculated with a suitable set of operation parameters at this frequency range. Moreover, the transient period for NR operation in 52.6 – 71 GHz range should be shorter than that for BS type 2-O considering the shorter symbol duration with the larger SCS and the target cell sizes in these frequency ranges.

2.6	Transmit signal quality

The frequency error and EVM requirements for BS type 1-O and BS type 2-O are the same, thus they should be applicable for NR operation in 52.6 – 71 GHz range. However, special consideration should be placed on the applicable higher order modulations for NR operation in 52.6 – 71 GHz range due to the higher phase noise expected in this frequency range.

Moreover, the CA time alignment error requirements for BS type 2-O are shorter than those for BS type 1-O considering the shorter symbol duration with the larger SCS and the target cell sizes in these frequency ranges. Similarly, the CA time alignment error requirements for NR operation in 52.6 – 71 GHz range should be shorter than that for BS type 2-O with the same considerations.

Proposal 6: The frequency error and EVM requirements for BS type 2-O should be applicable for NR operation in 52.6 – 71 GHz range. However, special consideration should be placed on the applicable higher order modulations for NR operation in 52.6 – 71 GHz range due to the higher phase noise expected in this frequency range. Moreover, the CA time alignment error requirements for NR operation in 52.6 – 71 GHz range should be shorter than that for BS type 2-O considering the shorter symbol duration with the larger SCS and the target cell sizes in these frequency ranges.

2.7	Unwanted emissions

The ACLR and ACS values from the coexistence studies at 70GHz in the NR study item (as recorded in TR 38.803 [4]) can be considered as baseline for NR operation in 52.6 – 71 GHz range. Note that the required ACIR values for channel bandwidth wider than 200MHz (which was used in the NR study item) should be smaller considering the relatively impact of the ACI on the higher noise floor with the wider receive bandwidth, so there should be no issue to use the ACLR and ACS values from the NR study item for channel bandwidth wider than 200MHz.

Moreover, the out-of-band emissions and unwanted emissions in the spurious domain specified in ETSI EN 303 722 [5] can be considered as baseline for at least unlicensed NR operation in 52.6 – 71 GHz range, as these requirements will be applicable in Europe. Additional regional requirements can be specified to align with regulatory requirements in certain regions.

Proposal 7: The ACLR and ACS values from the coexistence studies at 70GHz in the NR study item can be considered as baseline for NR operation in 52.6 – 71 GHz range. Moreover, the out-of-band emissions and unwanted emissions in the spurious domain specified in ETSI EN 303 722 can be considered as baseline for at least unlicensed NR operation in 52.6 – 71 GHz range. Additional regional requirements can be specified to align with regulatory requirements in certain regions.

# 3.	Conclusion

This contribution has provided proposals on BS transmitter requirements for extending current NR operation to 71 GHz according to the findings in the corresponding study item as recorded in TR 38.808. They are summarized as follows.

Proposal 1: The radiated transmitter characteristics requirements apply to the BS type 2-O should be considered as the baseline for NR operation in 52.6 – 71 GHz range.

Proposal 2: The radiated transmit power for NR operation in 52.6 – 71 GHz range should be declared by the manufacturer.

Proposal 3: The BS output power for NR operation in 52.6 – 71 GHz range should be declared by the manufacturer. Additional regional requirements can be specified to align with regulatory requirements in certain regions.

Proposal 4: The current total power dynamic range for BS type 2-O can be used as baseline for NR operation in 52.6 – 71 GHz range, but special consideration should be placed on the wider channel bandwidth with large SCS.

Proposal 5: The transmit OFF power for NR operation in 52.6 – 71 GHz range should be calculated with a suitable set of operation parameters at this frequency range. Moreover, the transient period for NR operation in 52.6 – 71 GHz range should be shorter than that for BS type 2-O considering the shorter symbol duration with the larger SCS and the target cell sizes in these frequency ranges.

Proposal 6: The frequency error and EVM requirements for BS type 2-O should be applicable for NR operation in 52.6 – 71 GHz range. However, special consideration should be placed on the applicable higher order modulations for NR operation in 52.6 – 71 GHz range due to the higher phase noise expected in this frequency range. Moreover, the CA time alignment error requirements for NR operation in 52.6 – 71 GHz range should be shorter than that for BS type 2-O considering the shorter symbol duration with the larger SCS and the target cell sizes in these frequency ranges.

Proposal 7: The ACLR and ACS values from the coexistence studies at 70GHz in the NR study item can be considered as baseline for NR operation in 52.6 – 71 GHz range. Moreover, the out-of-band emissions and unwanted emissions in the spurious domain specified in ETSI EN 303 722 can be considered as baseline for at least unlicensed NR operation in 52.6 – 71 GHz range. Additional regional requirements can be specified to align with regulatory requirements in certain regions.

# References

[1]	RP-210862, “Revised WID:  Extending current NR operation to 71 GHz”, CMCC.

[2]	RP-210129, “TR 38.808 v200: Study on supporting NR from 52.6 GHz to 71 GHz”, Intel Corporation.

[3]	3GPP TR 38.817-02: “NR; General aspects for Base Station (BS) Radio Frequency (RF) for NR”.

[4]	3GPP TR 38.803: “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”.

[5]	ETSI EN 303 722: “Wideband Data Transmission System (WDTS) for Fixed Network Radio Equipment operating in the 57 - 71 GHz band; Harmonised Standard for access to radio spectrum”.