3GPP TSG-RAN WG4 Meeting # 98-bis-e                	R4-2104623

Electronic Meeting, Apr. 12-20, 2021

Source: 	Nokia, Nokia Shanghai Bell

Title: 	PUCCH Demodulation performance requirements for operation in unlicensed bands

Agenda item: 	5.1.4.4.3

Document for: 	Discussion

Introduction

This paper extends the discussion with Nokia’s views on PUCCH BS demodulation requirements on [2] considering the outcome of RAN4 #98-e meeting on that topic [1].

PUCCH demodulation requirements

As a pat of the last RAN4 meeting, parameters for the PF3 were decided with 4 bits of information bits and UCI BLER as a test metric. However, also as part of the discussion, a point was raised that UCI BLER might not be the best test metric for this payload size.

Table 1 Summary of used test metric in Rel 15 NR requirements for PUCCH [3]

| PUCCH format | UCI information bits | Test metric |
| --- | --- | --- |
| Format 0 | 1 | ACK missed detection |
| Format 1 | 2 | ACK missed detection and ACK to NACK |
| Format 2 | 4 | ACK missed detection |
|  | 22 | UCI BLER |
| Format 3 | 16 | UCI BLER |
| Format 4 | 22 | UCI BLER |

PF3 with 4 bits is typically used for HARQ/ACK.

Existing RAN4 15 NR PUCCH requirements with 4 bits or less use ACK missed detection as test metric.

Considering the observations above, we propose to use ACK missed detection as test metric for PF3.

Define performance requirements of interlaced PUCCH format 3 with ACK missed detection metric.

Conclusion

This paper presented Nokia’s views on the demodulation requirements of interlaced PUCCH for use on unlicensed bands. As part of this discussion, the following observations and proposals were derived:

Observation 1: PF3 with 4 bits is typically used for HARQ/ACK.

Observation 2: Existing RAN4 15 NR PUCCH requirements with 4 bits or less use ACK missed detection as test metric.

Proposal 1: Define performance requirements of interlaced PUCCH format 3 with ACK missed detection metric.

References

- R4-2017467, Way forward on PUCCH demodulation requirements, 3GPP TSG-RAN WG4 Meeting #97-e

- R4-2100576, PUCCH Demodulation performance requirements for operation in unlicensed bands, 3GPP TSG-RAN WG4 Meeting #98-e, Nokia

- 3GPP TS 38.104 V16.6.0, NR; Base Station (BS) radio transmission and reception

- 3GPP TS 38.141-1 V16.6.0, NR; Base Station (BS) conformance testing Part 1: Conducted conformance testing

- 3GPP TS 38.141-2 V16.6.0, NR; Base Station (BS) conformance testing Part 2: Radiated conformance testing

- 3GPP TS 38.212 V16.3.0, NR; Multiplexing and channel coding

- 3GPP TS 38.214 V16.3.0, NR; Physical layer procedures for data

- 3GPP TS 38.306 V16.3.0, NR; User Equipment (UE) radio access capabilities