3GPP TSG-RAN WG4 Meeting #98-bis-e                                                         R4-2104730

Electronic Meeting, 12 - 20 Apr., 2021

Source: 	CATT

Title: 	Discussion on PUSCH demodulation requirements for FR1 256QAM

Agenda item:	8.14.3.1

Document for:	Approval

- Introduction

In RAN#89-e, the WID on further enhancement on NR demodulation performance was approved. The main objectives for PUSCH demodulation requirements for FR1 256QAM in WID [1] are shown as follows:

BS demodulation requirements: - PUSCH demodulation requirements for FR1 256QAM - Note 1: low mobility is considered for requirements definition - Note 2: Realistic phase noise modelling is left up to the contributing entities

In this contribution, we provide our views on PUSCH demodulation requirements for FR1 256QAM.

- Discussion

Waveform

Both CP-OFDM and DFT-s-OFDM for PUSCH are supported by NR standards. The DFT-s-OFDM is mainly used to enhance coverage with a low MCS. However, 256QAM characterized by higher MCS is mainly used to increase throughput. Therefore, it is proposed to adopt CP-OFDM for 256QAM demodulation.

Proposal 1: To adopt CP-OFDM for 256QAM demodulation.

Antenna configuration

Multiple RX antennas enable higher SNR, which is beneficial for reflecting the high throughput advantage of 256QAM. So the number of RX antennas 2, 4, 8 defined for 64QAM demodulation in TS 38.104 can be reused for 256QAM.

Proposal 2: To adopt the number of RX antennas 2, 4, 8 for 256QAM demodulation.

Only 1TX antenna for 256QAM demodulation in TS 36.104 is required. So 1TX antenna is sufficient for 25QAM for NR FR1.

Proposal 3: To adopt the number of TX antennas 1 for 256QAM demodulation.

DM-RS pattern

We believe that it is sufficient and more convenient to apply the same DM-RS pattern as for other modulation, i.e. single symbol DM-RS with pos1 additional DM-RS position.

Proposal 4: To adopt single symbol DM-RS with pos1 additional DM-RS position for 256QAM demodulation.

SCS/CBW combinations

For SCS/CBW combinations, 256QAM demodulation is not different from other modulation. The other modulations (QPSK, 16QAM, and 64QAM) testing covers the SCS/CBW combinations: 15 kHz SCS: 5MHz, 10MHz, 20MHz and 30 kHz SCS: 10MHz, 20MHz, 40MHz, 100MHz. So the same SCS/CBW combinations can be adopted.

Proposal 5: To adopt 15 kHz SCS: 5MHz, 10MHz, 20MHz and 30 kHz SCS: 10MHz, 20MHz, 40MHz, 100MHz for 256QAM demodulation.

FRC

Referring to the code rate (5/6) for 256QAM demodulation in TS 36.104, the code rate for MCS index =24 is 841/1024, which is close to 5/6. So it is sufficient to use MCS index =24 in Table 5.1.3.1-2: MCS index table 2 for PDSCH (256QAM MCS table) in TS 38.214.

Proposal 6: To adopt MCS index =24 in Table 5.1.3.1-2: MCS index table 2 for PDSCH in TS 38.214 for 256QAM demodulation.

Referring to above proposal for SCS/CBW combinations and DMRS pattern for 256QAM, the FRC for MCS index=24 is listed in Table 2-1.

Proposal 7: To adopt FRC parameters listed in Table 2-1 for 256QAM demodulation.

Table 2-1 FRC parameters for FR1 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos1 and 1 transmission layer (256QAM, R=841/1024)

| Reference channel | G-FR1-A6-8 | G-FR1-A6-9 | G-FR1-A6-10 | G-FR1-A6-11 | G-FR1-A6-12 | G-FR1-A6-13 | G-FR1-A6-14 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing [kHz] | 15 | 15 | 15 | 30 | 30 | 30 | 30 |
| Allocated resource blocks | 25 | 52 | 106 | 24 | 51 | 106 | 273 |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation | 256QAM | 256QAM | 256QAM | 256QAM | 256QAM | 256QAM | 256QAM |
| Code rate (Note 2) | 841/1024 | 841/1024 | 841/1024 | 841/1024 | 841/1024 | 841/1024 | 841/1024 |
| Payload size (bits) | 23568 | 49176 | 100392 | 22536 | 48168 | 100392 | 258144 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 3 | 6 | 12 | 3 | 6 | 12 | 31 |
| Code block size including CRC (bits) (Note 2) | 7888 | 8224 | 8392 | 7544 | 8056 | 8392 | 8352 |
| Total number of bits per slot | 28800 | 59904 | 122112 | 27648 | 58752 | 122112 | 314464 |
| Total symbols per slot | 3600 | 7488 | 15264 | 3456 | 7344 | 15264 | 39308 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos1, l0= 2 and l =11 for PUSCH mapping type A, l0= 0 and l =10 for PUSCH mapping type B as per table 6.4.1.1.3-3 of TS 38.211 [5]. NOTE 2: Code block size including CRC (bits) equals to K' in clause 5.2.2 of TS 38.212 [15]. |  |  |  |  |  |  |  |

PUSCH time domain resource allocation type

The PUSCH time domain resource allocation types for 256QAM should be both type A and B to allow for different types of BS implementation.

Proposal 8: To adopt the PUSCH time domain resource allocation type: type A and type B for 256QAM demodulation.

Channel model

Only TDLA30-10 Low was defined for 64QAM demodulation in TS38.104. So it is proposed to adopt single channel model TDLA30-10 Low for 256QAM.

Proposal 9: To adopt TDLA30-10 Low for 256QAM demodulation.

Other test parameters for testing PUSCH

It is sufficient and more convenient to reuse the same test parameters defined in Table 8.2.1.1-1 in TS 38.104 for other test parameters of 256QAM, including TDD UL-DL pattern, HARQ, time domain resource assignment, frequency domain resource assignment, etc. The other parameters for 256QAM are listed in Table 2-2.

Table 2-2 other test parameters for testing PUSCH for 256QAM

| Parameter |  | Value |
| --- | --- | --- |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| Time domain | PUSCH mapping type | A, B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |

Proposal 10：To adopt other parameters listed in Table 2-2 for 256QAM demodulation.

- Conclusion

This contribution provides analysis on PUSCH demodulation requirements for FR1 256QAM. The following proposals are concluded as follows:

Proposal 1: To adopt CP-OFDM for 256QAM demodulation.

Proposal 2: To adopt the number of RX antennas 2, 4, 8 for 256QAM demodulation.

Proposal 3: To adopt the number of TX antennas 1 for 256QAM demodulation.

Proposal 4: To adopt single symbol DM-RS with pos1 additional DM-RS position for 256QAM demodulation.

Proposal 5: To adopt 15 kHz SCS: 5MHz, 10MHz, 20MHz and 30 kHz SCS: 10MHz, 20MHz, 40MHz, 100MHz for 256QAM demodulation.

Proposal 6: To adopt MCS index =24 in Table 5.1.3.1-2: MCS index table 2 for PDSCH in TS 38.214 for 256QAM demodulation.

Proposal 7: To adopt FRC parameters listed in Table 2-1 for 256QAM demodulation.

Proposal 8: To adopt the PUSCH time domain resource allocation type: type A and type B for 256QAM demodulation.

Proposal 9: To adopt TDLA30-10 Low for 256QAM demodulation.

Proposal 10：To adopt other parameters listed in Table 2-2 for 256QAM demodulation.

- References

- RP-202000,New WID: Further enhancement on NR demodulation performance, China Telecom,RAN#89-e

- 3GPP TS 36.104 V17.0.0 (2020-12)

- 3GPP TS 38.104 V17.0.0 (2020-12)