3GPP TSG-RAN WG4 Meeting # 98-bis-e 	R4-2104806

Electronic Meeting, Apr. 12-20, 2021

Source: 	CATT

Title: 	Discussion on co-existence simulation assumption

Agenda item:	8.12.1

Document for:	Approval

- Introduction

In RAN4#98-e, co-existence simulation issue was raised [1] and discussed [2]. This contribution provides our consideration for the further study.

- Discussion

The following is the WF for the co-exist study in the last meeting.

Consider TR 38.803 as baseline and update more recent information after reviewing the study assumption

Some initial review has been provided in [1], some further analysis is conducted in this contribution. The initial proposals for the co-exit simulation are provided.

2.1 Simulation scenarios

In TS 38.803, the following simulation scenarios are used.

--------------------------------

Table 5.1: Summary of initial simulation scenarios for above 6GHz

| No. | Usage scenario | Aggressor | Victim | Direction | Simulation frequency | Deployment Scenario |
| --- | --- | --- | --- | --- | --- | --- |
| 7 | eMBB | NR, 200MHz | NR, 200MHz | DL to DL | 70 GHz | Indoor hotspot |
| 8 | eMBB | NR, 200MHz | NR, 200MHz | DL to DL | 70 GHz | Dense urban |
| 9 | eMBB | NR, 200MHz | NR, 200MHz | UL to UL | 70 GHz | Indoor hotspot |
| 10 | eMBB | NR, 200MHz | NR, 200MHz | UL to UL | 70 GHz | Dense urban |

--------------------------------

In TS 38.808, scenario indoor-A and indoor-B are used as high priority scenarios in the system simulation.

- Scenario indoor-A (for two operator case)

- Scenario indoor-C (for single operator case)

For 52.6-71 GHz, we think indoor scenario can be candidate scenario for co-existence simulation. For single operator case, the scenario indoor-C in TR 38.808 is same as layout of indoor hotspot in TR 38.803. For two operator case, the scenario indoor-A in TR 38.808 is not that tough as in TR 38.803. The minimum distance between BS of different operators of scenario indoor-A is 2m. The multi operators layout for indoor hotspot in TR 38.803 is coordinated operation (0% Grid Shift), which means the two operators are co-located. Our understanding is that TR 38.808 scenario is more reasonable that co-ordination in indoor area may not be very popular. So the Scenario indoor-A (for two operator case) from TS 38.808 can be taken as RAN4 co-exist simulation scenario for 52.6-71 GHz.

Proposal 1: Take Scenario indoor-A (for two operator case) from TS 38.808 as RAN4 co-exist simulation scenario for 52.6-71 GHz.

2.2 Co-existence simulation assumption

Carrier frequency

The carrier frequency difference between TR 38.803 and TR 38.808 is summarized in Table 2-1.

Table 2-1: Carrier frequency difference between TR 38.803 and TR 38.808

|  | TR 38.803 | TR 38.808 |
| --- | --- | --- |
| Carrier frequency | 70GHz | 60GHz Optional:70GHz |

From RF perspective, 70GHz path loss and hardware challenge are larger than 60GHz. The ACLR/ACS requirement for 70 GHz may be less stringent than 60 GHz, or at least the same as 60 GHz. So 70 GHz carrier frequency assumption is ok for above 52.6 GHz, 60 GHz can also be an extra choice if it’s also considered a typical center frequency.

Proposal 2: 70GHz is used as the carrier frequency assumption for 52.6-71 GHz co-existence simulation,  60 GHz can also be considered as an extra choice.

SCS, Bandwidth and RB

200MHz was used in TR 38.803, which needs to be modified for above 52.6 GHz. 2000 MHz and 400 MHZ is used in TR 38.808. There’re different understandings of the implementation of 2000MHz, larger CBW should be supported or CA can support 2000MHz. Therefore, 1.6GHz CBW and 480 KHz SCS can be chosen to be the simulation assumption. There’s no spectrum utilization agreement yet, the current FR2 264 RB can be assumed before the agreement in above 52.6 GHz.

Proposal 3: 1.6GHz CBW and 480KHz SCS/264 RB are the assumption for the configuration.

BS and UE antenna modelling

The BS and UE antenna modelling difference between TR 38.803 and TR 38.808 is summarized in Table 2-2. The Antenna element radiation pattern difference between TR 38.803 and TR 38.808 is summarized in Table 2-3.

Table 2-2: BS and UE antenna modelling difference between TR 38.803 and TR 38.808

|  |  | TR 38.803 Indoor | TR 38.808 Indoor |
| --- | --- | --- | --- |
| BS | (Mg, Ng, M, N, P) | (1, 1, 8, 16, 2) | (1,1,4,8,2) Optional: (1,1,8,16,2) |
|  | (dv, dh) | (0.5λ, 0.5λ) | (0.5λ, 0.5λ) |
|  | Antenna element gainGE,max | 5dBi | 5 dBi |
|  | Antenna element radiation pattern | Table 5.2.3.2.3-1 of TR38.803 | Table A.2.1-7 of TR38.802 for ceiling mount |
|  | Array gain per polarization per pannel | 26dBi | 20dBi Optional: 26dBi |
| UE | (Mg, Ng, M, N, P) | (1, 1, 2, 2, 2) | (1,2,2,2,2) Optional: (1,2,4,4,2) |
|  | (dv, dh) | (0.5λ, 0.5λ) | (0.5λ, 0.5λ) |
|  | Antenna element gainGE,max | 5dBi | 5 dBi |
|  | Antenna element radiation pattern | Table A.2.1-8 of TR38.802 | Table A.2.1-8 of TR38.802 |
|  | Array gain per polarization per pannel | 11dBi | 11dBi Optional: 17dBi |

Table 2-3:  Antenna element radiation pattern difference between TR 38.803 and TR 38.808

| Antenna element radiation pattern in Table 5.2.3.2.3-1 of TR38.803 | Antenna element vertical radiation pattern (dB) |  |
| --- | --- | --- |
|  | Antenna element horizontal radiation pattern (dB) |  |
|  | Combining method for 3D antenna element pattern (dB) |  |
| Antenna element radiation pattern for ceiling-mount in Table A.2.1-7 of TR38.802 | Antenna element vertical radiation pattern (dB) |  |
|  | Antenna element horizontal radiation pattern (dB) |  |
|  | Combining method for 3D antenna element pattern (dB) |  |

The antenna size assumption (Mg, Ng, M, N, P) in TR 38.803 is more appropriate than TR 38.808 because larger antenna size is needed for higher frequency. The assumed antenna element gain GE,max for BS for dense urban in TR 38.808 is 5 dBi, which is more reasonable than 8dBi in TR 38.803 considering element spacing (0.5λ, 0.5λ). Ceiling mount is considered in TR 38.808, which leads to different antenna element radiation pattern compared with the pattern in TR 38.803.

According to the discussion in the last RAN4 meeting, 8x16, 16x16 and 32x32 were some examples in TP [6]. We propose 16 x 16 as the assumption for co-exit simulation. For Antenna element radiation pattern, we propose to reuse the assumption in TR 38.808.

Proposal 4: For the BS and UE antenna model, use TR 38.808 assumption with the update of BS antenna size (1,1,16,16,2).

BS and UE max TX power

The BS and UE TX power difference between TR 38.803 and TR 38.808 is summarized in Table 2-4.

Table 2-4: BS and UE max TX power difference between TR 38.803 and TR 38.808

|  | TR 38.803 Indoor | TR 38.808 Indoor |
| --- | --- | --- |
| BS max TX power | 23dBm | 40 dBm EIRP Optional: 60 dBm EIRP Maximum TxP adjusted to meet EIRP limits |
| UE max TX power | 23dBm | 25 dBm EIRP with 21 dBm max TxP Optional: 40dBm EIRP with 21 dBm max TxP |

When R15 co-existence simulation was conducted, the BS/UE output power requirements were not decided yet. The power assumption was assumed much higher than the final requirements, especially for UE. TR 38.808 assumes EIRP capability, which is more reasonable when two NR releases have been finished. From that aspect, the assumption in TR 38.808 is more reasonable at least the EIRP assumption is correct from RF perspective. Considering the antenna size assumption in proposal 4, the BS Tx power assumption can be updated to be 61.8 dBm according to TP [6].

Proposal 5: EIRP is used as the BS/UE Tx power metric. BS Tx power is assumed as 61.8 dBm EIRP, UE Tx power is assumed as 25 dBm EIRP.

BS and UE noise figure

The BS and UE noise figure difference between TR38.803 and TR38.808 is summarized in Table 2-5.

Table 2-5, BS and UE noise figure difference between TR38.803 and TR 38.808

|  | TR 38.803 | TR 38.808 |
| --- | --- | --- |
| Noise figure | BS: 13 UE: 15 dB | 1) Referring to ETSI technical report TR 101 854, NF is in range of 10 – 13 dB. 2) Referring to the system level evaluation assumptions in annex A, BS: 7 dB, UE: 10 dB (13 dB optionally). |

Proposal 6: Noise figure assumption in TR 38.803 can be reused for co-existence simulation for 52.6-71GHz.

LBT

No LBT is used in this simulation assuming that the deployments are on licensed bands.

Proposal 7: No LBT is used for co-exit simulation.

Remaining assumptions

There’re some remaining assumptions such as channel model, power control, etc. The assumptions can reuse TR 38.803 assumption if there’re no conflicts with the above assumption.

Proposal 8: Remaining assumptions can reuse the assumptions in TR 38.803 if there’re no conflicts with the above assumptions.

- Conclusion

In this contribution, the some parameters of the co-existence study for 52.6-71GHz are discussed. The following observations and proposals are summarised as below:

Proposal 1: Take Scenario indoor-A (for two operator case) from TS 38.808 as RAN4 co-exist simulation scenario for 52.6-71 GHz.

Proposal 2: 70GHz is used as the carrier frequency assumption for 52.6-71 GHz co-existence simulation, 60 GHz can also be considered as an extra choice.

Proposal 3: 1.6GHz CBW and 480KHz SCS/264 RB are the assumption for the configuration.

Proposal 4: For the BS and UE antenna model, use TR 38.808 assumption with the update of BS antenna size (1,1,16,16,2).

Proposal 5: EIRP is used as the BS/UE Tx power metric. BS Tx power is assumed as 61.8 dBm EIRP, UE Tx power is assumed as 25 dBm EIRP.

Proposal 6: Noise figure assumption in TR 38.803 can be reused for co-existence simulation for 52.6-71GHz.

Proposal 7: No LBT is used for co-exit simulation.

Proposal 8: Remaining assumptions can reuse the assumptions in TR 38.803 if there’re no conflicts with the above assumptions.

- References

- R4-2100384, Discussion on co-existence for 52.6-71GHz, CATT, RAN4#98-e.

- R4-2103333, Email discussion summary for [98e][146] FS_NR_52_to_71GHz_Part_2

- R4-2103258, WF on NR 52 – 71 GHz: Part2

[4]   TR 38.803, Study on new radio access technology: Radio Frequency (RF) and co-existence aspects.

[5]   TR 38.808, Study on supporting NR from 52.6 GHz to 71 GHz (Release 17).

[6]   R4-2103256, TP to TR 38.808: BS antenna array