3GPP TSG-RAN4 Meeting #98-bis-e	R4-2104835

# Online, Apr. 12 - 20, 2021

Agenda item:	8.12.6

Source:	Apple Inc.

Title:	On the frequency range of 52.6 GHz to 71 GHz

WI/SI:	FS_NR_52_to_71GHz

Release:	Rel-17

Document for:	Approval

# Introduction

In RAN#90e, it was agreed in the revised WID [1] that

Note 5: RAN plenary will decide whether new FR (e.g. FR3) shall be defined for the frequency range from 52.6GHz-71GHz or the existing FR2 shall be extended to cover frequency range from 52.6GHz-71GHz.

Such decision made is mainly from the consideration of RAN4’s workload. The related issues have been discussed in both RAN#90e and RAN4#98e and opinions from interested companies are quite diverged.

In RAN#91e, this issue is further discussed and it is agreed that

As an agreement in RAN#91e, RAN1, RAN2 and RAN4 are asked to provide its analysis or recommendation to RAN#92E (June) on how to introduce the 52.6-71GHz frequency range

Based on the discussion in RAN#91e, the possible options include

- Option 1: reuse FR2 to cover the new frequency range 52.6-71GHz

- Option 2: introduce new FR3 for the new frequency range 52.6-71GHz

- Option 3a: introduce new FR2a and FR2b notation for 24.25-52.6GHz, 52.6-71GHz, respectively

- Option 3b: keep FR2 notation for 24.25-52.6GHz and introduce a new notation FR2a notation for 52.6-71GHz

In this contribution, we outline the pros and cons of these options and the related impacts upon the potential decision made among the options.

# 2	On the options to define the frequency range 52.6 GHz to 71 GHz

In this section, the pros and cons of different options are outlined

|  | Pros | Cons |
| --- | --- | --- |
| Option 1: reuse FR2 for cover the new frequency range 52.6-71GHz | - Easy for spec maintenance if majority of existing FR2 requirements and features can be reused. | - It is difficult to handle the existing FR2 requirements which cannot be extended to 52.6-71GHz |
| Option 2: introduce new FR3 for the new frequency range 52.6-71GHz | - No impact on existing spec - Flexibility to either reuse FR2 or introduce 60GHz-specific features and requirements | - New UE RF spec may be needed: one for FR3 SA and FR2+FR3, FR1+FR2+F3 - Duplication is expected for the feature and requirements which can be extended from existing FR2 - Otherwise, the spec title of 38.101-2/3 needs to be revised to include FR3. |
| Option 3a: introduce new FR2a and FR2b notation for 24.25-52.6GHz, 52.6-71GHz, respectively | - No new UE RF spec is needed - Flexibility to either reuse FR2 or introduce 60GHz-specific features and requirements | - Major revision is needed for existing FR2 features and requirements which cannot be extended to FR2b |
| Option 3b: keep FR2 notation for 24.25-52.6GHz and introduce a new notation FR2a notation for 52.6-71GHz | - Limited impact on existing spec - Flexibility to either reuse FR2 or introduce 60GHz-specific features and requirements | - It is ambiguous by definition if FR2a is part of FR2. If yes, the issues with Option 1 may happen here. If no, issues of Option 2 apply. |

Based on the analysis above, it is a challenge in Option 1 and 3b to handle the cases when 60GHz-specific requirements are to be introduced. Option 2 and 3a are more flexible for both backward and forward compatibility. Technically, the fundamental difference between Option 2 and 3a is not significant. With this, it is proposed:

Proposal 1: it is proposed to narrow down the list of options to Option 2 and 3a for further discussion.

Proposal 2: the following principles should be considered to define the frequency range of 52.6 GHz to 71 GHz

- No new spec should be introduced

- Ensure flexibility to either reuse FR2 or introduce 60GHz-specific features and requirements

- Strive to reduce the impact on existing spec

# 2	Some considerations on the frequency range 52.6 GHz to 71 GHz

Definition of the frequency range 52.6 GHz to 71 GHz largely depends how much the existing FR2 features and requirements can be extended to cover 52.6 GHz to 71 GHz. Although RAN4 needs to work on the details, the following aspects should be considered.

- UE architecture: The UE architecture for the frequency range from 52.6GHz-71GHz is still unclear. Since Rel-15, many of existing FR2 requirements are specified based on the assumptions of discrete RFICs for FR1 and FR2 separately. It is not clear as of now if 60GHz will be integrated with or separated from the FR2 RFIC. Depending on the different architectures for FR1+FR2+60GHz, existing FR2 features and requirements may not be easily extended to the frequency range from 52.6GHz-71GHz.

- Existing FR2 features: due to the uncertainty of the UE architecture and ongoing discussion in RAN1, it is not straightforward if and how the existing FR2 features and requirements can be directly extended.  If 60GHz specific new features and/or requirements are introduced under the umbrella of FR2, significant efforts on spec revisions are expected to differentiate 60GHz from the existing FR2 bands. Some preliminary analysis has been done on FR2 features and requirements which may not be able to be directly extended to 60GHz.

- Band agnostic principles: As discussed by RAN WG1 and WG4, supporting the 52.6 to 71GHz frequency range will require new system level parameters, such as new sub-carrier spacing, new minimum and maximum channel bandwidth, etc. Every frequency range has associated system parameters that inextricably define common requirements for all the bands in the corresponding range. If FR2 is reused for 52.6 to 71GHz range, then this logic will break as different bands will have different associated mandatory parameters.

In the following sub-sections, we present some preliminary analysis on existing FR2 features and requirements identifying which features it may not be able to extend directly to 60GHz. At the same time, we also present potentially negative consequences if a completely new frequency range, e.g. FR3, is introduced.

## 2.1 RRM requirements in RAN4

In the current RRM spec, the requirements are designed in a band agnostic manner for a specific FR and therefore it should be carefully evaluated if FR2 RRM requirements can be directly extended to cover 60GHz spectrum. Based on some preliminary analysis, at least the following issues are subject to further study:

- Measurement gap

In the RAN4 spec, independent measurement gaps (MG) were designed such that per-FR1 MG and per-FR2 MG can be configured separately. The current FR2 MG patterns are listed as below (pattern 12~23). The Measurement Gap Length (MGL) (1/3/5ms) for FR2 MG is based on the SSB duration under 120kHz and 240kHz, and the RF tuning margin is 250us for one way tuning in legacy FR2. However, if the SSB number is not increased in the SSB burst and the SCS is increased to 960kHz, the current FR2 MGL might be a big waste of the measurement resource. Furthermore, the RF tuning time with higher SCS might be even smaller than 250us, and therefore the per-FR MG for new 60GHz shall be redesigned and the old FR2 MG may not be applicable. It then would be necessary to have an individual FR concept to contain the new MG patterns.

Regarding the UE capability, so far per-FR MG capable UE would have different RF chains between FR1 and FR2, which means those UEs would not have cross FR interruption or interference. In the current RRM requirement, most of the RRM requirements are related with this UE capability, e.g., interruption, multiple SCell activation, multiple BWP switching and etc. If the new 60GHz design has a separated RF chain different from the  legacy FR2, it is also natural to introduce additional per-FR3 capability to indicate the 60GHz serving CC would not be interrupted or interfered by the UE activities in FR1 and legacy FR2.

- Beam sweeping based measurement delay requirements

In most of IDLE/CONNECTED measurement delay requirements of cell identification, measurement, handover, RLM, beam-management, re-establishment and redirection, the beam sweeping scaling factor is designed as 8 for FR2 (band agnostic), and some examples are shown as below.  However, if the narrower beam is used for the new 60GHz scenario, the scaling factor may need to be revisited (the beam sweeping number might be larger than 8). As a result, many of current FR2 band agnostic requirements will not be applicable to 60GHz if FR2 is extended to cover 60GHz.

IDLE mode cell detection requirement

RLM evaluation requirement

- Accuracy requirement

In the current RSRP/RSRQ/SINR accuracy requirements, the RF calibration margin is band agnostic in FR2. It has been taken into account in almost all measurement accuracy requirements. RF calibration margin is specified based on certain assumptions of the RF antenna array size and number. It is uncertain if such an assumption and calibration margin can be extended to 60GHz especially with the possibility of smaller array sizes or increased number of arrays with the reduction in wavelength compared with FR2. If not, the band agnostic FR2 accuracy requirements cannot be applied for this 60GHz measurement. Moreover, in IDLE mode, the re-selection rank is specified based on measurement accuracy and also on a per-FR basis(shown as below), and similar as accuracy requirement, the existing ranking requirement cannot be applied for cell reselection on 60GHz.

IDLE mode cell reselection rank requirement

- Timing requirement

In the current RAN4 timing requirement, e.g., MRTD/MTTD or SFTD measurement, FR specific requirements are defined to reflect the different scenarios. For instance, in inter-band NR CA MRTD requirement (shown as below), the FR2 MRTD is define based on the propagation delay difference between CCs and TAE. However, in the new 60GHz deployment, the cell coverage of such high spectrum would naturally shrink compared with the legacy FR2 deployment, and therefore the existing propagation delay difference shall be revised accordingly and the existing FR2 requirement cannot be used for new 60GHz case. On the other hand, in this 60GHz case, the legacy FR2 TAE in inter-band CA (3us) is much larger than 1 symbols length with SCS=960kHz, and RAN4 also needs to consider if a new FR shall be used for the TAE requirement.

The SFTD requirement in the current RRM spec is defined as 40Ts (1.3us) for FR1 and FR2 (inter-frequency SFTD requirement is shown as below), and the justification is: 40Ts is almost below 15% of the symbol length of 120kHz, however, with 960kHz SCS in 60GHz, the 40Ts is even longer than one symbol (120% of a symbol length), so it’s necessary to have a new FR for 60GHz with a new SFTD requirement.

TA offset is another aspect in the timing requirement, and the 60GHz TA offset may not follow the FR2 requirement. The existing TA offset requirement is derived based on UE capability of Tx-Rx/Rx-Tx transition time, but if new 60GHz UE has different Tx-Rx/Rx-Tx transition time, the existing FR2 TA offset cannot be applicable. Thus, a new FR of 60GHz with a new TA offset requirement is more reasonable.

- Interruption requirement with aggressor or victims

Even though some of the existing interruption requirements are designed based on the SCS of the victim CC, there are still many interruption requirements associated with FR (shown as below). The main reason is: the RF tuning time is differentiated from FR1 to FR2 and therefore the interruption from RF tuning/retuning shall be designed on per-FR basis. Similarly, any new 60GHz design would probably have shorter RF tuning/retuning time, and the existing FR2 interruption requirement cannot be applied any longer. Thus, it is reasonable to have a new FR of 60GHz with a new interruption requirement.

- Scheduling restriction

One example of existing scheduling restriction for mobility measurement is shown as below:

The one symbol before and after SSB is based on the assumption of cell phase synchronization error of 3us. However, on a 60GHz carrier, the symbol length with 960kHz is as low as 1.09us, and therefore the existing one symbols before and after SSB for FR2 scheduling restriction would be insufficient. Thus, using a new FR of 60GHz with a new cell phase synchronization requirement or with a new scheduling restriction would be the most reasonable solution.

## 2.2 Demod requirements in RAN4

Currently RAN1 is discussing potential enhancements to PT-RS and DM-RS for 60GHz operation. In case such new DM-RS and PT-RS are specified, the existing demodulation and CSI requirements for FR2 would not be applicable and a new set of requirements would need to be defined. Also, currently the radiated requirements apply to all FR2 bands and are band agnostic. Currently it is FFS if the existing FR2 demod requirements are applicable to band n262 / 47GHz. Even if DM-RS and PT-RS remained unchanged for 60GHz operation, it would need further study and discussion in RAN4 whether existing demodulation and CSI radiated test requirements are applicable to the new bands. From demodulation and CSI reporting requirements perspective, it is not possible to conclude if extending FR2 to include 60GHz band or defining a new frequency range is a better choice at this stage.

## 2.3 UE RF requirements in RAN4

For the frequency band from 52.6GHz to 71GHz, as the frequency goes higher, the RF components’ performance such PA, LO, filter are not expected to be as good as for FR2. Also, the coexistence scenarios with other systems will be different and potentially lead to different ACLR/ACS requirements. As a result, the RF requirements such as MOP, ACLR/SEM, ACS/blocking, etc. could be different. Depending on the gap with those FR2 requirements, which will require more discussions in RAN4, it seems reasonable to defer the decision on extending FR2 to cover this band or creating a new FR.

From the viewpoint of the specification impact, it bears mentioning that there are separate specifications for FR1 and FR2 frequency ranges, TS 38.101-1 and TS 38.101-2, respectively. There is also TS 38.101-3 that captures further FR1 and FR2 inter-working operation. Leveraging FR2 will allow 3GPP to re-use existing specifications, such as TS 38.101-2 and TS 38.101-3. Introduction of a new frequency range might instantiate at least one new specification, e.g. TS 38.101-x, in additional to which another specification might be needed to cover band combinations between a new frequency range and existing ranges. In fact, introduction of a new frequency range will trigger further discussions on how to handle band combinations and potential permutations between FR1/FR2/FR3.

Observation 1: At lease for the following FR2 features and requirements, it is not straightforward to reuse them for the frequency range from 52.6GHz to 71GHz

  - RRM:

    - Measurement gap

    - Beam sweeping based measurement delay requirements

    - Accuracy requirements

    - Timing requirement

    - Interruption requirement with aggressor or victims

  - Demod:

    - Existing demod and CSI requirements due to potential enhancements to PT-RS and DM-RS for 60GHz operation

    - Based on the ongoing discussion about the applicability of existing FR2 demod requirements to band n262 / 47GHz, similar approach should be done for 60GHz.

  - UE RF:

    - RF components’ performance such PA, LO, filter are not expected to be as good as for FR2. The related impacts should be further discussed

# 3	Conclusions

In this contribution, the potential impacts of how to define 52.6GHz-71GHz in either a new FR or existing FR2 are discussed from RAN4 perspective. Profound impacts have been identified from the aspects of UE architecture, existing FR2 features, band agnostic principles, signaling and 3GPP specification maintenance. Based on the analysis, it is proposed that

Proposal 1: it is proposed to narrow down the list of options to Option 2 and 3a for further discussion.

  - Option 2: introduce new FR3 for the new frequency range 52.6-71GHz

  - Option 3a: introduce new FR2a and FR2b notation for 24.25-52.6GHz and 52.6-71GHz, respectively

Proposal 2: the following principles should be considered to define the frequency range of 52.6 GHz to 71 GHz

- No new spec should be introduced

- Ensure flexibility to either reuse FR2 or introduce 60GHz-specific features and requirements

- Strive to reduce the impact on existing spec

Regarding the potential impact to extend the existing FR2 requirements and features to 52.6GHz-71GHz, it is shown that

Observation 1: At lease for the following FR2 features and requirements, it is not straightforward to reuse them for the frequency range from 52.6GHz to 71GHz

  - RRM:

    - Measurement gap

    - Beam sweeping based measurement delay requirements

    - Accuracy requirements

    - Timing requirement

    - Interruption requirement with aggressor or victims

  - Demod:

    - Existing demod and CSI requirements due to potential enhancements to PT-RS and DM-RS for 60GHz operation

    - Based on the ongoing discussion about the applicability of existing FR2 demod requirements to band n262 / 47GHz, similar approach should be done for 60GHz.

  - UE RF:

    - RF components’ performance such PA, LO, filter are not expected to be as good as for FR2. The related impacts should be further discussed

# 4	References

RP-202925, "Revised WID:  Extending current NR operation to 71 GHz", CMCC