3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2104959

# Electronic meeting, April 12th – April 16th, 2021

Agenda Item:	7.24.2.1

Source: 	MediaTek Inc.

Title: 	n5 REFSENS supporting 25MHz CBW

Document for:	Approval

- 1. Introduction

A new WID in [1] has been approved. UE is going to support n5 with 25MHz channel bandwidth. The contribution characterize REFSENS and propose on UL configuration.

- 2. Discussion

Discussion about UL channel bandwidth

In TS38.101-1 sub clause 6.2.2	UE maximum output power reduction, the MPR assumption is copied below:

UE is allowed to reduce the maximum output power due to higher order modulations and transmit bandwidth configurations. For UE power class 2 and 3, the allowed maximum power reduction (MPR) is defined in Table 6.2.2-2 and Table 6.2.2-1, respectively for channel bandwidths that meets both following criteria:

Channel bandwidth ≤ 100 MHz.

Relative channel bandwidth ≤ 4 % for TDD bands and ≤ 3 % for FDD bands

Where relative channel bandwith = 2*BWChannel / (FUL_low + FUL_high)

For n5 with 25MHz uplink CBW, relative channel bandwidth =25/837.5 = 2.98% which does not exceeds the “≤ 3 % for FDD bands”. Thus existing MPR table can be applied for this case.

Proposal 1: If UL bandwidth extend to 25MHz, it does not exceed existing MPR BW ratio assumption in TS38.101-1. The general MPR is still applicable and there’s no need to restrict UL bandwidth

REFSENS

The REFSENS calculation assumptions are listed in table 2.2-1 and REFSENS analysis is in table. Since UL bandwidth is up to 25MHz and the frequency gap between TX and RX is 20MHz, in addition to CIM5 spur, IMD3 of wanted TX and its image fall in the own receiving range shall be considered. From previous low band PA measurement, the total IMD3 level at PA output port is -17.5dBm. There would be 3.4/10.8 (the portion fall in the receiving range) IMD3 spur level fall into own receiving range. UL configuration is same with that in [2].

| Parameter | Value | Unit |
| --- | --- | --- |
| Antenna isolation | 10 | dB |
| n5 filter rejection from TX to RX | 51 | dB |
| n5 diversity receiver filter TX rejection | 44 | dB |
| Front-end loss | 4 | dB |
| CIM5 of n5 TX | -70 | dB |
| IMD3 level of n5 TX and its image | -17.5 | dBm |
| Thermal noise at n5 RX ANT port | -165 | dBm/Hz |
| PA wide band nosie | -125 | dBm/Hz |
| Transceiver effective phase noise due to wider BW | -144 | dBc/Hz |
| SNR requirement for QPSK | -1 | dB |

Table 2.2-1 Typical receiver performance parameters for MSD analysis

| Direct Signal Path |  |  |  |
| --- | --- | --- | --- |
| Parameter | Main | Diversity |  |
| n5 TX power at antenna port (Primary path) | 23 | 23 | dBm |
| n5 PA output noise power at RX freq | -125 | -125 | dBm/Hz |
| n5 TX noise power at PRX LNA port | -93.6 | NA | dBm/25MHz |
| n5 TX noise power at DRX LNA port | NA | -93.6 | dBm/25MHz |
| n5 receiver reciprocal mixing noise at LNA port | -94.2 | -94.2 | dBm/25MHz |
| Thermal noise at RX ant port | -165 |  | dBm/Hz |
| TX IMD3 due to wanted and its img at LNA input port | -73.5 | -84.5 | dBm |
| Total noise level refer to receiver LNA input port | -72.1 | -81.2 | dBm |
| Combined |  |  |  |
| REFSENS (25MHz BW) | -79.4 |  | dBm |

Table 2.2-2 Link analysis for n5 SCS=15 KHz, 25 MHz REFSENS calculation

Proposal 2: REFSENS for n5, 25MHz is proposed highlighted as below:

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n5 | 15 | -98.0 | -94.8 | -93.0 | -86.8 | -79.4 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -95.1 | -93.1 | -86.6 | -79.2 |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2.2-3: REFSENS values for n5 and 25 MHz CBW

And associated RB allocations should be as defined in 2.2-4.

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n5 | 15 | 25 | 251 | 201 | 201 | 201 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 121 | 101 | 101 | 101 |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2.2-4: RB allocations for REFSENS band n5 and 25 MHz CBW

Conclusion

Proposal 1: If UL bandwidth extend to 25MHz, it does not exceed existing MPR BW ratio assumption in TS38.101-1. The general MPR is still applicable and there’s no need to restrict UL bandwidth

Proposal 2: REFSENS for n5, 25MHz is proposed highlighted as below:

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n5 | 15 | -98.0 | -94.8 | -93.0 | -86.8 | -79.4 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -95.1 | -93.1 | -86.6 | -79.2 |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2.2-3: REFSENS values for n5 and 25 MHz CBW

And associated RB allocations should be as defined in 2.2-4.

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n5 | 15 | 25 | 251 | 201 | 201 | 201 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 121 | 101 | 101 | 101 |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2.2-4: RB allocations for REFSENS band n5 and 25 MHz CBW

Reference:

- RP-202277, Revised Basket WID on adding channel bandwidth support to existing NR bands, Ericsson

- R4-2103185 Way forward on the introduction of 25 MHz in band n5, RAN4#98-e