3GPP TSG-RAN WG4 Meeting #98BIS-e                                        	R4-2104963

Electronic meeting, 12th – 20th April, 2021

Source: 	LG Electronics

Title: 	TP on summary of interference studies for new x bands DL (x=3, 4, 5) with 2 bands UL

Agenda item:	10.5.3

Document for:	Approval

1. Introduction

This document provides a result of self-interference analysis for new LTE-A CA x bands (x=3, 4, 5) DL with 2 bands UL in Rel-17.

For 3 bands DL with 2 bands UL, the impact to the 3rd own receiving down link band by harmonics and IMD products when dual uplink transmission is activated.

For 4 bands DL with 2 bands UL, the impact to the 3rd and 4th own receiving down link bands by harmonics and IMD products when dual uplink transmission is activated.

For 5 bands DL with 2 bands UL, the impact to the 3rd, 4th, and 5th own receiving down link bands by harmonics and IMD products when dual uplink transmission is activated.

Since possible self-desense problems that have been already analysed in lower fallback mode such as 2 bands DL with 2 bands UL CA combos and 3 bands DL with 2 bands UL CA combos for the new 4 bands DL with 2 bands UL CA combinations and new 5 bands DL with 2 bands UL CA combinations. Therefore, RAN4 only defining MSD will be needed for new 3 bands DL with 2 bands UL CA combinations.

### ****************** Start of the TP in subclause 5.1.5 of TR36.717-03-02 ************************

### 5.1.5	Summary of interference studies

Table 5.1.5-1 summarizes a result of self-interference studies of proposed LTE-A inter-band CA combination for 3 bands DL with 2 bands UL in Rel-17.

Table 5.1.5-1: Summary of the self-interference studies for 3 bands DL with 2 bands UL

| Downlink CA configuration | Uplink CA configuration | Harmonic relation to 3rd band without uplink | IMD to 3rd band without uplink | Interference due to small frequency separation | MSD (Maximum Sensitivity Degradation) |
| --- | --- | --- | --- | --- | --- |
| CA_3C-7A-8A | CA_3A-8A | 3rd harmonic | 2nd and 3rd IMDs | - | No need to study for harmonic and IMD impacts since they have been already analyzed in TS36.101. |
| CA_1A-3C-38A | CA_1A-3A | - | - | Yes | Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. |
| CA_3C-8A-38A | CA_3A-8A | - | 2nd and 3rd IMDs | - | No need to study for IMD impacts since they have been already analyzed in TS36.101. |
| CA_1A-3C-20A | CA_1A-3A | - | - | Yes | Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. |
| CA_3A-8A-20A CA_3C-8A-20A | CA_3A-8A | - | 2nd , 4th and 5th IMDs | - | MSD (27dB) for 2nd IMD can follow from DC_3A-20A_n8A in TS38.101-3 |
| CA_1A-8A-20A | CA_1A-8A | - | 4th IMD | - | MSD (11.5dB) for 4th IMD can follow from DC_1A-20A_n8A in TS38.101-3 |
| CA_1A-8A-38A | CA_1A-8A | - | - | - | - |

Table 5.1.5-2 summarizes a result of self-interference studies of proposed LTE-A inter-band CA combination for 4 bands DL with 2 bands UL in Rel-17.

Table 5.1.5-2: Summary of the self-interference studies for 4 bands DL with 2 bands UL

| Downlink CA configuration | Uplink CA configuration | Harmonic relation to 3rd band without uplink | Intermodulation to 3rd band without uplink | Interference due to small frequency separation | MSD (Maximum Sensitivity Degradation) |
| --- | --- | --- | --- | --- | --- |
| CA_1A-3C-7A-8A | CA_1A-3A | - | - | Yes | Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. |
|  | CA_1A-8A | 2nd harmonic impact from B8 to B3 and B7 | - | - | No need to study for harmonic impacts since they have been already analyzed in TS36.101. |
|  | CA_3A-8A | 3rd harmonic impact from B8 to B7 | 2nd and 3rd IMDs in B7 | - | - No need to study for harmonic impacts since they have been already analyzed in TS36.101. - 2nd and 3rd IMD problems have been already covered in CA_3A-7A-8A/CA_3A-8A. |
| CA_1A-3C-8A-38A | CA_1A-3A | - | - | Yes | Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. |
|  | CA_1A-8A | 2nd harmonic impact from B8 to B3 | - | - | No need to study for harmonic impacts since they have been already analyzed in TS36.101. |
|  | CA_3A-8A | - | 2nd and 3rd IMDs in B38 | - | 2nd and 3rd IMD problems have been already covered in CA_3A-8A-38A/CA_3A-8A. |
| CA_1A-3A-8A-20A, CA_1A-3C-8A-20A | CA_1A-3A | - | - | Yes | Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. |
|  | CA_1A-8A | 2nd harmonic impact from B8 to B3 | 4th IMD in B20 | - | - No need to study for harmonic impacts since they have been already analyzed in TS36.101. |
|  |  | CA_3A-8A |  | - |  | 2nd and 5th IMDs in B20 |  | - |  | The MSD will study for IMD impacts in lower order DL_CA_3A-8A-20A with UL_CA_3A-8A |  |
| CA_1A-3A-20A-38A, CA_1A-3C-20A-38A | CA_1A-3A CA_1A-20A CA_3A-20A | - | - | Yes | Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. |
| CA_1A-3A-7A-38A | CA_1A-3A | - | - | Yes | Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. |
| CA_1A-7A-8A-38A | CA_1A-8A | 3rd harmonic impact from B8 to B7 | - | - | No need to study for harmonic impacts since they have been already analyzed in TS36.101. |
| CA_3A-7A-8A-38A | CA_3A-8A | 3rd harmonic impact from B8 to B7 | 2nd and 3rd IMDs in B7, 2nd and 3rd IMDS in B38 | - | - No need to study for harmonic impacts since they have been already analyzed in TS36.101. - All IMD problems have been already covered in CA_3A-7A-8A/CA_3A-8A and CA_3A-8A-38A/CA_3A-8A. |
| CA_1A-8A-20A-38A | CA_1A-8A | - | 4th IMD in B20 | - | The MSD will covered in lower order DL_CA_1A-8A-20A with UL_CA_1A-8A. |
| CA_3A-8A-20A-38A | CA_3A-8A | - | 2nd and 5th IMDs in B20, 2nd and 3rd IMDs in B38 | - | -The MSD for 2nd and 5th IMD problems will covered in lower order DL_CA_3A-8A-20A with UL_CA_3A-8A. - 2nd and 3rd IMD problems have been already covered in CA_3A-8A-38A/CA_3A-8A. |
| CA_3A-7A-20A-38A | CA_3A-20A | 3rd harmonic from B20 into B38 | 2nd IMD into B7 & B38, 3rd IMD into B7 & B38 | - | - The 2nd IMD already covered in DL_CA_3A-7A-20A with UL_CA_3A-20A. Also follow same MSD of DL_CA_3A-7A-20A for CA_3A-20A-38A |

Table 5.1.5-3 summarizes a result of self-interference studies of proposed LTE-A inter-band CA combination for 5 bands DL with 2 bands UL in Rel-17.

Table 5.1.5-3: Summary of the self-interference studies for 5 bands DL with 2 bands UL

| Downlink CA configuration | Uplink CA configuration | Harmonic relation to 3rd band without uplink | IMD to 3rd band without uplink | Interference due to small frequency separation | MSD (Maximum Sensitivity Degradation) |
| --- | --- | --- | --- | --- | --- |
| CA_1A-3A-7A-8A-38A | CA_1A-3A | - | - | Yes | Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. |
|  | CA_1A-8A | 2nd harmonic impact from B8 to B3 and 3rd harmonic impact from B8 to B7 | - | - | - No need to study for harmonic impacts since they have been already analyzed in TS36.101. |
|  | CA_3A-8A | 3rd harmonic impact from B8 to B7 | 2nd IMD in B7, 3rd IMD in B38 | - | - No need to study for harmonic impacts since they have been already analyzed in TS36.101. - All IMD problems have been already covered in CA_3A-7A-8A/CA_3A-8A and CA_3A-8A-38A/CA_3A-8A. |
| CA_1A-3A-8A-20A-38A | CA_1A-3A | - | - | Yes | Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. |
|  | CA_1A-8A | 2nd harmonic impact from B8 to B3 | 4th IMD in B20 | - | - No need to study for harmonic impacts since they have been already analyzed in TS36.101. - MSD for IMD impacts will covered in lower order DL_CA_1A-8A-20A with UL_CA_1A-8A. |
|  | CA_3A-8A | - | 2nd and 5th IMDs in B20, 2nd and 3rd IMDs in B38, | - | - 2nd and 5th IMD problems for lower order DL_CA_3A-8A-20Awith UL_CA_3A-8A need to be studied. - 2nd and 3rd IMD problems have been already covered in CA_3A-8A-38A/CA_3A-8A. |
| CA_1A-3A-7A-20A-38A | CA_1A-3A |  | - | Yes | Close proximity issue was already covered in Table 7.3.1A-0bA in TS 36.101. |
|  | CA_1A-20A | 3rd harmonic from B20 into B38 | - | - | - No need to study for harmonic impacts since they have been already analyzed in TS36.101. |
|  | CA_3A-20A | 3rd harmonic from B20 into B38 | 2nd & 3rd IMD into B7 & B38 | - | - No need to study for harmonic impacts since they have been already analyzed in TS36.101. - These IMD issues are already covered in lower fallback mode. |

According to Table 5.1.5.1, 5.1.5.2, and 5.1.5.3, it is proposed to define the following REFSENS exceptions and test configurations in the core specifications for 3 bands DL with 2 bands UL CA combinations that have some IMD issues.

Table 5.1.5.4 REFSENS exception and test configuration for proposed 3 bands DL with 2 bands UL CA combination in Rel-17

| E-UTRA band/ Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA DL Configuration | E-UTRA CA UL Configuration | E-UTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |

<End of TP>