3GPP TSG-RAN WG4 Meeting #98-bis-e                             R4-2105051

Online, 12th-20th Apr, 2021

Source:	Samsung, KDDI

Title:	TP for TR 37.717-21-11: DC_1-11_n41

Agenda item:			7.4.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.717-21-11 to include DC_1A-11A_n41A according to the request in [1].

2. Reference

- R4-2101509 WID revision: Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL).

# 3. Text Proposal

<Start of Text Proposal>

5.x	DC_1-11_n41

### 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-11A_n41A | DC_1A_n41A DC_11A_n41A |

### 5.x.2	Co-existence studies

Based on co-existence studies of DC_1A-11A_n41A with 2UL, it can get that:

- There are no additional harmonic and intermodulation impact for the additional band receiver considering KDDI spectrum (band n41 is limited to 2595MHz – 2645MHz).

### 5.x.3	∆TIB and ∆RIB values

For DC_1 A-11A_n41A, the TIB,c and RIB,c values are proposed in the tables below.

Table 5.x.3-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-11_n41 | 1 | 0.5 |
|  | 11 | 0.3 |
|  | n41 | 0.5 |

Table 5.x.3-2: ΔRIB due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-11_n41 | 1 | 0 |
|  | 11 | 0 |
|  | n41 | 0 |

### 5.x.4	REFSENS requirements

Based on 5.x.2 above, REFSENS exceptions are not expected.

<End of Text Proposal>