3GPP TSG-RAN WG4 Meeting # 98-bis-e 										                          R4-2105187

Electronic Meeting, 12th - 20th April, 2021

Agenda item:			7.26

Source:	Moderator (CATT)

Title:	Email discussion summary for [98-bis-e][114] NR_LTE_V2X_PC5_combos

Document for:	Information

Introduction

This email discussion summary will discuss the newly introduced band combinations of V2X con-current operation.

The candidate targets of this email discussion for 1st round and 2nd round:

- 1st round

  - Companies to provide comments on TPs and CRs involved.

- 2nd round

  - Further check the revised TPs and CRs if any.

  - Recommend the final status of TPs and CRs.

Topic #1: Introduce new band combination

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2104769 | CATT | Title: TP on V2X_n79A-n47A and V2X_n79A-47A coexistence study |
| R4-2104770 | CATT | Title: Draft CR for TS 38.101-1, Introduce new band combination of V2X_n79A-n47A |
| R4-2104771 | CATT | Title: Draft CR for TS 38.101-3, Introduce new band combination of V2X_n79A-47A |
| R4-2107297 | Huawei, HiSilicon | Title: Scope of NR V2X R17 combinations |

Open issues summary

Based on above contributions, no open issues are summarized. Associated CRs and TPs on newly introduced band combinations will be directly treated in sub-cluase 1.3.2.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |

CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2104769 (TP on V2X_n79A-n47A and V2X_n79A-47A coexistence study) | CATT: support this TP. |
| R4-2104770 (Draft CR for TS 38.101-1, Introduce new band combination of V2X_n79A-n47A) | Huawei: 1. Can we simplify the configuration table as we did in table 5.5A.3.1? 2. The spurious emission for frequency range 5925-5950 and 5815-5855 are additional requirements for band n47. Not sure they can be specified in the general requirements parts. |
|  | Qualcomm: Cannot agree with draft CR. There is TX band noise that couples from n79A to the RX of n47 and TX band noise of n47 that couples into the RX of n79. There needs to be a ΔRIB,V2X of 3dB for both n79 and n47 Reply to CATT Our analysis indicates that the RX band noise created by the n79 and n47/47 PAs cannot be filtered sufficiently by the external RF circuitry to prevent them from not impacting the RX sensitivity of both n79 and n47/B47. Please note that though the separation between these bands are over 800MHz the absolute frequency of operation of these bands is 5-6GHz which makes the design of RF components difficult given the Q-factor. So instead of considering absolute frequency separation of the 2 bands the Q factor should be considered in defining ΔRIB,V2X. This is the reason that we want a ΔRIB,V2X of 3dB specified for both n79 and n47 for inter-band combinations n79A-n47A and n79A-47A. Higher Q filters will make the insertion loss greater which will make the stand alone operation to require higher power in both bands. |
|  | CATT: Reply to Huawei 1. I am not clear of which configuration can be simplified compared with clause 5.5A.3.1. 2. This is just to follow the principle used in 36.101 that contains general requirements and additional requirements in one table. The additional spurious emission is also clarified with notes. So no confusion is observed here. Reply to Qualcomm: The frequency range of n47 is 5855-5925 while the frequency range of band n79 is 4400-5000Hz. It is expected no coupled noise from each other with a relatively large frequency separation. ΔRIB,V2X should be specified based on coexistence studies. |
|  | LGE: In TS38.101-1, the following A-SE requirements are defined with NS_33 as follow Table 6.5.3.4.2-1: Additional requirements for "NS_33" Protected band; Frequency range (MHz); Maximum Level (EIRP2); MBW (MHz); NOTE. Frequency range; 5925; -; 5950; -30; 1; 1. Frequency range; 5815; -; 5855; -30; 1; 3. NOTE 1: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. NOTE 2: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I in [11]. NOTE 3: Resolution BW is 10% of the measurement BW and the result should be integrated to achieve the measurement bandwidth. The sweep time shall be set larger than (symbol length)*(number of points in sweep) to improve the measurement accuracy.. So, for the V2X_n79A-n47A, RAN4 shall remove the coexistence proptection band as 5925-5950 MHz and 5815- 5855MHz.Since n47 V2X UE do not need to protect above freq. band in TS38.101-1 if NS_33 is not signaled to V2X UE. To QC, maybe 800MHz frequency gap can be eliminate the Tx noise into n47 Rx. |
| R4-2104771 (Draft CR for TS 38.101-3, Introduce new band combination of V2X_n79A-47A) | Qualcomm Cannot agree with draft CR. There is TX band noise that couples from n79A to the RX of n47 and TX band noise of B47 that couples into the RX of n79. There needs to be a ΔRIB,V2X of 3dB for both n79 and B47 Reply to CATT See reply to R4-2104770 |
|  | CATT: Support this CR. |
|  | LGE : LTE V2X can follow TS36.101 for coexistence requirements.So we are OK to add 5925-5950 MHz and 5815- 5855MHz for protection bands. So support draft CR. |
| R4-2107297 (Scope of NR V2X R17 combinations) | Qualcomm Cannot agree with TP. Once all the intra-band combinations in table 1-1 have been analyzed and the specification for these have been agreed to by interested companies then the issue of supporting release independence to an earlier release can be addressed. Reply to CATT We are agreeable to making these inter-band combinations agreed from past meetings release independent 39A-n47A, 40A-n47A, 41A-n47A, n39A-47A, n40A-47A, n41A-47A |
|  | CATT: Regarding release independence, we can include the already agreed band combination in table 1-1. |
|  | LGE : this is inter-band con-current V2X operation. So, it can be supported from rel-16. |
|  | Huawei: To address the concern of release independent, agree CATT the finished band combinations can be included in the Table. The purpose of the Table is to give a clear view which combinations have been introduced from Rel-17. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2104769 | Agreeable. |
| R4-2104770 | To be revised. The following open issues will be further discussed in 2nd round: - Whether to specify ΔRIB,V2X of 3dB for both n79 and n47. - Whether to remove the protection of frequency range 5925-5950MHz and 5815-5855MHz in UE co-existence spurious emission table. |
| R4-2104771 | To be revised. Whether to specify ΔRIB,V2X of 3dB for both n79 and 47 needs further discussion in 2nd round. |
| R4-2107297 | To be revised. Capture the already agreed band combinations in table 1-1. |

Discussion on 2nd round (if applicable)

Companies views’ collection for 2st round

Open issues

| Company | Comments |
| --- | --- |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-21xxxxx (Draft CR for TS 38.101-1, Introduce new band combination of V2X_n79A-n47A) |  |
| R4-21xxxxx (Draft CR for TS 38.101-3, Introduce new band combination of V2X_n79A-47A) |  |
| R4-21xxxxx (Scope of NR V2X R17 combinations) |  |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on … | YYY |  |
| LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2104769 | TP on V2X_n79A-n47A and V2X_n79A-47A coexistence study | CATT | Agreeable |  |
| R4-2104770 | Draft CR for TS 38.101-1, Introduce new band combination of V2X_n79A-n47A | CATT | Revised |  |
| R4-2104771 | Draft CR for TS 38.101-3, Introduce new band combination of V2X_n79A-47A | CATT | Revised |  |
| R4-2107297 | Scope of NR V2X R17 combinations | Huawei, HiSilicon | Revised |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents