3GPP TSG-RAN WG4 Meeting # 98-bis-e                                                        R4-2105227

Electronic Meeting, 12th– 20th April, 2021

Source:	Huawei, HiSilicon

Title:	TP for TR 36.717-03-02 to add UL configuration CA_1A-3A CA_1A-8A and CA_3A-8A for CA_1A-3A-7A-8A-38A

Agenda item:	10.5.3

Document for:	Approval

Background

This contribution provides text proposal on the LTE UL configuration CA_1A-3A CA_1A-8A and CA_3A-8A for CA_1A-3A-7A-8A-38A as defined in the revised WID [1].

Reference

[1]	RP-210200, “Revised WID on Rel-17 LTE-A inter-band CA for x bands (x= 3, 4, 5) DL with 2 bands UL”, LG Electronics

Text Proposal

---Start of changes---

8.X	LTE-A inter-band CA: Band 1 and Band 3 and Band 7 and Band 8 and Band 38 DL with 2 bands UL

8.X.1	List of specific combination issues

8.X.1.1	Channel bandwidths per operating band for CA

Table 8.X.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-7A-8A-38A | CA_1A-3A CA_1A-8A CA_3A-8A | 1 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 3 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

8.X.1.2	Co-existence studies for LTE-A UL CA_1A-3A, UL CA_1A-8A, UL CA_3A-8A and DL CA_1A-3A-7A-8A-38A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.X.1.2-1,Table 8.X.1.2-2 and Table 8.X.1.2-3.

Table 8.X.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 5130 | 5355 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 270 | 135 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2055 | 2250 | 1440 | 1650 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5550 | 5745 | 5340 | 5550 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3975 | 4230 | 3150 | 3435 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7470 | 7725 | 7050 | 7335 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 270 | 540 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5220 | 4860 | 6210 | 5895 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8760 | 9120 | 9390 | 9705 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1515 | 1170 | 2520 | 2190 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 | 9315 | 9180 | 9510 |

Based on Table 8.X.1.2-1, the 3rd IMD products by Band 1 and Band 3 will interfere own Rx frequency of Band 1. There are no harmonic or IMD products from Band 1 and Band 3 uplinks which would desensitize Band 7 and Band 8 and Band 38 downlink.

Table 8.X.1.2-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 2640 | 2745 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1100 | 1005 | 2800 | 2895 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2925 | 3080 | 220 | 90 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4720 | 4875 | 3680 | 3810 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4845 | 5060 | 660 | 825 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6640 | 6855 | 4560 | 4725 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2010 | 2200 | 5600 | 5790 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1740 | 1540 | 7040 | 6765 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1095 | 1320 | 4180 | 3930 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5440 | 5640 | 8560 | 8835 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6480 | 6705 | 7520 | 7770 |

Based on Table 8.X.1.2-2, 4th IMD products by Band 1 and Band 8 falls into the own Rx frequency of Band 1,and the 2nd harmonic from Band 8 falls into Rx frequency of Band 3, and the 3nd harmonic from Band 8 falls into Rx frequency of Band 7.

Table 8.X.1.2-3: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 5130 | 5355 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 795 | 905 | 2590 | 2700 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 25 | 120 | 2505 | 2690 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3470 | 3615 | 4300 | 4485 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 855 | 1035 | 4215 | 4475 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4350 | 4530 | 6010 | 6270 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1810 | 1590 | 5180 | 5400 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6260 | 5925 | 1950 | 1735 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7720 | 8055 | 5230 | 5445 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3595 | 3300 | 675 | 930 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6890 | 7185 | 6060 | 6315 |

Based on Table 8.X.1.2-3, 2nd IMD products by Band 3 and Band 8 falls into the own Rx frequency of Band 7,and 3th IMD products by Band 3 and Band 8 falls into the own Rx frequency of Band 38,and the 2nd harmonic from Band 8 falls into Rx frequency of Band 3, and the 3nd harmonic from Band 8 falls into Rx frequency of Band 7.

8.X.1.3	MSD

When uplink CA configurations CA_1A-3A, CA_1A-8A, CA_3A-8A is paired with downlink CA configuration CA_1A-3A-7A-8A-38A there are no additional analysis of MSD because it is solved in fallback band combination..

8.X.1.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values are shown in table 8.x.1.4-1 and table 8.x.1.4-2 according to CA_1-3-7-8-40 relaxation values, respectively.

Table 8.x.1.4-1: ΔTIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A-7A-8A-38A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.8 |
|  | 8 | 0.3 |
|  | 38 | 0.9 |

Table 8.x.1.4-2: ΔRIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A-7A-8A-38A | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0 |
|  | 8 | 0 |
|  | 38 | 0 |

---End of changes---