3GPP TSG-RAN WG4 Meeting#98-bis             	R4-2105238

E-meeting, 12th – 20th  April, 2021

Title:	TP to TR 38.717-02-01: CA_n30-n77

Source:	Nokia, AT&T

Agenda item:	7.2.2

Document for:	Approval

# Introduction

This is a TP into TR 38.717-02-01 to introduce CA_n30A-n77A.

********************** Start of TP *********************************

## 6.x		CA_n30-n77

### 6.x.1	Common for 1 band UL and 2 bands UL CA

#### 6.x.1.1 Operating bands for CA

Table 6.x.1-1: CA band combination of band CA_n30-n77

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n30-n77 | n30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

#### 6.x.1.2	Channel bandwidths per operating band for CA

Table 6.x.1.2-1: Supported bandwidths per CA band combination of band CA_n30-n77

| NR CA configuration | UL configuration | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n30A-n77A | CA_n30A-n77A | n30 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n30A-n77(2A) | CA_n77(2A) CA_n30A-n77A | n30 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 6.x.1.3	Co-existence studies

Table 6.x.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n30-n77.

Table 6.x.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 30 | 2305 | 2315 | 2350 | 2360 | 4610 | 4630 | 6915 | 6945 | 9220 | 9260 | 11525 | 11575 |
| 77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

Based on above table, there is no UL harmonic issue for CA_n30-n77.

Table 6.x.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 30 | 2305 | 2315 | 2350 | 2360 | 4700 | 4720 | 7050 | 7080 | 9400 | 9440 | 11750 | 11800 |
| 77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

#### UE receiver may suffer receiver 3rd order harmonic mixing from 2*TX of n77

#### 6.x.1.4	∆TIB and ∆RIB values

It is proposed to re- use relaxation values from DC_41_n77,

Table 6.x.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n30-n77 | n30 | 0.3 |
|  | n77 | 0.8 |

Table 6.x.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n30-n77 | n30 | 0 |
|  | n77 | 0.5 |

#### 6.x.1.5	REFSENs requirements

There is no REFSENS exceptions needed.

#### 6.x.1.6	OOB blocking exception requirements

There is no OOB exception for the CA combination.

### 6.x.2		Specific for 2 bands UL CA

#### 6.x.2.1	Maximum output power for inter-band CA

Table 6.x.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n30A-n77A | 23 | +2/-3 |

#### 6.x.2.2		UE co-existence

Table 6.x.2.2-1 gives IMD interference analysis for CA_ n30-n77 with 2 ULs.

Table 6.x.2.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2305 | 2315 | 3300 | 4200 |
| DL frequency (MHz) | 2350 | 2360 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4610 | 4630 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 6915 | 6945 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 9220 | 9260 | 16500 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 11525 | 11575 | 16500 | 21000 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 985 | 1895 | 5605 | 6515 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 410 | 1330 | 4285 | 6095 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7910 | 8830 | 8905 | 10715 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 2715 | 3645 | 7585 | 10295 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 10215 | 11145 | 12205 | 14915 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3790 | 1970 | 11210 | 13030 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14495 | 10885 | 5960 | 5020 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15505 | 19115 | 12520 | 13460 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7990 | 5270 | 345 | 1485 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14510 | 17230 | 13515 | 15345 |

Based on the table 6.x.2.2-1, there is IMD4 interfering n30.

Table 6.x.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.x.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n30-n77 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 38, 41, 53, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |

#### 6.x.2.3		REFSENS requirements

IMD5 interfering n30 own DL needs MSD requirement. MSD is re-used from CA_n1-n77 which is very similar combination.

Table 7.3A.5-1: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n30-n77 | n30 | 2310 | 5 | 25 | 2355 | 8.0 | FDD | IMD4 |
|  | n77 | 3487.5 | 10 | 50 | 3487.5 | N/A | TDD | N/A |

It is proposed to use MSD values from DC_40_n78.

Table 7.3B.2.3.2-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | 308 | 10.4 | 10.4 |  |  |  |  |  |  |  |  |  |
| NOTE 8: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.2-2: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n77 | 30 | 15 | 12 | 25 |  |  |  |  |  |  |  |  |  |

********************** End of TP *********************************