3GPP TSG-RAN WG4 Meeting #98-bis-e                                                         R4-2105258

Electronic Meeting, 12-20 April, 2021	Revision of R4-2106572

Source:	Ligado Networks, Huawei, HiSilicon

Title:	TP for TR 38.717-02-01: CA_n24-n41

Agenda item:			7.2.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.717-02-01 to include CA_n24-n41 as per [1].

# 2. Reference

- RP-210495, Revised WID on Rel-17 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2).

# 3. Text Proposal

<Start of Text Proposal>

## 6.x		CA_n24-n41

### 6.x.1		Common for 1 band UL and 2 bands UL CA

#### 6.x.1.1		Operating bands for CA

Table 6.x.1.1-1: CA band combination of band n24+n41

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n24 | 1626.5 MHz | – | 1660.5 MHz | 1525 MHz | – | 1559 MHz | FDD |
| n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

#### 6.x.1.2		Channel bandwidths per operating band for CA

Table 6.x.1.2-1: Supported bandwidths per CA band combination of band n24+n41

| NR CA Configuration | UL Configuration | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n24A-n41A | CA_n24A-n41A | n24 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n24A-n41(2A) | CA_n24A-n41A | n24 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 | See CA_n41(2A) BCS1 in Table 5.5A.2-1 from 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 6.x.1.3		Co-existence studies

Table 6.x.1.3-1 summarizes frequency ranges where harmonics occur for CA_n24-n41.

Table 6.x.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |  |
| n24 | 1626.5 | 1660.5 | 1525 | 1559 | 3253 | 3321 | 4879.5 | 4981.5 | 6506 | 6642 | 8132.5 | 8302.5 |  |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 | 12480 | 13450 |  |

Based on above table, there is no harmonic issue for the band combination of n24 and n41.

Table 6.x.1.3-2 summarizes frequency ranges where harmonics mixing occur for CA_n24-n41.

Table 6.x.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n24 | 1626.5 | 1660.5 | 1525 | 1559 | 3050 | 3118 | 4575 | 4677 | 6100 | 6236 | 7625 | 7795 |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 | 12480 | 13450 |

Based on above table, there is no harmonic mixing issue for the band combination of n24 and n41.

#### 6.x.1.4		∆TIB and ∆RIB values

For CA_n24-n41 , the ∆TIB,c and ∆RIB,c values are given according to CA_n25-n41 relaxation values.

Table 6.x.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n24-n41 | n24 | 0.3 |
|  | n41 | 0.41 |
|  |  | 0.92 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 6.x.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n24-n41 | n24 | 0 |
|  | n41 | 0 |

#### 6.x.1.5		REFSENS requirements

#### No additional MSD requirement need to be defined for this CA band combination.

#### 6.x.1.6		OOB blocking exception requirements

#### There is no OOB blocking exception for this CA band combination.

### 6.x.2		Specific for 2 bands UL CA

#### 6.x.2.1	Maximum output power for inter-band CA

Table 6.x.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n24A-n41A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

#### 6.x.2.2		UE co-existence studies

Table 6.x.2.2-1: Impact of Intermodulations

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1626.5 | 1660.5 | 2496 | 2690 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 835.5 | 1063.5 | 4122.5 | 4350.5 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 563 | 825 | 3331.5 | 3753.5 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 5749 | 6011 | 6618.5 | 7040.5 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 2189.5 | 2485.5 | 5827.5 | 6443.5 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 7375.5 | 7671.5 | 9114.5 | 9730.5 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 1671 | 2127 | 8245 | 8701 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 3816 | 4146 | 8323.5 | 9133.5 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 9002 | 9332 | 11610.5 | 12420.5 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 500.5 | 10.5 | 4167 | 4817 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 10741 | 11391 | 9871.5 | 10361.5 |

Based on the table above, there are no IMDs falling on top of n24 or n41 DL, and therefore no additional MSD needs to be specified.

Table 6.x.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.x.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n24-n41 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 25, 26, 29, 30, 48, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 in TS 38.101-1 or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |

#### 6.x.2.3		REFSENS requirements

No MSD needs to be specified.

<End of Text Proposal>