3GPP TSG-RAN WG4 #98bis-e	R4-2105415

e-Meeting, Apr 12 – 20, 2021

Agenda item:	8.12.4

Source: 	Qualcomm Incorporated

Title: 	Way forward on system simulations for 60 GHz

Document for:	Discussion

# Introduction

System simulations were proposed in R4-2104806 ( CATT), R4-210-5172 and 5173 (QCOM)).

# Rationale for 60 GHz band coexistence simulation

Companies discuss their rationale for system simulation, or their rationale for relying on past work.

| Company | Rational |
| --- | --- |
| QCOM | 60 GHz array directivities are higher – 60 GHz arrays can be bigger than FR2 - 8x2 element array is reasonable implementation that fits in the size of FR2 simulated array - Directivity affects both uplink and downlink results - Higher directivity will result in lower interference level 25 dBm assumption is too high for 60 GHz – somewhere in the 17-20 dBm range is realistic assumption - Based on commercial 802.11 ad/ay PA designs and realistic losses Additional comment regarding the need to do coexistence simulations. Although evaluation for FR2 coexistence studies has been carried out in TR 38.803, we believe that there are some differences between the simulation assumptions considered in TR 38.808 and TR 38.803, which calls the need for conducting additional coexistence simulations. For example, the indoor deployment scenarios in TR 38.803 are only scenario C, which is as realistic as scenario A. Additionally, more options regarding BW are considered in TR 38.808. Although it is plausible that the requirements might be like that reported in TR38.803, we believe that to validate this statement, coexistence simulations for 60GHz with an updated set of parameters needs to be conducted. |
| CATT | Large CBW, different EIRP capability assumption |
| ZTE | Fine to further evaluate 60GHz band since antenna array size and output power will be different compared with lower FR2 frequency range. However it should also be noted that coexistence simulation results is not only input to determine ACLR/ACS values, therefore we might end up with other factors like PA capability to determine the final values. |
| Huawei | For the system level simulations: it seems reasonable to discuss and align system simulation parameters (option1 from the first round summary), as the RAN1 TR 38.803 assumptions may require some updates e.g. due further updates on the channel BW agreements, consideration of RF impairments for the max output power, etc. We would be fine to collect the initial feedback on the system level simulation assumptions for indoor and outdoor scenarios. However, we are not ready to agree on details of the coex simulation assumptions. For sake of progress, we suggest to consider the discussed parameters and their values as the starting point and to aim for agreement next meeting. As we are discussing simulation assumptions for coex: it shall be clarified what are the scenarios in terms of aggressor and victim. Referring to the TR38.803, the coex simulations were performed at the 70GHz carrier frequency. Therefore question for clarification: shall we rather use 70GHz as carrier frequency, instead of 60GHz? Probably we shall not look as the 60GHz proxy frequency, but 70 (or even 71GHz) corner case. Please note that the latest version of TR38.803 is from 2017, as any reference to this TR shall be considered carefully (TR being quite old now). WF name to be clarified that it is related to the coexistence simulation assumptions. |
| Nokia | We do not see any need for additional simulations at given stage. We can rely on the results presented in 38.803 and/or for unlicensed deployments align applicable requirements to ETSI EN 303 753 harmonized standard. Noting that simulation studies have been conducted and aligned to other technologies deployed in the shared spectrum for the requirements captured in EN 303 753. Moreover, results in R4-2104429 and R4-2104401, as expected, have shown that ACIR requirements can be relaxed with non-collocated BS and larger channel bandwidth, as well as scenario indoor-A, so there is no need to redo simulation as those in TR 38.803 represent more stringent cases. |
| Ericsson | We do not see the need for additional simulation efforts. For those companies which believe new simulations are needed we would like to understand why the system simulations done in previously documented in TR 38.803 cannot be used for the work in this WI. |
| Apple | Even though the 60GHz array can be bigger (e.g. 8x2 elements) because its size can fit the size of FR2 simulated array, we do not see compelling reasons to consider it as the baseline option. For instance, referring to some evaluations done by RAN1, 2x2 element arrays were assumed. And since number of antenna elements will have a great impact on RAN4 requirements, such as peak EIRP and spherical coverage, our preference is to keep 4x1 as an option for the system simulations. |

# System coexistence sim parameters indoor

Table 1 List of system level coexistence simulation parameters

| Deployment |  | Indoor Office A and C |
| --- | --- | --- |
| System Parameters | Carrier Frequency | 60 GHz |
|  | Channel BW | 400 and 2000 MHz |
|  | Number of active UEs | 1 |
|  | Channel model | InH open office specified in [2] |
|  | LBT | LBT and No LBT |
|  | SCS | 120kHz and 960kHz |
| BS | (Mg, Ng, M, N, P) | (1,1,4,8,2) |
|  | (dv, dh) | (0.5 λ, 0.5 λ) |
|  | Antenna element gain | 5 dBi |
|  | Antenna element radiation pattern | Table A.2.1-7 in [3] for ceiling mount |
|  | Max Tx Power | 20 dBm (≈EIRP = 43 dBm) |
|  | Noise Figure | 13 dB |
| UE | (Mg, Ng, M, N, P) | (1,2,2,8,2) |
|  | (dv, dh) | (0.5 λ, 0.5 λ) |
|  | Antenna element gain | 5 dBi |
|  | Antenna element radiation pattern | Table A.2.1-8 in [3] |
|  | Max Tx Power | 20 dBm EIRP |
|  | Noise figure | 13 dB |
|  | LoS/ NLoS | LoS probability for indoor office defined in [4] |

Company comments on sim parameters

| Company | Scenario |
| --- | --- |
| QCOM | We agree with the all the parameters in the table. The critical difference for us from past work is the UE antenna array and the EIRP. At the shorter wavelengths the UE array can have more elements. Commercial PAs for 60 GHz are less efficient than FR2 and produce lower power. |
| Intel | May we get clarified why only no-LBT case is being studied in indoor and outdoor? Based on the approved WID, both LBT and no LBT are required to be specified. LBT is also mandatory by ETSI EN 302 567. We suggest to include LBT case for at least unlicensed band as well for both indoor and outdoor. |
| Huawei | Proposed WF: consider the above list of the coexistence simulation assumptions for indoor scenario as the starting point for discussion. Aim for decision in May meeting. |
| Nokia | Results in R4-2104429 and R4-2104401, as expected, have shown that ACIR requirements can be relaxed with scenario indoor-A compared to scenario indoor-C, so there is no need to redo simulation as those in TR 38.803 represent more stringent cases. |
| Apple | Referring back to our comment on the 4x1 array size, why do we consider channel bandwidth of 400MHz? At least for the 120kHz SCS the minimum channel bandwidth is 100MHz, which should be also considered. By the way, the table does not mention which SCS is assumed. |

Additional company comments and discussion:

These comments were entered as “Comments” in word. Copied them here since those type of comments are useful however they are not very readable:

|  | Scenario |
| --- | --- |
| Table 1 - Deployment | CATT: We think Indoor-A could be sufficient. Is there any reason for C? We’re ok if C is included. QCOM to CATT: We proposed C as it was the indoor scenario considered in TR 38.803. We agree with CATT that A might be more realistic in reflecting practical 60GHz indoor deployments |
| Table 1: System parameters - Frequency | Huawei: We would like to have more discussion on 60GHz vs. 70GHz. |
| Table 1: System parameters - Channel BW | ZTE: 100MHz with 120kHz SCS should be prioritized since this is worst case for coexistence study where it could have high in-band PSD and out-of-band PSD. QCOM to ZTE: Based on TR 38.808, we think that it is ok for the time being to consider 400 and 2000MHz as listed in Section A.2. |
| Table 1 – system parameters - LBT | Intel: Both LBT and no LBT are required by WID QCOM to Intel: Although LBT is in fact considered in TR 38.808, but we believe that we can consider non-LBT as a preliminary step to compare results (if possible) with the requirements derived in TR 38.803. We welcome views from other companies on this matter. Huawei to Intel: LBT applicability needs further clarifications (i.e. licensed vs. unlicensed operation). WID includes LBT and no-LBT for the channel access to the unlicensed spectrum. Intel to QCOM/Huawei: At least ETSI EN 302 567 requires LBT as the mandatory mechanism for accessing shared spectrum. So we suggest to include LBT for at least unlicensed spectrum. For licensed spectrum, we can further discuss once regulatory is clearer |
| Table 1 – system parameters - SCS | QCOM: proposal to use 120KHZ SCS for 400 MHz and 960KHz SCS for 2000MHz |
| Table 1 _ BS - (Mg, Ng, M, N, P) | CATT: It could be ok for indoor ZTE: this value is different from TR 38.803, as mentioned , antenna aperture might be too small for 70GHz. QCOM to ZTE: This is aligning with the indoor deployments considered in Section A.2 TR 38.808, where two options for the antenna configuration |
| Table 1 – Antenna element radiation pattern - | Huawei: Please note, that last meeting, there was an update to the radiation array section in TR 38.808 – probably better to refer to TR 38.808 for that reason. We would like to check more on this topic. |
| Table 1 – BS – Max TX power | CATT: We would like to know why this is so small. We proposed 61.8 dBm following the TR 38.808 BS part when the antenna size is (1,1,16,16,2). For indoor, if the antenna size is (1,1,4,8,2), EIRP is 52.8dBm. QCOM to CATT: In Section A.2 in TR 38.808, 40 and optional 60 dBm are listed. We believe that 40 dBm is a reasonable starting point given the considered antenna configuration and elemental gain. ZTE: : 5dBm per antenna element could be starting point, however this might need further discussion due to higher frequency. QCOM to ZTE: can ZTE elaborate the need to consider per antenna element conducted power/EIRP requirement. Generally, EIRP is defined for all the antenna array. |
| Table 1 – UE - (Mg, Ng, M, N, P) | CATT: We followed TR 38.808, it’s (1,2,2,2,2) Optional: (1,2,4,4,2), but we don’t have strong opinion. |
| Table 1 – UE - | CATT: We don’t have strong opinion, but it seems smaller than what’s assumed in TR 38.808, here the antenna size is already larger. QCOM to CATT: As listed in section 2, 25 dBm assumption is too high for 60 GHz – somewhere in the 17-20 dBm range is realistic assumption. Also, based on commercial 802.11 ad/ay PA designs and realistic losses. ZTE: Similar as BS side, output power per antenna element is needed. |

# System coexistence sim parameters outdoor

Baseline for comment

Table 2. List of simulation assumptions for dense urban micro scenario [1].

| Deployment |  | Dense urban scenario outdoor A as specified in [1] |
| --- | --- | --- |
| System Parameters | Carrier Frequency | 60GHz |
|  | Channel BW | 400 and 2000 MHz |
|  | Number of active UEs | 1 |
|  | Channel model | Umi model as specified in [2] |
|  | LBT | LBT and No LBT |
|  | SCS | 120kHz and 960KHz |
| BS | (Mg, Ng, M, N, P) | (1,1,1,8,2) |
|  | (dv, dh) | (0.5 λ, 0.5 λ) |
|  | Antenna element gain | 5 dBi |
|  | Antenna element radiation pattern | Table 7.3-1 of [2] |
|  | Max Tx Power | 20 dBm (≈EIRP = 43 dBm) |
|  | Noise Figure | 13 dB |
| UE | (Mg, Ng, M, N, P) | (1,2,2,8,2) |
|  | (dv, dh) | (0.5 λ, 0.5 λ) |
|  | Antenna element gain | 5 dBi |
|  | Antenna element radiation pattern | Table A.2.1-8 in [3] |
|  | Max Tx Power | 20 dBm EIRP |
|  | Noise figure | 13 dB |
|  | LoS/ NLoS | LoS probability for indoor office defined in [4] |

Company comments on sim parameters

| Company | Scenario |
| --- | --- |
| QCOM | We agree with the all the parameters in the table. The critical difference for us from past work is the UE antenna array and the EIRP. At the shorter wavelengths the UE array can have more elements. Commercial PAs for 60 GHz are less efficient than FR2 and produce lower power. Further analysis is need to see if we need to simulate narrower bandwidths for the uplink, downlink, or both for the edge of coverage case |
| CATT | We can have a set of assumptions to be a starting point. For the CBW, if some company think 100 MHz is worst case. We can consider 100MHz and 2000MHz. |
| Intel | We suggest to include LBT scenario. For detail, please see our comments for indoor. |
| Huawei | Proposed WF: consider the above list of the coexistence simulation assumptions for outdoor scenario as the starting point for discussion. Aim for decision in May meeting. |
| Nokia | Results in R4-2104429 and R4-2104401, as expected, have shown that ACIR requirements can be relaxed with non-collocated BS and larger channel bandwidth, so there is no need to redo simulation as those in TR 38.803 represent more stringent cases. |

Additional company comments and discussion:

These comments were entered as “Comments” in word. Copied them here since those type of comments are useful however they are not very readable:

|  | Scenario |
| --- | --- |
| Table 2: System parameters - Frequency | Same comment as indoor |
| Table 2: System parameters - Channel BW | Same comment as indoor |
| Table 2 – system parameters - LBT | Same comment as indoor |
| Table 2 – system parameters - SCS | Same comment as indoor |
| Table 2 _ BS - (Mg, Ng, M, N, P) | ZTE: this value is different from TR 38.803, as mentioned , antenna aperture might be too small for 70GHz. ZTE: (1,1,16,16,2) QCOM to ZTE: What is the motivation behind using 16x16 and not 16x8 as listed in Section A.2 TR 38.808? We would like to propose either the original proposed configuration or the outdoor macro considered scenario in 38.808 (1,1,8,16,2) Probably ZTE : Referring to TR38.808, arrays up to 32x32 were listed as examples. We would like to have more time to analyze the array size selection for BS. |
| Table 2 – BS – Max TX power | ZTE: Similar comments to indoor QCOM to ZTE: Similar reply to the indoor part, in Section A.2 in TR 38.808, 40 and optional 60 dBm are listed. We believe that 40 dBm is a reasonable starting point given the considered antenna configuration and elemental gain Huawei to ZTE/QCOM: using the same BS output power for indoor and outdoor looks strange. We need more discussion on this. |
| Table 2 – UE - (Mg, Ng, M, N, P) | Same comment as indoor |
| Table 2 – UE – Noise figure | Unknown company: To be verified. The values in TR 38.803 were 13 and 15 dB for UE and BS, respectively. |

# Conclusions and WF

WF Rationale for 60 GHz band coexistence simulation: Companies are split on the need for sim. Companies are welcome to further discuss whether coexistence simulations are needed in the next meeting.

WF on indoor coexistence simulation parameters. Although the need for simulation is not decided, interested companies are encouraged to work to define parameters in preparation for potential sims.

Indoor parameters WF

|  |  | Proposed WF |
| --- | --- | --- |
| Deployment |  | Indoor Office A agreed ; C is FFS |
| System Parameters | Carrier Frequency | 60 GHz or 70 GHz |
|  | Channel BW | 100, [400], and 2000 MHz |
|  | Number of active UEs | 1 |
|  | Channel model | InH open office specified in [2] |
|  | LBT | FFS. Do we do LBT only, no-LBT only, or both? |
|  | SCS | 100 MHz and [400] MHz 120kHz SCS 200 MHz 960kHz SCS |
| BS | (Mg, Ng, M, N, P) | FFS |
|  | (dv, dh) | (0.5 λ, 0.5 λ) |
|  | Antenna element gain | 5 dBi |
|  | Antenna element radiation pattern | FFS |
|  | Max Tx Power | FFS value and FFS whether to specify as total EIRP or per element |
|  | Noise Figure | FFS |
| UE | (Mg, Ng, M, N, P) | [(1,2,2,8,2) or (1,2,2,4,1)] |
|  | (dv, dh) | (0.5 λ, 0.5 λ) |
|  | Antenna element gain | 5 dBi |
|  | Antenna element radiation pattern | Table A.2.1-8 in [3] |
|  | Max Tx Power | FFS value and FFS whether to specify as total EIRP or per element |
|  | Noise figure | FFS |
|  | LoS/ NLoS | LoS probability for indoor office defined in [4] |

WF on outdoor coexistence simulation parameters:

|  |  | Proposed WF |
| --- | --- | --- |
| Deployment |  | Dense urban scenario outdoor A as specified in [1] |
| System Parameters | Carrier Frequency | 60 GHz or 70 GHz |
|  | Channel BW | 100, [400], and 2000 MHz |
|  | Number of active UEs | 1 |
|  | Channel model | InH open office specified in [2] |
|  | LBT | FFS. Do we do LBT only, no-LBT only, or both? |
|  | SCS | 100 MHz and [400] MHz 120kHz SCS 200 MHz 960kHz SCS |
| BS | (Mg, Ng, M, N, P) | FFS |
|  | (dv, dh) | (0.5 λ, 0.5 λ) |
|  | Antenna element gain | 5 dBi |
|  | Antenna element radiation pattern | FFS |
|  | Max Tx Power | FFS value and FFS whether to specify as total EIRP or per element |
|  | Noise Figure | FFS |
| UE | (Mg, Ng, M, N, P) | (1,2,2,8,2) or (1,2,2,4,1) |
|  | (dv, dh) | (0.5 λ, 0.5 λ) |
|  | Antenna element gain | 5 dBi |
|  | Antenna element radiation pattern | Table A.2.1-8 in [3] |
|  | Max Tx Power | FFS value and FFS whether to specify as total EIRP or per element |
|  | Noise figure | FFS |
|  | LoS/ NLoS | LoS probability for indoor office defined in [4] |

# Reference

- 3GPP TR 38.808: "Study on supporting NR from 52.6 GHz to 71 GHz".

- 3GPP TR 38.901: "Study on channel model for frequencies from 0.5 to 100 GHz".

- 3GPP TR 38.802: "Study on New Radio Access Technology Physical Layer Aspects".

- 3GPP TR 38.803: "Study on new radio access technology: Radio Frequency (RF) and co-existence aspects ".