3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2106035

# Electronic Meeting, Apr. 12-20, 2021

Title: 	Updated simulation assumptions for NR V2X multiple link test cases

Source: 	Huawei, HiSilicon

Agenda item:	5.2.1.3

Document for:	Discussion

Introduction

In this contribution, we provide the updated simulation assumptions for NR V2X demodulation requirements as per the latest agreements. Companies are encouraged to submit the simulation results for next meetings.

Simulation assumptions

  - Resource pool configuration

- The resource pool configuration specified in Table 2.1-1 is applicable for all V2X multiple link test cases listed in sub-clause 2.2~2.5

Table 2.1-1 Common resource pool configuration

| Resource pool configuration | PSCCH Time resource | Symbols | 2 |
| --- | --- | --- | --- |
|  | PSCCH Frequency resource | PRBs | 10 |
|  | PSFCH number of cyclic shift pairs |  | n1 |
|  | PSFCH hopping ID |  | 0 |
|  | PSFCH candidate resource type |  | allocSubCH |
|  | Set of PRBs for PSFCH transmission |  | ones(1,100) for 40 MHz and ones(1,50) for 20 MHz |
|  | PSSCH RSRP threshold |  | 66 (infinity dBm) |
|  | Synchronization reference |  | GNSS |
|  | Subchannel size | PRBs | 10 |
|  | Number of sub-channels |  | 5 for 20 MHz and 10 for 40 MHz |
|  | Start PRB for first sub-channel |  | 0 |
|  | Time resource bitmap |  | ones(1, 160) |
| Note 1: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-1 [6] for tested channel bandwidth and subcarrier spacing. |  |  |  |

Power imbalance test

- Test purpose: The purpose of this test is to check the demodulation performance when receiving PSSCH transmissions from two sidelink UEs with power imbalance in one slot.

- Methodology for power imbalance test:

  - The target requirement ICS = -27 dBc

  - Select SINR2 from simulation results for SNR@10% BLER point based on simulation assumptions in Table 2.1-2.

  - Select SNR2 such that SNR2 = SINR2+5dB

  - Compute SNR1 = 30.35dB from the relation: SINR2 = SNR2 – 10*log10(10^((SNR1 + ICS)/10)+1).

- The test setup is shown as Table 2.1-1:

Table 2.1-1: Test assumptions

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Active cell(s) |  |  | None |
| Active Sidelink UE(s) |  |  | Sidelink UE 1, Sidelink UE 2 |
| Sidelink UE 1 | Sidelink Transmissions |  | PSCCH + PSSCH |
|  | PSSCH DMRS pattern(Note 1) |  | {2,3} |
|  | RB allocation |  | PRB (0~9) |
|  | Time offset (Note 2) | s | 0 |
|  | Frequency offset (Note 3) | Hz | 0 |
|  | Antenna configuration |  | 1x2 Low |
|  | PSFCH periodicity | Slots | 4 |
|  | MinTimeGapPSFCH | Slots | 3 |
| Sidelink UE 2 | Sidelink Transmissions |  | PSCCH + PSSCH |
|  | PSSCH DMRS pattern(Note 1) |  | {2,3} |
|  | RB allocation |  | PRB (30~39) |
|  | Time offset (Note 2) | s | 0 |
|  | Frequency offset (Note 3) | Hz | 0 |
|  | Antenna configuration |  | 1x2 Low |
|  | PSFCH periodicity | Slots | 4 |
|  | MinTimeGapPSFCH | Slots | 3 |
| Note 1: {x, y}: x and y means the number of DMRS symbols for slot with PSFCH transmission and without PSFCH transmission, respectively. Note 2: Time offset of received signal by Sidelink UE with respect to GNSS reference timing. Note 3: Frequency offset of Sidelink UE with respect to GNSS reference frequency. |  |  |  |

- Companies are encouraged to provide simulation results for SINR2 for Sidelink UE 2 based on Table 2.1-2:

Table 2.1-2: Simulation assumptions and FRC for derivation of SINR2

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Channel bandwidth |  | MHz | 20 |
| Subcarrier spacing |  | kHz | 30 |
| Allocated resource blocks |  | RB | 10 |
| CP-OFDM symbols for slot with PSFCH(Note 1) |  |  | 9 |
| CP-OFDM symbols for slot without PSFCH |  |  | 12 |
| Modulation order |  |  | QPSK |
| MCS index |  |  | 4 |
| Number of MIMO layers |  |  | 1 |
| Number of DMRS REs |  |  | 15 |
| Number of REs for SCI format 1-A |  |  | 240 |
| 2nd stage SCI format 2-A configuration | Payloads | Bits | 35 |
|  | α |  | 1 |
|  | βoffset |  | 3.5 |
| Overhead for TBS determination |  |  | 0 |
| Transport Block Size for slots with PSFCH |  | Bits | 208 |
| Transport Block Size for slots without PSFCH |  | Bits | 432 |
| Transport block CRC |  | Bits | 24 |
| Maximum number of HARQ transmissions |  |  | 1 |
| Binary Channel Bits for slots with PSFCH |  |  | 744 |
| Binary Channel Bits for slots without PSFCH |  | Bits | 1464 |
| Note 1: OFDM symbols is for PSCCH/PSSCH transmission not including first symbol (AGC), PSFCH symbols, and guard symbols |  |  |  |

Soft buffer combing test

- Test purpose: This test is to verify soft buffer combining capability of UE and maximum number of receiving HARQ processes supported by UE per TTI

- Test design:

  - For verification of harq-RxProcessSidelink = n16 or n24, n16 or n24 UEs transmit signal one by one circularly for every slot and in the first subchannel.

  - For verification of harq-RxProcessSidelink = n32, the first 31 UEs transmit signal one by one circularly for every slot and in the first subchannel. The 32nd UE transmit signal in the same slot as the first UE but in the second subchannel.

  - For verification of harq-RxProcessSidelink = n48, the first 31 UEs transmit signal one by one circularly for every slot and in the first subchannel. The next 17 UEs transmit signal one by one circularly for every slot and in the second subchannel”

  - For verification of harq-RxProcessSidelink = n64, first 31 UEs transmit signal one by one circularly for every slot and in the first subchannel, the next 31 UEs transmit signal one by one circularly for every slot and in the second subchannel, the last 2 UEs transmit signal in the same slot as the first two UEs but in the third subchannel.

- According to the agreement of RAN4#98-e meeting, RAN4 will only define one set of performance requirements based on the results of harq-RxProcessSidelink = n64. Interested companies are encouraged to provide simulation results in next meeting based on the simulation assumptions given in Table 2.2-1 and Table 2.2-2.

Table 2.2-1： Simulation assumptions for soft buffer test

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Active cell(s) |  |  | None |
| Bandwidth |  | MHz | 20 |
| SCS |  | kHz | 30 |
| Subchannel size |  | RBs | 10 |
| Number of symbols for PSCCH |  | Symbol | 2 |
| Number of RBs for PSCCH |  | RBs | 10 |
| Active Sidelink UE(s) |  |  | 64 |
| Sidelink UE i, 0 ≤ i ≤ 63 | Sidelink Transmissions |  | First 31 UEs transmit PSCCH + PSSCH one by one circularly for every slot and in the first subchannel, the next 31 UEs transmit PSCCH + PSSCH one by one circularly for every slot and in the second subchannel, the last 2 UEs transmit PSCCH + PSSCH in the same slot as the first two UEs but in the third subchannel. |
|  | Time gap between initial transmission and retransmission | Slots | 31 |
|  | Timing offset (Note 1) | s | 0 |
|  | Frequency offset (Note 2) | Hz | 0 |
|  | Synchronization source |  | GNSS or GNSS-equivalent |
|  | Antenna configuration |  | 1x2 Low |
| Timing offset from receiving side (Note 3) |  | s | 0 |
| Frequency offset from receiving side (Note 4) |  | Hz | 0 |
| Propagation condition |  |  | AWGN |
| Test metric |  |  | SNR@5% BLER of PSSCH |
| Note 1: Time offset of Sidelink UE receive signal with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. Note 3: Timing offset from receiving side is with respect to GNSS reference timing. Note 4: Frequency offset from receiving side is with respect to GNSS reference frequency. |  |  |  |

Table 2.2-2： FRC for PSSCH

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Reference channel |  |  | R.PSSCH.2-1.x |
| Channel bandwidth |  | MHz | 20 |
| Subcarrier spacing |  | kHz | 30 |
| Allocated resource blocks |  | RB | 10 |
| CP-OFDM symbols for slot with PSFCH(Note 1) |  |  | 9 |
| CP-OFDM symbols for slot without PSFCH |  |  | 12 |
| Modulation order |  |  | 64QAM |
| MCS index |  |  | 27 |
| Number of MIMO layers |  |  | 1 |
| Number of DMRS REs |  |  | 12 |
| Number of REs for SCI format 1-A |  |  | 240 |
| 2nd stage SCI format 2-A configuration | payloads |  | 35 |
|  | α |  | 1 |
|  | βoffset |  | 2.5 |
| Overhead for TBS determination |  |  | 0 |
| Transport Block Size for slot with PSFCH |  | Bits | 3496 |
| Transport block CRC |  | Bits | 16 |
| Maximum number of HARQ transmissions |  |  | 2 |
| Binary Channel Bits for slots with PSFCH |  |  | 3816 |

PSCCH decoding capability test

The purpose of this test is to verify the maximum number of Sidelink processes supported by the V2X UE.

- Option 1: Use 40MHz bandwidth and AT command to calculate BLER based on test setup specified in Table 2.3-1.

Table 2.3-1 Simulation assumptions for PSCCHdecoding capability test

| Parameter |  | Unit | value |
| --- | --- | --- | --- |
| Bandwidth/SCS |  | MHz/kHz | 40/30 |
| Test metric |  |  | Prob(PSCCH miss detected)=1% |
| Sidelink UE i, 0 ≤ i ≤ 9 (Note 4) | Sidelink Transmissions |  | PSCCH + PSSCH |
|  | Timing offset (Note 1) | s | 0 |
|  | Frequency offset (Note 2) | Hz | 0 |
|  | Synchronization source |  | GNSS |
|  | Propagation Channel |  | Static propagation condition without external noise |
|  | Antenna configuration |  | 1x2 Low |
|  | PSFCH periodicity | Slots | 0 |
|  | PSSCH RMC |  | Same as PSSCH test with TDLA-30-2700 |
|  | PSCCH RMC (Note 3) |  | Same as single-link PSCCH test |
| Note 1: Time offset of received signal by Sidelink UE with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. Note 3: OCC index for PSCCH DMRS is randomly selected between {0, 1, 2} for each PSCCH transmission as per in Clause 8.4.1.3.2 of TS 38.211[9]. Note 4: Each UE occupies one sub-channel so that all sub-channels are filled. Note 6: The mapping procedure of PSSCH resource and PSFCH resource is specified in Clause 16.3 of TS 38.213 [11]. |  |  |  |

- Option2: Use following methodology based on test setup specified Table 2.3-1 with the modification for PSFCH periodicity from 0 to 1.

  - TE sets PSSCH priority (in PSCCH) when PSFCH Tx capability < 10, x PSSCH with higher priority, 10-x PSSCH with lower priority. Note that PSFCH is selected according to PSSCH priority, hence if PSFCH Tx capability = x, the x feedback corresponding to x high priority PSSCH is transmitted to TE.

  - TE can verify UE PSCCH decoding success or failure by checking whether all the higher priority PSSCH feedback is received. In order to always feedback all the high priority SCHs, UE has to decode all CCH to know the priority.

  - To avoid UE cheating, TE can randomize the location of higher priority PSSCHs

PSFCH decoding capability test

- The purpose of this test is to verify the maximum number of PSFCHs received by UE per slot in group cast scenario by using ACK/NACK feedback mode. In each slot, the tested UE transmits one PSSCH that occupies all the sub channels to a group of receiving UEs. The simulation assumptions are shown in Table 2.5-1.

- Test design:

- Option 1:

  - TE randomly transmit ACK or DTX on each PSFCH resource with equal probability

  - AT command adopted for this test case based on current available solution.

- Option 2: In every slot, TE transmits one of the two following options (1) all ACK (2) one NACK and all the rests are ACK

  - UE decodes all the PSFCH to decide PSSCH ReTx. For (1) no reTx; for (2) ReTx

  - Test metric: TE can verify whether UE successfully detect all the PSFCH by reTx received or not. If UE reTx behavior is correct, this slot is a “successful slot”. The requirement can be defined by “successful slot” exceeding 90%/99% or higher.

Table 2.5-1 Simulation assumptions for PSFCH decoding capability test

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth/SCS |  | MHz/kHz | 40/30 |
| Test metric |  |  | Prob (ACK miss)<=1% Prob (DTX->ACK)<=1% |
| HARQ-ACK information |  |  | ACK or NACK |
| Source ID of tested UE |  |  | 0 |
| Sidelink UE i, 0 ≤ i ≤ N-1(Note 3) | Sidelink transmissions for |  | PSFCH |
|  | Timing offset (Note 1) | s | 0 |
|  | Frequency offset (Note 2) | Hz | 0 |
|  | Synchronization source |  | GNSS or GNSS-equivalent |
|  | Propagation Channel |  | Static propagation condition No external noise sources are applied |
|  | Antenna configuration |  | 1x2 Low |
|  | Member ID(Note 4) |  | i |
|  | PSFCH resource allocation(Note 5) |  | N UEs transmit PSFCHs one by one on each RB with CS pair index 0. i.e. UE 0 transmits PSFCH on RB 0, UE 1 transmits PSFCH on RB 1,…, UE (N-1) transmits PSFCH on RB N-1 |
|  | PSFCH periodicity | Slots | 1 |
| Note 1: Time offset of received signal by Sidelink UE with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. Note 3: N equals to the number of PSFCH(s) resources that UE can receive in a slot as specified in Clause 4.2.16.1.6 of TS 38.306[14]( IE psfch-RxNumber)) . Note 4: Member ID is an identifier uniquely identifying a member Note 5: All PSFCHs in a slot are corresponding to one PSSCH that occupies all sub channels. |  |  |  |