3GPP TSG-RAN WG4 Meeting # 98-bis-e 												R4-2106146

Electronic Meeting, 12th – 20th April, 2021

Agenda item:			8.7.2, 8.7.5

Source:	Moderator (Samsung)

Title:	Email discussion summary for [98-bis-e][322] NR_HST_FR2_Scenarios_Demod

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

In RAN Plenary #89-e, the RAN4-led work item of NR support for high speed train (HST) scenario in FR2 has been approved [RP-202118] (which has been further revised to [RP-210800] with editorial revisions and updates on time schedule).

Based on the agreement captured in WF [R4-2103240], companies are encouraged to further study the FR2 HST deployment scenarios and the feasibility study of supported maximum speed from demodulation perspective. Furthermore, from this meeting, the performance part on demodulation will be started based on the WID TU planning and work item planning.

In this email thread, the following agenda items will be discussed:

- 8.7.2	High speed train deployment scenario in FR2

- 8.7.5	Demodulation requirements

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: TBA

- 2nd round: TBA

It is suggested to have the following target of 1st and 2nd round email discussion:

- 1st round: Further discussion on the deployment scenarios and demodulation related issues and requirements.

- 2nd round: Based on results from 1st round, achieve agreements as much as possible for deployment scenarios and demodulation related issues and requirements, as the basis for future discussion.

# Topic #1: Analysis on FR2 HST Deployment Scenarios

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2106825 | Huawei, HiSilicon | Observation 1: The minimum beam dwelling time can be 160 ms under HST FR2 scenario. Proposal 1: Assume 1 CPE per train as baseline for HST FR2 scenario. Proposal 2: Do not introduce any signaling for Bi-directional deployment for HST FR2 in Rel-17. |
| R4-2104905 | Qualcomm, Inc. | Proposal 1: Set UE antenna parameters the same as RRH except N=8, M=4 in simulation assumptions. Proposal 2: For the uni-directional model, the RRH boresight in azimuthal angle points to 780m from the projection of the RRH on the track. Proposal 3: For the uni-directional model, use one RRH beam for scenario 1 (Dmin = 10m). Proposal 4: For the uni-directional model, add an additional panel to the second and second last RRHs in a BBU to cover the HO region when Dmin is small. Define a network signaling to inform UE the presence of the additional panels. Proposal 5: For the bi-directional model, the RRH boresight in azimuthal angle points to 780m from the projection of the RRH on the track. Observation 1: Comparison of uni-directional and bi-directional models are listed in the following table: Uni-directional; Bi-directional. Boresight direction; Ds+Dadd; (1) Ds+Dadd (2) Ds/2 Our analysis suggests (1) to ensure coverage. Beam management; Each RRH covers [Dadd,Ds+Dadd] region, one region; Each RRH covers (1) [-Ds/2,-Dadd], [Dadd,Ds/2] and [Ds-Dadd, Ds+Dadd] three noncontiguous regions (2) [-Ds, -Ds+Dadd], [-Ds/2,-Dadd], [Dadd,Ds/2] and [Ds-Dadd, Ds] four noncontiguous regions. Throughput; About 10% worse than bi-directional model; About 10% gain over uni-directional channel. Handover; One additional panel is needed for some RRHs; No additional panel is needed.. Doppler spread (Dp = fc*v/speed of light); 0 if single path, <Dp if mult-path is considered; 2*Dp if signal is received from two RRHs closest to UE on opposite side. |
| R4-2104679 | Ericsson | Observation 1: In scenario 1, DPS with a single TX beam and a single RX beam is sufficient to achieve coverage. Observation 2: There is no need or benefit from JT in scenario 1. Observation 3: There is no benefit from bi-directional deployment in scenario 1. Observation 4: For 350km/h scenarios, track curvature is not sharp and coverage can be provided with 1 TX/RX beam also for curves with 700m BS separation. Observation 5: For 120km/h scenario, curves may in some cases be sharper. Coverage with 1TX/RX beam for the most extreme curves can be provided if BS are spaced around 400m around the curve. (Alternatively, multiple beams could be used). |
| R4-2104924 | ZTE Corporation | Observation 1: For bi-directional situation, CPE need to tackle the doppler shift hopping from minus to plus or plus to minus. Proposal 1: The ability of multi-beam Rx or Tx at a time can be considered for CPE under uni-directional situation of scenario-A. Proposal 2: Multiple panels per CPE can be considered for bi-directional situation of scenario-A. |
| R4-2105023 | Samsung | Observation 1: For uni-directional RRH deployment in Scenario-A, even with single narrow analog beam per RRH panel (i.e., [Mg, Ng, M, N, P]=[1, 1, 8, 16, 2]), there is still 30dB margin compared against PC4 REFSENS requirement. Observation 2: For uni-directional RRH deployment in Scenario-A, with one additional beam used per RRH panel, the cellular coverage for the beam’s intended region can be improved, even with one fixed RX beam used in UE side. Observation 3: For uni-directional RRH deployment for Scenario-A, the beam dwelling time can be in the range of [0.45, 0.96] seconds for UE maximum speed of 350kmph. Observation 4: To solve the issue of coverage hole for bi-directional deployment, there are two possible schemes: - Scheme-1: Connecting to 2nd-Nearest RRH; - Scheme-2: Connecting to Nearest RRH except Coverage Hole. Observation 5: If Scheme-1 for bi-directional deployment is adopted for Scenario-A, there is no benefit compared with uni-directional counterpart. Observation 6: Scheme-2 for bi-directional deployment can be used for solve the coverage-hole issue, at the expense of 3 TX beam switching within each Ds. Observation 7: For bi-directional RRH deployment for Scenario-A, the beam dwelling time can be in the range of [0.80, 1.99] seconds for UE maximum speed of 350kmph. |
| R4-2106503 | Intel Corporation | Observation 1: For Scenario A single fixed TX beam per RRH panel is enough for sufficient link budget Observation 2: Multiple TX beams at the RRH should be considered for Scenario B Observation 3: Either Network or CPE (or both) should support bidirectional operation to ensure service for trains moving in different directions. Observation 4: Single panel at the CPE covering both directions leads to non-optimal antenna gain exploitation on the most part of the distance. Proposal 1: RAN4 to consider CPE to be equipped with two panels pointed in opposite directions Observation 5: MU operation requires bidirectional deployment and DPS transmission Observation 6: No impact of MU operation on the RAN4 requirements identified. Proposal 2: RAN4 requirement can be defined based on the baseline of 1 CPE device per train Proposal 3: RAN4 to focus only on DPS transmission mode for FR2 HST. |
| R4-2106693 | Nokia, Nokia Shanghai Bell | On network deployment in Scenario-A: Observation 1: In Scenario A, the network covers mainly the area very close to the railway track. Proposal 1: RAN4 to use only one beam (i.e., one TCI state) per RRH in HST FR2 deployment Scenario A. Observation 2: For both bi- and uni-directional deployments, the discussed number of RRH sites per BBU is 4, i.e., only multi-RRH deployments are discussed. However, relatively large inter-RRH distance (Ds) makes a regular deployment with one RRH site per cell feasible. The mobility in this case is provided with the HO procedure. Proposal 2: RAN4 to consider also regular (non-SFN/non-DPS) deployment with 1 RRH site per BBU. Observation 3: In uni-directional deployments, it can also be sufficient to deploy only one RRH per RRH sight if CPE is capable of communicating with the network from opposite directions. Proposal 3: RAN4 to modify NOTE2 on the number of RRHs per sight in uni-directional deployment as follows: RAN4 focuses on 1 direction 1 train, but we are aware of the fact that either another panel to serve train towards the other direction is needed or a CPE is capable of Rx and Tx from the opposite direction. If this opposite direction is completely symmetric, the 1 direction study can apply directly. Observation 4: In regular (non-SFN/non-DPS) deployment, the beams' change happens together with the change of the RRH through the L3 HO procedure, which includes the synchronization to a target cell. Thus, the problem with different propagation delays when the CPE is switching serving RRH does not exist. However, the implications of different propagation delays can be experienced in Full SFN and DPS settings when the beams belong to the same cell, come from the same direction but from the different RRH sites. Such a situation can be observed both in uni- and bi-directional settings. Proposal 4: RAN4 to elaborate further on which deployments and propagation schemes are exposed to the very different propagation delays. Then, quantitively evaluate the implications in these scenarios both from the demodulation and RRM perspectives. Observation 5: The connection quality next to the RRH site in bi-directional deployment of Scenario A can be potentially improved by the signals coming from the neighboring RRHs. If it is the case, the Full-SFN scheme (i.e., PDSCH combining) can provide benefits. Proposal 5: RAN4 to discuss further if the connection quality in the area next to the RRH site in bi-directional deployment of Scenario A can be improved by switching to the reception from the neighboring RRH sites, e.g., using SFN or DPS schemes. On CPE configurations: Proposal 6: RAN4 to assume that in HST FR2 Scenario A, only high-speed CPEs installed on the roof of the train can be present in the network. Observation 6: We are not expecting any abnormal impacts on the system capacity or the inter-CPE interference when multiple CPEs per train are used. RAN4 requirements are formulated only for a single CPE/UE. Proposal 7: RAN4 to define requirements based on the assumption of 1 CPE per train. Observation 7: The utilization of only one panel pointing to the upside is less efficient in HST FR2 Scenario A in comparison to two panels per CPE oriented in opposite directions. However, we have not observed any mobility problem in this setting either, even though in our analysis, only one beam cooriented with the panel boresight was used. Proposal 8: RAN4 to decide if further analysis is needed regarding one panel per CPE pointing to upside and having analog beams directed to forward and backward in HST FR2 Scenario A. Observation 8: Uni- and bi-directional deployments can be mixed even in the same network. Proposal 9: If found to be needed, RAN4 to continue the discussion of issues related to the deployment type and UE capabilities signaling in the RRM track. |
| R4-2106826 | Huawei, HiSilicon | Observation 1: There is no any coverage issue under HST FR2 scenario assuming PC4 for both uplink and downlink. Proposal 1: Not consider Bi-directional deployment for Scenario A (700m/10m). Proposal 2: Use boresight parallel to the railway for Uni-directional deployment for Scenario A. |
| R4-2104680 | Ericsson | Observation 1: Scenario 2 can be adequately covered with 1 BS and 1 UE beam Observation 2: The coverage can be improved slightly using 2-3 BS beams and 1-2 UE beams Observation 3: Bi-directional deployment is inferior to uni-directional deployment for scenario 2. Observation 4: JT is not useful for scenario 2 Observation 5: For 350km/h scenarios, track curvature is not sharp and coverage can be provided with 1-3 TX/RX beam also for curves with 700m BS separation. Observation 6: For the 120km/h and low speed scenarios, closer spacing of the BS does not assist in the curve scenario and more beams may be needed. Proposal 1: RAN4 should confirm whether the 120 km/h curve scenario is important to investigate further for FR2 HST. |
| R4-2104926 | ZTE Corporation | Observation 1: If wider beam is considered for uni-directional situation of scenario-B, the number of TCI can be very small, e.g. 4. Observation 2: If wider beam is considered for bi-directional situation of scenario-B, the number of TCI can be very small, e.g. 4. Proposal 1: The ability of multi-beam Rx or Tx at a time should be considered for CPE under uni-directional situation of scenario-B. Proposal 2：To consider 4 TCIs for scenario-B of HST FR2. Proposal 3: Multiple panels per CPE should be considered for bi-directional situation of scenario-B. |
| R4-2105024 | Samsung | Observation 1: For uni-directional RRH deployment in Scenario-B, with single beam per RRH utilized, there is around 20dB margin compared against PC4 REFSENS requirement, which is 10dB lower than Scenario-A. Observation 2: For uni-directional RRH deployment in Scenario-B, with 2 beams used per RRH panel, the cellular coverage over the region near to RRH site can be significantly enhanced, even for one fixed RX beam used in UE side. Observation 3: For uni-directional RRH deployment for Scenario-B, the smallest beam dwelling time can be in the range of [1.61, 2.29] seconds for UE maximum speed of 350kmph. Observation 4: For Scenario-B, if scheme-1 used for bi-directional RRH deployment, around 20-25dB margin above PC4 REFSENS for [Mg, Ng, M, N, P]=[1, 1, 4, 8, 2] and [1, 1, 8, 8, 2], but significant performance loss is observed for narrower beam option, i.e., [Mg, Ng, M, N, P]=[1, 1, 8, 16, 2]. Observation 5: For Scenario-B, if scheme-1 bi-directional RRH deployment is used，  For RRH panel configuration [Mg, Ng, M, N, P]=[1, 1, 8, 16, 2], only single beam per RRH panel can’t provide satisfactory coverage. Observation 6: For Scenario-B, if scheme-1 used for bi-directional RRH deployment, two beams per RRH panel can provide satisfactory coverage. Observation 7: For bi-directional RRH deployment in Scenario-B, the scheme-2 (connecting to nearest RRH except coverage hole) is not recommended to be used. Observation 8: For bi-directional RRH deployment for Scenario-B, the beam dwelling time can be in the range of [1.68, 1.92] seconds for UE maximum speed of 350kmph. |
| R4-2106694 | Nokia, Nokia Shanghai Bell | Proposal 1: RAN4 to clarify based on the operators’ input if regular (i.e., low-speed non-HST) UEs can be connected to the same cell together with a HST CPE moving at maximum speed. Observation 1: In LoS conditions, without interference, the coverage area (over the railway track) of one RRH with one beam per panel is more than several Ds. Hence, even one beam per RRH can provide sufficient coverage. Observation 2: The usage of the beams pointed more perpendicular to the railway track is very limited. Out of a maximum of four beams per RRH, only two are reasonably used based on our simulation results. Even though one beam can provide sufficient coverage, we do not see a need to limit the number of beams per RRH only to one since the deployment with two beams is more general. Proposal 2: RAN4 to use 1 or 2 beams per RRH panel in uni-directional deployments for Scenario B. Proposal 3: RAN4 to use only 1 beam (TCI state) per RRH panel in uni-directional deployment with Full SFN transmission scheme for Scenario B. Proposal 4: RAN 4 not to use PDSCH combining in HST FR2 bi-directional deployment, Scenario B. Proposal 5: RAN4 to decide if more than two beams per RRH are beneficial in bi-directional deployment, scenario B. Observation 3: The utilization of only one panel pointing to upside is less efficient in HST FR2 Scenario B than two panels per CPE oriented into opposite directions. However, we have not observed any mobility problem in this setting either, even though in our analysis, only one beam cooriented with the panel boresight was used. Proposal 6: RAN4 to decide if further analysis is needed regarding one panel per CPE pointing to upside and having analog beams directed to forward and backward in HST FR2 Scenario B. |
| R4-2106827 | Huawei, HiSilicon | Observation 1: There is no any coverage issue under HST FR2 Scenario B assuming PC4 for both uplink and downlink. Proposal 1: Use 2 beams for Bi-directional deployment for Scenario B (700m/150m). Proposal 2: Use one beam for Uni-directional deployment for Scenario B. |
| R4-2104677 | Ericsson | Observation 1: The capacity for FR2 HST can be doubled by operating uni-directional connections in both directions. Observation 2: Attempting to operate with more than one UE / serving BS per direction may lead to significant inter-cell interference, removing most of the capacity benefit. Observation 3: The capacity limit for FR2 HST appears to be around 1Gbps / 100MHz. Proposal 1: Consider 1 UE panel per direction when setting requirements |
| R4-2104925 | ZTE Corporation | Observation 1: If (pre-)compensation of Doppler shift is considered at CPE or network side the max supported speed can be increased under the same RS density and SCS configuration. Observation 2: If 2 or more CPE per train are introduced, it needs to be clarified whether CPEs work jointly or independently. Proposal 1: To consider supporting handheld UE for HST_FR2 with lower priority. Proposal 2: If complexity is the concern, one CPE per train can be prioritized for HST_FR2. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1 General Assumptions

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 1-1-1: UE antenna element parameters

- Proposals

  - Proposal 1 (Qualcomm): Set UE antenna parameters the same as RRH except N=8, M=4 in simulation assumptions

  - Proposal 2 (Samsung): [Mg, Ng, M, N, P]=[1, 1, 4, 4, 2], 5dBi per element antenna gain

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 1-1-2: UE antenna panel(s) for forward and backward directions

- Proposals

  - Proposal 1 (Intel): RAN4 to consider CPE to be equipped with two panels pointed in opposite directions

  - Proposal 2 (Nokia): RAN4 to decide if further analysis is needed regarding one panel per CPE pointing to upside and having analog beams directed to forward and backward in HST FR2 Scenario A.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 1-1-3: Number of CPE devices per train/carriage

- Proposals

  - Proposal 1 (Huawei, ZTE, Nokia, Intel, Ericsson): RAN4 requirement can be defined based on the baseline of 1 CPE device per train

  - Observation 1 (Ericsson): Attempting to operate with more than one UE / serving BS per direction may lead to significant inter-cell interference, removing most of the capacity benefit.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 1-1-4: Necessity of JT in Scenario-A/B, Uni/Bi-directional RRH

- Proposals

  - Proposal 1 (Intel, Ericsson): RAN4 to focus only on DPS transmission mode for FR2 HST, don’t consider JT.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

### Sub-topic 1-2 Scenario-A, Uni-directional RRH

Issue 1-2-1: Number of Beam for uni-directional RRH deployment, Scenario-A

- For scenario-A, uni-directional, RRH parameter:

  - Proposal 1 (Qualcomm, Samsung, Intel, Ericsson, Nokia): 1 beam per RRH panel

- For scenario-A, uni-directional, UE parameter:

  - Proposal 1 (Ericsson, Samsung, Intel): 1 beam per UE panel (i.e., 1 beam per UE)

  - Proposal 2 (QC): 1 beam per panel (two panels in opposite direction)

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 1-2-2: Uni-directional operation

- Proposals

  - Proposal 1 (Nokia): RAN4 to modify NOTE2 on the number of RRHs per sight in uni-directional deployment as follows:

    - RAN4 focuses on 1 direction 1 train, but we are aware of the fact that either another panel to serve train towards the other direction is needed or a CPE is capable of Rx and Tx from the opposite direction. If this opposite direction is completely symmetric, the 1 direction study can apply directly.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 1-2-3: RRH boresight direction for uni-directional RRH deployment

- Proposals

  - Proposal 1 (Qualcomm): For the uni-directional model, the RRH boresight in azimuthal angle points to 780m from the projection of the RRH on the track.

  - Proposal 2 (Huawei): Use boresight parallel to the railway for Uni-directional deployment for Scenario A.

  - Proposal 3 (Samsung): RRH panel boresight pointed to the railway at the distance of Ds (projection of the neighboring RRH on the railway). Accordingly, for Scenario-A: Azimuth angle: 0.8 degree; Down-titling: 1.2 degree

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 1-2-4: Beam switching point

- Proposals

  - Proposal 1 (Samsung): Ds_offset (illustrated in below figure) in the range of [40-81]m for Scenario-A uni-directional RRH deployment.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 1-2-5: Handover

- Proposal (QC): For the uni-directional model, add an additional panel to the second and second last RRHs in a BBU to cover the HO region when Dmin is small. Define a network signaling to inform UE the presence of the additional panels.

T0

CID =1

CID =2

CID =2

HO

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

### Sub-topic 1-3 Scenario-A, Bi-directional RRH

Issue 1-3-1: Schemes for Bi-directional deployment

- Proposals

  - Proposal 1 (Samsung): To solve the issue of coverage hole for bi-directional deployment, there are two possible schemes:

    - Scheme-1: Connecting to 2nd-Nearest RRH;

    - Scheme-2: Connecting to Nearest RRH except Coverage Hole.

  - Proposal 2 (Samsung): For Scenario-A, bi-directional RRH deployment:

    - If Scheme-1 for bi-directional deployment is adopted for Scenario-A, there is no benefit compared with uni-directional counterpart.

    - Scheme-2 for bi-directional deployment can be used for solve the coverage-hole issue, at the expense of 3 TX beam switching within each Ds.

  - Proposal 3 (Nokia): RAN4 to discuss further if the connection quality in the area next to the RRH site in bi-directional deployment of Scenario A can be improved by switching to the reception from the neighboring RRH sites, e.g., using SFN or DPS schemes.

  - Proposal 4 (Huawei): For Scenario-A, not consider bi-directional RRH deployment.

  - Proposal 5 (QC): Use scheme 2 to resolve coverage issue in bi-directional channel

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 1-3-2: Number of Beam for bi-directional RRH deployment, Scenario-A

- For scenario-A, bi-directional, RRH parameter:

  - Proposal 1 (Nokia, Intel, Ericsson, QC): 1 beam per RRH panel, two panels in opposite directions

  - Proposal 1a (Samsung): Depends on scheme-1 or 2

    - For scheme-1: 1 beam per RRH panel for Scheme-1

    - For scheme-2: one additional beam per RRH site needs to cover neighboring RRH site.

- For scenario-A, bi-directional, UE parameter:

  - Proposal 3 (Samsung, Ericsson): 1 beam per UE panel (i.e., 2 beam per UE)

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 1-3-3: RRH boresight direction for bi-directional RRH deployment

- Proposals

  - Proposal 1 (Qualcomm): For the bi-directional model, the RRH boresight in azimuthal angle points to 780m from the projection of the RRH on the track.

  - Proposal 2 (Samsung): Different for Scheme-1 and 2:

    - For Scheme-1: Option-2: RRH panel boresight pointed to the railway at the distance of Ds (projection of the neighboring RRH on the railway)

    - For Scheme-2: RRH panel boresight pointed to the railway in the middle point between 2 RRHs

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 1-3-4: Beam Dwelling time

- For scenario-A, bi-directional:

  - Proposal 1 (Samsung): For single beam per Panel, For bi-directional RRH deployment for Scenario-A, the beam dwelling time can be in the range of [0.80, 1.99] seconds for UE maximum speed of 350kmph.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

  - Companies are encouraged to further disucss beam dwelling time based their own HST deployment scenario study, and the discussion outcome will be served as the basis for RRM discussion, e.g., the dwelling time for each beam may have implication on the required time duration for some RRM operation.

### Sub-topic 1-4 Scenario-B, Uni-directional RRH

Issue 1-4-1: Number of Beam for uni-directional RRH deployment, Scenario-B

- For scenario-B, uni-directional, RRH parameter:

  - Proposal 1 (Ericsson, Huawei, Samsung): 1 beam per RRH panel

  - Proposal 2 (Nokia): 1 or 2 beams per RRH panel

  - Proposal 2a (Nokia): RAN4 to use only 1 beam (TCI state) per RRH panel in uni-directional deployment with Full SFN transmission scheme for Scenario B.

  - Proposal 3 (ZTE): 4 beams per RRH panel

  - Proposal 4 (QC): 4 beams with uneven separation ([0 7.5 15 22.5 37.5] relative angle in degree to boresight direction) per RRH panel

  - Proposal 5 (Intel): 2 beams per RRH panel

- For scenario-B, uni-directional, UE parameter:

  - Proposal 1 (Ericsson, Samsung): 1 beam per UE panel (i.e., 1 beam per UE)

  - Proposal 2 (QC) 7 beams with separation ([0 7.5 15 22.5 30 37.5 45] relative angle in degree to boresight direction) on one side, 13 UE beams if consider RRHs on two sides, per UE panel

  - Proposal 3 (Intel): 2 beams per UE panel

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 1-4-2: Beam switching point

- Proposals

  - Proposal 1 (Samsung): Ds_offset (illustrated in below figure) in the range of [370-457]m for Scenario-B uni-directional RRH deployment.

  - Proposal 2(QC): 4 switching point per Ds (4 beams in total)

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

### Sub-topic 1-5 Scenario-B, Bi-directional RRH

Issue 1-5-1: Schemes for Bi-directional deployment

- Proposals

  - Proposal 2 (Samsung): For Scenario-B, bi-directional RRH deployment:

    - If Scheme-1 if scheme-1 used for bi-directional RRH deployment, two beams per RRH panel can provide satisfactory coverage.

    - The scheme-2 (connecting to nearest RRH except coverage hole) is not recommended to be used.

  - Proposal 3 (QC): Use scheme 2 for bi-directional model to resolve coverage issue

  - Proposal 4 (Nokia): RAN4 not to use PDSCH combining in HST FR2 bi-directional deployment, Scenario B.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 1-5-2: Number of Beam for bi-directional RRH deployment, Scenario-B

- For scenario-B, bi-directional, RRH parameter:

  - Proposal 1 (Ericsson): 1 beam per RRH panel

  - Proposal 2 (Huawei, Nokia, Intel): 2 beam per RRH panel

  - Proposal 2a (Samsung): if scheme-1 (connecting to 2nd-nearest RRH) is used, 2 beam per RRH panel

  - Proposal 3 (ZTE): 4 beams per RRH panel

  - Proposal 4 (QC): 4 beams with uneven separation ([0 7.5 15 22.5 37.5] relative angle in degree to boresight direction) per RRH panel

  - Proposal 5 (Nokia): RAN4 to decide if more than two beams per RRH are beneficial in bi-directional deployment, scenario B.

- For scenario-B, bi-directional, UE parameter:

  - Proposal 1 (Ericsson, Samsung, Huawei): 1 beam per UE panel (i.e., 1 beam per UE)

  - Proposal 2 (QC) 7 beams with separation ([0 7.5 15 22.5 30 37.5 45] relative angle in degree to boresight direction) on one side, 13 UE beams if consider RRHs on two sides, per UE panel

  - Proposal 3 (Nokia): RAN4 to decide if further analysis is needed regarding one panel per CPE pointing to upside and having analog beams directed to forward and backward in HST FR2 Scenario B.

  - Proposal 4 (Intel): 2 beams per UE panel (i.e., 4 beams per UE)

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 1-5-3: Beam Dwelling time

- For scenario-B, bi-directional:

  - Proposal 1 (Samsung): For two beam per Panel, for bi-directional RRH deployment for Scenario-B, the beam dwelling time can be in the range of [1.68, 1.92] seconds for UE maximum speed of 350kmph.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

  - Companies are encouraged to further disucss beam dwelling time based their own HST deployment scenario study, and the discussion outcome will be served as the basis for RRM discussion, e.g., the dwelling time for each beam may have implication on the required time duration for some RRM operation.

### Sub-topic 1-6 Comparison between Uni-/Bi-directional RRH Deployment

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-6-1: Comparison between uni- and bi-directional RRH deployment

- Proposals

  - Observation 1 (Ericsson): Bi-directional deployment is inferior to uni-directional deployment for scenario B.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

  - Companies are also welcomed to provide further views between bi-directional and uni-directional deployment.

### Sub-topic 1-7 Signaling

Issue 1-7-1: Necessity of Signaling

- Proposals

  - Proposal 1 (Huawei): Do not introduce any signaling for Bi-directional deployment for HST FR2 in Rel-17.

  - Proposal 2 (Nokia): If found to be needed, RAN4 to continue the discussion of issues related to the deployment type and UE capabilities signaling in the RRM track.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

### Sub-topic 1-8 Other Issues Identified in This Meeting

Issue 1-8-1: Track curvature and impact on RRH separation

- Proposals

  - Observation 1 (Ericsson): For 350km/h scenarios, track curvature is not sharp and coverage can be provided with 1 TX/RX beam also for curves with 700m BS separation.

  - Observation 2 (Ericsson): For 120km/h scenario, curves may in some cases be sharper. Coverage with 1TX/RX beam for such curves can be provided if BS are spaced around 400m around the curve. (Alternatively, multiple beams could be used).

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 1-8-2: Consider 1 RRH site per BBU

- Proposals

  - Proposal 1 (Nokia): RAN4 to consider also regular (non-SFN/non-DPS) deployment with 1 RRH site per BBU.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 1-8-3: High difference in propagation delays

- Proposals

  - Proposal 1 (Nokia): RAN4 to elaborate further on which deployments and propagation schemes are exposed to the very different propagation delays. Then, quantitively evaluate the implications in these scenarios both from the demodulation and RRM perspectives.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 1-8-4: Dedicated network for roof-mounted CPE

- Proposals

  - Proposal 1 (Nokia): RAN4 to assume that in HST FR2 Scenario A, only high-speed CPEs installed on the roof of the train can be present in the network.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 1-8-5: Handheld UE for FR2 HST

- Proposals

  - Proposal 1 (ZTE): To consider supporting handheld UE for HST_FR2 with lower priority.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 1-8-6: The ability of Multi-beam RX or TX at UE

- [Moderator]: It is noted that the following agreement is achieved in RAN4#98-e:

- Bi-directional operation for two panels (if any): - Follow Rel-15/16 principle of “only one panel to TX/RX at a time”. - FFS signaling is needed.

- Proposals

  - Proposal 1 (ZTE): The ability of multi-beam Rx or Tx at a time can be considered for CPE under uni-directional situation of scenario-A/B.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 1-8-7: Presence of regular UE in the network

- Proposals

  - Proposal 1 (Nokia): RAN4 to clarify based on the operators’ input if regular (i.e., low-speed non-HST) UEs can be connected to the same cell together with a HST CPE moving at maximum speed.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

## Companies views’ collection for 1st round

### Open issues

Sub topic 1-1

| Company | Comments |
| --- | --- |
| Ericsson | Issue 1-1-1: UE antenna element parameters We assumed 4x4 UE array. Most likely the assumption does not make much difference to the conclusions on the reference scenario and requirements. Issue 1-1-2: UE antenna panel(s) for forward and backward directions We assume 2 panels pointing in opposite directions. In this scenario though it is more optimal to operate as 2 UEs, each operating uni-directional than bi-directional (double throughput obtained). We do not see any use for the upside pointing UE as it does not achieve full coverage. Support proposal 1. Issue 1-1-4: Necessity of JT in Scenario-A/B, Uni/Bi-directional RRH JT does not work efficiently in uni-directional as the two paths will be separated by a delay larger than the CP. For bi-directional, we do not see any advantages over uni-directional and it requires double panel operation for the same UE. It is better to transmit different data from opposite directions than the same signal (doubles capacity). Hence no need to consider JT. |
| ZTE | Issue 1-1-1: UE antenna element parameters 4x4 or 2x4 UE array can be considered Issue 1-1-2: UE antenna panel(s) for forward and backward directions Support proposal 1 Issue 1-1-3: Number of CPE devices per train/carriage 1 CPE per train as baseline for RAN4 requirement Issue 1-1-4: Necessity of JT in Scenario-A/B, Uni/Bi-directional RRH JT(full SFN) is inferior to DPS. |
| Intel | Issue 1-1-1: UE antenna element parameters Prefer to keep both 4x4 and 2x4 options for UE antenna array Issue 1-1-2: UE antenna panel(s) for forward and backward directions Support proposal 1. @Ericsson: we agree that we can double throughput by serving 2 UEs in different directions, but it’s better to have them non-co-located to avoid interference (even though it is low due to antenna patterns filtering). Anyway, it doesn’t affect the requirements definition. Issue 1-1-3: Number of CPE devices per train/carriage Support proposal 1. Issue 1-1-4: Necessity of JT in Scenario-A/B, Uni/Bi-directional RRH Support proposal 1. We do not see any benefits in JT |
| Nokia, Nokia Shanghai Bell | Issue 1-1-1: UE antenna element parameters In our simulations, we used PC4 assumptions agreed in the WF of RAN4#97e meeting (R4-2017828) with a configuration [Mg, Ng, M, N, P] = [1, 1 or 2, 4, 4, 2], i.e., with 1 or 2 CPE panels. This parametrization includes Proposal 2. We propose to use it as a reference, e.g., in simulation parameters. However, we do not think that UE/CPE antenna panel configuration needs to be defined strictly as a part of deployment configuration. Issue 1-1-2: UE antenna panel(s) for forward and backward directions In our simulations, we have observed that CPE can be potentially equipped with only one antenna panel oriented upward. Such configuration results in the considerable loss in system performance due to the lower SINR in comparison to two-panel CPE. However, we have not observed mobility problems either even though the CPE beam configuration was not optimized. We studied only one beam co-oriented with the panel. Issue 1-1-3: Number of CPE devices per train/carriage In our simulations with multiple CPEs per train, briefly reported in R4-2106639 (Figure 5), we observed an increase in interference in comparison to only one CPE per train (no interference). As the signal stays on sufficient level there is no considerable impact on mobility performance. Just a few occasional failures were observed in bi-directional setting. Hence, we can build the RRM requirement based on 1 CPE per train. Issue 1-1-4: Necessity of JT in Scenario-A/B, Uni/Bi-directional RRH We do not see enough reasons to exclude JT/Full-SFN scheme for the analysis now. There is a number of issues that are still under discussion which will impact this decision. For example, one-panel CPEs can benefit from JT operation in bi-directional setting (Issues 1-1-2), JT can be more efficient in improving signal level next to the RRH (Issue 1-3-1). Additionally, the DPS scheme without the assumption of ToC or perfect timing offset knowledge can also face similar issues as JT in uni-directional deployments (Issue 1-8-3). |
| QC | Issue 1-1-1: UE antenna element parameters We are open to discuss both options. Ideally, RAN4 should first decide RRH antenna configurations, then with agreements on pathloss model and deployment parameters, link budget analysis can decide between the two options. Issue 1-1-2: UE antenna panel(s) for forward and backward directions Proposal 1 is more reasonable, given that trains can travel in opposite directions, and it is not guarantee that all the uni-directional deployments are with the same direction. |
| Samsung | Issue 1-1-1: UE antenna element parameters We found that 4x4 UE array shall be assumed based on our knowledge of current antenna penal implementation. If conclusion and analysis can be confirmed based on this assumption, this assumption should be used by RF session to further drive the RF requirement definition. Issue 1-1-2: UE antenna panel(s) for forward and backward directions Agree with P1. 2 panels pointing in opposite directions. Issue 1-1-3: Number of CPE devices per train/carriage Support proposal 1. |
| Huawei | Issue 1-1-1: UE antenna element parameters We prefer to use 4x4 to achieve better performance. Issue 1-1-2: UE antenna panel(s) for forward and backward directions We are OK with the Proposal 1. We don’t think it reasonable for the CPE with panel pointing to upside. Issue 1-1-3: Number of CPE devices per train/carriage One CPE per RRH should be considered. Issue 1-1-4: Necessity of JT in Scenario-A/B, Uni/Bi-directional RRH We don’t think SFN JT is appropriate considering the large Doppler spread and large delay spread to be handled. |

Sub topic 1-2

| Company | Comments |
| --- | --- |
| XXX | Issue 1-2-2: Uni-directional operation Agree with the statement. Also, the “CPE” could be configured as 2 UEs both operating uni-directional but in different directions. Still from a requirements point of view it would look like single direction though. Issue 1-2-3: RRH boresight direction for uni-directional RRH deployment We assumed boresight parallel, but in our opinion any of these options works well. Issue 1-2-4: Beam switching point It is not critical exactly where the switching is as long as it is in the range 740-800m. The Samsung proposal is OK. Issue 1-2-5: Handover We do not see the need for the extra panel. Good coverage around a BS can be obtained from the previous BS and only 1 beam per BS. |
| ZTE | Issue 1-2-1: Number of Beam for uni-directional RRH deployment, Scenario-A For scenario-A,uni-directional 1 beam per RRH panel can be supported Since candidate scheme of full SFN is not excluded, the number of beam for UE panel should not be restricted to 1, as multi-beam is helpful to deal with propagation difference. Issue 1-2-2: Uni-directional operation Agree Issue 1-2-3: RRH boresight direction for uni-directional RRH deployment Considering dmin of scenario-A, the 3 options are very close. The height of the RRH, the height of the antenna and the switching point should be considered to reflect a more realistic deployment. Issue 1-2-4: Beam switching point Agree the proposal |
| Intel | Issue 1-2-1: Number of Beam for uni-directional RRH deployment, Scenario-A Support one fixed beam per panel both on RRH and UE sides. We think that even in uni-directional deployment UE should be equipped with 2 panels. It should sweep over both panels at least when it enters the network to define the direction of uni-directional deployment. Issue 1-2-2: Uni-directional operation Ok with Proposal 1 Issue 1-2-3: RRH boresight direction for uni-directional RRH deployment There is almost no difference between the proposals: 0.08° difference between P1 and P3 and 0.8° difference between P2 and P3. Prefer Proposal 3 as it is more straightforward. Issue 1-2-4: Beam switching point Ok with the proposal Issue 1-2-5: Handover Based on our analysis HO is not a problem for HST in FR2. However, we are ok to keep it for further study |
| Nokia, Nokia Shanghai Bell | Issue 1-2-1: Number of Beam for uni-directional RRH deployment, Scenario-A We agree with Proposal 1. Issue 1-2-2: Uni-directional operation From the previous version of the Note it followed that in uni-directional deployment it should always be two panels at RRH site pointed into opposite directions to support the trains moving in opposite directions (even though one of the mis used at a time). In our opinion, it is enough is also possible to have only one panel at RRH. Then, if the CPE has two panels, then one of those can be used depending on the train direction. Issue 1-2-3: RRH boresight direction for uni-directional RRH deployment We used Proposal 3 in our simulations because it was originally included in the WF at the RAN4#97-e. Further optimizations can be made if the problems in RRM performance are found. For the moment it is not the case. Hence, we preferer to keep Proposal 3 as a reference for simulations, but other options should not be precluded. Issue 1-2-4: Beam switching point In general, we agree that the beam switching will, probably, happen not exactly under the RRH site location but at some distance from it. However, the exact point of beam switching might depend on many factors including environmental (e.g. slow fading), implementational (the orientation of antenna panels and beam configuration, side lobes, etc.), reason of beam switch (e.g. HO-based or based on L1 measurements), etc. Therefore, in our opinion, beam switching point cannot be used as a deployment parameter. We do not see a need to target for an agreement in this issue. The discussion can continue in the propagation models topic, where Ds_offset can be present as a parameter. Issue 1-2-5: Handover In the scenario under discussion, up to our best knowledge, that are no issues neither with the coverage nor with the signal strength. Interference between the SSB resources of neighbouring cells can be avoided by using TDM or FDM mechanisms available in the NR. In our simulations, we have not observed any issues with HO performance. Therefore, it is not very clear to us why additional panel is needed. |
| QC | Issue 1-2-1: Number of Beam for uni-directional RRH deployment, Scenario-A We support 1 beam per RRH panel (proposal 1) and 1 beam per UE panel and two panels per UE (proposal 2). Note that proposal 2 for UE is consistent with proposal 1 in issue 1-1-2. If proposal 1 is agreed in 1-1-2, proposal 1 for UE panel is invalid. Issue 1-2-3: RRH boresight direction for uni-directional RRH deployment Based on our understanding, proposal 3 is calculated based on boresight pointing to 700m. In this case, the difference between proposal 1 and 3 is minor, we can compromise to proposal 3. Issue 1-2-4: Beam switching point In our study 80m is where the SNR from the closest RRH beam exceed the previous serving RRH beam. But this distance depends on the codebook and antenna parameters, we suggest revising the proposal as: Ds_offset (switching point) is where the SNR from the target RRH (currently closest RRH) beam exceeds the SNR from the source RRH beam. Issue 1-2-5: Handover In the proposal, instead of pushing for the solution, we would like to raise this concern of handover and neighboring cell detection issue. We are open to discuss better solutions than the proposed one, since not only additional RRH panels are needed, UE has to active another panel, too. However, the lack of coverage from the neighboring cell until the neighboring cell suddenly has much larger power than the serving cell is indeed an issue we have to resolve in uni-directional model with small Ds. Instead of the proposal of solution, we can compromise to the two potential agreements: - UE half cone coverage of antenna arrays on one panel is between 0 to 60 degrees on azimuthal plane, which leads to coverage hole from RRH beams when UE is passing the RRH - RAN4 to study the resolution to the issue in uni-directional model with small Ds: lack of coverage from the neighboring cell until the neighboring cell suddenly has much larger power than the serving cell, which may fail the handover procedure. In fact, the first potential agreement can also justify issue 1-2-4 switching point, and the bi-directional coverage issue 1-3-1. |
| Samsung | Issue 1-2-1: Number of Beam for uni-directional RRH deployment, Scenario-A Based on our observation and analysis in our paper, for uni-directional RRH deployment for Scenario-A, 1 beam per RRH panel can provide good enough system performance. Issue 1-2-2: Uni-directional operation The statement “but we are aware of the fact that either another panel to serve train towards the other direction is needed or a CPE is capable of Rx and Tx from the opposite direction.” Need more clarification: - We already agree use the R15/16 assumption that UE can only simultaneously TX/RX with one panel, so: - (a) Here in the statement, “another panel” seems means another UE from air-interface perspective; - (b) “CPE capable of RX and TX from opposite direction”, but two panel can’t work simultaneously. If above is correct understanding, then the statement with clarification will help. Issue 1-2-3: RRH boresight direction for uni-directional RRH deployment Here the intention to list these options are obviously not preclude other implementation, or mandate something, but provide the assumption for the basis of analysis. From our understanding, if these options can’t be narrowed down to one, it is companies’ responsibility to list their chosen option in their evaluation, and that assumption can be informative to be captured in TR. Issue 1-2-4: Beam switching point As proponent of P1, the intention is in two-folds: - If further alignment of evaluation is needed, the switching point (similar to above Issue 1-2-3, boresight direction) should be listed along with companies’ analysis, which is informative to be captured in TR. - This beam switching point will help to derive the demodulation performance channel model, and the conclusion here will be delivered to demod session as the basis to derive channel model. Issue 1-2-5: Handover This is the issue newly proposed in this meeting and we need more time to analysis the handover region is large enough or not, to confirm the problem identified by QC. |
| Huawei | Issue 1-2-1: Number of Beam for uni-directional RRH deployment, Scenario-A We prefer to use 1 beam per panel for both RRH and UE. Issue 1-2-2: Uni-directional operation We are OK with the Proposal 1. Issue 1-2-3: RRH boresight direction for uni-directional RRH deployment We prefer Option 2. Issue 1-2-4: Beam switching point The specific beam switching point closely depends on the Issue 1-2-1 and Issue 1-2-3. Issue 1-2-5: Handover Maybe this is a coverage issue when UE is near to the edge of two BBU based on the current RRM requirements. But we need to further check this issue for next meeting. |

Sub topic 1-3

| Company | Comments |
| --- | --- |
| Ericsson | Issue 1-3-1: Schemes for Bi-directional deployment We generally do not see a use or need for bi-directional. Either scheme may be feasible, connecting to 2nd nearest is more useful. However it would be better to operate uni-directional in both directions than bi-directional (doubles capacity). Issue 1-3-4: Beam Dwelling time We should take care that the beam dwelling time for bi-directional is not the thing driving RRM requirements if the bi-directional deployment anyhow does not have gains above uni-directional. |
| ZTE | Issue 1-3-1: Schemes for Bi-directional deployment Scheme-2 is preferred for scenario-A Issue 1-3-2: Number of Beam for bi-directional RRH deployment, Scenario-A Proposal 1 is supported. For UE parameter, if 2 panels are assumed for CPE proposal 3 can be supported. Issue 1-3-3: RRH boresight direction for bi-directional RRH deployment Scheme 1 is supported |
| Intel | Issue 1-3-1: Schemes for Bi-directional deployment Scheme-1 is less efficient than uni-directional deployment in terms of link budget. @Samsung: How does beam switch is assumed to be triggered in Scheme-1? Is it some location-based or measurement-based decision? Scheme-2 is preferred. We have similar observation as Ericsson. Uni-directional deployment seems to be more straightforward while it still has sufficient link budget. We think that it is more beneficial to use bi-directional deployment as two uni-directional deployments which can serve 2 UEs looking in different directions. Issue 1-3-2: Number of Beam for bi-directional RRH deployment, Scenario-A Support Proposal 1 and 3. For Proposal 1a, based on Samsung’s tdoc single beam should also work well for scheme-2. Issue 1-3-3: RRH boresight direction for bi-directional RRH deployment Ok with Proposal 2, Scheme-2 Issue 1-3-4: Beam Dwelling time Ok with Proposal 1 |
| Nokia, Nokia Shanghai Bell | Issue 1-3-1: Schemes for Bi-directional deployment We do not see a need to consider two different schemes in this issue. Both schemes are possible if bi-directional deployment is considered, i.e. scheme two cannot be prohibited. The main reason why Scheme-2 might not be feasible is because the CPE does not have enough time to switch to the beams of the neighbouring RRHs. For example, if neighbouring RRHs belong to different cells, then CPE usually might not be able to HO to them in the area next to the RRH. However, if JT or DPS is used, CPE might be still able use the neighbouring RRHs. That what we are proposing to study further. Issue 1-3-2: Number of Beam for bi-directional RRH deployment, Scenario-A Regarding Proposal 1a, in our simulation results we observe that the area under the RRH site is in the coverage of the neighbouring RRHs. Moreover, there is hardly any space available for one more beam in this scenario. Hence, only one beam can be used in practice (Proposal 1). Regarding the number of CPE beams, it is necessary to clarify Issue 1-1-2 before. However, if two panels per CPE are used, then 1 beam per panel assumption is fine for us. Issue 1-3-3: RRH boresight direction for bi-directional RRH deployment We think that in Scenario A the proposed differences in the configuration of the beams have minor impact on minima performance requirements because the beams a almost parallel to the track. Issue 1-3-4: Beam Dwelling time In general, we agree that beam dwelling time is important statistics. However, the exact values may depend on many factors and implementation. The core of the proposal still left unclear to us. Regarding the observed values, in scenario A, bidirectional setting, in our simulations we see the median value of Time-of-Stay per cell on the level of 2 seconds. Note that slow fading model was used in our simulations and other parameters can be found in our RRM contribution (R4-2106583). |
| QC | Issue 1-3-1: Schemes for Bi-directional deployment We can support scheme 2 (proposal 5 or part of proposal 1/2) or not consider bi-directional model (proposal 4) Issue 1-3-2: Number of Beam for bi-directional RRH deployment, Scenario-A We support proposal 1 for RRH, and proposal 3 for UE. Issue 1-3-3: RRH boresight direction for bi-directional RRH deployment We support proposal 1. This is coupled with issue 1-3-1 proposal. In order to cover the coverage hole under the next RRH, the boresight direction should point to Ds (or Ds+Ds_offset) to compensate for pathloss. |
| Samsung | Issue 1-3-1: Schemes for Bi-directional deployment We propose this issue of “how the beam should be directed to cover the track for bi-directional scenario” because we found the RRH-site coverage issue. We list two schemes for companies’ view and analysis. As we present in our paper, Scheme-1 is not good enough because at least for Scenario-A, it don’t show performance advantage compared with uni-directional deployment. Agree that uni-directional deployment is more straightforward in this scenario-A. To answer Intel’s question, scheme-1 can be achieved based on R15 beam measurement – beam (TCI) indication, but it could be different from normal operation because gNB need to switch the beam from the active serving one if its RSRP becomes good. If bi-directional is preferred, Scheme-2 also need more evaluation, because we found that not only one beam switching is needed, but the cell-site area will also Doppler shift change. Issue 1-3-2: Number of Beam for bi-directional RRH deployment, Scenario-A For RRH side, need to discuss scheme-1 and 2 firstly. For UE side, P3 is straightforward, and obviously supported by us as proponent. Issue 1-3-3: RRH boresight direction for bi-directional RRH deployment Agree with P2 as proponent depends on Scheme-1 and 2 to be used. Similar to Issue 1-2-3, it is companies’ responsibility to list their chosen option in their evaluation if the assumption can’t be aligned, and that assumption can be informative to be captured in TR along with their analysis. Issue 1-3-4: Beam Dwelling time Based on our analysis, we provide the ideal beam dwelling time, and it should be noted that any hysteresis of beam switching procedure is not yet considered. From our perspective, we think this beam dwelling time needs to be analysis, and purpose of this analysis: - This information should be used by RRM session to define their requirement If beam dwelling time is small, this will impose the restriction of UE RX beam number for a realistic system implementation. |
| Huawei | Issue 1-3-1: Schemes for Bi-directional deployment For scheme 1, it is difficult for CPE to perform beam switching since the received signal power from the nearest RRH is the strongest among all RRHs based on our evaluations. For scheme 2, as per our evaluation, the side-lobe of the nearest RRH may be stronger than the main-lobe of the second nearest RRH when CPE is directly under the RRH. It is also difficult for CPE to perform beam switching. In general, we do not think that bi-directional is suitable for Scenario-A. |

Sub topic 2-4

| Company | Comments |
| --- | --- |
| Ericsson | Issue 1-4-1: Number of Beam for uni-directional RRH deployment, Scenario-B We are OK to consider 1-3 RRH beams per panel for scenario B; although operation with 1 beam is feasible, for straight track the coverage can be improved slightly with 2-3 beams and also curves in the track may be better covered. For the UE, we are OK to consider 1-2 beams; 2 beams may enable further coverage optimization and robustness where there are curves. Issue 1-4-2: Beam switching point We are OK with proposal 1; this aligns with our observations. |
| ZTE | Issue 1-4-1: Number of Beam for uni-directional RRH deployment, Scenario-B 3 or 4 beams can be considered for scenario-B uni-directional deployment. For UE parameter, 2 or more beams per panel. |
| Intel | Issue 1-4-1: Number of Beam for uni-directional RRH deployment, Scenario-B Based on our analysis single beam per RRH panel is not enough to cover full inter-RRH distance. Symmetrically, single beam at UE is also not enough. Prefer to have 2 beams per panel both at UE and RRH sides Issue 1-4-2: Beam switching point Ok with Proposal 1. |
| Nokia, Nokia Shanghai Bell | Issue 1-4-1: Number of Beam for uni-directional RRH deployment, Scenario-B In non-JT schemes, the number of RRH beams per panel can be over two but as it is shown in our contribution (R4-2106694) the usage of the beams oriented perpendicular to the railways track is very limited. Hence, two beams per RRH panel looks to be a reasonable assumption for us. Regarding the number of UE beams, we have not observed any performance problem in simulations with only one beam per CPE. Thus, 1 or 2 beams per CPE panel is a reasonable choice. Issue 1-4-2: Beam switching point Similarly to our comment in Issue 1-2-4, we propose to continue the discussion together with propagation conditions in Topic 2, where Ds_offset can be considered as model parameter. |
| QC | Issue 1-4-1: Number of Beam for uni-directional RRH deployment, Scenario-B Exact number of beam can be revisited after RRH and UE antenna parameters assumption for evaluation are finalized. The idea we would like to highlight is multiple RRH beams with uneven separations. We see significant benefit in our evaluation, and we would like to recommend RAN4 to consider this as part of evaluation assumptions. For UE beams, we see performance gain from multiple UE beams but the exact number can be revisited after RRH and UE antenna parameters assumption for evaluation are finalized. Issue 1-4-2: Beam switching point This depends on the conclusion from issue 1-4-1. |
| Samsung | Issue 1-4-1: Number of Beam for uni-directional RRH deployment, Scenario-B RRH side: Based on our analysis, P1 (i.e., 1 beam per RRH panel) is provided. In other words, 1 beam per RRH panel can provide sound system performance, but adding more beams will give better performance. UE side: P1 as the proponent of this proposal. Issue 1-4-2: Beam switching point P1 as the proponent of this proposal. |
| Huawei | Issue 1-4-1: Number of Beam for uni-directional RRH deployment, Scenario-B As per our evaluation, 1 beam per panel for both RRH and CPE is feasible. Issue 1-4-2: Beam switching point Beam switching point depends on the beam configurations of number of beams and beam direction. |

Sub topic 2-5

| Company | Comments |
| --- | --- |
| Ericsson | Issue 1-5-1: Schemes for Bi-directional deployment We do not see any benefit from bi-directional operation. If we would do it, we agree with proposals 2 and 4. However a more efficient use of panels is to operate unidirectional in both directions and operate the CPE as if it is 2 UEs. Issue 1-5-2: Number of Beam for bi-directional RRH deployment, Scenario-B We are OK with 1-3 beams per RRH and 1-2 per UE. Issue 1-5-3: Beam Dwelling time We should take care that the bi-directional beam dwelling time does not drive RRM requirements if it is not clear that there is a benefit from bi-directional operation compared to uni-directional. |
| ZTE | Issue 1-5-1: Schemes for Bi-directional deployment Scheme 2 (Nearest RRH) is supported . Issue 1-5-2: Number of Beam for bi-directional RRH deployment, Scenario-B 2 or more beams per RRH panel and 4 or more beam per CPE(2 or more beams per panel if 2 panels are assumed) |
| Intel | Issue 1-5-1: Schemes for Bi-directional deployment Same comment as for Scenario A. Uni-directional operation seems to be more beneficial in bi-directional deployment. In case of bi-directional operation, we support Proposal 3 (Scheme-2). Issue 1-5-2: Number of Beam for bi-directional RRH deployment, Scenario-B 2 beams per RRH panel and 2 beams per UE panel should be enough for sufficient coverage Issue 1-5-3: Beam Dwelling time Ok with the proposed values. Agree with Ericsson’s comment: these values should not be used in RRM requirements definition until the benefits of bi-directional operation will be clarified. |
| Nokia, Nokia Shanghai Bell | Issue 1-5-1: Schemes for Bi-directional deployment We have a similar comment as for the Issue 1-3-1. There is no difference in the described schemes 1 and 2. It is just a matter of CPE to be able to switch/HO to the neighbouring RRHs. Issue 1-5-2: Number of Beam for bi-directional RRH deployment, Scenario-B Based on our simulations, if the number of beams is over two, the beams that are more perpendicular to the railway track are used much less frequently then the two beams that are more parallel to the track. Hence, two beams per RRH panel looks to us as a reasonable choice in scenario B. On the CPE side, one beam per CPE panel should be sufficient, but two beams per RRH can be considered as well. Issue 1-5-3: Beam Dwelling time Similar comment as for the Issue 1-3-4. For reference, based on our simulations, the median beam dwelling time in the scenario B with two beams per RRH panel was around 0.8 second. |
| QC | Issue 1-5-1: Schemes for Bi-directional deployment In proposal 2, scheme 2 is not recommended mainly because only 1 RRH beam is considered in the evaluation. 1 beam is not enough to cover the RRH’s serving area and the next RRH’s coverage hole. However, if multiple beams are considered, scheme 2 is applicable to scenario B bi-directional model to resolve the coverage issue. Therefore, we support proposal 3. Issue 1-5-2: Number of Beam for bi-directional RRH deployment, Scenario-B Same comment as issue 1-4-1. |
| Samsung | Issue 1-5-1: Schemes for Bi-directional deployment We found out for Scenario-B, if Scheme-1 used for bi-directional RRH deployment, two beams per RRH panel can provide satisfactory coverage. The scheme-2 (connecting to nearest RRH except coverage hole) is not recommended to be used. Issue 1-5-2: Number of Beam for bi-directional RRH deployment, Scenario-B Based on our analysis, P2a if scheme-1 is adopted for RRH side, and P1 for UE side. Issue 1-5-3: Beam Dwelling time Similar to Issue 1-3-4. |
| Huawei | Issue 1-5-1: Schemes for Bi-directional deployment We prefer the following scheme, two RRH beams point to Ds and Ds/2 respectively, panel boresight point to Ds/2. Actually as per our evaluation, there is no much difference for the panel boresight pointing to a certain position in the range of [Ds/2, Ds]. Issue 1-5-2: Number of Beam for bi-directional RRH deployment, Scenario-B As per our evaluation, 2 beam per panel for RRH and 1 beam per CPE is feasible. Issue 1-5-3: Beam Dwelling time The beam dwelling time depends on the panel and beam configurations. For our evaluation, the beam dwelling time can be about 1.2 to 2.4 second range that is sufficient to perform beam switching. |

Sub topic 2-6

| Company | Comments |
| --- | --- |
| Ericsson | Note we have the same observation for scenario A. It is not obvious why to use bi-directional. With 2 panels in each direction at the UE and RRHs, the system could be operated as uni-directional in each direction with the CPE acting as 2 UEs. This would double the capacity compared to operating as bi-directional. |
| ZTE | Issue 1-6-1: Comparison between uni- and bi-directional RRH deployment The bi-directional means that the RRH is mounted with 2 panels. We think bi-directional deployment can improve coverage or capacity. Or maybe the bi-directional deployment should be clarified. |
| Intel | Issue 1-6-1: Comparison between uni- and bi-directional RRH deployment As commented before, we find it more beneficial to use bi-directional deployment for uni-directional operation. |
|  | Issue 1-6-1: Comparison between uni- and bi-directional RRH deployment In our opinion, the requirements should be designed taking both possible deployments into account. |
| Samsung | Issue 1-6-1: Comparison between uni- and bi-directional RRH deployment The intention of comparison between uni- and bi-directional deployment needs clarification: (1) If companies’ can’t be fully aligned with narrow-down to one option, either uni- or bi-directional, seems it is straightforward to keep both options. (2) From RAN4 requirement perspective, we need to consider applicability rule and/or how to define requirement to enable UE can work in both uni-/bi-directional RRH deployment scenarios, if that is our target. |
| Huawei | We have different observations with Ericsson. There is large timing jump when UE performs beam switching that is a very serious issue that needs to be handled for Uni-directional. Also the CPE performing Bi-directional or Uni-directional reception is independent on The RRH deployment. We don’t think the capacity will be doubled since the total system capacity is fixed with certain bandwidth, these two-like UEs just share the total capacity. Also considering the possible interface between two CPEs, we don’t see any benefit for using 2 UEs. And also it is inconsistent with agreement in last meeting that only one Rx/Tx at a time. |

Sub topic 1-7

| Company | Comments |
| --- | --- |
| Ericsson | Issue 1-7-1: Necessity of Signaling This can be discussed in RRM if decided that bi-directional should be included. |
| Intel | Issue 1-7-1: Necessity of Signaling Need to agree first, whether bi-directional operation should be considered. |
| QC | Issue 1-7-1: Necessity of Signaling As we commented in the RRM thread, network signaling and UE capability for FR2 HST are needed, whether separate signaling/capability is needed for uni-directional and bi-directional can be FFS. We are ok to discuss this in RRM. Only one thread should cover this discussion, not both. |
| Samsung | Suggest the following guidance for following discussion: - Deployment scenario session focus on the feasibility of each scenario, in which gNB and UE parameters are studied to give the insight of how the system should be designed to enable such FR2 HST operation. - For the necessity and how to define signalling for RRM operation, it is discussed in RRM session. For the necessity and how to define signalling for Demod operation, it is discussed in Demod session. |
| Huawei | OK to discuss this issue in RRM. |

Sub topic 1-8

| Company | Comments |
| --- | --- |
| Ericsson | Issue 1-8-2: Consider 1 RRH site per BBU This can be considered in addition to DPS; not clear does it make any difference to the requirements though. Issue 1-8-3: High difference in propagation delays There is a need for uni-directional for a mechanism to re-set the UL TA when the serving BS is changed; this needs to be further considered (but a solution should be possible). Issue 1-8-4: Dedicated network for roof-mounted CPE Makes sense; there will not be other types of UE directly along the track as in scenario A. Issue 1-8-5: Handheld UE for FR2 HST Wall losses would prevent such UEs from connecting. Also, the orientation of the UEs would be random. Scenarios and RRM requirements could be very different. The WID states that the focus should be on roof mounted UEs. We prefer to exclude handheld UEs and keep the focus on roof mounted UE. Issue 1-8-6: The ability of Multi-beam RX or TX at UE We do not see a need for multi-beam UE. However, operating each panel of the UE as a separate UE with uni-directional transmission would double the capacity. |
| ZTE | Issue 1-8-2: Consider 1 RRH site per BBU Can be considered if only HST CPE is supported for HST_FR2. Issue 1-8-3: High difference in propagation delays Agree |
| Intel | Issue 1-8-2: Consider 1 RRH site per BBU Agree that such deployment can be taken into account. But it should not be considered as a baseline Issue 1-8-3: High difference in propagation delays Agree with Proposal 1 Issue 1-8-4: Dedicated network for roof-mounted CPE Other FR2 UEs can be present inside of the train. Even though we don’t expect to consider them in requirements definition, we should take them into account while defining Network signalling. Issue 1-8-5: Handheld UE for FR2 HST Requirements definition should be focused on roof-mounted CPE device. Issue 1-8-6: The ability of Multi-beam RX or TX at UE Based on WID multi-beam RX or TX is out of scope. Issue 1-8-7: Presence of regular UE in the network We assume that non-HST UEs can also be present in HST network. The railway is not always in rural environment. It is questionable to deploy RRHs (and gNB) every 700 meter along the whole railway for serving only very limited number of UEs a day without an option to reuse it for non-HST UEs |
| Nokia, Nokia Shanghai Bell | Issue 1-8-1: Track curvature and impact on RRH separation We agree with observation 1. However, in our opinion, 120 kmp/h scenario mentioned in Observation 2 is not HST scenario defined by the WID. Hence, it should not be considered further. Issue 1-8-3: High difference in propagation delays Both Full-SFN and DPS schemes without the assumption of ToC or perfect timing offset knowledge can also be challenging in uni-directional deployments. Issue 1-8-5: Handheld UE for FR2 HST According to the WID, we need to be “focused on train roof-mounted high-power devices”. We need to be realistic about the scope of the work in the WI and focus primarily on CPEs. Issue 1-8-6: The ability of Multi-beam RX or TX at UE The proposal needs further clarification. Is it related to the support of multiple active TCI states? If it is the case, it is necessary to verify if such joint transmission scheme is Rel-16 compatible. |
| QC | Issue 1-8-2: Consider 1 RRH site per BBU Average cell dwelling time is 7s if 1 RRH site per BBU, HO overhead might be large in this case. Issue 1-8-3: High difference in propagation delays Large propagation delay between beams can lead to the following issue: While UE can derive timing for any beam by PSS/SSS detection, the large propagation delay difference can introduce large ISI and signal power (of PSS/SSS) degradation. 700m distance leads to 2.3us propagation delay, which is 4 times CP and more than ¼ symbol duration. Here is our proposal to this issue: RAN4 needs to study the scheme to alleviate ISI and the requirement should take signal power degradation into consideration. |
| Samsung | Issue 1-8-1: Track curvature and impact on RRH separation Right now, we are not clarified that how to do analysis based on track curvature and necessity of that. Issue 1-8-2: Consider 1 RRH site per BBU Agree that such deployment can be taken into account but not baseline. Issue 1-8-3: High difference in propagation delays Agree with Nokia’s original proposal and QC’s proposal above. Issue 1-8-4: Dedicated network for roof-mounted CPE Based on WID, only train-roof-mounted CPE is considered. For this work item, agree with Nokia’s P1 to only consider dedicated network. Issue 1-8-5: Handheld UE for FR2 HST Based on WID, requirements definition should be focused on roof-mounted CPE device. Issue 1-8-6: The ability of Multi-beam RX or TX at UE Based on WID multi-beam RX or TX is out of scope. Issue 1-8-7: Presence of regular UE in the network Similar to Issue 1-8-4, only train-roof-mounted CPE UE is considered based on WID. For handheld UE, we are not convinced that the penetration loss will not block all signals. |
| Huawei | Issue 1-8-1: Track curvature and impact on RRH separation The 120km/h train speed is not expect in real high speed scenario. We prefer to only consider the line area that is more common for performance requirements definition. Coverage can also be ensured by adding more RRHs at the “curve position” by real deployment. Issue 1-8-2: Consider 1 RRH site per BBU Target deployment scenario in the WID is multi-RRHs share the same cell-ID. Different HST transmission schemes are all aimed to reduce cell handover that is very frequently to happen in HST scenario, so we prefer to only consider 4 or more RRHs. Issue 1-8-3: High difference in propagation delays This should be considered and solved before we conclude the specific deployment schemes. Issue 1-8-4: Dedicated network for roof-mounted CPE We are OK with the Proposal 1. Issue 1-8-5: Handheld UE for FR2 HST The handheld UE is not in the WID, also considering heavily penetration loss, we don’t think such kind of UE is feasible for FR2 HST. Issue 1-8-6: The ability of Multi-beam RX or TX at UE It is fine to consider this kind of CPE for further evaluations. Issue 1-8-7: Presence of regular UE in the network In the real network, HST dedicated cell only serves high speed UEs, we should assume that there is no non-HST UE in the network. |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

N/A because no CRs/TPs submitted under Topic-1.

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

Sub-topic 1-1: General Assumptions

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 | Issue 1-1-1: RRH and UE antenna element parameters [Moderator] Based on the discussion, companies are not aligned with UE antenna element parameters to be used for deployment scenario study analysis. One company suggest to discuss RRH antenna element parameters firstly. Candidate options: - For each panel in UE: - Option-1: N=4, M=4 with 2 polarization - Option-2: N=8, M=4 with 2 polarization - Option-3: N=2, M=4 (or N=4, M=2) with 2 polarization - For antenna array configuration in RRH： - Option-1: [Mg, Ng, M, N, P]=[1, 1, 4, 8, 2] - Option-2: [Mg, Ng, M, N, P]=[1, 1, 8, 8, 2] - Option-3: [Mg, Ng, M, N, P]=[1, 1, 8, 16, 2] Recommendations for 2nd round: - [Moderator] We observed different antenna element parameters for UE and RRH panel are proposed and utilized by individual company. Suggest to discuss the principle of how the options can be narrow-down. As moderator, we suggest this should be done by considering (1) to more match product implementation plan; (2) feasibility study on deployment scenario can be confirmed based on the proposed option. ----------GTW Discussion --------- E///: Option 1 for UE and option 2 for RRH side. If no consensus reached, we can consider which case be worst case. QC: We need to decide RRH element first and then link-budget analysis (operators’ feedback recommended), with mean Peak EIRP and then we can decide UE panel elements. Huawei: Share similar view from E///. With above observations, we think all above options feasible, we may need to first decide what level of link-budget needed. What’s E/// proposal for selections fine for us. Samsung: What’s the reference for worst case? From link-budget aspect with sharp beams vs wide beams with less number of beams need to be considered as tradeoff. Option 1 for UE and option 2/option 3 for RRH side. QC: RRH side should be decided firstly. UE we can open the options for further discussion and take one as starting point. Nokia: With option 1 on UE side, we didn’t see any issues with our analysis. Option 1 with UE side as starting points. For RRH side, we agree with option 2 for RRH side as baseline. E///: For worst case, we agree to think about number of beams in UE side and BS side to minimize the number of beams. We agree the companies’ proposal to select some of options as starting points. QC: For EIRP required need operators input and link-budget analysis; the selection of UE elements will imply the EIRP and PC requirements. Samsung: We are OK with the proposal. Option 1 aligned with the assumption derived PC4 requirements. Agreement: RRH side: Option-2: [Mg, Ng, M, N, P]=[1, 1, 8, 8, 2] UE side: Option 1: N=4, M=4 with 2 polarizations as starting point, and other options not precluded pending on further discussion RF session can trigger relevant discussion on RF requirements taking above agreements into account. |
|  | Issue 1-1-2: UE antenna panel(s) for forward and backward directions [Moderator] Views are aligned on P1, based on discussion till now. Tentative agreements: - RAN4 to consider CPE to be equipped with two panels pointed forward and backward along the track. - Detailed boresight directions of each panel can be adjusted based on companies’ analysis. Recommendations for 2nd round: - Discuss the above tentative agreement (slightly changed from P1) is acceptable or not. |
|  | Issue 1-1-3: Number of CPE devices per train/carriage [Moderator] Views are aligned on P1, based on discussion till now. Tentative agreements: - RAN4 requirement can be defined based on the baseline of 1 CPE device per train Recommendations for 2nd round: - Agree the tentative agreement. |
|  | Issue 1-1-4: Necessity of JT in Scenario-A/B, Uni/Bi-directional RRH [Moderator] Views are aligned on P1, based on discussion till now. Tentative agreements: - RAN4 to focus only on DPS transmission mode for FR2 HST - Joint Transmission (full SFN) is precluded. Recommendations for 2nd round: - Agree the tentative agreement. |

Sub-topic 1-2 Scenario-A, Uni-directional RRH

| Sub-topic 1-2 | Issue 1-2-1: Number of Beam for uni-directional RRH deployment, Scenario-A [Moderator] Options are discussed and summarized as below: - For scenario-A, uni-directional, RRH parameter: - Option 1 (Qualcomm, Samsung, Intel, Ericsson, Nokia, ZTE): 1 beam per RRH panel - For scenario-A, uni-directional, UE parameter: - Option 1 (Ericsson, Samsung, Intel): 1 beam per UE panel (i.e., 1 beam per UE) - Option 2 (QC): 1 beam per panel (two panels in opposite direction) Tentative agreements: - For scenario-A, uni-directional, RRH parameter: - 1 beam per RRH panel Recommendations for 2nd round: - Agree on the above tentative agreement - Discuss more on two options for UE parameter. ---GTW discussion---------- QC: In general, we agree beam per panel; from deployment scenario, not sure all the RRHs along with the track? If trains go opposite directions, 2 panels maybe needed. 2 panel with opposite directions in UE side. E///: What’s the difference from proposals. Intel: We agree with QC, UE with 2 panels, and 1 panel activated per time and 1 beam per panel. Samsung: From UE side, 2 panels needed. Only one panel active for this deployment scenario. QC: We agree with Samsung, only one panel active at any given time with 2 panels implemented in UE. For some handover scenario, we may need to two panels activated simultaneously and which can be further discussed as a corner case during certain region. Samsung: Product implementation aspect, we all aligned; whether NW can guarantee the deployment scenario condition or UE need to detect and identify the conditions. Huawei: We share the same understanding for UE implementation. We think UE need to have capability to enable beam switch across panels. Nokia: We are fine to have 1 beam per panel, how many panels needed is a separate issue ; and we are OK with the 2 panels in UE side if companies prefer. E///: NW can’t always guarantee with one side direction especially for scenario B. QC: All RRH with same BBU, with same direction. FFS whether 2 panels required or not for neighbour cell search and handover. Samsung: Two UEs with separate panel directions, this is can be captured in TR. But we would like to focus on single UE operation. Huawei: We don’t think there is benefit with capacity with two UEs. We prefer to focus on per UE per train /coverage. We are open to further discuss some special cases handling. Nokia: For E/// proposal with 2 separate UE, for this scenario, no benefits. Used panel is active panel, UE can implement two panels, but according WID, only panel used to transmit and receive at any time. E///: Two UEs can be transparent to specifications and leave to implementation. We can focus on 1 UE per train for introducing requirements. We should not exclude the possibility of UE with single panel Intel: 2nd panel can be needed in some corner cases. Whether we need to cover the corner case when defining requirements or focused on typical scenario. Agreement: 1 beam per panel; 2 panels assumed to be implemented in the UE side; Only the one active panel per UE can be used for Tx and Rx; and FFS whether another panel can be used for beam search |
| --- | --- |
|  | Issue 1-2-2: Uni-directional operation [Moderator] The statement “but we are aware of the fact that either another panel to serve train towards the other direction is needed or a CPE is capable of Rx and Tx from the opposite direction.” Need more clarification: - We already agree use the R15/16 assumption that UE can only simultaneously TX/RX with one panel, so: - (a) Here in the statement, “another panel” seems means another UE from air-interface perspective; - (b) “CPE capable of RX and TX from opposite direction”, but two panel can’t work simultaneously. If above is correct understanding, then the statement with clarification will help. Tentative agreements: N/A Recommendations for 2nd round: - Discussion on the above understanding and agree with a clarified statement. |
|  | Issue 1-2-3: RRH boresight direction for uni-directional RRH deployment [Moderator] Three options are listed here: - Option 1: For the uni-directional model, the RRH boresight in azimuthal angle points to 780m from the projection of the RRH on the track. - Option 2: Use boresight parallel to the railway for Uni-directional deployment for Scenario A. - Option 3: RRH panel boresight pointed to the railway at the distance of Ds (projection of the neighboring RRH on the railway). Accordingly, for Scenario-A: Azimuth angle: 0.8 degree; Down-titling: 1.2 degree Tentative agreements: - Suggest RAN4 may not need to specify RRH/UE boresight direction for deployment scenario study, but left for companies’ choice: - RRH/UE boresight directional information can be provided by individual company to accompany their deployment scenario analysis result, which can be captured in TR. Recommendations for 2nd round: - Discuss the tentative agreement. |
|  | Issue 1-2-4: Beam switching point Tentative agreements: - For Scenario-A uni-directional RRH deployment - Ds_offset (switching point) is where the SNR from the target RRH (currently closest RRH) beam exceeds the SNR from the source RRH beam, illustrated in below figure. - Ds_offset: in the range of [40-81]m. Recommendations for 2nd round: - Agree the tentative agreement (based on P1 and capture QC’s comment). - FFS Ds_offset value which could be used to derive channel model. |
|  | Issue 1-2-5: Handover [Moderator] The following proposal is proposed in this meeting, which raise the concern on the newly proposed issue. Proposal 1 (QC): UE half cone coverage of antenna arrays on one panel is between 0 to 60 degrees on azimuthal plane, which leads to coverage hole from RRH beams when UE is passing the RRH Proposal 2 (QC): RAN4 to study the resolution to the issue in uni-directional model with small Ds: lack of coverage from the neighboring cell until the neighboring cell suddenly has much larger power than the serving cell, which may fail the handover procedure. Proposal 2 is a consequence of proposal 1. They are not contradicting proposals. T0 CID =1 CID =2 CID =2 HO Tentative agreements: N/A Recommendations for 2nd round: - Since it is first time to discuss this issue, suggest to discuss and analyse more. ---GTW discussion----- QC: We encourage companies to further think about the issue we pointed out and any resolutions to address this issue. Nokia: We think this issue not valid for scenario A, but for scenario B we can further discuss. Samsung: Whether or not enough overlapped regions for hand over. We have similar observation as Nokia, we also open to further discuss. E///: We have similar observation as Nokia, the overlapped region is large enough with sufficient SNR for scenario A. Huawei: With our understanding, cell handover require long time compared to beam switch within cell, more detailed analysis for the overlapped region. Further discuss this issue in 2nd round and future RAN4 meetings if needed. Details description for this potential issue can be captured in WF. |

Sub-topic 1-3 Scenario-A, Bi-directional RRH

| Sub-topic 1-3 | Issue 1-3-1: Schemes for Bi-directional deployment [Moderator] Two schemes are proposed to solve “RRH-site” coverage issue for bi-directional deployment: - Scheme-1: Connecting to 2nd-Nearest RRH; - Scheme-2: Connecting to Nearest RRH except Coverage Hole. Furthermore, some companies propose to preclude bi-directional deployment for Scenario-A. Tentative agreements: N/A Recommendations for 2nd round: - Discuss bi-directional RRH deployment can be precluded from Scenario-A or not. - If bi-directional RRH is considered in Scenario-A, FFS Scheme-2 can be precluded for Scenario-A. ---------------GTW Discussion-------------------- Question 1: whether need to consider bi-directional deployment? Question 2: if needed, which scheme can be further considered? Nokia: UE should be capable of Beam handover/switch during this transition period. This can be resolved the coverage hole issue in scenario B and can be further studied. E///: We can further discuss and decide in next meeting, what’s the benefits of bi-directional over uni-directional? Huawei: For scenario A, Dmin small, the power levels among Erhus/Beams is small and hard for UE to select which beam can be selected. We think uni-directional enough for this scenario. This issue also for scenario B. QC: For question 1, we are open. For question 2, UE in the transition region, UE need to rely on other RRHs other than the nearest RRH. The same problem also exists for scenario B also; we prefer to enable scheme 2 with small path loss compared to scheme 1. Samsung: For scenario A, we share similar view as Huawei and E///, this deployment no benefits observed. For scheme 1, we agree not perfect solution, just one candidate solution for consideration. We are ok to focus on scheme 2. Intel: we also didn’t see any benefits for bi-directional deployment under scenario A. For scenario B , we can further discuss. Nokia: Any special setting for scheme 1 and scheme 2? For scenario A, no possibility to add more beams and scenario B can be considered. Huawei: We didn’t observe the coverage hole issue under suitable panel adjusting with our analysis. E///: We didn’t conclude whether coverage issue. For scenario A, further evaluate the benefits with bi-directional deployment compared to uni-directional deployment - Further analysis needed for the potential issue of coverage when close to RRH locations; Scheme 2 can be used as starting points for further analysis |
| --- | --- |
|  | Issue 1-3-2: Number of Beam for bi-directional RRH deployment, Scenario-A [Moderator] Discussion is based on: - For scenario-A, bi-directional, RRH parameter: - Proposal 1 (Nokia, Intel, Ericsson, QC): 1 beam per RRH panel, two panels in opposite directions - Proposal 1a (Samsung): Depends on scheme-1 or 2 - For scheme-1: 1 beam per RRH panel for Scheme-1 - For scheme-2: one additional beam per RRH site needs to cover neighboring RRH site. - For scenario-A, bi-directional, UE parameter: - Proposal 3 (Samsung, Ericsson): 1 beam per UE panel (i.e., 2 beam per UE) Recommendations for 2nd round: - Depends on Issue 1-3-1 on necessity of Scheme-2. - If Scheme-2 is not needed, P1 and P3 are suggested to be agreed. |
|  | Issue 1-3-3: RRH boresight direction for bi-directional RRH deployment [Moderator] For the discussion till now, two proposals are based upon: - Proposal 1 (Qualcomm): For the bi-directional model, the RRH boresight in azimuthal angle points to 780m from the projection of the RRH on the track. - Proposal 2 (Samsung): Different for Scheme-1 and 2: - For Scheme-1: Option-2: RRH panel boresight pointed to the railway at the distance of Ds (projection of the neighboring RRH on the railway) - For Scheme-2: RRH panel boresight pointed to the railway in the middle point between 2 RRHs Tentative agreements: - Suggest RAN4 may not need to specify RRH/UE boresight direction for deployment scenario study, but left for companies’ choice: - RRH/UE boresight directional information can be provided by individual company to accompany their deployment scenario analysis result, which can be captured in TR. Recommendations for 2nd round: - Discuss the tentative agreement. |
|  | Issue 1-3-4: Beam Dwelling time [Moderator] Beam dwelling time based on one company’s analysis is provided for discussion. - For scenario-A, bi-directional: - Proposal 1 (Samsung): For single beam per Panel, For bi-directional RRH deployment for Scenario-A, the beam dwelling time can be in the range of [0.80, 1.99] seconds for UE maximum speed of 350kmph. Tentative agreements: - (If bi-directional RRH deployment is not precluded from Scenario-A), for single beam per Panel, in bi-directional RRH deployment for Scenario-A, the beam dwelling time can be in the range of [0.80, 1.99] seconds for UE maximum speed of 350kmph. Recommendations for 2nd round: - Discussion tentative agreement based on the necessity of bi-directional deployment in Scn-A . |

Sub-topic 1-4 Scenario-B, Uni-directional RRH

| Sub-topic 1-4 | Issue 1-4-1: Number of Beam for uni-directional RRH deployment, Scenario-B [Moderator] Options are discussed for RRH and UE side. : - For scenario-B, uni-directional, RRH parameter: - 1 beam per RRH panel - 2 beam per RRH panel - 3 beam per RRH panel - 4 beam per RRH panel - 4 beams per RRH panel with uneven separations - For scenario-B, uni-directional, UE parameter: - 1 beam per UE panel (i.e., 1 beam per UE) - 7 beams with separation ([0 7.5 15 22.5 30 37.5 45] relative angle in degree to boresight direction) on one side, 13 UE beams if consider RRHs on two sides, per UE panel - 2 beams per UE panel Tentative agreements: N/A Recommendations for 2nd round: - Need more discussion on how to proceed. |
| --- | --- |
|  | Issue 1-4-2: Beam switching point [Moderator] Beam switching point is dependent on Issue 1-4-1, which will impact the definition of channel modelling used for performance requirement. Tentative agreements: N/A Recommendations for 2nd round: - Need more discussion and based on upon Issue 1-4-1. |

Sub-topic 1-5  Scenario-B, Bi-directional RRH

| Sub-topic 1-5 | Issue 1-5-1: Schemes for Bi-directional deployment [Moderator] Similar to Scenario-A, how to solve coverage issue around RRH-site for bi-directional case is discussed. Tentative agreements: N/A Recommendations for 2nd round: - Need more discussion on how to proceed. |
| --- | --- |
|  | Issue 1-5-2: Number of Beam for bi-directional RRH deployment, Scenario-B [Moderator] Options are discussed for RRH and UE side: - For scenario-B, bi-directional, RRH parameter: - 1 beam per RRH panel - 2 beam per RRH panel - 3 beam per RRH panel - 4 beam per RRH panel - 4 beams per panel with uneven separations - For scenario-B, bi-directional, UE parameter: - 1 beam per UE panel (i.e., 1 beam per UE) - 7 beams with separation ([0 7.5 15 22.5 30 37.5 45] relative angle in degree to boresight direction) on one side, 13 UE beams if consider RRHs on two sides, per UE panel - 2 beams per UE panel Tentative agreements: N/A Recommendations for 2nd round: - Need more discussion on how to proceed. |
|  | Issue 1-5-3: Beam Dwelling time [Moderator] Since the number of RRH beams are needed to be confirmed firstly, and the benefits of bi-directional RRH deployment needs to be confirmed firstly, this issue can be discussed later. Tentative agreements: N/A Recommendations for 2nd round: - Need more discussion on how to proceed. |

Sub-topic 1-6 Comparison between Uni-/Bi-directional RRH Deployment

| Sub-topic 1-6 | Issue 1-6-1: Comparison between uni- and bi-directional RRH deployment [Moderator] We see different views toward bi-directional RRH deployment. Furthermore, the intention of comparison between uni- and bi-directional deployment needs clarification: (1) If companies’ can’t be fully aligned with narrow-down to one option, either uni- or bi-directional, seems it is straightforward to keep both options. (2) From RAN4 requirement perspective, we need to consider applicability rule and/or how to define requirement to enable UE can work in both uni-/bi-directional RRH deployment scenarios, if that is our target. Tentative agreements: N/A Recommendations for 2nd round: - Need more discussion on how to proceed. |
| --- | --- |

Sub-topic 1-7 Signaling

| Sub-topic 1-7 | Issue 1-7-1: Necessity of Signaling [Moderator] Have the following suggestion for how to drive the discussion on signalling: Deployment scenario session focus on the feasibility of each scenario, in which gNB and UE parameters are studied to give the insight of how the system should be designed to enable such FR2 HST operation. - For the necessity and how to define signalling for RRM operation, it is discussed in RRM session. - For the necessity and how to define signalling for Demod operation, it is discussed in Demod session. Tentative agreements: N/A Recommendations for 2nd round: - Discuss more on how to proceed. |
| --- | --- |

Sub-topic 1-8 Other Issues

| Sub-topic 1-8 | Issue 1-8-1: Track curvature and impact on RRH separation [Moderator] Suggest the proponent of O1 and O2 to give more specific proposals and also suggestion how to proceed. Tentative agreements: N/A Recommendations for 2nd round: - Discuss more on how to proceed. |
| --- | --- |
|  | Issue 1-8-2: Consider 1 RRH site per BBU [Moderator] Company suggest to also consider regular (non-SFN/non-DPS) deployment with 1 RRH site per BBU. This could be considered together with the handover issue in Sub-topic 1-2. Tentative agreements: - RAN4 to consider regular (non-SFN/non-DPS) deployment with 1 RRH site per BBU: - To be considered with low priority; - Analysis baseline is still 4 RRH site per BBU Recommendations for 2nd round: - Discuss the above tentative agreement. |
|  | Issue 1-8-3: High difference in propagation delays [Moderator] No company propose opposite view against P1, Tentative agreement is drafted based on P1 and comment from QC. Tentative agreements: - RAN4 to elaborate further on which deployments and propagation schemes are exposed to the very different propagation delays. - Qquantitively evaluate the implications in these scenarios both from the demodulation and RRM perspectives. - RAN4 needs to study the scheme to alleviate ISI and the requirement should take signal power degradation into consideration. Recommendations for 2nd round: - Discuss the above tentative agreement. |
|  | Issue 1-8-4: Dedicated network for roof-mounted CPE [Moderator] Based on the P1, it is discussed that the network is dedicated for roof-mounted CPE or not. Tentative agreements: - RAN4 to assume that in HST FR2 Scenario A, only high-speed CPEs installed on the roof of the train can be present in the network. Recommendations for 2nd round: - Since there is one company had different view, suggest to FFS tentative agreement in 2nd round. |
|  | Issue 1-8-5: Handheld UE for FR2 HST [Moderator] One company propose to consider handheld UE for FR2 HST with low priority. Tentative agreements: N/A Recommendations for 2nd round: - Suggest to still follow WID’s scope on roof-mounted CPE. |
|  | Issue 1-8-6: The ability of Multi-beam RX or TX at UE [Moderator] One company propose to consider the ability of multi-beam RX or TX at UE. Tentative agreements: N/A Recommendations for 2nd round: - Suggest to still follow WID’s scope and not consider multi-beam RX or TX at UE. |
|  | Issue 1-8-7: Presence of regular UE in the network [Moderator] Based on the discussion, several company think presence of regular UE in the network is possible. Tentative agreements: - RAN4 to clarify based on the operators’ input if regular (i.e., low-speed non-HST) UEs can be connected to the same cell together with a HST CPE moving at maximum speed. Recommendations for 2nd round: - Suggest to agree tentative agreement for further study. |

## Discussion on 2nd round (if applicable)

[Moderator] Suggest companies to have the discussion and comments on the following table for WF on deployment scenarios for 2nd round.

| Pages/Topics | Comment Collection |
| --- | --- |
| General Assumption (Page 2-3) | Nokia: In our opinion, JT/Full-SFN scheme cannot be excluded completely fro HST FR2 due to its benefits. We also can agree to keep FFS: the use of JT/Full-SFN for HST FR2. Nokia: We think that it is importatnt to update the Note on the uni-directional operatoin that was not corect in the previous WF. Taking into account that there is an agreement that that "CPE to be equipped with two panels pointed forward and backward along the track", the explanation about another RRH panel is not needed. |
|  | Samsung: for Nokia’s comment on JT, we prefer this to be discussed in GTW, because I think if precluding JT is confirmed in this meeting, that will be a big and important progress, and till now we don’t see technical analysis for how JT works, not comparable to what companies did for DPS. Samsung: for the note, I add more description to make sure people know what note we are talking about here. |
| Scenario-A, Uni-directional (Page 4) | Intel: We think that it is importatnt to update the Note on the uni-directional operatoin that was not corect in the previous WF. Taking into account that there is an agreement that that "CPE to be equipped with two panels pointed forward and backward along the track", the explanation about another RRH panel is not needed. |
|  | Samsung: We can’t agree on this revision, because it is revision on GTW’s agreement. Furthermore, I don’t think the agreement preclude any further discussion on RRM session. On the contrary, the deployment scenario discussion here will impact RF, RRM and Demod requriements. I need to revert your revision here. Pls. let me know if you still have concern on using original GTW agreement. |
| Scenario-A, Uni-directional (Page-5, switching point) | Nokia: In our opinion, Ds_offset value can be discussed only in terms of channle model for perfromance requirements. RRH switching point cannot be defined as a part of deployment. It is result of deployment that also depend on a lagre number of many other factors. |
|  | Huawei: Ds_offset value is FFS, no need to limit to the specific range. |
|  | Samsung: Nokia’s proposed definition of RRH switching point is okay for us, and I just remove “(nearest)” because for UE Moving toward RRH, the target RRH is not nearest RRH. Nokia’s comment on Ds_offset is okay to us. Samsung: Huawei’s removal of Ds_offset range is okay to us for waiting more companies’ input in next meeting. |
| Scenario-A, Uni-directional (Page-6, Handover issue) | Ericsson: We have agreed to assume Ds = 700m for this work. If another scenario with smaller Ds is to be considered, then it should be proposed and added to the list of scenarios Ericsson: We haven't seen this. The neighbour cell coverage is good for all of the H0 distance in our modelling. So we do not think that the WF should state that there is such a lack of coverage Ericsson: We have not seen any handover issue; so there should first be a confirmation if an issue exists before discussing how to resolve it. |
|  | Nokia: Agree with Ericsson. We do not oberve Handover issue in uni-direction Scenario A. Good coverage can be easily observed from the propogation maps. HO does not need to happen in the area next to the RRH site (CID=2). |
|  | Samsung: Ericsson’s revision (current version) is okay for us. |
| Scenario-A, Bi-directional (Page 7) | Huawei: We should objectively analyze possible pros and cons for one specific deployment, not only provide benefit of one deployment scenario compared to another one. |
|  | Samsung: Huawei’s revision on bi- vs. uni- is okay to us. Samsung: we still suggest to have the sentence to describe Scheme-1 and 2 on the pages, otherwise it is not clear. I have the revision on this sentence to make it is more like an observation. |
| Scenario-A, Bi-directional (Page 8) | Company-A: |
|  | Company-B: |
| Scenario-B, Uni-directional (Page 9) | Intel: Same comment as for Scenario-A: We expect that the use of the second panel in uni-directional deployment is a corner case: CPE needs to use it for beam search only when it enters the network or deployment suddenly changes it's direction of service. We see that it is RRM related issue. RRM session already has discussion on how to handle cases which are not typical for HST FR2 CPE (e.g. IDLE mode, DRX mode). We think that the the issue of the number of panels for beam search can also be further discussed in RRM session. Suggest to send current agreement to RRM session as it is |
|  | Nokia: Same comment as in scenario A. |
|  | Qualcomm: Add 7 UE beams option to page 9 and 10 |
|  | Samsung: For FFS to “further discuss in RRM session”, same wording should be used as Page-4. |
| Scenario-B, Bi-directional (Page 10) | Huawei: whether the coverage is an issue or not is dependent on the specific deployment scheme |
|  | Samsung: If we want to open the discussion for Scheme-2 and/or 3, we prefer to also consider Scheme-1. |
| Scenario-B, Bi-directional (Page 11 – Originally contained in Page-10) | Ericsson: We are also not yet convinced of the benefits of bi-directional for scenario B (but realize that there may be more likelihood of benefits than scenario A). The benefits should be confirmed. |
|  | Huawei: As per our analysis, there is some benefits for bi-directional deployment comparing to uni-directional deployment. e.g. There is larger timing jump when UE performs beam switching that is a very serious issue that needs to be handled for Uni-directional. |
|  | Qualcomm: Add 7 UE beams option to page 9 and 10 |
| Necessity of signaling (Page 12 – Originally Page-11) | Nokia: Exacrly the same issues are already in discussion in the RRM thread. It will only add confusion if similar discussions continues in two different treads. We propose to focus only on the Demod aspect here. |
|  | Samsung: Again, this deployment scenario here is not only for Demod. I don’t believe FFS UE capability signaling can be removed. |
| Others (Page 13 – Originally Page-12) | Ericsson: It is not so obvious where this ISI comes from if we do not have JT ? So reworded to firstly check whether there is a scenario with ISI. There is a need to deal with propagation delay changes for uni-directional. |
|  | Samsung: Similar view as Ericsson. Current wording from Ericsson seems a compromise for companies to FFS. |
| Others (Page 14 – Originally Page-13) | Intel: We still have concerns on that. We agree that the requirements should be defined for roof-mounted CPEs only. But in real network there can be other FR2 devices in the train. Typical placement of handheld devices in the train is near the windows. The penetration loss for windows is not so crucial, so they may be able to connect to the network. How will the network recognize them and differentiate from CPEs without any capability signalling? Intel: As we mentioned in the comment above, even though there are no LOW-SPEED NON-HST UEs in the network, FR2 UEs (like handheld devices) can still be present IN THE TRAIN and can connect to the network. We don't have such term as "roof-mounted CPE" in the spec so the ways to differentiate such UEs from other FR2 UEs need to be clarified. The most straightforward way is to define UE capability for HST FR2 support. Or if it is some cell barring mechanism, it shoiuld also be clarified and mentioned in WF. |
|  | Samsung: Reply to Intel’s 1st comment, NW at least can bar other handheld UE from accessing just like one operator’s NW can bar other operators’ UE to access, but it is obviously out of RAN4 scope. For the bullet “RAN4 focus on roof-mounted CPE in Rel-17 WI.”, is there specific proposal on how to change this? |

Furthermore, some general comments from Nokia are provided as follows:

- Issues 1-1-4: Necessity of JT in Scenario-A/B, Uni/Bi-directional RRH
[Nokia] Nokia comment in the first round was not taken into account. We still see some benefits in JT/Full-SFN scheme, e.g., simplified and transparent mobility in a cell.
We prefer to keep this Issue open for the next meeting with the following options:

  - Option 1: Joint Transmission (full SFN) is precluded

  - Option 2: Joint Transmission (full SFN) can be used in HST FR2

- Issues 1-2-2: Uni-directional operation
[Nokia] The Note was formulated in such a way because it was not agreed if the use of one panel oriented upwards is feasible. Based on the agreement in the issues 1-1-2 that "CPE to be equipped with two panels pointed in opposite directions", we would like to propose the following update to the Note:

  - “RAN4 focuses on 1 direction 1 train, but we are aware of the fact that either another panel to serve train towards the other direction is needed or a CPE can use one of its panels from the appropriate direction. If this opposite direction is completely symmetric, the 1 direction study can apply directly.”

- Issue 1-2-4: Beam switching point
[Nokia] We still do not understand why it should be an agreement on the switching point in the deployment. It can only be discussed as a mode parameter.
Hence we suggest to modify the definition:

  - Ds_offset is the channel model parameter describing the switching point where the SNR from the target RRH (currently closest RRH) beam exceeds the SNR from the source RRH beam, illustrated in below figure.

- Issue 2-2-1 and Issue 2-3-1:
[Nokia] In our understanding, sing-tap is not fading model. Therefore, the tentative agreement in 2-2-1 is not fully correct.
We are proposing to join the Issues with a sigle WF:

  - The single tap propagation model can be assumed for each single Tx-Rx link for both scenario-A and scenario-B.

- Issue 2-4-1: Consider JT model for DL? 
[Nokia] As far as we do not accept that the agreement in the Issue 1-1-4 is achieved, one additional Option has to be added:

  - Option 2: Use both DPS and SFN channel models for both Uni-directional/Bi-directional

- Issues 3-1-1: Uplink maximum speed feasibility study and requested RS configuration
[Nokia] Based on our simulation results, it is not always feasible to support maximum speed with 260km for Uplink only DMRS configuration without PT-RS configuration for frequency offset tracking (Table 2 in R4-2106916).
Our observation was not listed correctly in the Issue. We were referring only to the uni-directional setting when observed that 260 kmph speed can be supported without PT-RS. 
We cannot agree with the statement above.

# Topic #2: Channel Modeling

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2104678 | Ericsson | Proposal 1: Adopt RMa pathloss model for scenario 2. Proposal 2: Adopt the single tap fading model for scenario 2. |
| R4-2105025 | Samsung | Proposal-1: If there is no further evidence from measurement campaign, RAN4 choose TS38.901 RMa LoS pathloss model used for link budget evaluation for Scenario-B. Proposal-2: The single-tap can be assumed for a single TX-RX link for Scenario-B. Observation-1: As a candiate channel profile for bi-directional RRH deployment sceanrio, the single tap channel profile is obtained by applying the parameters of Scenario-A and B into the profile in TS38.101-4 B.3.1. Observation-2: For HST-DPS for uni-directional RRH deployment, the Doppler shift trajectory is dependent on the switching point configured between two RRHs. Proposal-3: The switching point between two RRHs are assumed as the Table 1. Table 1. Switching point between two RRHs (Ds_offset is defined as below Figure 2/4) UE moving towards serving beam; UE moving away from serving beam. Scenairo-A (Ds = 700m , Dmin = 10m); Ds_offset = 700 + 40 (meter); Ds_offset = 40 (meter). Scenario-B (Ds = 700m , Dmin = 150m); Ds_offset = 700 + 370 (meter); Ds_offset = 370 (meter). Proposal-4: For HST-DPS channel for uni-directional RRH deployment (Alt-1, UE moving towards serving beam), the cosine of angle θt used in Doppler shift fst=fd cosθt is provided as below: - cosθt=Ds_offset-vtDmin2+Ds_offset-vt2, 0<t≤Dsv (eq. 1) - cosθt=cosθt modDsv, t>Ds/v (eq. 2) where Ds≤Ds_offset<2Ds (eq. 3) Observation-3: Doppler shift trajectory is demonstrated in Fig.3 for HST-DPS Alt-1: UE Moving towards Serving Beam. Proposal-5: For HST-DPS channel for uni-directional RRH deployment (Alt-2, UE moving away from serving beam), the cosine of angle θt used in Doppler shift fst=fd cosθt is provided as below: - cosθt=-Ds_offset+vtDmin2+Ds_offset+vt2, 0<t≤Dsv (eq. 4) - cosθt=cosθt modDsv, t>Ds/v (eq. 5) where 0≤Ds_offset<Ds (eq. 6) Observation-4: Doppler shift trajectory is demonstrated in Fig.4 for HST-DPS Alt-2: UE Moving Away from Serving Beam. Observation-5: For HST-DPS for bi-directional RRH deployment, the Doppler shift trajectory is dependent on the beam management scheme to be concluded from deployment scenario study. Proposal-6: For HST-DPS channel for HST-DPS for bi-directional deployment, the cosine of angle θt used in Doppler shift fst=fd cosθt is provided as below: - cosθt=Ds-vtDmin2+Ds-vt2, 0<t≤(0.5*Ds)/v (eq. 7) - cosθt=vtDmin2+vt2, (0.5*Ds)/v<t≤Ds/v (eq. 8) cosθt modDsv, t>Ds/v (eq. 9) Observation-6: If we assume UE is served by 2nd-nearest RRH, Doppler shift trajectory is demonstrated in Fig.6 for FR2 HST bi-directional RRH deployment. Proposal 7: Channel models for uplink and downlink performance evaluation are proposed as follows: Table 2. Proposed Channel Model Selection for UL and DL Performance Evaluation Scenario (applicable to both A&B); Uplink; Downlink. Bi-directional RRH Deployment; Single Tap Channel for FR2 HST; HST-DPS Channel for FR2 HST Bi-Directional RRH Deployment. Uni-directional RRH Deployment; HST-DPS Channel for FR2 HST Uni-Directional RRH Deployment: Alt-1: UE Moving towards Serving Beam. |
| R4-2106828 | Huawei, HiSilicon | Proposal 1: Use RMa Los pathloss model for Scenario B. Proposal 2: Use DPS channel model for both Uni-directional/Bi-directional and not consider SFN JT channel model for performance requirements. |
| R4-2106911 | Nokia, Nokia Shanghai Bell | On the Sceanio-B channel mode for link budget evaluation: Observation 1: LoS conditions can be assumed in the areas where the train is allowed to move at the maximum speeds that are evaluated in the WI. Proposal 1: RAN4 to choose TS38.901 RMa LoS pathloss model also for the evaluation of Scenario-B. On channel models for performance requirements in UL: Observation 2: Only single TX-RX link is used for UL transmission from CPE. This link is LoS and, hence, has only one strongly dominating path. Proposal 2: RAN4 to consider only single-tap propagation model for BS performance requirements, both in Scenario-A and Scenario-B. Observation 3: Doppler shift trajectories proposed for Single-tap high-speed train channel conditions in FR1 describe bi-directional setting. Historically, the model maintains the continuity of the frequency offset and avoids the alternation of Doppler shift sign when handing over from one RRH to another. However, we see it more realistic to have the alternation of the Doppler shift sing at RRH site change. This also makes sense for comparability between uni-direction and bi-directional channel models. Proposal 3: RAN4 to modify the single-tap propagation channel model for HST FR2 in UL to take into account the Doppler shift sign alternation in bi-directional setting when CPE is handing over from one RRH site to another. Observation 4: In uni-directional HST FR2 setting, the signal is always coming to the CPE from one direction. Doppler shift does not change the sign when CPE switches from one RRH to another. Hence, a different single-tap prorogation conditions should be considered in uni-directional setting. Below, we adapt the single-tap high-speed train channel conditions for uni-directional setting. The cosine of angle θt is given by: cosθt=Ds2-vtDmin2+Ds2-vt2, 0≤t≤Ds2v cosθt=1.5Ds-vtDmin2+1.5Ds-vt2, Ds2v<t≤Dsv cosθt= cosθt mod(Dsv), t>Dsv, where the initial distance of the train from RRH site over the railways track is Ds2 , and Ds is the distance between RRH sites, Dmin is RRH site - railway track distance, both in meters; v is the velocity of the train in m/s, t is time in seconds. Proposal 4: RAN4 to use single-tap propagation channel, as described above, in HST FR2 uni-directional setting for UL. On channel models for performance requirements in DL: Observation 5: A single-tap, SFN, and DPS propagation models were introduced in HST FR1 for DL. FR1 SFN channel profile cannot be re-used directly in FR2 because omni-directional transmission and reception cannot be assumed. In general, a larger variety of channel profiles can be considered in HST FR2: uni-directional JT, bi-directional JT, uni-directional DPS, uni-directional DPS. Proposal 5: RAN4 to decide which of the channel profiles (uni-directional JT, bi-directional JT, uni-directional DPS, uni-directional DPS) shall be considered for the CPE performance requirements. Proposal 6: Consider only two simultaneously received taps (i.e., only signals from two transmitting RRHs) in JT channel profile. Observation 6: DPS model in FR2 is nothing else than a single-tap channel profile with Doppler sign alternation at RRH site change. Proposal 7: RAN4 to modify the single-tap propagation channel model for HST FR2 in DL to take into account the Doppler shift sign alternation in bi-directional setting when CPE is handing over from one RRH site to another. Use this model in bi-directional DPS setting. Similarly to UL, we can adapt the single-tap high-speed train channel conditions for uni-directional setting. The cosine of angle θt is given by: cosθt=Ds2-vtDmin2+Ds2-vt2, 0≤t≤Ds2v cosθt=1.5Ds-vtDmin2+1.5Ds-vt2, Ds2v<t≤Dsv cosθt= cosθt mod(Dsv), t>Dsv, where the initial distance of the train from RRH site over the railways track is Ds2 , and Ds is the distance between RRH sites, Dmin is RRH site - railway track distance, both in meters; v is the velocity of the train in m/s, t is time in seconds. Proposal 8: RAN4 to use single-tap propagation channel, as described above, in HST FR2 uni-directional setting for DL. |
| R4-2106865 (Moved from AI 8.7.5) | Ericsson | Proposal 1: RAN4 specify the unidirectional HST single tap channel model for UE/BS demodulation requirements for HST FR2 as follows: fduni(t)=fmaxcosθt cosθt=αDs+vtDmin2+αDs+vt2, 0≤t≤Dsvcosθt modDsv, t>Dsv Where fmax (Hz) is the maximum Doppler frequency, Ds (m) is the distance between RRHs, Dmin (m) is the distance between RRH and railway track, v (m/h) is the velocity of the train, and α>0. Proposal 2: If RAN4 use the unidirectional HST single tap model, RAN4 should discuss further the parameters, i.e., fmax, Ds, Dmin, v, and α, according to the conclusion of the deployment scenario discussion. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1 Channel Model for Scenario-B Link Budget Analysis

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 2-1-1: Channel Model for Scenario-B Link Budget Analysis

- Proposals

  - Proposal 1 (Ericsson, Nokia, Samsung, Huawei): RAN4 to choose TS38.901 RMa LoS pathloss model also for the evaluation of Scenario-B.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

### Sub-topic 2-2 Fading model for a single TX-RX link in Scenario-B

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 2-2-1: One-Tap fading model for a single TX-RX link in Scenario-B

- [Moderator] Issue 2-2-1 is related to Issue 2-1-1, and if LoS pathloss is adopted for Scenario, it is straightforward to use a one tap fading model to represent a single TX-RX link. This is the basis for the follow-up discussion on channel modelling for performance requirement, i.e., if one tap fading model is realistic enough to present a single TX-RX link in Scenario-B (same as Scenario-A), then the following model selection will make sense for performance requirement among (a) modified single-tap model, (b) modified HST-DPS model, (c) others.

- Proposals

  - Proposal 1 (Ericsson, Samsung): The single-tap can be assumed for a single TX-RX link for Scenario-B.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

### Sub-topic 2-3 Uplink Channel Model for Performance Requirement

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 2-2-1: Channel model for BS performance requirements

- Proposals

  - Proposal 1 (Nokia): RAN4 to consider only single-tap propagation model for BS performance requirements, both in Scenario-A and Scenario-B.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 2-3-1: Channel Model for Uplink Uni-directional RRH deployment

- Proposals

  - Proposal 1 (Nokia): Use single-tap propagation channel for UL uni-directional RRH deployment, as described below:

cosθt=Ds2-vtDmin2+Ds2-vt2, 0≤t≤Ds2v

cosθt=1.5Ds-vtDmin2+1.5Ds-vt2, Ds2v<t≤Dsv

cosθt= cosθt mod(Dsv), t>Dsv,

  - Proposal 2 (Samsung, Ericsson (based on R4-2106865)): HST-DPS Channel for FR2 HST Uni-Directional RRH Deployment: Alt-1: UE Moving towards Serving Beam the cosine of angle θ(t)  used in Doppler shift fst=fd cosθt is provided as below

- cosθt=Ds_offset-vtDmin2+Ds_offset-vt2,  0<t≤Dsv 											(eq. 1)

- cosθt=cosθt modDsv,  t>Ds/v  											(eq. 2)

where Ds≤Ds_offset<2Ds    												(eq. 3)

|  | UE moving towards serving beam |
| --- | --- |
| Scenairo-A (Ds = 700m , Dmin = 10m) | Ds_offset = 700 + 40 (meter) |
| Scenario-B (Ds = 700m , Dmin = 150m) | Ds_offset = 700 + 370 (meter) |

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 2-3-2: Channel model for Uplink Bi-directional RRH deployment

- Proposals

  - Proposal 1 (Nokia): RAN4 to modify the single-tap propagation channel model for HST FR2 in UL to take into account the Doppler shift sign alternation in bi-directional setting when CPE is handing over from one RRH site to another.

cosθt=Ds2-vtDmin2+Ds2-vt2, 0≤t≤Dsv,

cosθt= cosθt mod(Dsv), t>Dsv.

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
|  | Scenario-A-260 | Scenario-A-350 | Scenario-B-260 | Scenario-B-350 |
| Ds | 700 m | 700 m | 700 m | 700 m |
| Dmin | 10 m | 10 m | 150 m | 150 m |
| v | 260 km/h | 350 km/h | 260 km/h | 350 km/h |
| fd | 14454 Hz | 19458 Hz | 14454 Hz | 19458 Hz |

  - Proposal 2 (Samsung): Reuse Single Tap Channel in TS38.104 for FR2 HST by updating parameters.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

### Sub-topic 2-4 Downlink Channel Model for Performance Requirement

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 2-4-1: Consider JT model for DL?

- Proposals

  - Proposal 1 (Nokia): RAN4 to decide which of the channel profiles (uni-directional JT, bi-directional JT, uni-directional DPS, uni-directional DPS) shall be considered for the CPE performance requirements.

  - Proposal 1a (Nokia): If JT adopted, consider only two simultaneously received taps (i.e., only signals from two transmitting RRHs) in JT channel profile.

  - Proposal 2 (Huawei, Samsung): Use DPS channel model for both Uni-directional/Bi-directional and not consider SFN JT channel model for performance requirements.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 2-4-2: Channel model for Downlink Uni-directional RRH deployment

- Proposals

  - Proposal 1 (Nokia): Use single-tap propagation channel for DL uni-directional RRH deployment, as described below:

cosθt=Ds2-vtDmin2+Ds2-vt2, 0≤t≤Ds2v

cosθt=1.5Ds-vtDmin2+1.5Ds-vt2, Ds2v<t≤Dsv

cosθt= cosθt mod(Dsv), t>Dsv,

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
|  | Scenario-A-260 | Scenario-A-350 | Scenario-B-260 | Scenario-B-350 |
| Ds | 700 m | 700 m | 700 m | 700 m |
| Dmin | 10 m | 10 m | 150 m | 150 m |
| v | 260 km/h | 350 km/h | 260 km/h | 350 km/h |
| fd | 7227 Hz | 9729 Hz | 7227 Hz | 9729 Hz |

  - Proposal 2 (Samsung, Ericsson (based on R4-2106865)): HST-DPS Channel for FR2 HST Uni-Directional RRH Deployment: Alt-1: UE Moving towards Serving Beam the cosine of angle θ(t)  used in Doppler shift fst=fd cosθt is provided as below

- cosθt=Ds_offset-vtDmin2+Ds_offset-vt2,  0<t≤Dsv 											(eq. 1)

- cosθt=cosθt modDsv,  t>Ds/v  											(eq. 2)

where Ds≤Ds_offset<2Ds    												(eq. 3)

|  | UE moving towards serving beam |
| --- | --- |
| Scenairo-A (Ds = 700m , Dmin = 10m) | Ds_offset = 700 + 40 (meter) |
| Scenario-B (Ds = 700m , Dmin = 150m) | Ds_offset = 700 + 370 (meter) |

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 2-4-3: Channel model for Downlink Bi-directional RRH deployment

- Proposals

  - Proposal 1 (Nokia): RAN4 to modify the single-tap propagation channel model for HST FR2 in DL to take into account the Doppler shift sign alternation in bi-directional setting when CPE is handing over from one RRH site to another. Use this model in bi-directional DPS setting.

  - cosθt=Ds2-vtDmin2+Ds2-vt2, 0≤t≤Dsv,

cosθt= cosθt mod(Dsv), t>Dsv.

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
|  | Scenario-A-260 | Scenario-A-350 | Scenario-B-260 | Scenario-B-350 |
| Ds | 700 m | 700 m | 700 m | 700 m |
| Dmin | 10 m | 10 m | 150 m | 150 m |
| v | 260 km/h | 350 km/h | 260 km/h | 350 km/h |
| fd | 7227 Hz | 9729 Hz | 7227 Hz | 9729 Hz |

  - Proposal 2 (Samsung): HST-DPS Channel for FR2 HST Bi-Directional RRH Deployment. the cosine of angle θt used in Doppler shift fst=fd cosθt is provided as below:

- cosθt=Ds-vtDmin2+Ds-vt2,  0<t≤(0.5*Ds)/v 											(eq. 7)

- cosθt=vtDmin2+vt2,  (0.5*Ds)/v<t≤Ds/v 											(eq. 8)

cosθt modDsv,    t>Ds/v    												(eq. 9)

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

## Companies views’ collection for 1st round

### Open issues

Sub topic 2-1

| Company | Comments |
| --- | --- |
| ZTE | Issue 2-1-1: Channel Model for Scenario-B Link Budget Analysis Agree |
| Intel | Agree with proposal 1. |
| Samsung | As proponent of P1, support P1. |
| Huawei | We are OK with the Proposal 1. |

Sub topic 2-2

| Company | Comments |
| --- | --- |
| Ericsson | Issue 2-2-1: Channel model for BS performance requirements We agree to proposal 1 |
| Intel | We support proposal that for both scenarios A and B propagation condition with single tap can be assumed. |
| Nokia, Nokia Shanghai Bell | Issue 2-2-1: One-Tap fading model for a single TX-RX link in Scenario-B In general, we agree with the proposal, however, we prefer to discuss needed the channel models in separate issues, e.g., as proposed in the Issues 2-2-1 for UL and Issues 2-4-1 in DL. |
| QC | We can agree with single tap model for link budget analysis, since link budget analysis is for coverage discussion. However, for performance requirement in demod, a separate discussion is needed. Performance analysis is based on the entire trajectory instead of focusing on the farthest point the RRH can reach UE. |
| Smsung | Here is our understanding for these channel model related issues: - LoS pathloss needs to be confirmed for both Scenario A and B, then it is assumed that one tap channel btw one RRH and UE. - Whether SFN or DPS will be used, will impact channel modelling for demodulation. If SFN is out of the scope, RAN4 need to define demod channel for DPS-based scheme. How to define demodulation channel modelling is another story to be discussed in Sub-topic 2-3 and 2-4. |
| Huawei | We are OK with the Proposal 1. |

Sub topic 2-3

| Company | Comments |
| --- | --- |
| Ericsson | Issue 2-3-2: Channel model for Uplink Bi-directional RRH deployment We should clarify whether bi-directional is really beneficial. If we do bi-directional requirements, either approach can work. |
| ZTE | Issue 2-2-1: Channel model for BS performance requirements Agree (issue index needs to be updated) Issue 2-3-1: Channel Model for Uplink Uni-directional RRH deployment Proposal 2 is preferred |
| Intel | Issue 2-3-1: Channel model for BS performance requirements Similar to issue 2-2-1 we support proposal 1. Issue 2-3-2: Channel Model for Uplink Uni-directional RRH deployment In order to agree on exact channel model, we need to discuss two main aspects: 1. Should we assume continuous or non-continuous Doppler profile in UL direction? 2. In which point we should assume UE switch operation from one RRH to another? Regardless of Tx scheme UE will instantly switch Tx frequency when switching from one RRH to another is performed. In this case we should also assume instant change of UL RX frequency. Therefore, non-continuous Doppler frequency profile should be assumed for UL channel model. Both proposal 1 and 2 if we properly understood them, suggest considering non-continuous Doppler frequency profile (HST DPS like channel model). The difference between these two options is starting point and switching point. We suggest further discuss whether some switching point offset should be introduced to the actual channel model profile or not, Issue 2-3-3: Channel model for Uplink Bi-directional RRH deployment Support proposal 1. It should be non-continuous Doppler frequency profile due to UE switching from one RRH to another. |
| Nokia, Nokia Shanghai Bell | Issue 2-3-1: Channel Model for Uplink Uni-directional RRH deployment We think that the model from Proposal 2 makes sense as well. It is necessary to analyse further the proposed values of Ds_offset and if the difference to the case when UE is moving against serving beams. |
| Samsung | Issue 2-3-1: Channel model for BS performance requirements Need to differentiate the discussion between uni- and bi-direcitonal deployment. To combine with the following two Issues together. Issue 2-3-2: Channel Model for Uplink Uni-directional RRH deployment As we proposed in P2, non-continuous Doppler shift profile (like HST DPS) is desired. Switching point can be decided based on deployment scenario discussion, but based on our current evaluation, switching point’s impact on performance is limited. Regarding P1, we would like to clarify whether these two equations is experienced by the same severing RRH. What is the switching point for different RRH? Issue 2-3-3: Channel model for Uplink Bi-directional RRH deployment If we follow similar logic as FR1, the single tap model used in TS38.104 (with modified parameters) can also be served to evaluate performance results. |
| Huawei | Issue 2-3-1: Channel Model for Uplink Uni-directional RRH deployment We think the Proposal 2 is generally OK, but the offset value should be changed if needed when the beam orientation is finally decided. Issue 2-3-2: Channel model for Uplink Bi-directional RRH deployment We prefer to use Option 2. |

Sub topic 2-4

| Company | Comments |
| --- | --- |
| Ericsson | Issue 2-4-1: Consider JT model for DL? Proposal 2. JT is not feasible for unidirectional due to the time difference between the paths and not beneficial (and requiring two panel operation at the same UE) for bi-directional. |
| ZTE | Issue 2-4-1: Consider JT model for DL? JT(full SFN) is faced with propagation delay difference and doppler shift hopping so DPS is prioritized. |
| Intel | Issue 2-4-1: Consider JT model for DL? We support Proposal 2 because we do not see any value to configure JT scheme in HST FR2 deployments. We also should discuss which scenarios should be adopted for requriements definition: unidirectional or bidirectional. We do not see big difference between them from performance verification. Slightly prefer unidirectional scenario. Issue 2-4-2: Channel model for Downlink Uni-directional RRH deployment Regardless of Tx scheme Doppler frequency profile will be non-continuous in DL. In this case both proposals are aligned. Same time proposal 2 suggest considering some practical switching point. We can go with proposal 2 and further discuss exact offset value. It can be 0 if we agree that there is no need to consider it at all for performance verification. Issue 2-4-3: Channel model for Downlink Bi-directional RRH deployment Actual Doppler frequency profile depends on beam management assumptions and both proposals are valid but correspond to different beam management approaches. We suggest discuss this issue after outcome of deployment study. |
| Nokia, Nokia Shanghai Bell | Issue 2-4-1: Consider JT model for DL? This discussion should be based on the conclusions of HST FR2 Deployment in Topic 1. If JT scheme is agreed to be relevant to HST FR2, we do not see reasons to exclude corresponding model from pefromance requirements. Issue 2-4-2: Channel model for Downlink Uni-directional RRH deployment Similar comment as in Issue 2-3-1. Issue 2-4-3: Channel model for Downlink Bi-directional RRH deployment The difference in the equations between Proposal 1 and Proposal 2 might be just due to the different starting point of the Doppler trajectory. In our proposal, the CPE starts at point Ds/2 that is also used in the HST FR1 single-tap propagation model. |
| QC | Issue 2-4-1: Consider JT model for DL? We can support proposal 2, but how many beams and how to test multiple beams with different AoA in DPS should be discussed, maybe in demod agenda. Issue 2-4-2: Channel model for Downlink Uni-directional RRH deployment The Doppler shift model should be identical to FR1 single tap model with corresponding Ds and Dmin, no additional model is needed. In FR2, the key issue is whether and how to test multiple beams with different AoA. Doppler shift has relatively minor impact on performance. |
| Samsung | Issue 2-4-1: Consider JT model for DL? We support Proposal 2 as proponent. Issue 2-4-2: Channel model for Downlink Uni-directional RRH deployment P2 as proponent, but the switching point can be further refined based on other companies’ input. Regarding P1, we would like to clarify whether these two equations is experienced by the same severing RRH. What is the switching point for different RRH? Issue 2-4-3: Channel model for Downlink Bi-directional RRH deployment P2 as proponent, and the channel model in P2 is similar to HST-DSP for FR1. |
| Huawei | Issue 2-4-1: Consider JT model for DL? We don’t think SFN JT is appropriate considering the large Doppler spread and large delay spread to be handled. Issue 2-4-2: Channel model for Downlink Uni-directional RRH deployment Same as Issue 2-3-1, we think the Proposal 2 is generally OK, but the offset value should be changed if needed when the beam orientation is finally decided. Issue 2-4-3: Channel model for Downlink Bi-directional RRH deployment The channel model should be settled after the beam orientation is settled, maybe different number of switching points for different beam design. We prefer to discuss this issue later. |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2105025 (TP to TR 38.854) | Samsung: As drafter of this TP, we suggest to approve to capture agreement from last meeting. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#2-1 | Issue 2-1-1: Channel Model for Scenario-B Link Budget Analysis [Moderator] For Issue 2-1-1 and below Issue 2-2-1, here is our understanding: (1) LoS pathloss needs to be confirmed for both Scenario A and B (already confirmed for A in last meeting), then it is assumed that one tap channel btw one RRH and UE. (2) Whether SFN or DPS will be used, will impact channel modelling for demodulation. If SFN is out of the scope, RAN4 need to define demod channel for DPS-based scheme. (3) How to define demodulation channel modelling is another story to be discussed in Sub-topic 2-3 and 2-4. For this Issue, it is no other views different from P1 received. Tentative agreements: - RAN4 to choose TS38.901 RMa LoS pathloss model also for the evaluation of Scenario-B. Recommendations for 2nd round: - Agree the tentative agreement. |
| Sub-topic#2-2 | Issue 2-2-1: One-Tap fading model for a single TX-RX link in Scenario-B [Moderator] Issue 2-2-1 is related to Issue 2-1-1: if LoS pathloss is adopted for Scenario, it is straightforward to use a one tap fading model to represent a single TX-RX link. This is the basis for the follow-up discussion on channel modelling for performance requirement, i.e., if one tap fading model is realistic enough to present a single TX-RX link in Scenario-B (same as Scenario-A), then the following model selection will make sense for performance requirement among (a) modified single-tap model, (b) modified HST-DPS model, (c) others. Tentative agreements: - The single-tap can be assumed for a single TX-RX link for Scenario-B. Recommendations for 2nd round: - Agree the tentative agreement. |
| Sub-topic#2-3 | Issue 2-3-1: Channel model for BS performance requirements [Moderator] Views are provided on the following proposal: - Proposal-1: RAN4 to consider only single-tap propagation model for BS performance requirements, both in Scenario-A and Scenario-B. Need to be further clarify: - If P1 is related Issue 2-2-1, i.e., one tap (rather than multi-tap) is used for UL, seems Issue 2-2-1 already cover this. Tentative agreements: N/A Recommendations for 2nd round: - Further discussion based on the detailed proposed UL channel modeling below. Issue 2-3-2: Channel Model for Uplink Uni-directional RRH deployment [Moderator] Two models for UL uni-directional RRH deployment are provided. Tentative agreements: N/A Candidate options: - Option 1: Use single-tap propagation channel for UL uni-directional RRH deployment, as described below: cosθt=Ds2-vtDmin2+Ds2-vt2, 0≤t≤Ds2v cosθt=1.5Ds-vtDmin2+1.5Ds-vt2, Ds2v<t≤Dsv cosθt= cosθt mod(Dsv), t>Dsv, - Optioin 2: HST-DPS Channel for FR2 HST Uni-Directional RRH Deployment: Alt-1: UE Moving towards Serving Beam the cosine of angle θ(t) used in Doppler shift fst=fd cosθt is provided as below - cosθt=Ds_offset-vtDmin2+Ds_offset-vt2, 0<t≤Dsv (eq. 1) - cosθt=cosθt modDsv, t>Ds/v (eq. 2) where Ds≤Ds_offset<2Ds (eq. 3) UE moving towards serving beam. Scenairo-A (Ds = 700m , Dmin = 10m); Ds_offset = 700 + 40 (meter). Scenario-B (Ds = 700m , Dmin = 150m); Ds_offset = 700 + 370 (meter). Note: switching point can be FFS. Recommendations for 2nd round: - Suggest to further discuss based on the above two options. Issue 2-3-3: Channel model for Uplink Bi-directional RRH deployment [Moderator] Two models for UL bi-directional RRH deployment are provided. Tentative agreements: N/A Candidate options: - Option 1: RAN4 to modify the single-tap propagation channel model for HST FR2 in UL to take into account the Doppler shift sign alternation in bi-directional setting when CPE is handing over from one RRH site to another. cosθt=Ds2-vtDmin2+Ds2-vt2, 0≤t≤Dsv, cosθt= cosθt mod(Dsv), t>Dsv. Parameter; Value. Scenario-A-260; Scenario-A-350; Scenario-B-260; Scenario-B-350. Ds; 700 m; 700 m; 700 m; 700 m. Dmin; 10 m; 10 m; 150 m; 150 m. v; 260 km/h; 350 km/h; 260 km/h; 350 km/h. fd; 14454 Hz; 19458 Hz; 14454 Hz; 19458 Hz. - Option 2: Reuse Single Tap Channel in TS38.104 for FR2 HST by updating parameters. Recommendations for 2nd round: - Suggest to further discuss based on the above two options. |
| Sub-topic#2-4 | Issue 2-4-1: Consider JT model for DL? [Moderator] Based on the discussion so far, all companies are okay with following P2 to only consider DPS and preclude SFN JT. The discussion is similar to Issue 1-1-4. Tentative agreements: N/A - Use DPS channel model for both Uni-directional/Bi-directional and preclude SFN JT channel model for performance requirements. Recommendations for 2nd round: - Agree the tentative agreement. Issue 2-4-2: Channel model for Downlink Uni-directional RRH deployment [Moderator] Two models for UL bi-directional RRH deployment are provided. Tentative agreements: N/A Candidate options: - Option 1: Use single-tap propagation channel for DL uni-directional RRH deployment, as described below: cosθt=Ds2-vtDmin2+Ds2-vt2, 0≤t≤Ds2v cosθt=1.5Ds-vtDmin2+1.5Ds-vt2, Ds2v<t≤Dsv cosθt= cosθt mod(Dsv), t>Dsv, Parameter; Value. Scenario-A-260; Scenario-A-350; Scenario-B-260; Scenario-B-350. Ds; 700 m; 700 m; 700 m; 700 m. Dmin; 10 m; 10 m; 150 m; 150 m. v; 260 km/h; 350 km/h; 260 km/h; 350 km/h. fd; 7227 Hz; 9729 Hz; 7227 Hz; 9729 Hz. - Option 2: HST-DPS Channel for FR2 HST Uni-Directional RRH Deployment: Alt-1: UE Moving towards Serving Beam the cosine of angle θ(t) used in Doppler shift fst=fd cosθt is provided as below - cosθt=Ds_offset-vtDmin2+Ds_offset-vt2, 0<t≤Dsv (eq. 1) - cosθt=cosθt modDsv, t>Ds/v (eq. 2) where Ds≤Ds_offset<2Ds (eq. 3) UE moving towards serving beam. Scenairo-A (Ds = 700m , Dmin = 10m); Ds_offset = 700 + 40 (meter). Scenario-B (Ds = 700m , Dmin = 150m); Ds_offset = 700 + 370 (meter). Note: switching point can be FFS. Recommendations for 2nd round: - Suggest to further discuss based on the above two options. Issue 2-4-3: Channel model for Downlink Bi-directional RRH deployment [Moderator] Two models for UL bi-directional RRH deployment are provided. Tentative agreements: N/A Candidate options: - Option 1: RAN4 to modify the single-tap propagation channel model for HST FR2 in DL to take into account the Doppler shift sign alternation in bi-directional setting when CPE is handing over from one RRH site to another. Use this model in bi-directional DPS setting. - cosθt=Ds2-vtDmin2+Ds2-vt2, 0≤t≤Dsv, cosθt= cosθt mod(Dsv), t>Dsv. Parameter; Value. Scenario-A-260; Scenario-A-350; Scenario-B-260; Scenario-B-350. Ds; 700 m; 700 m; 700 m; 700 m. Dmin; 10 m; 10 m; 150 m; 150 m. v; 260 km/h; 350 km/h; 260 km/h; 350 km/h. fd; 7227 Hz; 9729 Hz; 7227 Hz; 9729 Hz. - Option 2: HST-DPS Channel for FR2 HST Bi-Directional RRH Deployment. the cosine of angle θt used in Doppler shift fst=fd cosθt is provided as below: - cosθt=Ds-vtDmin2+Ds-vt2, 0<t≤(0.5*Ds)/v (eq. 7) - cosθt=vtDmin2+vt2, (0.5*Ds)/v<t≤Ds/v (eq. 8) cosθt modDsv, t>Ds/v (eq. 9) Recommendations for 2nd round: - Suggest to further discuss based on the above two options. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2105025 | Suggest the TP part of this paper (TP to TR 38.854) is approved. |

## Discussion on 2nd round (if applicable)

View are collected for WF on Channel Modeling for FR2 HST:

[Ericsson] One question; on slide 7, the option 2 is highlighted in yellow. Does it’s highlighting in yellow imply that you propose to agree option 2 ?

If so, we do not agree and prefer option 1, because in our understanding JT does not work for this scenario as the difference in propagation times between the nodes is too high. Also, for bi-directional JT would require simultaneously receiving on two panels.

[Nokia] Sorry for the slight confusion with the yellow highlight on Slide 7:

Yellow marking just meant that this option was not present before and was added by us. No proposal to continue with Option 2 only.

Regarding your comment that “for bi-directional JT would require simultaneously receiving on two panels,” I think that the signal from the previous RRH is still strong enough and can be received by the same panel, e.g., like in a uni-directional setting.

[Samsung] For the highlighted Option 2 (both DSP and SFN) in P7, we already agree in last meeting’s WF:

•	“Bi-directional operation for two panels (if any):

•	Follow Rel-15/16 principle of “only one panel to TX/RX at a time”.

•	FFS signaling is needed.”

And also from WID:

•	“Focused on train roof-mounted high-power devices

-	Single panel, i.e. only one active antenna panel at a time, as baseline antenna assumption ”

We have the concern on the newly added option 2 (if you intention is for bi-directional case), because it is not aligned with previous agreement.

If you intention is for uni-directional, as commented by Ericsson, the propagation delay difference will be the issue making it impossible to be used.

So I suggest to remove this newly proposed option2 here.

@Sean,

For your proposed Option-3 for DL, I think option 3 should be similar as Option 2 (just different UE movement), but of course it is not complete because we need the express for t not within [0, Ds], right?

At least we need some revision on Option-3, right?

[Nokia] Thank you for commenting on the Issues on Page 7!

However, we do not think that the agreements you listed contradict the JT/SFN way of operation.

As we already commented in our reply to Ericsson:

“the signal from the previous RRH is still strong enough and can be received by the same panel, e.g., like in a uni-directional setting.”

Regarding a  difference in propagation delays, we would like to study this Issue further (taking into account also the Issue in the WF on the deployments, page 13).

As a result, we cannot agree that JT/Full-SFN modeling shall be excluded entirely from the WF.

We propose the following revision for Slide 7:

•	RAN4 agrees to use DPS channel model for both Uni-directional/Bi-directional for performance requirements.

•	Consider JT model for DL?

o	FFS: a need for JT/Full-SFN channel models for performance requirements

Regarding Slide 8, we also think that the model for “UE Moving away from Serving RRH” is worth checking further for the next meeting.

To allow possible corrections to the models, we propose to add

“Other options are not precluded”.

A few additional updates made by us in the WF (marked in RED):

-	Slide 2: corrected wrong references

-	Slide 4: nobody objected to the proposed WF, and it is changed to an agreement.

# Topic #3: Demodulation Requirement

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2105028 | Samsung | Theoretical analysis for maximum supported velocity with different RS configuration  Observation 1: It’s feasible to support up to 350km/h with 120kHz TRS configured for frequency offset tracking from DL perspective.  Observation 2: It is not feasible to support maximum 350km/h velocity with targeting 30GHz carrier frequency under only DMRS configuration for UL .  Observation 3: It’s feasible to support up to 350km/h with combined DMRS + PTRS (120kHz) used for frequency offset tacking for UL. Then we bring evaluation results for both uplink and downlink with different channel modelling/deployment scenarios. UL performance evaluation  Observation 4: Similar performance can be achieved for both bi-directional and un-directional deployment scenario.  Observation 5: With 1 DMRS+PTRS (L=1, K=2) configuration, better performance can be achieved in terms of maximum throughput.  Observation 6: With only DMRS (1+1+1) configuration, the achievable throughput is very lower, it is not feasible to support 350km/h @ 30GHz carrier frequency.  Observation 7: From UL demodulation perspective, it is feasible to support the maximum speed with 350km/h, even with high carrier frequency up to 30GHz with PTRS configured. DL demodulation –Single Tap  Observation 8: For rank1 transmission, it is feasible to support UE speed up to 350km/h with single tap channel  Observation 9: For rank1 transmission with 260km/h, the maximum throughput for MCS 17 and MCS19 can be achieved around 10 dB SNR.  Observation 10: For rank1 transmission with 350km/h, the maximum throughput for MCS 13 can be achieved around 4 dB SNR. The maximum throughput for MCS 17 can be achieved around 12 dB SNR. DL demodulation - Uni-direction RRH deployment with DPS scheme  Observation 11: Similar performance can be archived for scenario A (Dmin=10) and scenario B (Dmin=150) in uni-directional scenario with DPS transmission scheme.  Observation 12: For rank1 transmission, it is feasible to support UE speed up to 350km/h in uni-directional scenario with DPS transmission scheme  Observation 13: For rank1 transmission with 260km/h UE speed, the maximum throughput for MCS 17 can be achieved round 10 dB SNR  Observation 14: For rank2 transmission with 260km/h UE speed, the maximum throughput of MCS 17 can be achieved around 15 dB  Observation 15: For rank1 transmission with 350km/h UE speed, the maximum throughput for MCS 17 can be achieved around 18 dB  Observation 16: For rank2 transmission with 350km/h UE speed, the maximum throughput for MCS 17 cannot be achieved, the SNR of 70% maximum throughput is about 12 dB , the maximum throughput of MCS13 can be achieved around 12 dB DL demodulation- Bi-direction RRH deployment with DPS scheme  Observation 17: Similar performance can be archived for scenario A (Dmin=10) and scenario B (Dmin=150) in bi-directional scenario with DPS transmission scheme.  Observation 18: For rank1 transmission, it is feasible to support UE speed up to 350km/h in bi-directional scenario with DPS transmission scheme  Observation 19: For rank1 transmission with 260km/h UE speed, the maximum throughput for MCS 17 can be achieved round 10 dB SNR  Observation 20: For rank2 transmission with 260km/h UE speed, the maximum throughput of MCS 17 can be achieved around 12 SNR  Observation 21: For rank1 transmission with 350km/h UE speed, the maximum throughput for MCS 17 can be achieved around 19 dB SNR  Observation 22: For rank2 transmission with 350km/h UE speed, the maximum throughput of MCS 17 cannot achieved, the SNR of 70% maximum throughput is about 12 dB, the maximum throughput of MCS 13 can be achieved around 12 dB SNR To summary, based on our evaluation results and theoretical analysis over different deployment scenarios for DL and UL, the initial conclusion as following: Initial conclusion: It’s feasible to support UE velocity up to 350km/h under various deployment scenarios with 120kHz PTRS configured in uplink and 120kHz TRS configured in downlink. |
| R4-2106435 | Intel Corporation | Proposal #1: Assume the following reference signal(s) for DL frequency offset tracking: • For unidirectional deployment TRS, DMRS or TRS + PTRS • For bidirectional deployment TRS+ PTRS Proposal #2: Assume the following reference signal(s) for UL frequency offset tracking: PTRS or DMRS + PTRS where PTRS density is not less than every second symbol. Proposal #3: Define different set of UL requirements to cover different frequency offset compensation implementation. |
| R4-2106473 | Qualcomm Incorporated | Observation 1: Current assumptions on the HST scenario, geometry of the deployment and UE movement with respect to the signal source might have implication on the design of the demodulation performance tests; Observation 2: Single probe OTA chambers, fixed beam and fixed UE position have been assumed to design FR2 radiated requirements; Proposal 1: RAN4 to discuss on the impact of the assumptions of a static UE and single probe OTA chambers on the FR2 high speed train demodulation test design; Proposal 2: For the definition of radiated demodulation requirements for FR2 HST, RAN4 should keep into account the testability of high power devices inside OTA chambers; |
| R4-2106829 | Huawei, HiSilicon | Observation 1: It is not feasible to support 350km/h without using PTRS for uplink. Observation 2: DMRS+PTRS can provide more accurate frequency offset estimation than PTRS for uplink. Proposal 1: To support 350km/h, RAN4 define performance requirements using TRS+SSB for tracking frequency offset for downlink. Proposal 2: To support 350km/h, RAN4 define performance requirements using DMRS+PTRS for tracking frequency offset for uplink. |
| R4-2106916 | Nokia, Nokia Shanghai Bell | On maximum supported speed in UL: Observation 1: In PUSCH with mapping Type B, allocation length 8 and 9 has the same DM-RS patterns. Since the performance of these two cases is very close. It would make more sense to introduce allocation length 10 instead of 9 to study sparser DM-RS pattern. Observation 2: Looking at PUSCH demodulation performance in single-tap propagation conditions, it can be concluded that: - 3 DM-RS (1+1+1) reference symbols are not sufficient to cope with frequency offset in all priority scenarios - 3 DM-RS (1+1+1) can be potentially used only with 260 kmph maximum train speed in uni-directional setting - The presence of PT-RS (time density 2) is needed to provide reliable connectivity at 350 kmph. Observation 3: PT-RS support is mandatory UE capability in FR2. Proposal 1: RAN4 to necessitate transmitting of PT-RS with PUSCH in HST FR2 deployments if it is decided to support 350kmph maximum train speed. Proposal 2: If RAN4 decides to use PT-RS signals in UL, then their time density should be 2. Proposal 3: If RAN4 to discuss whether 260 kmph speed can be supported without PT-RS, and what reference signals configuration can be used in this case. On maximum supported speed in DL: Observation 4: PT-RS were not introduced neither in Option 2 (DMRS (1+1+1)) nor in the PDSCH demodulation parameters. However, their usage is highly recommended in FR2. They are always present in UE FR2 PDSCH test configurations defined in TS 38.101-04. Observation 5: PDSCH transmission at 350 km/h speed can be supported in HST FR2 in single-tap propagation conditions if 3 DM-RS (1+1+1) and PT-RS (frequency density 1, time density 2) are used. Proposal 4: RAN4 to necessitate transmitting of PT-RS with PDSCH in HST FR2 deployments. Proposal 5: RAN4 to use PT-RS signals with time density 2 in DL. |
| R4-2105029 | Samsung | Proposal 1: No PDSCH requirement with HST single tap channel model in FR2 Proposal 2: if needed to define PDSCH requirement with both RRH deployment scenarios, applicability rule can be further discuss to reduce the test efforts Proposal 3: No PDSCH requirement for JT-SFN transmission scheme in bi-directional scenario. FFS on PDSCH requirement for JT-SFN transmission scheme in unidirectional scenario Proposal 4: DPS scheme 1a and 1b can be considered for PDSCH requirement in unidirectional scenario. Proposal 5: Only DPS scheme 1a can be considered for PDSCH requirement in bi-directional scenario Proposal 6: For 120 KHz, it is feasible to use the maximum Doppler frequency as 9722Hz for PDSCH requirement. |
| R4-2106436 | Intel Corporation | Proposal #1: Define DL demodulation performance requirements only with 120 kHz SCS and 100 MHz CBW. Proposal #2: Define DL demodulation performance requirements with 350 km/h UE speed. Proposal #3: Define DL demodulation performance requirements only with one deployment scenario (A or B). Proposal #4: Analyse impact of UE frequency error on DL demodulation performance and after that conclude on necessity of explicit modelling of UE frequency error during the test procedure. Proposal #5: Define DL demodulation performance requirements only with DPS Tx scheme. Proposal #6: Do not define PDCCH demodulation performance requirements for HST FR2. |
| R4-2106830 | Huawei, HiSilicon | - Define PDSCH performance requirements for HST FR2 with the following parameters: Parameter; Value. Maximum Doppler; 9596Hz. Channel model; single-tap, DPS. CBW/SCS; 100MHz/120kHz. PUSCH mapping; Type A, start symbol 1, duration 13. DMRS; 1+1+1. PTRS; KPTRS=2, LPTRS=1. Antenna configuration; 2x2. MCS; 17. Test metric; 70% of maximum throughput. |
| R4-2106866 | Ericsson | Proposal 1: Assume UE uses TRS (4 symbol interval) for frequency offset tracking. Observation 1: Maximum Doppler frequency based on TRS is 14,000Hz if we don’t assume frequency error. Observation 2: Maximum Doppler frequency based on TRS is 11,000Hz if we assume frequency error of 0.1ppm at 30GHz. Observation 3: Considering the maximum throughput achievement, the maximum Doppler shift can be set up to about 12,000Hz for FR2 with SCS=120kHz if we configure DMRS 1+1+1 with the assumption UE does not perform frequency offset compensation. Observation 4: Considering the maximum throughput achievement, the maximum Doppler shift can be set up to about 12,000Hz for FR2 with SCS=120kHz even if we configure no additional DMRS symbols with the assumption UE performs frequency offset compensation. Proposal 2: Assume DMRS configuration with 1+1+1 for UE demodulation requirements. Proposal 3: Assume also DMRS configuration without additional DMRS symbols for single tap scenario. Proposal 4: Set the maximum Doppler shift for PDSCH demodulation requirements for HST FR2 by assuming the UE speed of 350km/h at the carrier frequency of 30GHz. Observation 5: Reception difference between two RRHs exceeds the CP with SCS=120kHz for both Scenarios A and B in the case of HST-SFN joint transmission. Proposal 5: For FR2 HST UE demodulation requirements, RAN4 should define the PDSCH demodulation requirements with the assumption UE receives PDSCH only from one RRH, e.g., HST single tap, multi-path fading (TDL), or HST-DPS, with the assumption of single FFT receiver. Proposal 6: RAN4 discuss whether to define PDSCH demodulation requirements for joint transmission assuming UE is capable of multiple FFT receiver. |
| R4-2104681 | Ericsson | Observation 1: Double-symbol DM-RS configuration is typically for MU-MIMO scenario and due to the presence of phase-noise, such configuration is generally not preferrable for FR2 BS. Observation 2: Taking frequency drift into consideration, under worst-case scenarios where Doppler reversal is expected, frequency offset estimation (FOE) based on PT-RS may be necessary for HST travelling faster than 250 km/h. Proposal 1: For single-tap channel model, PT-RS (with time density LPT-RS = 1) based FOE and (1+0) DM-RS symbol for channel estimation can be considered. Proposal 2: In case fading shall be considered, a low Doppler spread is most realistic (e.g. TDLA30-75). PT-RS (with time density LPT-RS = 1) based FOE and (1+0) DM-RS symbol for channel estimation can still be considered for low Doppler spread. Proposal 3: We think that the single tap channel model is sufficient and there is no real need to include TDLA30-75 (or any other fading channel) in addition. Proposal 4: If substantial fading would be expected for scenario B, more DM-RS would be required for channel estimation and DM-RS based FOE shall be considered. Potentially, a lower speed may need to be considered. Proposal 5: Check the PRACH demod performance with simulation with the following setup. - UE speed: same as PUSCH uses; - Carrier frequency: 28GHz and/or 30GHz; - Channel: AWGN, and/or TDL-A (if needed); - Format: A2, A3, B4, C2; - SCS: 120kHz; - Antenna: 1T2R; - Ncs: 0; - Test metric: missed detection rate = 1% while false alarm rate = 0.1%. |
| R4-2105030 | Samsung | PUSCH requirements: Proposal 1: Define PUSCH requirement with HST single tap channel model. Further discussion the test applicability rule of requirement for two RRH deployment scenario if needed. Proposal 2: Focus on the PUSCH requirement with open space scenario firstly Proposal 3: For 120 KHz SCS, it is feasible to use the maximum Doppler frequency as 19444Hz for PUSCH requirement. Proposal 4: The following simulation assumption for PUSCH requirement with HST single tap setup can be considered as - Waveform: CP-OFDM - SCS&BW: 120 KHz SCS, 100 MHz - RS Configuration: 1 DMRS symbol+ PTRS (L=1, K=2) - Resource mapping type: type B - Length of data symbol: 9 - MCS: 16 - Antenna configuration: 1Tx 2Rx low Observation 1: PUSCH requirement with UL timing adjustment can be considered. PRACH requirements: Observation 2: No limitation for PRACH to support 350km/h velocity with carrier frequency 30GHz Proposal 5: For PRACH with short sequence format, only define the performance requirement for format C2 in Rel-17 FR2 HST WI. Proposal 6: Set frequency offset as 19444Hz for PRACH format requirement to align the Doppler shift assumption of PUSCH Proposal 7: Reuse the following test parameters for PRAH format requirement PRACH; PRACH SCS; Time error tolerance. preamble; (kHz); AWGN. C2; 120; 0.26us. PRACH preamble; SCS (kHz); Ncs; Logical sequence index; v. C2; 120; 69; 0; 0. PUCCH requirements Proposal 8: No PUCCH requirement for HST scenario |
| R4-2106437 | Intel Corporation | Proposal #1: Define UL demodulation performance requirements only with 120 kHz SCS and consider 50, 100 and 200 MHz CBW. Proposal #2: Define DL demodulation performance requirements only with one deployment scenario (A or B). Proposal #3: Define UL demodulation performance requirements only with transform precoding disabled. Proposal #4: Define two different sets of UL demodulation performance requirements to distinguish two different possible frequency offset compensation approaches. Proposal #5: Assume PTRS or PTRS+DMRS based frequency offset tracking for UL demodulation performance requirements definition. PTRS density should not be less than every second symbol. Proposal #6: Do not define PUCCH demodulation performance requirements for HST FR2. Proposal #7: Define PRACH demodulation performance requirements only with AWGN conditions with 9722 Hz frequency offset and consider PRACH configuration from Table 1 Table 1. Proposed PRACH preambles and configuration. Burst format SCS (kHz) Ncs Logical sequence index v A2, B4, C2 120 69 0 0 Proposal #8: Define UL TA demodulation performance requirements for HST FR2 with scenario Y. |
| R4-2106780 | Nokia, Nokia Shanghai Bell | Channels to test Proposal 1: Follow NR_HST specification changes as baseline; test PUSCH, PUSCH UL TA, and PRACH. PUSCH Proposal 2: Only test CP-OFDM waveform. Proposal 3: Only have 2Rx requirements. Proposal 4: Limit MCS to 16QAM max. Proposal 5: Configure both DM-RS (1+1) and PT-RS (K=2, L=1) to allow for up to 350km/h in single tap models. Proposal 6: Align all remaining configurations with FR1 HST. PRACH Proposal 7: Have requirements for A2, B4, and C2. Proposal 8: Limit channel model to AWGN with frequency offset decided by maximum observable doppler shift. Proposal 9: Align all remaining configurations with FR1 HST. |
| R4-2106831 | Huawei, HiSilicon | - Preamble format 0/1/2/3 cannot be used for HST FR2. Preamble format A1/A2/A3/B1/B2/B3/B4/C0 cannot be used for HST FR2. - Define PUSCH performance requirements for HST FR2 with the following parameters: Parameter; Value. Maximum Doppler; 19191Hz. Channel model; single-tap. CBW/SCS; 200MHz/120kHz. PUSCH mapping; Type B, start symbol 0, duration 10. DMRS; 1+1+1. PTRS; KPTRS=2, LPTRS=1. Waveform; CP-OFDM. Antenna configuration; 1x2. MCS; 16. Test metric; 70% of maximum throughput. Use C2 for HST FR2 performance requirements definition under AWGN channel with 19191 Hz frequency offset. Define UL TA requirements with the following parameters: Parameter; Value. Channel model; Stationary UE: AWGN, Moving UE: AWGN. UE speed; 350 km/h. CP length; Normal. A; 1.25 μs. Δω; 1.04 s-1. MCS; 16. CBW; 200MHz. PUSCH resource allocation; 0 to 65 RB for moving UE, 66 to 131 for stationary UE. SRS resource allocation; last symbol in slot #3 in radio frames, CSRS = 33, BSRS =0, for 132 RB. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 3-1 Maximum Speed Feasibility Study

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 3-1-1: Uplink maximum speed feasibility study and requested RS configuration

- Proposals

  - To support 350kmph maximum speed:

    - Observation 1 (Samsung, Nokia, Huawei): Not feasible for only DM-RS configuration without PT-RS

    - Observation 2 (Samsung, Nokia, Huawei, Intel): Feasible for combined DM-RS+ PT-RS (density no less than 2) used for frequency offset tracking.

      - Observation 2a (Samsung): With 1 DMRS+PTRS (L=1, K=2) configuration, better performance can be achieved in terms of maximum throughput.

      - Observation 2b (Nokia): With 1+1 DMRS+PTRS (L=1, K=2) configuration.

    - Observation 3 (Intel): Feasible for PT-RS or DM-RS + PT-RS (density no less than 2) for frequency tracking.

    - Observation 4 (Ericsson): Feasible for

      - For single-tap channel model, PT-RS (with time density LPT-RS = 1) based FOE and (1+0) DM-RS symbol for channel estimation

      - In case fading shall be considered, a low Doppler spread is most realistic (e.g. TDLA30-75). PT-RS (with time density LPT-RS = 1) based FOE and (1+0) DM-RS symbol for channel estimation can still be considered for low Doppler spread.

      - We think that the single tap channel model is sufficient and there is no real need to include TDLA30-75 (or any other fading channel) in addition.

      - If substantial fading would be expected for scenario B, more DM-RS would be required for channel estimation and DM-RS based FOE shall be considered. Potentially, a lower speed may need to be considered.

    - Observation 5 (ZTE): If (pre-)compensation of Doppler shift is considered at CPE or network side the max supported speed can be increased under the same RS density and SCS configuration.

  - To support 260kmph maximum speed:

    - Observation 6 (Nokia): Feasible for only DM-RS configuration without PT-RS

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 3-1-2: Downlink maximum speed feasibility study and requested RS configuration

- Proposals

  - To support 350kmph maximum speed:

    - Observation 1 (Nokia): Feasible in single-tap propagation conditions if 3 DM-RS (1+1+1) and PT-RS (frequency density 1, time density 2) are used.

    - Proposal 1 (Nokia): RAN4 to necessitate transmitting of PT-RS with PDSCH in HST FR2 deployments.

    - Observation 2 (Huawei, Samsung, Ericsson): Feasible by using TRS+SSB for tracking frequency offset for downlink.

    - Observation 3 (Intel): Feasible by using:

      - TRS, DMRS or TRS + PTRS, for unidirectional deployment

      - TRS+ PTRS, for bidirectional deployment

    - Observation 4 (Ericsson):

      - TRS (4 symbol interval) for frequency offset tracking

      - DMRS configuration with 1+1+1 for UE demodulation requirements (with frequency offset compensation before the demodulation process).

      - DMRS configuration without additional DMRS symbols for single tap scenario (without frequency offset compensation before the demodulation process).

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 3-1-3: Carrier frequency for Doppler frequency calculation

- Proposals

  - Proposal 1: 28 GHz

  - Proposal 2: 29.5 GHz

  - Proposal 3: 30 GHz

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

### Sub-topic 3-2 UE Demodulation Requirement

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 3-2-1: General test scope for UE demodulation requirements

- Proposals

  - Proposal 1 (Samsung, Intel, Huawei, E///, Nokia):

    - Introduce PDSCH requirement

  - Proposal 2 (Intel):

    - Do not define PDCCH demodulation performance requirements for HST FR2.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 3-2-2: Applicability rule

- Proposals of Applicability rule for uni- and bi-directional RRH deployment scenarios

  - Proposal 1 (Samsung): if needed to define PDSCH requirement with both RRH deployment scenarios, applicability rule can be further discussed.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 3-2-3: Requirement for Scenario A or B

- Proposals

  - Proposal 1 (Intel): Define DL demodulation performance requirements only with one deployment scenario (A or B).

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 3-2-4: Transmission schemes for Scenario A or B

- [Moderator] Similar discussion on channel modelling selection for JT or DPS, in Issue 2-4-1.

- Proposals

  - Observation 1 (Ericsson, Intel, Samsung): Reception difference between two RRHs exceeds the CP with SCS=120kHz for both Scenarios A and B in the case of HST-SFN joint transmission.

  - Proposal 1 (Intel): Define DL demodulation performance requirements only with DPS Tx scheme.

  - Proposal 2 (Ericsson):

    - For FR2 HST UE demodulation requirements, RAN4 should define the PDSCH demodulation requirements with the assumption UE receives PDSCH only from one RRH, e.g., HST single tap, multi-path fading (TDL), or HST-DPS, with the assumption of single FFT receiver.

    - RAN4 discuss whether to define PDSCH demodulation requirements for joint transmission assuming UE is capable of multiple FFT receiver.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 3-2-5: DPS schemes for DL demodulation requirements (if agreed)

- Proposals

  - Proposal 1 (Samsung):  DPS scheme 1a and 1b can be considered for PDSCH requirement in unidirectional scenario.

  - Proposal 2 (Samsung): Only DPS scheme 1a can be considered for PDSCH requirement in bi-directional scenario.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 3-2-6: SCS and channel bandwidth

- Proposals

  - Proposal 1 (Intel): Define requirement only with 120kHz SCS and 100MHz CBW.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 3-2-7: UE frequency error

- Proposals

  - Proposal 1 (Intel): Analyse impact of UE frequency error on DL demodulation performance and after that conclude on necessity of explicit modelling of UE frequency error during the test procedure.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 3-2-8: Other simulation assumption for PDSCH

- Proposals

  - Proposal 1 (Huawei): Define PDSCH performance requirement with:

| Parameter | Value |
| --- | --- |
| Maximum Doppler | 9596Hz |
| Channel model | single-tap, DPS |
| CBW/SCS | 100MHz/120kHz |
| PDSCH mapping | Type A, start symbol 1, duration 13 |
| DMRS | 1+1+1 |
| PTRS | KPTRS=2, LPTRS=1 |
| Antenna configuration | 2x2 |
| MCS | 17 |
| Test metric | 70% of maximum throughput |

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

### Sub-topic 3-3 BS Demodulation Requirement

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 3-3-1: General test scope for UL requirements

- Proposals

  - Proposal 1 (Samsung, Intel, Huawei, E///, Nokia):

    - PUSCH

    - PUSCH with UL timing adjustment

    - PRACH requirement

  - Proposal 2 (Samsung, Intel):

    - No PUCCH requirement for FR2 HST scenario.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 3-3-2: Test Setup for PUSCH requirement (if agreed)

Issue 3-3-2-1: Requirement for scenario A or B

- Proposals

  - Proposal 1 (Intel): Define UL demodulation performance requirements only with one deployment scenario (A or B).

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 3-3-2-2: Requirement for uni-and bi-directional RRH deployment scenarios

- Proposals

  - Proposal 1 (Samsung): if needed to define PUSCH requirement with both RRH deployment scenarios, applicability rule can be further discussed.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 3-3-2-3: Waveform for PUSCH requirement

- Proposals

  - Proposal 1 (Intel, Nokia, Samsung): Define UL demodulation performance requirements only with transform precoding disabled.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 3-3-2-4: SCS & BW

- Proposals

  - Proposal 1 (Intel): Define UL demodulation performance requirements only with 120 kHz SCS and consider 50, 100 and 200 MHz CBW.

  - Proposal 2 (Samsung): 120 KHz SCS, 100 MHz CBW.

  - Proposal 3 (Huawei): one typical CBW, e.g., 200MHz.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 3-3-2-5: Antenna configuration

- Proposals

  - Proposal 1 (Samsung): Antenna configuration: 1Tx 2Rx low

  - Proposal 2 (Nokia): Only have 2Rx requirements

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 3-3-2-6: Whether to define different set of PUSCH requirement to cover different FO compensation implementation

- Proposals

  - Proposal 1 (Intel): Define different sets of UL requirements to cover different frequency offset compensation implementations (pre-FFT and post-FFT processing).

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 3-3-2-7: Other assumption for PUSCH demodulation requirement

- Proposals

  - Proposal 1 (Samsung): Check the PUSCH demod performance with simulation with the following setup.

    - Resource mapping type: type B

    - Length of data symbol: 9

    - MCS: 16

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 3-3-3: Test Setup for UL timing adjustment requirement

Issue 3-3-3-1: Test Scenario

- Proposals

  - Proposal 1 (Intel, Huawei, Samsung): scenario Y

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 3-3-3-2: Simulation Assumption for scenario Y (if agreed)

- Proposals

  - Proposal 1 (Huawei):

| Parameter | Value |
| --- | --- |
| Channel model | Stationary UE: AWGN, Moving UE: AWGN |
| UE speed | 350 km/h |
| CP length | Normal |
| A | 1.25 μs |
| Δω | 1.04 s-1 |
| MCS | 16 |
| CBW | 200MHz |
| PUSCH resource allocation | 0 to 65 RB for moving UE, 66 to 131 for stationary UE |
| SRS resource allocation | last symbol in slot #3 in radio frames, CSRS = 33, BSRS =0, for 132 RB |

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 3-3-4: Test Setup for PRACH requirement (if agreed)

Issue 3-3-4-1:  PRACH Formats

- Proposals

  - Proposal 1 (Ericsson): A2, A3, B4, C2

  - Proposal 2 (Samsung, Huawei): C2,

  - Proposal 3 (Intel, Nokia): A2, B4, C2

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 3-3-4-2:  Channel

- Proposals

  - Proposal 1 (Ericsson): AWGN, and/or TDL-A (if needed);

  - Proposal 2 (Samsung): AWGN

  - Proposal 3 (Nokia): Limit channel model to AWGN with frequency offset decided by maximum observable doppler shift.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 3-3-4-3:  Frequency offset

- Proposals

  - Proposal 1 (Samsung): align with PUSCH

  - Proposal 2 (Intel): 9722Hz with 350km/h

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 3-3-4-4: Test Preamble Configuration

- Proposals for NCS

  - Proposal 1 (Ericsson): NCS = 0

  - Proposal 2 (Samsung, Intel, Nokia): NCS = 69

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

### Sub-topic 3-4 Testability Aspects

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 3-4-1: Testability issues for FR2 HST UE

- Proposals

  - Proposal 1 (Qualcomm): RAN4 to discuss on the impact of the assumptions of a static UE and single probe OTA chambers on the FR2 high speed train demodulation test design;

  - Proposal 2 (Qualcomm): For the definition of radiated demodulation requirements for FR2 HST, RAN4 should keep into account the testability of high power devices inside OTA chambers.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

## Companies views’ collection for 1st round

### Open issues

Sub topic 2-1

| Company | Comments |
| --- | --- |
| XXX |  |

Sub topic 2-2

| Company | Comments |
| --- | --- |
| XXX |  |

Sub topic 3-1-1

| Company | Comments |
| --- | --- |
| Ericsson | We share the observation that FOE is not feasible with DM-RS without PT-RS. The PT-RS density should be sufficient that the receiver can perform FOE based on PT-RS (i.e. PT-RS density 2 for 350 km/h). |
| Intel | All companies concluded that PTRS is needed to support 350 km/h. We should agree that PTRS is transmitted during the test. |
| Nokia, Nokia Shanghai Bell | Issue 3-1-1: Uplink maximum speed feasibility study and requested RS configuration Issue 3-1-2: Downlink maximum speed feasibility study and requested RS configuration The important point is that the use of PT-RS is essential both in UL and DL directions to support 350 km/h. Another question, if 350 kmp/h speed needs to be supported as such. Operator’s input would be helpful to decide. Issue 3-1-3: Carrier frequency for Doppler frequency calculation Both Proposals 1 and 3 are OK for us. |
| Qualcomm | Transmit PTRS during the test. |
| Samsung | Issue 3-1-1: Uplink maximum speed feasibility study and requested RS configuration From the maximum estimation capability of RS, It’s feasible to support UE velocity up to 350km/h under various deployment scenarios with 120kHz SCS configured in uplink, Without PTRS configuration, 260 is feasible for only DMRS configuration For detail RS configuration of DMRS and PTRS, we can further discuss during the demodulation performance discussion |
| Huawei | Considering the test effort and the test load, we prefer to only define one set of requirements, i.e. for 350km/h by using DMRS+PTRS to provide large and accurate frequency tracking range simultaneously. We prefer PTRS configuration K=2 and L=1 that is the typical configuration for other existing FR2 cases, also considering that PTRS can be used for phase noise estimation. For the DMRS configuration, we prefer to use 1+1+1 to provide more accurate frequency offset estimation. |

Sub topic 3-1-2

| Company | Comments |
| --- | --- |
| Ericsson | Since PT-RS and DMRS are transmitted only when PDSCH is transmitted, we prefer to use TRP for frequency tracking. According to the analysis, it is possible to support 350km/h at 30GHz with TRS with SCS=120kHz. For the PDSCH demodulation, two additional DMRS can compensate the phase rotation caused by Doppler shift. However if it is for the dedicated UE and UE knows it is single tap channel model (bidirectional and/or unidirectional), single DMRS can also compensate the phrase rotation. RAN4 should also capture this scenario in the demodulation requirements. |
| Intel | We do not need agreement on baseline RS for frequency offset tracking since even with TRS we can support 350 km/h considering also UE frequency tracking error. Using of DMRS/PTRS in addition should be up to implementation. |
| Qualcomm | Agree to introduce TRS for frequency offset tracking. DMRS configuration needs to be further discussed. |
| Samsung | Issue 3-1-2: Downlink maximum speed feasibility study and requested RS configuration From the maximum estimation capability of TRS, It’s feasible to support UE velocity up to 350km/h under various deployment scenarios with 120kHz SCS configured in downlink for unidirectional deployment. For Bi- unidirectional deployment, with PTRS configuration, the maximum 350km UE velocity can be supported For detail RS configuration for DMRS , we can further discuss during the PDSCH demodulation performance discussion |
| Huawei | For the downlink, it is not appropriate to track frequency offset by using DMRS or PTRS. If UE is not scheduled PDSCH for a long time, especially when UE pass the RRH, the Doppler jump may not be solved with high probability. As per our evaluation, TRS+SSB is feasible for frequency offset tracking. For the DMRS configuration, considering 10ms periodicity of TRS, we prefer to use 1+1+1 to better solve large residual frequency offset. |

Sub topic 3-1-3

| Company | Comments |
| --- | --- |
| Ericsson | Proposal 3. We should follow the WID. |
| Intel | Support Proposal 3 to align with WID. |
| Qualcomm | Since the WID specifies up to 30GHz, Doppler frequency computed with Option 3 should to be considered |
| Samsung | We are ok with option1 and option 3 to align with WID for maximum Doppler calculation, while details carrier frequency for deployment depends on operator input. |
| Huawei | As per TS 38.101-2, the highest carrier frequency for the candidate band (i.e. n257, n258 and n261 in the WID) is 29.5GHz. However, the WID also mention that the carrier frequency is up to 30GHz. We slightly prefer to use 29.5GHz for the Doppler value derivation, since it is the highest valid carrier frequency within 30GHz. |

Sub topic 3-2-1

| Company | Comments |
| --- | --- |
| Ericsson | Both proposals are fine (define PDSCH and not define PDCCH) |
| Intel | We support both proposals. |
| Qualcomm | Both proposals are fine; |
| Samsung | We support for both proposals, only define PDSCH requirement for Rel-17 FR2 HST WI |
| Huawei | We are OK with Proposal 1 + Proposal 2. |

Sub topic 3-2-2

| Company | Comments |
| --- | --- |
| Ericsson | We should discuss it after we finalize the scope of UE demodulation requirements for FR2 HST. |
| Intel | We support proposal to discuss applicability rule between bi/uni-directional scenarios if both will be introduced. |
| Qualcomm | Discuss applicability rule once the demodulation scenarios are agreed. |
| Huawei | Different deployment scenario can be deployed in different region and the different algorithm can be used for different deployment scenario. If such applicability rule is defined, the UE maybe not work under certain deployment scenario. So it is not suitable to only define performance for only deployment scenario A or B. |

Sub topic 3-2-3

| Company | Comments |
| --- | --- |
| Ericsson | Requirements can defined with the worst case of the two scenarios |
| Qualcomm | Further discuss specifying different requirements based on expected demodulation performance differences. |
| Samsung | Generally, we are ok with the proposal, only define the requirements for one scenario. From the Doppler shift trajectory, the Doppler variation in scenario A is faster than scenario B. While, the impact of demodulation performance for both scenarios is minor, based on our simulation results. We are open to discuss which scenario is more challenge from demodulation perspective. If RAN4 concludes both scenario A and scenario B are feasible from link budget analysis perspective. Similarly, if requirements for both scenarios is defined, from test perspective, only one of them will be selected for test based on test applicability rule, to reduce the test efforts. |
| Huawei | The issue is that how we determine Scenario A or Scenario B is the worst case. One beam can be used for Scenario A while two beams or more can be used for Scenario B, also different algorithm can be used for different deployment scenario. Same reason as Issue 3-2-2, we prefer to define both requirements if needed. |

Sub topic 3-2-4

| Company | Comments |
| --- | --- |
| Ericsson | Proposal 2. We are also fine to exclude the joint transmission. |
| Qualcomm | We can agree not to include SFN JT in the requirements. Further discuss which schemes to include in the requirements. |
| Intel | We do not think that JT is suitable Tx scheme for HST FR2 and prefer to define requirements only for DPS. |
| Huawei | We don’t think SFN JT is appropriate considering the large Doppler spread and large delay spread to be handled. For the single-tap, it is too ideal and not likely happen in FR2, also DPS channel model can verify performance in single-tap, so we prefer to define DL performance requirements only with DPS Tx scheme. |

Sub topic 3-2-5

| Company | Comments |
| --- | --- |
| Ericsson | We can discuss it after we decide bidirectional and/or unidirectional deployment. |
| Qualcomm | DPS scheme can be further discussed once the deployment is agreed. |
| Intel | Support both proposals. It is not feasible to consider DPS scheme 1b in bidirectional deployment under assumption of one active panel on CPE side. |
| Huawei | Both DPS 1a and DPS 1b should be considered, we don’t see any issue for both DPS transmission schemes. |

Sub topic 3-2-6

| Company | Comments |
| --- | --- |
| Ericsson | OK with proposal 1. |
| Qualcomm | 120kHz/100MHz can be used as baseline for demodulation requirements definition |
| Samsung | We are ok with proposal 1 |
| Huawei | For the channel bandwidth, considering that the CPE is serving for all users in the train, we prefer to use larger channel bandwidth, such as 200MHz. |

Sub topic 3-2-7

| Company | Comments |
| --- | --- |
| Ericsson | We think it is up to UE implementation similarly to phase noise model. |
| Intel | We need to agree whether we need to consider UE frequency error as additional frequency offset for testing setup. We expect that there will be no difference since UE can use pre-FFT frequency offset correction and additional frequency offset on top of the Doppler shift will not cause baseband performance degradation. Same time it is better to check further the impact of frequency offset and conclude do we need to model it or not. |
| Qualcomm | UE frequency offset can be studied with simulation results, but it should be up to UE implementation |
| Huawei | We prefer to not consider this same as Rel-16 HST FR1 requirements. |

Sub topic 3-2-8

| Company | Comments |
| --- | --- |
| Ericsson | Regarding the parameters in Proposal 1, maximum Doppler should depends on the carrier frequency since we believe 9596Hz is derived based on fc=29.5GHz. We prefer to consider single DMRS case for HST single tap. For HST single-tap, antenna configuration should be 1x2 if we follow FR1 HST single tap. |
| Intel | Parameter; Value. Maximum Doppler; 9596Hz. Channel model; single-tap, DPS. CBW/SCS; 100MHz/120kHz. PDSCH mapping; Type A, start symbol 1, duration 13. DMRS; 1+1+1. PTRS; KPTRS=2, LPTRS=1. Antenna configuration; 2x2. MCS; 17. Test metric; 70% of maximum throughput. Maximum Doppler frequency can be updated according to issue 3-1-3. Exact channel model is still under discussion. As for DMRS configuration we agree to consider 2 additional DMRS symbols which is baseline assumption for HST operation. |
| Qualcomm | Simulation parameters are being discussed in other issues, so keep discussing. |
| Samsung | Regarding the maximum Doppler value, we prefer to discuss the maximum velocity and carrier frequency firstly Regarding for channel model, we suggestion to focus on DPS firstly, single tap channel is not a realistic scenario for HST, especially for FR2. it is not feasible to configure only one TCI state to track different RRH (the same SSB index) For MCS, we are ok with MCS17 as starting point, which is used for FR1 HST, For DMRS, we are ok with 2 additional DMRS |
| Huawei | Maybe there is a typo on the proposal. For the channel bandwidth, considering that the CPE is serving for all users in the train, we prefer to use larger channel bandwidth, such 200MHz. |

Sub topic 3-3-1

| Company | Comments |
| --- | --- |
| Ericsson | Issue 3-3-1: General test scope for UL requirements Agree proposal 1 and 2. |
| Intel | Support both proposals. |
| Nokia, Nokia Shanghai Bell | Issue 3-3-1: General test scope for UL requirements Both proposal 1 and 2 are OK. |
| Samsung | Issue 3-3-1: General test scope for UL requirements Agree proposal 1 and 2. |
| Huawei | We are OK with both Proposal 1 and Proposal 2. |

Sub topic 3-3-2

| Company | Comments |
| --- | --- |
| Ericsson | Issue 3-3-2-1: Requirement for scenario A or B Agree proposal 1; we can define requirements based on the worst case scenario for demodulation. Issue 3-3-2-2: Requirement for uni-and bi-directional RRH deployment scenarios We have not seen any benefit from bi-directional. If there are multi panels ad BS and UE, it is better to operate uni-directional in both directions. Before creating requirements, we should clarify gain. Is the requirement impact basically Doppler reversal or not ? Issue 3-3-2-3: Waveform for PUSCH requirement Agree proposal 1. Issue 3-3-2-5: Antenna configuration Agree proposals 1 & 2. Issue 3-3-2-7: Other assumption for PUSCH demodulation requirement The parameters are OK |
| Intel | Issue 3-3-2-2: Requirement for uni-and bi-directional RRH deployment scenarios We support proposal to discuss applicability rule between bi/uni-directional scenarios if both will be introduced. Issue 3-3-2-3: Waveform for PUSCH requirement Since only one type of UE is assumed for which it is suitable to consider only CP-OFDM waveform we do not need to consider DFT-S-OFDM waveform for requirements definition. Issue 3-3-2-4: SCS & BW Excluding 50 MHz from proposal 1 is also fine for us. We can consider only typical configurations for requirements definition based on companies’ inputs. Issue 3-3-2-5: Antenna configuration Support both proposals. Issue 3-3-2-6: Whether to define different set of PUSCH requirement to cover different FO compensation implementation Pre-FFT and Post-FFT processing at BS side is to possible approaches for HST FR2 scenario when number of served UEs is quite limited. Each of them leads to different UL performance and based on our observations post-FFT cannot guarantee 64QAM operation with 350 km/h UE speed. In this case we suggest defining to set of requirements one with 16QAM and another one with 64QAM and make 64QAM requirements up to BS declaration. In this case RAN4 will guarantee minimum performance in HST FR as well as 64QAM operation. Issue 3-3-2-7: Other assumption for PUSCH demodulation requirement We also suggest considering 64QAM (MCS 17) in order to check issue related to different possible BS implementations. |
| Nokia, Nokia Shanghai Bell | Issue 3-3-2-1: Requirement for scenario A or B In FR1 HST, both scenarios with RRH close (tunnel) and further always from the railways track were tested explicitly. However, the proposal might make sense to reduce the testing burden. We need to study further which of the scenarios is more challenging from the demodulation point of view. Issue 3-3-2-2: Requirement for uni-and bi-directional RRH deployment scenarios The testing of equipment that supports only one type of deployment scenario cannot be enforced all scenarios. Hence, we agree with the proposal. Issue 3-3-2-4: SCS & BW In our opinion, we need to test 50 MHz CBW to support minimal requirements. Above that, either 100 MHz or 200 MHz CBS can be tested. Issue 3-3-2-5: Antenna configuration Proposal 1 is also OK. Issue 3-3-2-6: Whether to define different set of PUSCH requirement to cover different FO compensation implementation In our understanding, FO compensation realisation if fully implementation issue. Requirements should assume any implementation. Hence, we do not need to defined different set of requirements in this case. Issue 3-3-2-7: Other assumption for PUSCH demodulation requirement The parameters are fine but what is the difference to the parameters agreed in the previous meeting for PUSCH maximum supported speed evaluation? |
| Samsung | Issue 3-3-2-1: Requirement for scenario A or B Generally, we are ok with the proposal, only define the requirements for one scenario. From the Doppler shift trajectory, the Doppler variation in scenario A is faster than scenario B. While, the impact of demodulation performance for both scenarios is minor, based on our simulation results. We are open to discuss which scenario is more challenge from demodulation perspective. If RAN4 concludes both scenario A and scenario B are feasible from link budget analysis perspective. Similarly, if requirements for both scenarios is defined, from test perspective, only one of them will be selected for test bases on the manufacturer of declaration, to reduce the test efforts. Issue 3-3-2-2: Requirement for uni-and bi-directional RRH deployment scenarios Based on RRH deployment scenario, currently both bi-directional scenario and uni-directional scenario are feasible from the beam coverage analysis. The Doppler shift trajectory will slightly different for these two scenarios. From the demodulation perspective, there is no different receiver processing foreseen. Based on our initial simulation, similar performance can be achieved. Therefore, it is not necessary to define the demodulation requirement for both scenario. If both scenarios are introduced for PUSCH requirements in RAN4, some test applicability rule should be considered to reduce the test effort. Only one of them will be selected for test bases on the manufacturer of declaration. Issue 3-3-2-3: Waveform for PUSCH requirement We prefer to define the PUSCH requirement with CP-OFDM only, similar as FR1, only CP-OFDM waveform is considered. Considering the distance of RRH to the track is close, which can be regarded as the cell center. While the main use case for DFT-s-OFDM is for cell edge UE. Therefore, it is not practical to define the requirement for DFT-s-OFDM for HST scenario. Issue 3-3-2-4: SCS & BW We prefer only to define the typical CBW requirement to reduce the test effort. Issue 3-3-2-5: Antenna configuration Issue 3-3-2-6: Whether to define different set of PUSCH requirement to cover different FO compensation implementation Generally, we agree the FO compensation is belong to the BS implementation issue, RAN4 should not specific the implementation method into specification for requirement definition. Therefore, we do not prefer to define different set of PUSCH requirement. Alternately, we can consider the additional impairment margin for requirement considering the Post-FOC degradation. Issue 3-3-2-7: Other assumption for PUSCH demodulation requirement Generally, the simulation assumption is same with previous meeting for maximum speed evaluation. Regarding the MCS level, MCS 16 is used FR1 HST and Rel-15 NR BS demod, we can use it as the starting point. |
| Huawei | Issue 3-3-2-1: Requirement for scenario A or B Same view as Issue 3-2-2 and Issue 3-2-3, we prefer to define both requirements if needed. Issue 3-3-2-2: Requirement for uni-and bi-directional RRH deployment scenarios Same view as Issue 3-2-2 and Issue 3-2-3, we don’t support to define such applicability rule. Issue 3-3-2-3: Waveform for PUSCH requirement We are OK with the Proposal 1. Issue 3-3-2-4: SCS & BW We prefer to use 200MHz channel bandwidth for requirements definition considering that the CPE is serving for all users in the train. Issue 3-3-2-5: Antenna configuration We are OK with both Proposal 1 and Proposal 2. Issue 3-3-2-6: Whether to define different set of PUSCH requirement to cover different FO compensation implementation We think it is the implementation issue and not required to be aligned for different manufactures. For minimum requirements definition, we can use the typical method, i.e. post-FFT processing. Issue 3-3-2-7: Other assumption for PUSCH demodulation requirement For the Length of data symbol, we prefer to use 10 that is same as other requirements defined for FR2. |

Sub topic 3-3-3

| Company | Comments |
| --- | --- |
| Ericsson | Issue 3-3-3-1: Test Scenario Agree proposal 1 Issue 3-3-3-2: Simulation Assumption for scenario Y (if agreed) Proposed parameters are Ok Issue 3-3-4-1: PRACH Formats C2 is the most important to include Issue 3-3-4-2: Channel AWGN only is OK (proposal 2) Issue 3-3-4-3: Frequency offset Proposal 1 is OK to align with PUSCH Issue 3-3-4-4: Test Preamble Configuration We should double check that the test preamble is not an outlier case |
| Intel | Issue 3-3-3-2: Simulation Assumption for scenario Y (if agreed) Channel BW and MCS value depends on outcome of issues 3-3-2-4 and 3-3-2-7. Issue 3-3-4-1: PRACH Formats Support proposal 2 given that only format C2 with 120 kHz SCS can provide enough coverage for deployments with 700m inter-site distance. Issue 3-3-4-2: Channel Support Option 3. We do not see necessity to introduce requirements with fading channel model. Frequency offset should be considered to model HST conditions. Issue 3-3-4-3: Frequency offset In general, we support proposal 1 but 9722 Hz value was derived under assumption of 350km/h UE speed and 30 GHz carrier frequency that we believe will be adopted for PUSCH. Issue 3-3-4-4: Test Preamble Configuration Proposal 2 is based on exiting PRACH requirements for 120 km/h. We think we can reuse same assumptions for HST. |
| Nokia, Nokia Shanghai Bell | Issue 3-3-3-1: Test Scenario We agree with Proposal l. Issue 3-3-3-2: Simulation Assumption for scenario Y (if agreed) In general, the simulation assumptions looks fine. However, it is necessary to wait until is is finally agreed that 350 km/h is maximum UE speed in HST FR2 deployment, and discuss what CBW to be tested, e.g. 100MHz vs 200 MHz. Issue 3-3-4-2: Channel Proposals 2 is OK as well. Issue 3-3-4-3: Frequency offset Proposal 1 is fine. |
| Samsung | Issue 3-3-3-1: Test Scenario We are ok with scenario Y Issue 3-3-3-2: Simulation Assumption for scenario Y (if agreed) In current stage, the maximum speed is still under discussion, also the channel bandwidth for requirement is not agreed yet, we suggest to further discuss the practical parameters, such as A, delta, subframes in which PUSCH is transmitted, PUSCH resource allocation and SRS allocation schemes Therefore, we should include other options is no precluded. Issue 3-3-4-1: PRACH Formats We prefer to define requirement with only C2 format Based on the deployment scenario discussion, the Ds between each RRH is about 700m in both scenario A and scenario B. With 120 KHz SCS, the coverage of format C2 is scaled as 1.15km, while for other formats A or B, the support coverage is less than 700m Issue 3-3-4-2: Channel For FR2, fading channel is not a typical scenario. Based on the link budget analysis with different scenario A and scenario B, one tap fading channel model. We are open to discuss whether requirement for fading channel is necessary Issue 3-3-4-3: Frequency offset We prefer option 1 Regarding the frequency offset setting for PRACH requirement, we prefer to align the maximum Doppler value assumption for PUSCH, and The detail value of Doppler value can depend on the maximum speed as well as the target carrier frequency. Issue 3-3-4-4: Test Preamble Configuration For other test parameters of PRACH, we can reuse the same assumption of FR2 for short sequence format as starting point. |
| Huawei | Issue 3-3-3-1: Test Scenario We are OK with the Proposal 1. Issue 3-3-3-2: Simulation Assumption for scenario Y (if agreed) We are OK with the Proposal 1. Issue 3-3-4-1: PRACH Formats We prefer to use C2 that is the only one feasible PRACH format for FR2 HST. Issue 3-3-4-2: Channel We prefer to use AWGN only with the Doppler offset. Issue 3-3-4-3: Frequency offset We are OK with the Proposal 1. Issue 3-3-4-4: Test Preamble Configuration The Preamble Configuration should be further check until next meeting. |

Sub topic 3-4-1

| Company | Comments |
| --- | --- |
| Ericsson | For proposal 1, maybe we can assume two probes in chamber, as RRM assumes 2AoA tests. For proposal 2, it depends on UE RF discussion, but we don’t think the max UE Tx power should not exceed PC1. In this case, we assume FR2 HST device can be tested in chamber. |
| Intel | Only static UE and single probe can be assumed for performance requirements verification. Demodulation testing should not assume exact modeling of real field scenario. We think that baseband processing can be effectively verified with current test methodology. Definitely, beam management performance is very important aspect, but it will be guaranteed by RRM requirements. We can think of combined set of RRM and Demod requirements as a single feature to support HST FR2 operation. |
| Nokia, Nokia Shanghai Bell | Regarding proposal 1, DPS scheme should not be a problem. For SFN scheme, when several geographically distributed TRPs are modelled, we think that it might be still possible to go with a single probe if the simulator setup is capable enough. Regarding proposal 2, we think that the power will not go above 20 dB that is acceptable for OTA chambers. |
| Qualcomm | If UE and single probe positions are assumed to be fixed for the duration of the test, the effectiveness of the demodulation performance testing might be reduced, so it could be further discussed whether to study an approach that takes into account continuous UE movement from RRH to RRH. |

### CRs/TPs comments collection

N.A because no CRs/TPs submitted under Topic-2 related AIs.

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |
| Sub-topic 3-1 | Issue 3-1-1: Uplink maximum speed feasibility study and requested RS configuration Tentative agreements: - It is feasible to support maximum speed with 350km for Uplink with PTRS or DMRS+PTRS configuration used for frequency offset tracking with 120KHz SCS - It is feasible to support maximum speed with 260km for Uplink only DMRS configuration without PTRS configuration for frequency offset tracking - Configure PTRS during the PUSCH demodulation test - FFS on DMRS+PTRS configuration for PUSCH demodulation requirement with single-tap channel model - Option 1: 1 DMRS +PTRS (L=1,K=2) (Samsung, Ericsson) - Option 2: 1+1 DMRS +PTRS (L=1.K=2) (Nokia) - Option 3: 1+1+1 DMRS+PTRS(L=1, K=2) (Huawei) - FFS on DMRS+PTRS configuration for PUSCH demodulation requirement with fading channel if needed - Option 1: 1 DMRS +PTRS (L=1,K=2) (Ericsson) - Option 2: 1+1+1 DMRS+PTRS(L=1, K=2) (Huawei) - FFS on DMRS+PTRS configuration for PUSCH demodulation requirement with substantial fading channel for scenario B if needed - Option 1: more DMRS configuration (Ericsson) - Option 2: 1+1+1 DMRS+PTRS(L=1, K=2) (Huawei) Recommendations for 2nd round: - Suggest companies to discuss based on the above candidate options Issue 3-1-2: Downlink maximum speed feasibility study and requested RS configuration Tentative agreements: - It is feasible to support maximum speed with 350km/h for downlink with TRS( 4 symbol interval) +SSB for frequency offset tracking under unidirectional RRH deployment with 120KHz SCS - It is feasible to support maximum speed with 350km/h for downlink with TRS (4 symbol interval) +SSB and PTRS (L=1,K=2) for frequency offset tracking under bidirectional RRH deployment with 120KHz SCS - Configure PTRS during the PDSCH demodulation test - FFS on RS as baseline for frequency offset tracking to support 350km/h - Option 1: SSB+TRS (Huawei, Ericsson, Samsung) - Option 2: TRS+PTRS for directional deployment (Intel) - Option 3: TRS, DMRS or TRS+PTRS for unidirectional deployment (Intel) - FFS on DMRS configuration for PDSCH demodulation requirement - Option 1: 1 DMRS assumption for single tap channel model (bidirectional and or unidirectional ) (Ericsson) - Option 2: 1+1+1 DMRS (with frequency offset compensation before the demodulation process) (Ericsson, Huawei) Recommendations for 2nd round: - Suggest companies to discuss based on the above candidate options Issue 3-1-3: Carrier frequency for Doppler frequency calculation Candidate options: - Option 1: 28GHz (Nokia, Samsung) - Option 2: 29.5GHz (Huawei) - Option 3: 30GHz (Ericsson, Nokia, Intel, Samsung, Qualcomm,) Recommendations for 2nd round: - Suggest companies to discuss based on the above candidate options |
| Sub-topic 3-2 | Issue 3-2-1: General test scope for UE demodulation requirements Tentative agreements: - Only define the PDSCH demodulation requirements in Rel-17 FR HST WI (Samsung, Huawei, Qualcomm, Ericsson, Intel) Recommendations for 2nd round:N.A Issue 3-2-2: Applicability rule for uni-and bi-directional RRH deployment scenarios Tentative agreements: - FFS to define both PDSCH demodulation requirements for uni-and bi-directional RRH deployment scenarios - FFS to define the test applicable rule if both PDSCH demodulation requirements for uni-and bi-directional RRH deployment scenarios are defined. Recommendations for 2nd round: - Suggest companies to discuss based on the above candidate options in the next meeting Issue 3-2-3: Requirement for Scenario A or B Candidate options: - Option 1: Define PDSCH demodulation performance requirements only with one deployment scenario (A or B)(Intel, Samsung) - Option 2:Define PDSCH demodulation performance requirements with the worst cases of two scenarios (Ericsson) - Option 3:Define PDSCH demodulation performance requirements with both scenarios if needed (Huawei, Qualcomm) Recommendations for 2nd round: - Suggest companies to discuss based on the above candidate options in the next meeting Issue 3-2-4: Transmission schemes for Scenario A or B Tentative agreements: - No PDSCH requirement with SFN joint transmission scheme in Rel-17 FR2 HST WI (Ericsson, Huawei, Qualcomm, Intel) Candidate options: - Option 1: Define PDSCH demodulation performance requirements only with DPS Transmission scheme (Samsung, Intel, Huawei, [Qualcomm]) - Option 2: FFS on PDSCH requirements of HST single tap or multi-path fading (TDL) (Ericsson, [Qualcomm]) Recommendations for 2nd round: - Suggest companies to discuss based on the above candidate options Issue 3-2-5: DPS schemes for DL demodulation requirements Tentative agreements: Candidate options: - DPS transmission scheme in Uni-directional RRH deployment scenario - FFS on DPS scheme 1a and DPS scheme 1b - DPS transmission scheme in Bi-directional RRH deployment scenario - FFS on DPS scheme 1a and DPS scheme 1b Recommendations for 2nd round: - Suggest companies to discuss based on the above candidate options in the next meeting Issue 3-2-6: SCS and channel bandwidth Candidate options: - Option 1: 120KHz with 100MHz (Samsung, Intel, Ericsson, Qualcomm) - Option 2: 120KHz with 200MHz (Huawei) Recommendations for 2nd round: - Suggest companies to discuss based on the above candidate options in the next meeting Issue 3-2-7: UE frequency error Tentative agreements: - FFS on considering the impact of UE frequency error on DL demodulation performance Recommendations for 2nd round: - Suggest companies to discuss based on the above tentative agreements in the next meeting Issue 3-2-8: Other simulation assumption for PDSCH Candidate options: - Option 1(Huawei) Parameter; Value. Maximum Doppler; [9596Hz]. Channel model; [single-tap], [DPS]. CBW/SCS; [200MHz/120kHz]. PDSCH mapping; Type A, start symbol 1, duration 13. DMRS; [1+1+1]. PTRS; KPTRS=2, LPTRS=1. Antenna configuration; [2x2]. MCS; [17]. Test metric; 70% of maximum throughput. - Other options are not precluded Recommendations for 2nd round: - Suggest companies to discuss based on the above tentative agreements |
| Sub-topic 3-3 | Issue 3-3-1: General test scope for UL requirements Tentative agreements: - Only define the following BS demodulation requirements in Rel-17 FR HST WI - PUSCH - UL timing adjustment - PRACH Recommendations for 2nd round:N.A Issue 3-3-2: Test Setup for PUSCH requirement Issue 3-3-2-1: Requirement for scenario A or B Candidate options: - Option 1: Define PUSCH demodulation requirements based on the worst cases scenario (Ericsson, Nokia) - Option 2: Define PUSCH demodulation requirements only with one deployment scenario (A or B) (Intel, Samsung) - Option 3: Define PUSCH demodulation requirements for both two scenarios if needed (Huawei) Recommendations for 2nd round: - Suggest companies to discuss based on the above candidate options in the next meeting Issue 3-3-2-2: Requirement for uni-and bi-directional RRH deployment scenarios Tentative agreements: - FFS to define both PUSCH demodulation requirements for uni-and bi-directional RRH deployment scenarios - FFS to define the test applicable rule if both PUSCH demodulation requirements for uni-and bi-directional RRH deployment scenarios are defined. Recommendations for 2nd round: - Suggest companies to discuss based on the above candidate options in the next meeting Issue 3-3-2-3: Waveform for PUSCH requirement Tentative agreements: - Define PUSCH demodulation performance requirement only with CP-OFDM waveform Recommendations for 2nd round:N.A Issue 3-3-2-4: SCS & BW Candidate options: - Option 1: 120KHz with 50MHz, 100 MHz or 200MHz (Intel, Nokia, ) - Option 2: 120KHz with 100MHz (Samsung) - Option 3: 120KHz with 200MHz (Huawei) Recommendations for 2nd round: - Suggest companies to discuss based on the above candidate options in the next meeting Issue 3-3-2-5: Antenna configuration Tentative agreements: - Define PUSCH demodulation requirement only with 1Tx 2Rx antenna configuration (Samsung, Huawei, Ericsson, Intel, Nokia) Recommendations for 2nd round:N.A Issue 3-3-2-6: Whether to define different set of PUSCH requirement to cover different FO compensation implementation Tentative agreements: - FFS to define different set of PUSCH requirement to cover different FO compensation implementation Recommendations for 2nd round: - Suggest companies to discuss based on the above candidate options in the next meeting Issue 3-3-2-7: Other assumption for PUSCH demodulation requirement Candidate options: - Resource mapping type: type B - Length of data symbol - Option 1 (Samsung, Ericsson, Nokia): 9 - Option 2 (Huawei):10 - MCS - Option 1 (Samsung, Huawei, Ericsson, Nokia, Intel): MCS 16 - Option 2 (Intel):MCS 16 and MCS17 Recommendations for 2nd round: - Suggest companies to discuss based on the above candidate options in the next meeting Issue 3-3-3: Test Setup for UL timing adjustment requirement Issue 3-3-3-1: Test Scenario Tentative agreements: - Define the UL timing adjustment requirement with scenario Y in Rel-17 FR2 HST WI Recommendations for 2nd round: N.A Issue 3-3-3-2: Simulation Assumption for scenario Y Candidate options:: - Option 1 (Huawei): Parameter; Value. Channel model; Stationary UE: AWGN, Moving UE: AWGN. UE speed; 350 km/h. CP length; Normal. A; 1.25 μs. Δω; 1.04 s-1. MCS; 16. CBW; 200MHz. PUSCH resource allocation; 0 to 65 RB for moving UE, 66 to 131 for stationary UE. SRS resource allocation; last symbol in slot #3 in radio frames, CSRS = 33, BSRS =0, for 132 RB. - Other options are not precluded Recommendations for 2nd round: - Suggest companies to discuss based on the above candidate options in the next meeting Issue 3-3-4: Test Setup for PRACH requirement Issue 3-3-4-1: PRACH Formats Tentative agreements: - Define PRACH requirement for Rel-17 FR2 HST WI with following format(s) - C2 (Samsung, Ericsson, Intel, Huawei, Nokia) - FFS on A2, A3, B4 Recommendations for 2nd round: - Suggest companies to discuss based on the above tentative agreement. Issue 3-3-4-2: Channel Issue 3-3-4-3: Frequency offset Tentative agreements: - Define PRACH requirement for Rel-17 FR2 HST WI only with AWGN channel, no PRACH requirement is defined for fading channel. - FFS on the Frequency offset with AWGN channel - Option 1: align the Doppler value with PUSCH (Samsung, Huawei, Ericsson, Nokia, Intel) - Option 2: 9722Hz with 350km/h at 30GHz carrier frequency (Intel) Recommendations for 2nd round: - Suggest companies to discuss based on the above tentative agreement Issue 3-3-4-4: Test Preamble Configuration Candidate options: - Ncs - Option 1: Ncs =0 (Ericsson) - Option 2: Ncs =69(Samsung, Intel, Nokia) - Other options are not precluded (Huawei, Ericsson) Recommendations for 2nd round: - Suggest companies to discuss based on the above candidate options in the next meeting |
| Sub-topic 3-4 | Issue 3-4-1: Testability issues for FR2 HST UE Candidate options: - RAN4 to discuss on the impact of the assumption of a static UE and single probe OTA chambers on the FR2 high speed train demodulation test design - Option 1: FFS the assumption two probes in chamber, as RRM assumption with 2AOA tests (Ericsson) - Option 2: FFS on combining RRM and Demod requirement as a single feature to support HST FR2 operation (Intel) - Option 3: FFS on study an approach with taking into account continuous UE movement from RRH to RRH (Qualcomm) - FFS on keep into account the testability of high power devices inside OTA chambers , for the definition of radiated demodulation requirements for FR2 HST Recommendations for 2nd round: - Suggest companies to discuss based on the above candidate options in the next meeting |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Issue 3-1-1: Uplink maximum speed feasibility study and requested RS configuration

Ericsson: The conclusion on the speed is fine for us. Regarding the RS configuration, for single tap our simulations indicated only one DM-RS needed. Could Nokia, Huawei elaborate on why more than one ?

Intel: In Rel-15/16 for Single Tap channel model we assumed 2 additional DMRS symbols. It is typical configuration for HST conditions. We do not want to emphasize that 1 DMRS symbols is enough for high speed scenarios.\

Huawei: Same as Intel’s comment, 2 additional DMRS is more typical in HST scenario.

Samsung: From our side, we prefer to only 1 DMRS configuration. We agree that in FR1, more DMRS symbols should be configured, which can increase the range of frequencey offset, considering there is no PTRS configured. For FR2, as mentioned, to enable 350km/h maximum speed, the PTRS shuold be configured. From the maximum doppler estimation capacity, with 1 DMRS +PTRS or 2 DMRS +PTRS shuld be the same, since the density of RS (DMRS or PTRS) is 1 in time domain if joint DMRS+PTRS for doppler estimation. The benifit of more DMRS configuration is to overcome fading impact,

Considering the channel is a single tap, we think 1 DMRS should be enough.

Beside the doppler impact in FR2 HST, phase noise should be considered. With configured more PTRS symbols can be benifical for phase noise estimation, since the phase noise impact can be changed per symbol depending on the nonlinearity of PA in the real product.

Meanwhile, as mentioned in WID, this UE is dedicated CPE  for HST scenario in FR2, less UE is served by RRH, where it is not a interference limited scenario or reource limited scenario, different for FR1, more UE can access the network in UL, Therefore, more resouces can be allocated for UL to improve the UL throughput in FR2 for practical scheduling. With more DMRS symbols, the overhead of RS is higher than 1 DMRS+PTRS. Based on our simulation, with configured 1 DMRS +PTRS, better throughput can be acheived.

[Nokia]: We agree with the feasibility of support of 350 kmph speed. Regarding the number of DMRS symbols, we think that 3 DM-RS symbols (i.e., two additional DMSRS symbols) is excessive. However, in practical systems, the fading channel will be rather quickly changing in HST application. For UL allocations of more than 6 symbols, we see at least DM-RS 1+1 (in addition to PT-RS) as a quite expected configuration.

[Nokia]: here, we have a few comments and revisions marked with BLUE text, as follows:

- Slide 4:

  - We cannot agree with the statement that
“It is feasible to support maximum speed with 260km for uplink only DMRS configuration without PTRS configuration for frequency offset tracking with 120KHz SCS”

  - Based on our simulation results, it is not always feasible to support maximum speed with 260km for Uplink only DMRS configuration without PT-RS configuration for frequency offset tracking (Table 2 in R4-2106916).
Our observation was not listed correctly in the Issue 3-1-1. We were referring only to the uni-directional setting when we observed that 260 kmph speed could be supported without PT-RS.

- Slide 5:

  - It is not clear to us why the first statement that
“It is feasible to support maximum speed with 350km/h for downlink with PTRS or DMRS(1+1+1)+PTRS (L=1, K=2) configuration used for frequency offset tracking under single tap propagation conditions with 120KHz SCS” shall be removed?
Is this statement wrong?

  - We fixed a small typo in RS Option 2.

Issue 3-1-2: Downlink maximum speed feasibility study and requested RS configuration

Ericsson: Actually in our view a single tap channel is all that is needed as FR2 scenarios only work when there is LoS. Our proposals were only illustrative of what would be needed for the fading channel. So we would actually propose to agree on single tap channel and remove the bullets from slide 4 about fading channel.

Intel: Based on deployment discussion for both scenarios A and B single tap propagation conditions can be assumed. In this case we do not see reason to define demodulation requirements under fading propagation conditions.

Huawei: We don’t think it is needed for fading channel for both Scenario A and Scenario B.

Samsung: Simialr with other companies, we think fading channel is not a typical scenario for FR2 HST scenario, consideirng the Dmin is close to track.

[Nokia]: We agree that only single-tap channel model was on the table in the HST FR2 deployment discussions before. We do not see a need to introduce fading propagation conditions when only LoS propagation is considered

Huawei: Same as Intel’s comment, 2 additional DMRS is more typical in HST scenario.

[Nokia]:  Fine for speed and 1+1+1 is also the most common configuration

[Huawei]:

Slide#4:

-          We had long discussion during NR Rel-15 FR2 BS demodulation requirements definition. For the phase noise estimation, there are different implementations, 1 DMRS+PTRS is not feasible for some implementations.

-          Fading channel related is confirmed by Ericsson, it can be removed.

Slide#5:

-          Huawei’s response to Nokia for removal of the first bulletin in Slide#5 is added.

-          Also as per most companies’ analysis, SSB+TRS is feasible for frequency offset tracking to support 350km/h for both uni-directional and bi-directional, so the third bulletin is not needed.

Slide#6: As per the confirmation from companies, 30GHz carrier frequency for Doppler frequency calculation is fine for all companies that have replied.

Slide#9:

-          From our evaluation, DPS 1b is feasible for both uni-directional and bi-directional deployment scenario. Also as per GTW discussion, most companies have the observation that UE should have the capability to use another panel for beam searching, it should be removed.

-          Multi-fading channel is removed as per company’s feedback.

Slide#12: Considering Ericsson and Nokia are OK to only consider format C2 for PRACH, Option 1 can be agreed.

[Samsung]:

Slide # 4

In Rel-15 FR2, both 1 DMRS and 2 DMRS are considered and defined requirement. if my understanding is correctly, there is no statement that PTRS+1DMRS is not feasible for some implementions.

Slide # 5

Generally , the statement of "SSB+TRS is feasible for frequency offset tracking to support 350km/h" for uni-directional is true, since the doppler jump only from the maximum to zero (the worst case, if the switching point is at the projection of adjacent RRH)

In case of bi-directional scneario, we have different understanding, assuming the switching piont is Ds/2, the dopper will jump from the postive maximum  value to the negative maximum value, Therefore, the doppler should be double.  Based on the TRS configuration (4 symbol interval), the maximum doppler of TRS is 14000Hz. Based on the 350km/h and 30GHz carrier frequency, the maximum doppler should be 9722Hz. At the Ds/2, the maximum doppler change can be up to 19437Hz. It is out of estimation capacity of TRS.

Therefore,   we think PTRS should be considered to enable 350km/h under bi-directional scenario. We suggest the third bulletion should be captured in the WF.

[Huawei]:

Slide#4: considering different implementation, RAN4 defined requirements for both 1 DM-RS and 2 DM-RS with manufacture declaration D.101.

For Slide#5:

As per our evaluation, “TRS only” can handle Doppler 13989Hz and “SSB+TRS” can handle Doppler 20829Hz that is within the range of twice of max Doppler.

For the downlink, it is not appropriate to track frequency offset by using DMRS or PTRS. If UE is not scheduled PDSCH for a long time, especially when UE pass the RRH, the Doppler jump may not be solved with high probability.

[Samsung]:

Slide#4: yes, that is true in Rel-15 FR2,  we agree to define the requirement with both 1DMRS and 2 DMRS with test applicability,  based on the declaration of manufacture . Companies have different implementation, while it  does not mean that 1 DMRS+PTRS is not feasible for phase noise estimation. We have no common understanding that "1 DMRS+PTRS is not feasible for some implementations".

[Nokia]:

Thank you for providing a response to our question about the removal of the first bulletin in Slide#5!

However, we do not completely agree with your explanation. In our understanding, the FOE schemes we are currently discussing are all "single shot". I.e., we don't track the FOE. Thus, it does not matter if a UE was not scheduled for a long time.

Could it be possible for you to agree then that

"It is at least feasible to support maximum speed with 350km/h for downlink with PTRS or DMRS(1+1+1)+PTRS (L=1, K=2) configuration used for frequency offset tracking under single tap propagation conditions with 120KHz SCS"?

[Samsung]:

Further comments on slide#5. from the estimation capability of TRS+SSB perspective, higher doppler value can be achieved, as mentioned by your contributions. It has the same doppler tracking capability of PTRS (L=2). While for PDSCH requirement perspective, your intention is that enabling SSB transmission and decoding PBCH during PDSCH test?

[Huawei]:

During the test, it is not necessary to decode PBCH. For our understanding, frequency offset can be estimated by PSS/SSS, or by PBCH DMRS. SSB transmission is mandatory. Also as per current typical configuration in TS 38.101-4, SSB is transmitted in slot#0 every 20ms. It feasible to use SSB+TRS for frequency estimation.

[Samsung]:

Thanks for your further clarfication. It is true that SSB can be configured for PDSCH, while whether applying SSB for frequency offset tracking to compensate PDSCH should be UE implemention.

Meanwhile, we do not fully agree with you that "it is not necessary to decode the PBCH". In order to enable DMRS frequency offset tracking, as least the SSB index information and SSB carrieroffset contained in MIB should be known, although it can be configured.

Regarding the PSS/SSS for frequencey offset tracking, we donot think it is a typical method, considering that different sequence are generated for PSS/SSS

Again, from feasility perspective, we agree with TRS+SSB is a feasible implementation to eable 350km/h UE velocity. While we also still see other possible implementation methods as TRS+PTRS.

Based on that, can we agree that the following statements?

- It is feasible to support maximum speed with 350km/h for downlink with TRS( 4 symbol interval) +SSB for frequency offset tracking under unidirectional and bi-directional RRH deployment  with 120KHz SCS

- It is feasible to support maximum speed with 350km/h for downlink with TRS( 4 symbol interval) for frequency offset tracking under uni-directional RRH deployment  with 120KHz SCS

- It is feasible to support maximum speed with 350km/h for downlink with TRS( 4 symbol interval) +PTRS (L=1) for frequency offset tracking under bi-directional RRH deployment  with 120KHz SCS

[Intel]

- Slide 5: Clarify our Option 2.

- Slide 8: Update Notes on how to conclude on different deployment scenarios for requirements definition

  - There is no direct discussion in deployment analysis whether Scenario A/B or bidirectional/unidirectional is a better approach from link budget perspective. All of them can be supported. In this case it is better to say “consider deployment analysis” rather than “rely on deployment analysis outcome”

- Slide 13: Clarify our Option 2.

- Slide 18: Allow other options for PUSCH MCS considering that is a first meeting for demod discussion

[Huawei]:

@Nokia, for our understanding, post-FFT frequency compensation is needed for one-shot FOE that is not typical implementation for UE. the performance degradation needs be further checked.

@Samsung: For TRS+PTRS, if UE is not scheduled PDSCH for a long time, then no PT-RS will be transmitted, especially when UE pass the RRH, the Doppler jump may not be solved with high probability.

Companies may have different implementation or evaluation assumptions that results in different observations, it is not so necessary to reach common observation or agreements for this part, we can focus the discussion of RS baseline for the final performance requirements definition.

Issue 3-1-3: Carrier frequency for Doppler frequency calculation

Ericsson: Fine for us
  Intel: We support to consider carrier frequency as 30 GHz.

Huawei: We are OK with 30GHz.

Samsung: We are with OK to use 30GHz for maximum Doppler calculation. Similar as FR1 HST, we can add a note as "The same Doppler shift requirement shall be applied regardless of the frequency of operation of the basestation and thus for lower frequencies, the supported speed is higher"

[Nokia]: 30 GHz is fine for us.

Issue 3-2-5: DPS schemes for DL demodulation requirements

Intel: Not feasible to support scheme 1b in bidirectional deployments. In unidirectional deployments both 1a and 1b can be supported.

Huawei: It is the agreement in GTW that “FFS whether another panel can be used for beam search”, the issue can be discussed later when we make the conclusion about that.

Issue 3-3-4-1:  PRACH Formats

Ericsson: OK for us
  Intel: We support to define requirements with only PRACH format C2.

Huawei: OK for us.

Samsung: We are with OK to only define the requirement with PRACH format C2, since the cell coverage is less than 700m for other formats..

[Nokia]: C2 only is OK for us, but we also not against having Ax and Bx PRACH formats

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on FR2 HST Deployment Scenario Analysis | Samsung |  |
| WF on Channel Modeling for FR2 HST | Nokia |  |
| WF on Demodulation requirement for FR2 HST | Samsung |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2105025 | Channel modeling for FR2 HST and TP to TR 38.854 | Samsung | Suggest the TP part of this paper (TP to TR 38.854) is approved. |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2106100 | WF on FR2 HST Deployment Scenario Analysis | Samsung | To be treated in GTW on Tuesday |  |
| R4-2106101 | WF on Channel Modeling for FR2 HST | Nokia | Agreeable |  |
| R4-2106102 | WF on Demodulation requirement for FR2 HST | Samsung | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents