3GPP TSG-RAN WG4 Meeting # 98-bis-e 												R4-2106154

Electronic Meeting, 12th – 20th April, 2021

Agenda item:			8.14.1, 8.14.2

Source:	Moderator (China Telecom)

Title:	Email discussion summary for [98-bis-e][323] NR_perf_enh2_Demod_Part1

Document for:	Information

Introduction

This email thread discusses the general and UE parts for Rel-17 demodulation performance enhancement WI in AI 8.14.1 and 8.14.2.

List of candidate target of email discussion for 1st round and 2nd round:

- 1st round: Invite companies to review the recommended WF in section 1~3, and provide comments (if any) in section 1.3, 2.3 and 3.3.

- 2nd round: Focus on the following revised WP and WFs:

  - WF on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC (led by Intel)

    - Note: The release independent Issue 2-7-1 should be included in this WF.

  - WF on CQI reporting requirements for inter-cell interference MMSE-IRC (led by Ericsson)

  - WF on MMSE-IRC receiver for intra-cell inter-user interference (led by Huawei)

  - Work plan for Further enhancement on NR demodulation performance WI (led by China Telecom)

Topic #1: General

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2104951 | China Telecom | Work plan for Further enhancement on NR demodulation performance WI |
| R4-2104952 | China Telecom | TR skeleton (V0.0.1) for Inter-user interference suppression for NR Multiple-User Multiple-Input Multiple-Output (MU-MIMO) |

Open issues summary

Sub-topic 1-1: Work plan

Issue 1-1: Work plan

- Proposals

  - Initial work plan in  R4-2104951

- Recommended WF

  - Encourage comments if any

Sub-topic 1-2: TR skeleton

Issue 1-2: TR skeleton

- Proposals

  - TR skeleton in R4-2104952: According to the approved WID, the TR skeleton (V0.0.1) is proposed to capture Phase I outcome of Inter-user interference suppression for NR MU-MIMO.

    - Note: With the new objective on LTE CRS-IM performance evaluation added at RAN #91e, the TR title in the WID would probably be updated in the June RAN #92e meeting.

- Recommended WF

  - Encourage comments if any

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Company A | Issue 1-1: Work plan Issue 1-2: TR skeleton |
| Huawei, HiSilicon | Issue 1-1: Work plan We are fine with the work plan Issue 1-2: TR skeleton We are fine with the TR skeleton. |
| Ericsson | Issue 1-1: For LTE CRS-IM, we don’t think it is possible to provide the initial simulation results in RAN4#99 in May. The following work plan is preferred from Ericsson’s view. RAN4 #99 - Discussion on the link simulation assumptions RAN4 #100 - Continue the discussion on the link simulation assumptions - Discussion on the possible impact to other WGs, if necessary. RAN4 #101 - Collection of initial ideal simulation results - Update of link simulation assumptions if needed RAN4 #102 - Collection of updated ideal and impairment results - Update of link simulation assumptions if needed - Draft CRs endorsed RAN4 #102b - Collection of updated ideal and impairment results - CRs approved |
| Qualcomm | Issue 1-1: Work plan Similar concern as Ericsson. Issue 1-2: TR skeleton Additional sections will be needed for LTE CRS-IM performance evaluation. |
| ZTE | Issue 1-1: Work plan Several comments from us: 1) Since Scenario 2 has a lower priority than Scenario 1, for discussions on CQI reporting in RAN4#98-b-e(this meeting), we can focus on necessity of CQI reporting for Scenario 1, and similar discussion for Scenario 2 can be left to RAN4#102 with a condition that Scenario 1 works are completed; 2) For LTE CRS-IM, the very first simulation collection scheduled in May, same meeting for discussion on simulation assumption, might be based on un-aligned assumptions, and may have no meaning to collect such un-calibrated numerical results, therefore, better to schedule the initial simulation results one meeting after the assumptions are agreed. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1 | Issue 1-1: Work plan - Recommendations for 2nd round: - Marked as ‘To be revised’ and check whether further updates are needed based on the comments. Issue 1-2: TR skeleton - Tentative agreements: - The TR skeleton can be agreeable. - As noted in issue 1-2 and in the cover of the TR skeleton, CRS-IM related changes to this TR will be made based on the revised WID in the June RAN #92e meeting |

Discussion on 2nd round

R4-2106119	Work plan for Further enhancement on NR demodulation performance WI

Type: Work Plan		For: Approval
					Source: China Telecom

Decision:		Return to.

Discussion:

[CTC]: Some reply to the comments:

Comment #1 on the time to collect results for CRS-IM:

Companies commented that it is too early to collect initial simulation results in RAN4#99 in May, and it is meaningless to collect results based on un-aligned assumptions.

CTC reply:

As seen in the Note 3 of the work plan table, according to RAN plenary guidance, RAN4 is asked to perform this evaluation until RAN#93E (September), to be reviewed at RAN#92E (June).

If RAN plenary will review the status in June, it is better to have a look at some initial simulation results in May. Like MU-MIMO IRC in this WI, although it is the first meeting to discuss this objective, several companies have submitted initial simulation results.

But we agree that the results based on un-aligned assumptions may not be that helpful, so we have changed this bullet as follows, i.e., submission of simulation results is allowed, but the results will not be put together based on a template.

•       Collection Submission of initial simulation results if any

Comment #2 on the other WG impact analysis for CRS-IM:

Thanks E/// for the good suggestion, which has been added in the revised version.

Comment #3 on the plan after RAN#93E for CRS-IM:

From our company view, we fully support the whole plan from E///. Meanwhile, considering currently the WID only includes the study phase, we may need to add the plan after RAN#93E later.

Comment #4 on separating the discussion on whether to introduce CQI reporting requirements for scenario 1 and scenario 2

We are fine with the comment from ZTE, and has made the change accordingly.

[Intel]: Thank you for work plan update. We agree with your comments.

As for collection of CRS-IM results, we have rather same comment. In the previous RAN meeting WF RP-210906 was agreed. In this WF we have the following note: Ask RAN4 to perform this evaluation until RAN#93E (September), to be reviewed at RAN#92E (June).

Based on our understanding, it is assumed that intermediate check of evaluation will be done in June RAN meeting (i.e. after RAN4 #99). Therefore, we expect that interested companies can bring initial simulation results in the next RAN4 meeting. We think that original or new wording with clarification note “if any” should be fine, because it does not mandate every company to provide result in the next RAN4 meeting.

[Ericsson]: Thank you very much for your revision. We have concern the plan of the CSI-RS in May meeting.

We are wondering if companies can submit the simulation results without any simulation assumption. Companies can provide the results, but we don’t want to mention it the work plan.

| RAN4 #99 (May 2021) | - Discussion and initial agreement on link level simulation assumptions - Submission of initial simulation results if any |
| --- | --- |
| RAN4 #100 (Aug 2021) | - Agreement on baseline assumptions - Discussion on the possible impact to other WGs, if necessary. - Collection of simulation results if any |

[CTC]: If it is common understanding that interested companies will bring initial simulation results for CRS-IM in RAN4 #99, we are ok with Ericsson’s update.

Topic #2: MMSE-IRC receiver for inter-cell interference

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2104606 | CMCC | Proposal 1: Introduce the Micro-cell (victim cell) and Macro-cell (interfering cell) scenario for R17 demodulation enhancements inter-cell interference suppressing. Proposal 2: the following scenarios should be considered: - FR1, FDD sync scenario; - FR1, FDD async scenario; - FR1, TDD sync scenario; Proposal 3: the following SCS and bandwidth combination should be considered: - FDD - 10MHz/15kHz - 50MHz/15kHz - TDD - 40MHz/30kHz - 100MHz/30kHz Proposal 4: For TDD 30kHz, use typical 7D1S2U(S=6D+4G+4U) for TDD configuration. Proposal 5: - For victim cell, first consider rank-1 transmission. - For MIMO rank transmission probability on the interference cells, take rank-1 transmission probability 70% as the starting point. Proposal 6: - For victim cell, take 2Tx as the baseline. - For interference cells, consider both 4Tx and 2Tx. Proposal 7: SSB configuration for victim cells and interference cells can be aligned. - SSB position in burst: first SSB in Slot#0 - SSB periodicity: 20ms. Proposal 8: Use throughput vs SNR as the test metric for MMSE-IRC receiver demodulation performance requirements, and consider 70% relative throughput for the test points. |
| R4-2104846 | Apple | Observation #1: DMRS based interference covariance estimation suffers degradation when there is no interference on DMRS REs. Proposal #1: For requirements with DMRS based interference covariance estimation, limit scenarios to cases where interference can be rejected with DMRS based covariance estimation. Proposal #2: Introduce requirements with ICI for QPSK and 16QAM. Proposal #3: Further discuss interference model for NR. |
| R4-2104953 | China Telecom | The following proposals were made w.r.t. the network scenario: Proposal 1: Cover both sync and async network scenarios for FDD, and sync network scenario for TDD. Proposal 2: For the SCS, cover at least 15kHz SCS for FDD and 30kHz SCS for TDD. Proposal 3: For the channel bandwidth, cover 10 MHz and 40 MHz channel bandwidth for FDD 15kHz SCS, 40MHz and 100MHz channel bandwidth for TDD 30kHz SCS. Proposal 4: Consider two TDD configurations for 30kHz SCS: - Configuration 1: 7D1S2U, S = 6D:4G:4U - Configuration 2: DDDSUDDSUU, S1=10G: 2G: 2U, S2 = 10G: 2G: 2U Proposal 5: Only consider PDSCH to PDSCH interference. Use symbols #0 and #1 of each slot for control channel in all the considered cells. The following observation and proposals were made w.r.t. the reference receiver: Observation 1: It is unknown whether the precoding matrix in any two contiguous PRBs in the neighboring/interfering cell(s) is the same. Proposal 6: The estimation of interference covariance matrix can be performed at per PRB and per slot basis. Proposal 7: Reuse the interference profiles for LTE MMSE-IRC receiver, i.e., assume target geometry of -2.5dB, use DIP1/2= -1.73/-8.66 dB for synchronous network, and use DIP1/2= -2.23/-8.06 dB for asynchronous network. Proposal 8: The number of explicit interferers can be 2 or 1 for different tests, by considering the tradeoff between test complexity and the gain of MMSE-IRC over MMSE receiver. Proposal 9: Assume 70% and 30% probability for rank 1 and rank 2 transmission in the interfering cell(s). Proposal 10: For the interferers, assume random precoding per slot and per PRB bundling granularity, with PRB bundling size of 2. Proposal 11: Assume 16QAM randomly modulated symbols in the interfering PDSCH. The following proposals were made w.r.t. the target PDSCH parameters: Proposal 12: Use rank 1 for target PDSCH. Proposal 13: As baseline, choose MCS for which the resulting geometry value falls in the range ~[-2.5dB, 0dB] with typical margin considered. Proposal 14: For the precoding model of target PDSCH, further discuss whether to use random precoding or follow PMI. Proposal 15: For the throughput test point, use 70% of max TP as baseline. Proposal 16: Re-use the Rel-15 assumptions on HARQ process number, i.e., 4 for FDD 15kHz SCS and 8 for TDD 30kHz SCS. The following proposals were made w.r.t. the common parameters for target and interfering PDSCH: Proposal 17: For the antenna configuration, use 2Tx as baseline, and cover both 2Rx and 4Rx. Proposal 18: For the propagation condition, include both TDLA30-10 and TDLC300-100 at this early phase, and decide whether down-selection is needed later. Use ULA low for the antenna correlation. Proposal 19: In both target and interfering PDSCH, assume PDSCH mapping type A with full PRB allocation, use DMRS Type 1 with single symbol front loaded and 1 additional DMRS, with FDM applied between DMRS and data. Proposal 20: For SSB in both target and interfering cells, reuse the Rel-15 assumptions , i.e., configure the first SSB in slot #0 in every 20 slots, and the slot #0 in every 20 slots is not scheduled for PDSCH transmission. Proposal 21: Use Physical cell ID of 0 for the serving cell, and cell ID 1 and 2 for the two interfering cells Proposal 22: Further discuss whether TRS/CSI-RS are collided among cells, as well as whether DMRS and TRS/CSI-RS are collided among cells. The following observation and proposals were made w.r.t. the CQI reporting requirements: Observation 2: Two categories of scenarios can be considered for further discussion on CQI reporting requirement: - A) Interference is precoded as data, i.e., in serving cell’s NZP CSI-RS REs, DMRS and/or data is transmitted in interfering cell. For these scenarios, MMSE-IRC based CQI calculation is beneficial for the PDSCH performance. - B) Interference is not precoded as data, i.e., in serving cell’s NZP CSI-RS REs, NZP CSI-RS or ZP CSI-RS is transmitted in interfering cell. For these scenarios, check the performance benefits brought by MMSE-IRC based CQI calculation over MMSE based CQI calculation if needed. Proposal 23: For MMSE-IRC based CQI reporting, the interference covariance can be estimated and averaged among multiple PRBs, and the exact PRB number for interference covariance averaging needs further discussion. Proposal 24: As the starting point, model one inter-cell interferer with DIP of -0.41dB and static propagation condition. Proposal 25: Reuse the LTE test metric as a starting point, which include: 1) the relative increase of the throughput obtained when the transport format is that indicated by the reported CQI subject to an interference model compared to the case with a white Gaussian noise model, and 2) when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified DIP, the average BLER for the indicated transport formats shall be greater than or equal to 2%. |
| R4-2104977 | NTT DOCOMO, INC. | Proposal 1: For MMSE-IRC receiver for inter-cell interference test, the following test parameters are applied; Parameters; Value. Duplex mode/SCS; FDD 15kHz, TDD 30kHz. Channel Bandwidth; FDD 10MHz, TDD 40MHz. TDD pattern; 7D1S2U (S= 6D:4G:4U). Antenna configuration; 2x2, 2x4. PDSCH mapping; Type A, Start symbol 2, Duration 12. PDCCH allocation; Symbol 0,1. Test metric; SNR @70% of maximum throughput. |
| R4-2106426 | Intel Corporation | Proposal 1: Define CQI reporting requirements for MMSE-IRC receiver for scenario with inter-cell interference. Proposal 2: Use the following LTE NAICS assumptions for interference modelling for initial alignment of NR MMSE-IRC performance: - Explicit modelling of 2 interference cells with INRs 13.91 and 3.34 dB or 7.77 and 2.29 dB - Interference signal is transmitted in each slot with 16QAM randomly modulated symbols and with random rank (Rank 1 – 80% and Rank 2 – 20 %) Proposal 3: Analyse the SINR and INRs distributions for “Urban macro” NR scenarios using methodology from TR 36.866 and system level assumptions from TR 38.913 and 38.901. Proposal 4: Consider the following simulation assumptions for Scenario 1 requirements: - SCS/CBW: 15 kHz / 10 MHz for FDD and 30 kHz / 40 MHz for TDD - TDD pattern: 7D1S2U, S = 6D+4G+4U - PDSCH configuration for serving and interference cells - Type A PDSCH mapping with starting symbol 2 and duration 12 - DMRS Type 1 with 1 additional RS - Number of PDSCH DMRS CDM group(s) without data = 1 - Serving PDSCH Rank 1, MCS 5 or 13 or 19. - Channel model: TDL-A, 30 ns, 10 Hz - Antenna configuration: 2x2 and 2x4 with ULA Low correlation - Receiver assumptions: MMSE-IRC with DMRS based covariance matrix estimation and without time selective interference handling Proposal 5: Consider the following simulation assumptions for Scenario 2 requirements: - SCS/CBW: 15 kHz / 10 MHz for FDD and 30 kHz / 40 MHz for TDD - TDD pattern: 7D1S2U, S = 6D+4G+4U - PDSCH configuration for serving cell - Type A PDSCH mapping with starting symbol 2 and duration 12 - DMRS Type 1 with 1 additional RS - Number of PDSCH DMRS CDM group(s) without data = 1 - RMC: Rank 1, MCS 5 or 13 or 19. - PDSCH configuration for interference cells - Option 1: Type A PDSCH mapping with starting symbol 2 and duration 5 for both cells - Option 2: - Type A PDSCH mapping with starting symbol 2 and duration 5 for cell #1 - Type B PDSCH mapping with starting symbol 7 and duration 7 for cell #2 - Option 3: Type B PDSCH mapping with starting symbol 4 and duration 7 for both cells - Channel model: TDL-A, 30 ns, 10 Hz - Antenna configuration: 2x2 and 2x4 with ULA Low correlation - Receiver assumptions: - Option 1: MMSE-IRC with DMRS based covariance matrix estimation and without time selective interference handling - Option 2: MMSE-IRC with DMRS based covariance matrix estimation and with time selective interference handling - Option 3: MMSE-IRC with DMRS and Data based covariance matrix estimation Observation #1: MMSE-IRC allows to achieve ~3.5 dB performance benefits in comparison to MMSE-MRC for Scenario 1 with considered simulation assumptions Observation #2: MMSE-IRC allows to achieve ~3-4 dB performance benefits in comparison to MMSE-MRC for Scenario 2-1 and 2-2 with considered simulation assumptions. Observation #3: No MMSE-IRC performance benefits in comparison to MMSE-MRC are observed for Scenario 2-3. Observation #4: MMSE-IRC processing with time selective interference handling allows to achieve additional ~1-2 dB performance benefits in comparisons to basic MMSE-IRC processing for Scenario 2-1 and 2-2 with considered simulation assumptions. Observation #5: MMSE-IRC processing with time selective interference handling allows to achieve additional ~6-8 dB performance benefits in comparison to MMSE-MRC or basic MMSE-IRC processing for Scenario 2-3 with considered simulation assumptions. |
| R4-2106833 | Huawei, HiSilicon | Proposal 1: We propose the following configurations for PDSCH and DMRS: - PDSCH configuration for serving cell and neighboring cell: - PDSCH Mapping Type: Type A - Starting symbol: 2 - Length: 12 - PDSCH DMRS configuration for serving cell and neighboring cell: - DMRS Type: Type 1 - Number of additional DMRS: 1 - Scheduling with data multiplexed on the DMRS symbols Proposal 2: We propose to define the requirements for 15 kHz/10 MHz for FDD and 30 kHz/40 MHz for TDD. Proposal 3: We propose to use DIP ratio to define the interference profiles. Proposal 4: We propose to consider two neighbour cells as the interference cells. Proposal 5: We propose to use TDLA30-10 as the propagation condition. Proposal 6: We propose to use 16QAM as the target MCS for inter-cell MMSE IRC requirements. Proposal 7: We propose to further discuss whether to introduce the corresponding CQI reporting test. |
| R4-2106867 | Ericsson | Proposal 1: RAN4 should agree with the inter-cell interference model before discussing the detailed simulation assumption for inter-cell interference scenario. Proposal 2: RAN4 defines the interference modes for NR performance enhanced requirements as follows, by reusing LTE interference model. Some values in [] in the texts below may be discussed further. This subclause provides interference modelling for each explicitly modelled interfering cell in the requirement scenario. In each slot, each interfering cell shall transmit randomly modulated data over the entire PDSCH region and the full transmission bandwidth according to the probabilities of occurrence. Transmitted physical channels shall include SS/PBCH block. Probabilities of occurrence in each slot are as specified in the requirement scenario. If the probabilities of occurrence in each slot are not specified in the requirement scenario, as default, they are equal to 1. For each slot and each [smallest] subband as defined in subclause 5.2.1.4 of TS38.214, a transmission rank shall be randomly determined independently from other subbands as well as other interfering cells. Probabilities of occurrence of each possible transmission rank are as specified in the requirement scenario. For each slot and each subband, a precoding matrix for the number of layers v associated to the selected rank shall be selected randomly from [single panel type I]. The generic beamforming model in subclause B.4 shall be applied assuming DM-RS and CSI reference signals as specified in the requirement scenario. Random precoding with selected rank and precoding matrices for each slot and each subband shall be applied to [16QAM] randomly modulated layer symbols including the DM-RS over antenna port [1000] when the rank is one and antenna ports [1000/1001] when the rank is two. For unallocated REs in the CORESET region, precoding of randomly selected from single panel type I, per slot with equal probability of each applicable i1, i2 combination, and with REG bundling granularity for number of Tx larger than 1, shall be applied to QPSK randomly modulated layer symbols.. Proposal 3: Neighboring cell(s) schedule SSB at the same location as the serving cell. Proposal 4: Neighboring cell(s) schedule CSI reference signals (for both tracking and CSI acquisition) at the same location as the serving cell. Proposal 5: RAN4 use the dominant interference proportion (DIP) to specify the received signal powers from interfering cells: DIPi=Iori+1Noc' Where ior(i+1) is the average received power spectral density from the i-th strongest interfering cell involved in the requirement scenario and Noc'=j=2Nioc(j)+Noc, where Noc is the average power spectral density of a white noise source consistent with the definition provided in TS38.101-4 subclause 4.4.3, and N is the total number of cells involved in a given requirement scenario. Note ior(1) is assumed to be the power spectral density associated with the serving cell.. Proposal 6: RAN4 specify the NR UE demodulation requirements for MMSE-IRC suppressing the inter-cell interference with regard to SINR to achieve 70% of maximum throughput. Proposal 7: Define SINR (for FR1) as follows, and capture it in TS38.101-4 Clause 4.4.2. SINR=j=1NRXEsjj=1NRXN'ocj Where Es(j) is the averaged received energy per Hz of the wanted signal during the useful part of the symbol, i.e. excluding the cyclic prefix, at the j-th UE receiver antenna connector ; average power is computed within a set of REs used for the transmission of physical, divided transmission bandwidth within the set. And Noc'(j) is the power spectral density (average power per RE normalised to the subcarrier spacing) of the summation of the received power spectral densities of the strongest interfering cells explicitly defined in a test procedure plus Noc(j), as measured at the j-th UE receiver antenna connector. The respective power spectral density of each interfering cell relative to Noc(j) is defined by its associated DIP value, or the respective power spectral density of each interfering cell relative to Noc(j) is defined by its associated Es/Noc value.. Proposal 8: RAN4 should study first the MMSE-IRC receiver performance with interfering cell(s) whether it shows the gain compared with the case without neighboring interfering cell(s), i.e., AWGN-only scenario. Proposal 9: Use Rel-11 LTE DIP settings as the starting point, i.e., (DIP Cell 2, DIP Cell 3) = (-2.23, -8.06), (-2.23, -Inf), (-1.73, -8.66), (-1.73, -Inf). Proposal 10: In Rel-17, MMSE-IRC receiver performance requirements with interference cell condition is defined only for single carrier scenario. Proposal 11: For the Rel-17 MMSE-IRC receiver performance evaluation, RAN4 should assume the following parameters as the starting point: Parameters; FDD; TDD. CBW; 10MHz (single carrier only); 40MHz (single carrier only). TDD configuration; N/A; 7DS2U, S=6D+4G+4U Schedule PDSCH in special slots. SCS; 15kHz; 30kHz. PDSCH configuration; Type A, Start symbol 2, Duration 12​; Type A, Start symbol 2, Duration 12​. PRB bundling size; 2; 2. PDSCH DMRS configuration; Type 1, 1+1; Type 1, 1+1. MCS; MCS 4 (QPSK, CR=0.3) MCS 13 (16QAM, CR=0.5); MCS 4 (QPSK, CR=0.3) MCS 13 (16QAM, CR=0.5). Rank; 1; 1. PDSCH precoder; Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity; Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity. Antenna configuration; 2x2, 2x4; 2x2, 2x4. Propagation condition; TDLA30-10 TDLC300-100; TDLA30-10 TDLC300-100. DIP; (DIP1, DIP2) = (-2.23, -8.06) (DIP1, DIP2) = (-2.23, -Inf) (DIP1, DIP2) = (-1.73, -8.66) (DIP1, DIP2) = (-1.73, -Inf); (DIP1, DIP2) = (-2.23, -8.06) (DIP1, DIP2) = (-2.23, -Inf) (DIP1, DIP2) = (-1.73, -8.66) (DIP1, DIP2) = (-1.73, -Inf). Time Offset relative to serving cell; 0 us; 0 us. Metric; SINR to achieve 70% of maximum throughput; SINR to achieve 70% of maximum throughput. Proposal 12: RAN4 defines the CSI reporting tests with neighboring cell(s) interference condition. Proposal 13: RAN4 discuss whether the UE demodulation and CQI reporting requirements with inter-cell interference is released independent from Rel-15 or not, after RAN4 agree with the detailed simulation assumption. |
| R4-2107093 | MediaTek inc. | Proposal 1: Determine the number of interferers and the corresponding interference profiles first. Proposal 2: Regarding CQI report, discuss the assumption of interference for DM-RS and PDSCH first. |

Open issues summary

Note: The issues in sub-topic 2-1 to 2-5 are for scenario 1 with slot-based transmission and aligned SCS among cells.

Sub-topic 2-1: Interference model for scenario 1

Issue 2-1-1: Sync and async network for FR1

- Proposals

  - Option 1: (CMCC, China Telecom)

    - FDD sync scenario

    - FDD async scenario

    - TDD sync scenario

  - Option 2: FDD and TDD sync scenario (E///)

- Recommended WF

  - Encourage feedback from more companies

Issue 2-1-2: Interference profile

- Proposals

  - Option 1: Reuse the DIP based interference profiles from LTE MMSE-IRC receiver (China Telecom, Huawei, E///, [MediaTek])

    - China Telecom:

      - Assume target geometry of -2.5dB, use DIP1/2= -1.73/-8.66 dB for synchronous network, and use DIP1/2= -2.23/-8.06 dB for asynchronous network.

      - The number of explicit interferers can be 2 or 1 for different tests, by considering the tradeoff between test complexity and the gain of MMSE-IRC over MMSE receiver.

    - HW: Consider two neighbour cells as the interference cells

    - E///: Use Rel-11 LTE DIP settings as the starting point, i.e., (DIP Cell 2, DIP Cell 3) = (-2.23, -8.06), (-2.23, -Inf), (-1.73, -8.66), (-1.73, -Inf).

  - Option 2: In addition to the Macro cell scenario considered for LTE MMSE-IRC receiver, introduce the Micro-cell (victim cell) and Macro-cell (interfering cell) scenario  (CMCC)

    - CMCC: we observe the interference in realistic deployment

  - Option 3: (Intel)

    - Use the following LTE NAICS assumptions for interference modelling for initial alignment of NR MMSE-IRC performance

      - Explicit modelling of 2 interference cells with INRs 13.91 and 3.34 dB or 7.77 and 2.29 dB

    - Analyse the SINR and INRs distributions for “Urban macro” NR scenarios using methodology from TR 36.866 and system level assumptions from TR 38.913 and 38.901.

- Recommended WF

  - Encourage more discussion

Issue 2-1-3: Transmission rank of interfering PDSCH

- Proposals

  - Option 1: Reuse LTE MMSE-IRC assumption for DMRS-based transmission mode, i.e., random rank with 70% and 30% probability for rank 1 and rank 2 transmission in the interfering cell(s), as the starting point  (CMCC, China Telecom)

  - Option 2: Reuse LTE NAICS assumption, i.e., random rank with 80% and 20% probability for rank 1 and rank 2 transmission in the interfering cell(s) (Intel)

- Recommended WF

  - Encourage feedback from more companies

Issue 2-1-4: Precoding of interfering PDSCH

- Proposals

  - Option 1: Random precoding  (China Telecom, Ericsson)

    - Option 1A: Random precoding  per slot and per PRB bundling granularity, with PRB bundling size of 2 (China Telecom)

    - Option 1B: Random precoding  per slot and per [smallest] subband as defined in subclause 5.2.1.4 of TS38.214; [single panel type I] codebook (Ericsson)

- Recommended WF

  - Encourage feedback from more companies

Issue 2-1-5: Modulation order of interfering PDSCH

- Proposals

  - Option 1: Reuse LTE assumption for MMSE-IRC receiver, i.e., 16QAM randomly modulated symbols (China Telecom, [E///], Intel)

- Recommended WF

  - Encourage feedback from more companies

Sub-topic 2-2: DMRS configuration and reference receiver for scenario 1

Issue 2-2-1: DMRS configuration

- Proposals

  - Option 1: limit scenarios to cases where interference can be rejected with DMRS based covariance estimation. (Apple)

    - Apple: DMRS based interference covariance estimation suffers degradation when there is no interference on DMRS REs.

  - Option 2: for both serving and interfering cells, DMRS Type 1 with single symbol front loaded and 1 additional DMRS, with FDM applied between DMRS and data (China Telecom, Intel, Huawei)

  - Option 3: DMRS Type 1 with single symbol front loaded and 1 additional DMRS (E///)

- Recommended WF

  - Encourage more discussion

Issue 2-2-2: Interference covariance estimation granularity

- Proposals

  - Option 1: The estimation of interference covariance matrix can be performed at per PRB and per slot basis (China Telecom, Intel)

    - China Telecom: It is unknown whether the precoding matrix in any two contiguous PRBs in the neighboring/interfering cell(s) is the same.

- Recommended WF

  - Encourage feedback from more companies

Sub-topic 2-3: Target PDSCH parameters for scenario 1

Issue 2-3-1: Transmission rank

- Proposals

  - Option 1: rank 1  (CMCC, China Telecom, Intel, E///)

- Recommended WF

  - Use rank 1 as baseline

Issue 2-3-2: MCS

- Proposals

  - Option 1: QPSK and 16QAM (Apple, E///)

    - Option 1A: MCS 4 (QPSK, CR=0.3), MCS 13 (16QAM, CR=0.5) (E///)

  - Option 2: choose MCS for which the resulting geometry value falls in the range ~[-2.5dB, 0dB] with typical margin considered (China Telecom)

  - Option 3: MCS 5 or 13 or 19 (Intel)

  - Option 4: 16QAM (HW)

- Recommended WF

  - Encourage more discussion

Issue 2-3-3: Precoding model

- Proposals

  - Option 1: Further discuss whether to use random precoding or follow PMI (China Telecom)

  - Option 2: Single Panel Type I, Random precoder selection (E///)

- Recommended WF

  - Encourage more discussion

Issue 2-3-4: PRB bundle size

- Proposals

  - Option 1: 2 (E///)

- Recommended WF

  - Encourage more discussion

Issue 2-3-5: Performance measurement point

- Proposals

  - Option 1: 70% relative throughput (CMCC, China Telecom, DCM, E///)

    - Option 1A: SINR (not SNR) at 70% TP (E///)

- Recommended WF

  - Use Option 1 + Option 1A as baseline

Issue 2-3-6: HARQ process number

- Proposals

  - Option 1: Re-use the Rel-15 assumptions on HARQ process number, i.e., 4 for FDD 15kHz SCS and 8 for TDD 30kHz SCS (China Telecom)

- Recommended WF

  - Use Option 1 as baseline

Sub-topic 2-4: Other parameters for target and interfering PDSCH for scenario 1

Issue 2-4-1: SCS

- Proposals

  - Option 1: FDD 15kHz, TDD 30kHz (CMCC, China Telecom, DCM, Intel, Huawei, E///)

- Recommended WF

  - Agree Option 1

Issue 2-4-2: Channel bandwidth

- Proposals

  - Option 1 (CMCC)

    - FDD 15kHz: 10MHz, 50MHz

    - TDD 30kHz: 40MHz, 100MHz

      - CMCC: We also support to consider the maximum bandwidth for FDD and TDD, which is 50MHz for FDD and 100MHz for TDD.

  - Option 2 (China Telecom)

    - FDD 15kHz: 10MHz, 40MHz

    - TDD 30kHz: 40MHz, 100MHz

  - Option 3 (DCM, Intel, Huawei, E///)

    - FDD 15kHz: 10MHz

    - TDD 30kHz: 40MHz

- Recommended WF

  - Encourage more discussion

Issue 2-4-3: TDD DL/UL configuration for 30kHz SCS

- Proposals

  - Option 1: 7D1S2U(S=6D+4G+4U) (CMCC, DCM, Intel)

  - Option 2: Consider two TDD configurations (China Telecom)

    - Configuration 1: 7D1S2U, S = 6D:4G:4U

    - Configuration 2: DDDSUDDSUU, S1=10G: 2G: 2U, S2 = 10G: 2G: 2U

- Recommended WF

  - Encourage more discussion

Issue 2-4-4: Number of carriers

- Proposals

  - Option 1: In Rel-17, MMSE-IRC receiver performance requirements with interference cell condition is defined only for single carrier scenario. (E///)

- Recommended WF

  - Encourage feedback from more companies

Issue 2-4-5: PDCCH and PDSCH allocation

- Proposals

  - Option 1: for serving and interfering cells (China Telecom, Intel, DCM, Huawei, E///)

    - Use symbols #0 and #1 of each slot for PDCCH

    - PDSCH Start symbol 2, Duration 12

- Recommended WF

  - Agree Option 1

Issue 2-4-6: Tx antenna number

- Proposals

  - Option 1: 2Tx as baseline for serving cell, both 4Tx and 2Tx for interfering cells (CMCC)

  - Option 2: 2Tx as baseline for serving cell and interfering cells (China Telecom, DCM, Intel, E///)

- Recommended WF

  - Encourage more discussion

Issue 2-4-7: Propagation condition

- Proposals

  - Option 1: Include both TDLA30-10 and TDLC300-100 at this early phase (China Telecom, E///)

  - Option 2: TDLA30-10 (Intel, HW)

- Recommended WF

  - Encourage more discussion

Issue 2-4-8: Antenna correlation

- Proposals

  - Option 1: ULA low (China Telecom, Intel)

- Recommended WF

  - Can we use Option 1 as baseline?

Issue 2-4-9: PDSCH mapping type

- Proposals

  - Option 1: PDSCH mapping type A for serving and interfering PDSCH (China Telecom, DCM, Huawei)

- Recommended WF

  - Can we agree Option 1?

Issue 2-4-10: PRB allocation

- Proposals

  - Option 1: Full PRB allocation for serving and interfering PDSCH (China Telecom)

- Recommended WF

  - Can we agree Option 1?

Issue 2-4-11: SSB configuration for serving and interfering cells

- Proposals

  - Option 1: Aligned SSB configuration for serving and interference cells (CMCC, China Telecom, E///)

    - Option 1A: (CMCC, China Telecom)

      - SSB position in burst: first SSB in Slot#0; SSB periodicity: 20ms.

      - The slot #0 in every 20 ms is not scheduled for PDSCH transmission

- Recommended WF

  - Encourage feedback from more companies

Issue 2-4-12: Physical cell ID

- Proposals

  - Option 1: Physical cell ID of 0 for the serving cell, and cell ID 1 and 2 for the two interfering cells (China Telecom)

- Recommended WF

  - Encourage feedback from more companies

Issue 2-4-13: TRS/CSI-RS among cells

- Proposals

  - Option 1: Further discuss whether TRS/CSI-RS are collided among cells (China Telecom)

  - Option 2: Neighboring cell(s) schedule CSI reference signals (for both tracking and CSI acquisition) at the same location as the serving cell (E///)

- Recommended WF

  - Encourage feedback from more companies

Issue 2-4-14: DMRS and TRS/CSI-RS among cells

- Proposals

  - Option 1: Further discuss whether DMRS and TRS/CSI-RS are collided among cells (China Telecom)

- Recommended WF

  - Encourage feedback from more companies

Sub-topic 2-5: CQI reporting requirements for scenario 1

Issue 2-5-1: Whether to define CQI reporting requirements

- Proposals

  - Option 1: Yes (China Telecom, Intel, E///)

  - Option 2: Further discuss (Huawei, MediaTek)

    - MediaTek: Enhancement on CQI can be considered only if network can guarantee same interference is experienced by CSI-RS and PDSCH.

- Recommended WF

  - Encourage more discussion

Issue 2-5-2: Interference covariance estimation granularity for CQI reporting

- Proposals

  - Option 1: the interference covariance can be estimated and averaged among multiple PRBs, and the exact PRB number for interference covariance averaging needs further discussion (China Telecom)

- Recommended WF

  - Encourage feedback from more companies

Issue 2-5-3: Interference model for CQI reporting

- Proposals

  - Option 1: As the starting point, model one inter-cell interferer with DIP of -0.41dB and static propagation condition (China Telecom)

- Recommended WF

  - Encourage feedback from more companies

Issue 2-5-4: Test metric for CQI reporting

- Proposals

  - Option 1: Reuse the LTE test metric as a starting point, which include (China Telecom)

    - 1) the relative increase of the throughput obtained when the transport format is that indicated by the reported CQI subject to an interference model compared to the case with a white Gaussian noise model, and

    - 2) when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified DIP, the average BLER for the indicated transport formats shall be greater than or equal to 2%.

- Recommended WF

  - Encourage feedback from more companies

Sub-topic 2-6: Scenario 2 with non-slot-based transmission

Issue 2-6-1: Test parameters

- Proposals

  - Option 1 (Intel)

    - SCS/CBW: 15 kHz / 10 MHz for FDD and 30 kHz / 40 MHz for TDD

    - TDD pattern: 7D1S2U, S = 6D+4G+4U

    - PDSCH configuration for serving cell

      - Type A PDSCH mapping with starting symbol 2 and duration 12

      - DMRS Type 1 with 1 additional RS

      - Number of PDSCH DMRS CDM group(s) without data = 1

      - RMC: Rank 1, MCS 5 or 13 or 19.

    - PDSCH configuration for interference cells

      - Option 1: Type A PDSCH mapping with starting symbol 2 and duration 5 for both cells

      - Option 2:

    - Type A PDSCH mapping with starting symbol 2 and duration 5 for cell #1

    - Type B PDSCH mapping with starting symbol 7 and duration 7 for cell #2

      - Option 3: Type B PDSCH mapping with starting symbol 4 and duration 7 for both cells

    - Channel model: TDL-A, 30 ns, 10 Hz

    - Antenna configuration: 2x2 and 2x4 with ULA Low correlation

    - Receiver assumptions:

      - Option 1: MMSE-IRC with DMRS based covariance matrix estimation and without time selective interference handling

      - Option 2: MMSE-IRC with DMRS based covariance matrix estimation and with time selective interference handling

      - Option 3: MMSE-IRC with DMRS and Data based covariance matrix estimation

- Recommended WF

  - Encourage feedback from more companies

Sub-topic 2-7: Release independence

Issue 2-7-1: Release independence

- Proposals

  - Option 1: RAN4 discuss whether the UE demodulation and CQI reporting requirements with inter-cell interference is released independent from Rel-15 or not, after RAN4 agree with the detailed simulation assumption (E///)

- Recommended WF

  - Encourage feedback from more companies

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Company A | Sub-topic 2-1: Interference model Issue 2-1-1: Sync and async network for FR1 Issue 2-1-2: Interference profile Issue 2-1-3: Transmission rank of interfering PDSCH Issue 2-1-4: Precoding of interfering PDSCH Issue 2-1-5: Modulation order of interfering PDSCH Sub-topic 2-2: DMRS configuration and reference receiver Issue 2-2-1: DMRS configuration Issue 2-2-2: Interference covariance estimation granularity Sub-topic 2-3: Target PDSCH parameters Issue 2-3-1: Transmission rank Issue 2-3-2: MCS Issue 2-3-3: Precoding model Issue 2-3-4: PRB bundle size Issue 2-3-5: Performance measurement point Issue 2-3-6: HARQ process number Sub-topic 2-4: Other parameters for target and interfering PDSCH Issue 2-4-1: SCS Issue 2-4-2: Channel bandwidth Issue 2-4-3: TDD DL/UL configuration for 30kHz SCS Issue 2-4-4: Number of carriers Issue 2-4-5: PDCCH and PDSCH allocation Issue 2-4-6: Tx antenna number Issue 2-4-7: Propagation condition Issue 2-4-8: Antenna correlation Issue 2-4-9: PDSCH mapping type Issue 2-4-10: PRB allocation Issue 2-4-11: SSB configuration for serving and interfering cells Issue 2-4-12: Physical cell ID Issue 2-4-13: TRS/CSI-RS among cells Issue 2-4-14: DMRS and TRS/CSI-RS among cells Sub-topic 2-5: CQI reporting requirements Issue 2-5-1: Whether to define CQI reporting requirements Issue 2-5-2: Interference covariance estimation granularity for CQI reporting Issue 2-5-3: Interference model for CQI reporting Issue 2-5-4: Test metric for CQI reporting Sub-topic 2-6: Scenario 2 with non-slot-based transmission Issue 2-6-1: Test parameters Sub-topic 2-7: Release independence Issue 2-7-1: Release independence |
| China Telecom | Sub-topic 2-1: Interference model Issue 2-1-1: Sync and async network for FR1 Support option 1. LTE UE MMSE-IRC test has covered both sync and async for FDD. For the simulation work, sync FDD/TDD can be prioritized over async FDD. Issue 2-1-2: Interference profile For HomNet, support to reuse the DIP based interference profile from LTE, i.e., option 1. We also support to cover HetNet as proposed in option 2. In HetNet with co-channel micro cells, the interference will be severer, and MMSE-IRC can help suppress the inference. The DIP values for HetNet need further discussion; one option could be to translate the INRs in NAICS scenario 2 to DIPs, in order to avoid additional system simulation. Issue 2-1-3: Transmission rank of interfering PDSCH Not sure if there will be obvious performance difference for the two options. Maybe we can select one as baseline by following the majority view after the 1st round. Issue 2-1-4: Precoding of interfering PDSCH For the codebook, ok to use single panel type I as proposed in option 1B. For the granularity of precoding in frequency-domain, either option 1A or 1B is fine. Issue 2-1-5: Modulation order of interfering PDSCH Support option 1. Sub-topic 2-2: DMRS configuration and reference receiver Issue 2-2-1: DMRS configuration It seems the three options do not conflict with each other. Option 2 gives more details, and we support option 2. Issue 2-2-2: Interference covariance estimation granularity Support option 1. Sub-topic 2-3: Target PDSCH parameters Issue 2-3-1: Transmission rank Support the recommended WF. Issue 2-3-2: MCS Given the different options here, for the initial simulation, maybe we can select one MCS for QPSK and one MCS for 16QAM, e.g., MCS 5 and 13, and the MCSs can adjusted later after more simulation results are collected. Issue 2-3-3: Precoding model It is fine to use option 2 in the initial simulation, while follow PMI is not precluded. Issue 2-3-4: PRB bundle size Ok with option 1. Issue 2-3-5: Performance measurement point Support the recommended WF. With explicit inter-cell interference modelling, SINR instead of SNR should be used. Issue 2-3-6: HARQ process number Support the recommended WF. Sub-topic 2-4: Other parameters for target and interfering PDSCH Issue 2-4-1: SCS Support the recommended WF. Issue 2-4-2: Channel bandwidth In addition to FDD 15kHz 10MHz and TDD 30kHz 40MHz, a larger CBW should be considered for FDD and TDD respectively. The difference between option 1 and option 2 is to include 40MHz or 50MHz for FDD 15kHz, and we still prefer 40MHz since 50MHz may not be implemented in many bands/UEs. So, we still support option 2. Issue 2-4-3: TDD DL/UL configuration for 30kHz SCS We will not insist on option 2, and option 1 can be fine for us. Issue 2-4-4: Number of carriers Fine with option 1. Issue 2-4-5: PDCCH and PDSCH allocation Support option 1. Issue 2-4-6: Tx antenna number Either option 1 or 2 is ok. Issue 2-4-7: Propagation condition Support option 1. MMSE-IRC performance should be checked in different typical scenarios, which is an important aspect for performance testing. Issue 2-4-8: Antenna correlation Option 1. Issue 2-4-9: PDSCH mapping type Option 1. Issue 2-4-10: PRB allocation Option 1. Issue 2-4-11: SSB configuration for serving and interfering cells Option 1. Issue 2-4-12: Physical cell ID Option 1. Issue 2-4-13: TRS/CSI-RS among cells Ok with option 1 or use option 2 as starting point. Issue 2-4-14: DMRS and TRS/CSI-RS among cells Sub-topic 2-5: CQI reporting requirements Issue 2-5-1: Whether to define CQI reporting requirements Support option 1. It is important to define CQI reporting requirements, otherwise the gain by MMSE-IRC will be only on BLER reduction but not MCS/throughput increase. Issue 2-5-2: Interference covariance estimation granularity for CQI reporting Support option 1 as a general consideration for further discussion. Issue 2-5-3: Interference model for CQI reporting Support option 1, to reuse the LTE CQI reporting test setup. Issue 2-5-4: Test metric for CQI reporting Support option 1 Sub-topic 2-6: Scenario 2 with non-slot-based transmission Issue 2-6-1: Test parameters Probably can be discussed later. Sub-topic 2-7: Release independence Issue 2-7-1: Release independence Support to release independent from Rel-15, and option 1 is ok to us. |
| Huawei, HiSilicon | Sub-topic 2-1: Interference model Issue 2-1-1: Sync and async network for FR1 We prefer to only consider the sync scenario for both of FDD and TDD. Issue 2-1-3: Transmission rank of interfering PDSCH Option 1 is fine for us. We prefer to reuse LTE MMSE-IRC assumptions as the starting point. Issue 2-1-5: Modulation order of interfering PDSCH Option 1 is fine for us. Sub-topic 2-2: DMRS configuration and reference receiver Issue 2-2-1: DMRS configuration Option 2 is fine for us. In addition, number of CDM group should be defined to 1 for both of serving cell and interference cells. Sub-topic 2-3: Target PDSCH parameters Issue 2-3-1: Transmission rank The recommended WF is fine for us. Issue 2-3-3: Precoding model Option 2 is fine for us. Issue 2-3-5: Performance measurement point The recommended WF is fine for us. As SINR is also used for LTE MMSE-IRC test case. Issue 2-3-6: HARQ process number The recommended WF is fine for us. Sub-topic 2-4: Other parameters for target and interfering PDSCH Issue 2-4-1: SCS The recommended WF is fine for us. Issue 2-4-3: TDD DL/UL configuration for 30kHz SCS Prefer option 1 as it is commonly used. Issue 2-4-4: Number of carriers We agree with option 1 that only define test cases for single carrier scenario. Issue 2-4-5: PDCCH and PDSCH allocation The recommended WF is fine for us. Issue 2-4-6: Tx antenna number We prefer option 2. Issue 2-4-7: Propagation condition We support to only consider TDLA30-10 for cell-edge users. Issue 2-4-8: Antenna correlation The recommended WF is fine for us. Issue 2-4-9: PDSCH mapping type The recommended WF is fine for us. Issue 2-4-10: PRB allocation The recommended WF is fine for us. Issue 2-4-11: SSB configuration for serving and interfering cells Option 1+1A is fine for us. Issue 2-4-12: Physical cell ID Option 1 is fine for us. Sub-topic 2-5: CQI reporting requirements Issue 2-5-1: Whether to define CQI reporting requirements We prefer to further discuss this issue. Before making the decision, test scenario should be discussed clearly to make sure the CQI reporting requirements is meaningful. Sub-topic 2-6: Scenario 2 with non-slot-based transmission Issue 2-6-1: Test parameters We should focus on the scenario 1 firstly. Sub-topic 2-7: Release independence Issue 2-7-1: Release independence Option 1 is fine for us. |
| DOCOMO | Sub-topic 2-1: Interference model Issue 2-1-3: Transmission rank of interfering PDSCH Both Option 1 and Option 2 are acceptable for us. Issue 2-1-5: Modulation order of interfering PDSCH We support Option 1. Sub-topic 2-2: DMRS configuration and reference receiver Issue 2-2-1: DMRS configuration We support Option 2. Sub-topic 2-3: Target PDSCH parameters Issue 2-3-1: Transmission rank We support Option 1. Issue 2-3-5: Performance measurement point We support the recommended WF. Issue 2-3-6: HARQ process number We support the recommended WF. Sub-topic 2-4: Other parameters for target and interfering PDSCH Issue 2-4-1: SCS We support the recommended WF. Issue 2-4-2: Channel bandwidth Since “FDD 15kHz:10MHz” and “TDD 30kHz:40MHz” are included in all Options, Option 3 can be agreed first. Also, we are fine to consider the additional CBW, i.e., both Option 1 and Option 2 are acceptable for us. Issue 2-4-3: TDD DL/UL configuration for 30kHz SCS We support Option 1. Issue 2-4-4: Number of carriers We support Option 1. It is acceptable for us to assume only single carrier scenario in Rel-17. Issue 2-4-5: PDCCH and PDSCH allocation We support the recommended WF. Issue 2-4-6: Tx antenna number Both Option 1 and Option 2 are acceptable for us. Issue 2-4-7: Propagation condition We support Option 1. Issue 2-4-8: Antenna correlation We support Option 1. Issue 2-4-9: PDSCH mapping type We support Option 1. Issue 2-4-10: PRB allocation We support Option 1. Issue 2-4-11: SSB configuration for serving and interfering cells We support Option 1 + 1A. Issue 2-4-12: Physical cell ID We support Option 1. Sub-topic 2-6: Scenario 2 with non-slot-based transmission Issue 2-6-1: Test parameters We prefer to discuss this issue later. Sub-topic 2-7: Release independence Issue 2-7-1: Release independence We support Option 1. |
| Ericsson | Sub-topic 2-1: Interference model Issue 2-1-1: Sync and async network for FR1 We prefer Option 2. In NR, we think not only TDD but also FDD is also synchronized. We would like to understand how this sync/async assumption affects to the simulation parameters. Even if the TDD synchronized network, the signal reception timing could not be aligned between the serving cell and neighbouring cell(s) according to the assumption of UE location. Issue 2-1-2: Interference profile Option 1. From our understanding, the NW deployment in FR1 is basically similar as LTE. Thus, we suggest to reuse the DIP based interference profiles from LTE MMSE-IRC receiver as a start point. If we cannot obtain enough gains with MMRS-IRC receiver compared with AWNG-only condition, we may further evaluate other DIP models. To option 2, as SS/PBCH blocks only use 4 OFDM symbols with 240 subcarriers, we don’t think the interference from SSB to PDSCH is too much. Also RAN4 demodulation requirements dose not scheduled PDSCH in slot 0, where SSB is transmitted. Issue 2-1-3: Transmission rank of interfering PDSCH Option 1. It’s reasonable to reuse LTE IRC assumption as a start point, and we also don’t see too much difference with option 2. Issue 2-1-4: Precoding of interfering PDSCH We slight prefer Option 1B, to apply the random precoding per subband size instead of PRB bundling size because PRB bundling is configured per UE. But we are also fine with Option 1A if companies want to assume the minimum PRB size for precoding. For Precoder codebook, we prefer to use the single panel type I. Issue 2-1-5: Modulation order of interfering PDSCH Option 1 is fine with us. Sub-topic 2-2: DMRS configuration and reference receiver Issue 2-2-1: DMRS configuration Option 2 is fine with us. Issue 2-2-2: Interference covariance estimation granularity Option 1 is fine, but we think it is up to UE implementation. Sub-topic 2-3: Target PDSCH parameters Issue 2-3-1: Transmission rank Support the recommended WF. Issue 2-3-2: MCS Option 1. But we also tend to agree with Option 2. We should revisit MCS/Rank if UE cannot achieve 70% of maximum throughput with a given condition. Issue 2-3-3: Precoding model Option 2. We would like to consider the same method as other UE demodulation requirements. We are wondering if UE can report the correct PMI because CSI-RS is also collided by the neighbouring cells. Issue 2-3-4: PRB bundle size Option 1. Reuse the existing PDSCH demodulation requirement parameter. Issue 2-3-5: Performance measurement point Support the recommended WF. Issue 2-3-6: HARQ process number Support the recommended WF. Sub-topic 2-4: Other parameters for target and interfering PDSCH Issue 2-4-1: SCS Support the recommended WF. Issue 2-4-2: Channel bandwidth Option 3. We would like to keep the Rel-15/16 assumption. In our understanding, we set 10MHz for FDD SCS=15kHz and 40MHz for TDD SCS=30kHz, considering the supported bands in RF core requirements (TS38.101-1). For example, we are wondering how many FDD bands support CBW of 50MHz. Issue 2-4-3: TDD DL/UL configuration for 30kHz SCS Option 1. Configuration 2 in Option 2 is also fine in addition to Configuration 1, depending on the number of test cases. Issue 2-4-4: Number of carriers Option 1. Issue 2-4-5: PDCCH and PDSCH allocation Support the recommended WF. Issue 2-4-6: Tx antenna number Option 2. We prefer to keep the same number of Tx antennas for both serving cell and interfering cell(s). Issue 2-4-7: Propagation condition Option 1. Depending on the number of test cases, we configure TDLA30-10 for some test cases and TDLC300-100 for other test cases. Issue 2-4-8: Antenna correlation Support the recommended WF. Issue 2-4-9: PDSCH mapping type Support the recommended WF. Issue 2-4-10: PRB allocation Support the recommended WF. Issue 2-4-11: SSB configuration for serving and interfering cells We support Option 1A. It is aligned with the practical deployment. Issue 2-4-12: Physical cell ID Option 1 could be fine. Issue 2-4-13: TRS/CSI-RS among cells Option 2. It’s important to align CSI-RS configuration for serving and interference cells which also align with practical deployment. Also this assumption can minimize the CSI-RS/TRS interference from neighbouring cell(s) to PDCSH in the serving cell. Issue 2-4-14: DMRS and TRS/CSI-RS among cells As we mentioned before, from real NW deployment, NW will align the DMRS and TRS/CSI-RS configuration among cells. Thus, we don’t need to consider the collision between DMRS and TRS/CSI-RS. Sub-topic 2-5: CQI reporting requirements Issue 2-5-1: Whether to define CQI reporting requirements Option 1. As we mentioned before, from real NW deployment, NW will align TRS/CSI-RS configuration among cells. It is therefore beneficial to define CQI reporting requirements under the neighbouring cell interference environment. We think LTE CQI tests with neighbouring interference are good starting point. Issue 2-5-2: Interference covariance estimation granularity for CQI reporting Option 1 seems fine, but we think it is up to UE implementation. Issue 2-5-3: Interference model for CQI reporting Fine with Option 1 as a starting point. Issue 2-5-4: Test metric for CQI reporting Support Option 1. Sub-topic 2-6: Scenario 2 with non-slot-based transmission Issue 2-6-1: Test parameters We should follow the WID. RAN4 should discuss scenario 2 (non-slot-based transmission) after we conclude the scenario/feasibility of scenario 1 (slot-based transmission). Sub-topic 2-7: Release independence Issue 2-7-1: Release independence Option 1. |
| Qualcomm | Sub-topic 2-1: Interference model Issue 2-1-1: Sync and async network for FR1 Prefer Option 2. MMSE-IRC receiver based on DMRS based interference covariance won’t have different implementation for sync and async scenarios since it will just look at the interference seen by its DMRS. Issue 2-1-2: Interference profile Prefer to keep DIP levels open and decide based on simulations. Also, prefer to keep only 1 interfering cell. Question to proponents of 2 interfering cells: What is the motivation for introducing the requirements with 2 interfering cells? What additional demod algorithm testing are we doing for 2 interfering cells that we can’t test with just 1 interfering cell? Issue 2-1-3: Transmission rank of interfering PDSCH Prefer to keep this open. Issue 2-1-4: Precoding of interfering PDSCH Prefer Option 1A, similar to existing test cases. Issue 2-1-5: Modulation order of interfering PDSCH Prefer to keep it open in this meeting. For initial simulation assumptions, ok to assume 16QAM but other options not precluded. Sub-topic 2-2: DMRS configuration and reference receiver Issue 2-2-1: DMRS configuration In our opinion, these options are not mutually exclusive and we can combine all three options and focus on scenarios where interference can be rejected with DMRS based covariance estimation. Issue 2-2-2: Interference covariance estimation granularity This is up to UE implementation, so RAN4 should not enforce any particular granularity. For simulation assumptions, RAN4 can assume granularity of PRB bundling size similar to other test cases. Sub-topic 2-3: Target PDSCH parameters Issue 2-3-1: Transmission rank Ok with recommended WF. Issue 2-3-2: MCS Prefer QPSK. Ok with QPSK and 16QAM as two separate options and decide later based on simulation results. Issue 2-3-3: Precoding model Prefer random precoding. It is not a good idea to mix fixed MCS simulations with CSI reporting. Otherwise, it will be hard to focus the test on only demod performance in presence of interferer. Issue 2-3-4: PRB bundle size Ok with Option 1. Issue 2-3-5: Performance measurement point Ok with recommended WF. Issue 2-3-6: HARQ process number Ok with recommended WF. Sub-topic 2-4: Other parameters for target and interfering PDSCH Issue 2-4-1: SCS Ok with Option 1. Assumption is same duplex/SCS for serving and interfering cell? Issue 2-4-2: Channel bandwidth Prefer Option 3. From UE demod perspective, testing for multiple CBWs does not bring any new insights. Issue 2-4-3: TDD DL/UL configuration for 30kHz SCS Prefer Option 1. Issue 2-4-4: Number of carriers Ok with Option 1. Issue 2-4-5: PDCCH and PDSCH allocation Ok with recommended WF. Issue 2-4-6: Tx antenna number Prefer Option 2. In LTE, both 2Tx and 4Tx interfering cell scenarios were considered because it changed number of CRS symbols/ports. In case of NR, that is not the case. So, there is no need to test for both configurations. Issue 2-4-7: Propagation condition Prefer Option 2. If PRB bundling granularity is 2 PRBs, it doesn’t matter whether channel is frequency selective or not, because UE will do channel estimation based on PRB Bundling size. Issue 2-4-8: Antenna correlation Ok with Option 1. Issue 2-4-9: PDSCH mapping type Ok with Option 1. Issue 2-4-10: PRB allocation Ok with Option 1. Issue 2-4-11: SSB configuration for serving and interfering cells This should be discussed together with Issue 2-4-13. It doesn’t make sense to have both SSB and TRS colliding because UE will not have any good quality RS to run time/frequency tracking. It can’t be assumed for UE to do interference cancellation on these RS as baseline. Our preference is that at least TRS should not see any interference. If it’s not possible to protect TRS, we would at least want SSB to not see any interference. Issue 2-4-12: Physical cell ID Question to China Telecom: How does physical cell ID matters for this scenario? In LTE, CRS vshift was related to cell ID but that is not the case in NR. Am I missing something here? Issue 2-4-13: TRS/CSI-RS among cells Prefer TRS to not see any interference. See comment for Issue 2-4-11. Issue 2-4-14: DMRS and TRS/CSI-RS among cells Prefer TRS from serving cell to not collide with interference. Serving cell DMRS can collide with DMRS or PDSCH as long as TRP is same for both interfering DMRS and PDSCH. If that’s not possible, then serving cell DMRS should collied with interfering cell PDSCH. Sub-topic 2-5: CQI reporting requirements Issue 2-5-1: Whether to define CQI reporting requirements Agree with MediaTek’s observation. Also, CSI-IM of serving cell should also see PDSCH interference from interfering cell. Issue 2-5-2: Interference covariance estimation granularity for CQI reporting Same comment as Issue 2-2-2. This is up to UE implementation, so RAN4 should not enforce any particular granularity. For simulation assumptions, RAN4 can assume granularity of PRB bundling size similar to other test cases. Issue 2-5-3: Interference model for CQI reporting For static condition, test metric (2) in Issue 2-5-4 may not work. In LTE, these requirements were defined for fading conditions where this may make more sense. Issue 2-5-4: Test metric for CQI reporting Further discuss whether this test metric makes sense for static channel condition. Sub-topic 2-6: Scenario 2 with non-slot-based transmission Issue 2-6-1: Test parameters As per WID, this scenario is second priority. So, we prefer to discuss this issue after Scenario 1 is stable. Sub-topic 2-7: Release independence Issue 2-7-1: Release independence Prefer to keep this open. |
| Intel | Issue 2-1-1: Sync and async network for FR1 We fine to consider sync and async cases for FDD. Same time, we suggest to prioritize sync case. Rx processing will be the same for both cases. Therefore, there is no big benefits from introduction of requirements for both cases. Issue 2-1-2: Interference profile LTE interference profile was analysed in different release. The latest analysis was done for NACS and CRS-IM WIs. Therefore, we think that it is better to rely on analysis from these WIs. Based on our understanding, it also was discussed whether to use DIP or INR methodology for NAICS analysis. It was agreed to use INR methodology. Based on our understanding, INR methodology is more straightforward in comparison to DIP and provide clear information on signal power difference between serving and interference signals. Therefore, INR methodology and values from NAICS are more preferable for us. Same time, we think that it is rather important to define requirements which reflect practical NR interference conditions and show MMSE-IRC performance under such conditions. Therefore, we suggest to make system level analysis to find proper interference modelling. Issue 2-1-3: Transmission rank of interfering PDSCH Both options are fine for us as starting point. We don’t expect big performance difference. We can also use system level analysis to find typical rank distribution for NR system. Issue 2-1-4: Precoding of interfering PDSCH Option 1A is fine for us. Issue 2-2-1: DMRS configuration Based on our understanding, all options are rather same in case of Type A full slot PDSCH mapping Issue 2-2-2: Interference covariance estimation granularity Based on our understanding, at least we can assume per slot time domain granularity. As for frequency domain granularity, we probably can keep it up to implementation. Issue 2-3-1: Transmission rank Support recommended WF Issue 2-3-2: MCS There is a typo in our proposal. There should be MCS 4 instead of MCS 5. Same time, option 1A is also fine for us. Issue 2-3-3: Precoding model Support Option 2. We think that random precoding can be used. (i.e. similar to Rel-15 and Rel-16 PDSCH requirements) Issue 2-3-4: PRB bundle size Option 1 is fine for us Issue 2-3-5: Performance measurement point We suggest to SNR at 70% TP (i.e. similar to NAICS). Such methodology allows to consider existing SNR definition in Section 4.4.2 without any modifications. Issue 2-3-6: HARQ process number Option 1 is fine for us Issue 2-4-2: Channel bandwidth Based on our understanding, testing of single CBW per SCS is sufficient from coverage point of view. Therefore, we suggest to go with Option 3 as more typical configuration for Rel-15 and Rel-16 PDSCH requirements. Issue 2-4-3: TDD DL/UL configuration for 30kHz SCS Support Option 1. Testing of single UL/DL configuration is from coverage point of view. Issue 2-4-4: Number of carriers Support Option 1. Issue 2-4-6: Tx antenna number Analysis for scenarios with 2 Tx case should be sufficient. Issue 2-4-7: Propagation condition We are fine to consider several conditions at the initial stage (i.e. Option 1). Same time, requirements should be defined for one channel model. Issue 2-4-9: PDSCH mapping type Support Option 1 Issue 2-4-10: PRB allocation Support Option 1 Issue 2-4-11: SSB configuration for serving and interfering cells Support Option 1A Issue 2-4-12: Physical cell ID Support Option 1 Issue 2-4-13: TRS/CSI-RS among cells We can check PDSCH performance for scenarios with colliding and non-colliding TRS configuration. Issue 2-5-1: Whether to define CQI reporting requirements We support definition of CQI requirements because it allows to verify that UE makes IRC processing for both blocks (demodulation and CSI). Issue 2-5-2: Interference covariance estimation granularity for CQI reporting We think that more analysis is needed to understand the impact of different covariance matrix estimation granularities on reporting statistics and requirements, Issue 2-5-3: Interference model for CQI reporting We suggest to align with assumptions for demodulation requirements. If INR methodology will be used for demod then we suggest to use same methodology for CQI. Issue 2-5-4: Test metric for CQI reporting We are fine to check LTE metric as starting point. Issue 2-6-1: Test parameters We understand that definition of requirements for Scenario 1 is prioritized for this meeting. Our intention is to collect early comments from interested companies. Issue 2-7-1: Release independence Option 1 is fine for us. We can comeback to this question later. |
| Apple | Sub-topic 2-1: Interference model Issue 2-1-1: Sync and async network for FR1 Option 2. Sync assumptions for FDD and TDD. Issue 2-1-2: Interference profile The interference profile will have to be decided based on simulations, we support keeping both DIP and NAICS based modelling open (Option1,3). Also, propose to introduce 1 interferer cell as a starting point. Issue 2-1-3: Transmission rank of interfering PDSCH Support option 2. Issue 2-1-4: Precoding of interfering PDSCH Support option 1A. Issue 2-1-5: Modulation order of interfering PDSCH We can agree to option 1 as baseline. Sub-topic 2-2: DMRS configuration and reference receiver Issue 2-2-1: DMRS configuration We prefer to have non-overlapping DMRS between target and interferer. If companies have strong preference for option 2, we propose to study both cases where DMRS is seeing PDSCH vs DMRS from interferer. Option 1 should be the guiding principle while configuring DMRS for target cell and interferer. Issue 2-2-2: Interference covariance estimation granularity We are not sure if we should discuss this as a configuration parameter. Setting the precoding granularity of interferer is sufficient. Sub-topic 2-3: Target PDSCH parameters Issue 2-3-1: Transmission rank We support the recommended WF. Issue 2-3-2: MCS We support option 1. QPSK and 16QAM are more practical for interference limited scenarios. OMCS 4, 13 can be re-used as other demod requirements. Issue 2-3-3: Precoding model We support option 2- random precoder. Since Rel-15 we have tried to separate demod and CSI reporting requirements and we should aim to maintain it. Issue 2-3-4: PRB bundle size Option 1 is fine. Issue 2-3-5: Performance measurement point SINR @ 70% Max TP. Issue 2-3-6: HARQ process number We support the recommended WF. Sub-topic 2-4: Other parameters for target and interfering PDSCH Issue 2-4-1: SCS We support the recommended WF. Issue 2-4-2: Channel bandwidth We support option 3. We don’t see the necessity to include additional CBW. Issue 2-4-3: TDD DL/UL configuration for 30kHz SCS We support option 1. Issue 2-4-4: Number of carriers We support option 1. Issue 2-4-5: PDCCH and PDSCH allocation We support the recommended WF. Issue 2-4-6: Tx antenna number We support option 2. Issue 2-4-7: Propagation condition We support to have option 2 as the baseline. Issue 2-4-8: Antenna correlation We support the recommended WF. Issue 2-4-9: PDSCH mapping type We support the recommended WF. Issue 2-4-10: PRB allocation We support the recommended WF. Issue 2-4-11: SSB configuration for serving and interfering cells We don’t see a reason to aligned SSB configuration between target and interferer. Issue 2-4-12: Physical cell ID Does PCI matter for demod requirements, since we don’t test SSB detection/initial access? Issue 2-4-13: TRS/CSI-RS among cells We don’t see why TRS/CSI-RS should collide. There is so much flexibility in scheduling these and seems very pessimistic to assume they collide between target and interferer. Issue 2-4-14: DMRS and TRS/CSI-RS among cells Same view as issue 2-4-13 and 2-2-1. Sub-topic 2-5: CQI reporting requirements Issue 2-5-1: Whether to define CQI reporting requirements We don’t see strong motivation to introduce CQI reporting in the presence of interferer. We should avoid introducing requirements just because we also had them in LTE. We don’t think it provides any additional coverage. We would like to understand the motivation and added coverage with this. Issue 2-5-2: Interference covariance estimation granularity for CQI reporting This should be left to UE implementation in our understanding and not be a config / sim parameter. Issue 2-5-3: Interference model for CQI reporting Issue 2-5-4: Test metric for CQI reporting Sub-topic 2-6: Scenario 2 with non-slot-based transmission Issue 2-6-1: Test parameters We think this is a bit early to discuss this as the first priority is Scenario 1. Sub-topic 2-7: Release independence Issue 2-7-1: Release independence We think this is very early to discuss this. |
| MediaTek | Sub-topic 2-1: Interference model Issue 2-1-1: Sync and async network for FR1 Prefer Option 2. Issue 2-1-3: Transmission rank of interfering PDSCH Option 1 is fine for us. Issue 2-1-5: Modulation order of interfering PDSCH Option 1 is fine for us. Sub-topic 2-2: DMRS configuration and reference receiver Issue 2-2-1: DMRS configuration Option 2 is fine for us. Issue 2-2-2: Interference covariance estimation granularity We think the estimation granularity is up to UE implementation. Sub-topic 2-3: Target PDSCH parameters Issue 2-3-1: Transmission rank Support the recommended WF. Issue 2-3-3: Precoding model Prefer Option 2. Issue 2-3-5: Performance measurement point Support the recommended WF. Issue 2-3-6: HARQ process number Support the recommended WF. Sub-topic 2-4: Other parameters for target and interfering PDSCH Issue 2-4-1: SCS Support the recommended WF. Issue 2-4-2: Channel bandwidth Prefer Option 3. Issue 2-4-3: TDD DL/UL configuration for 30kHz SCS Support Option 1. Issue 2-4-4: Number of carriers Support Option 1. Issue 2-4-5: PDCCH and PDSCH allocation Support the recommended WF. Issue 2-4-6: Tx antenna number Prefer Option 2. Issue 2-4-8: Antenna correlation Support the recommended WF. Issue 2-4-9: PDSCH mapping type Support the recommended WF. Issue 2-4-10: PRB allocation Support the recommended WF. Sub-topic 2-5: CQI reporting requirements Issue 2-5-1: Whether to define CQI reporting requirements We prefer Option 2. Sub-topic 2-7: Release independence Issue 2-7-1: Release independence Option 1 is fine for us. |
| CMCC | Sub-topic 2-1: Interference model Issue 2-1-1: Sync and async network for FR1 Option1. Issue 2-1-2: Interference profile We prefer Option1+Option2. The DIP values for Option2 need further discussion; To Ericsson: In our contribution, we give an observation about the micro will be infected severely when macro transmit SSBs. It does not mean that there is interference only in this case. The reason for this observation is the workload of marco is light for now, so the interference is generated when marco transmit mandatory SSBs. In the future, if the workload of macro is heavy, the interference will be obvious in all slots. In a word, the interference in this “micro-macro” scenario is common and have limited relations with SSBs position. Issue 2-1-3: Transmission rank of interfering PDSCH Option1 can be the starting point. Issue 2-1-4: Precoding of interfering PDSCH Option1A is OK for us. Issue 2-1-5: Modulation order of interfering PDSCH We support Option1 Sub-topic 2-2: DMRS configuration and reference receiver Issue 2-2-1: DMRS configuration These options are not conflict with each other; We are OK to go with Option2. Issue 2-2-2: Interference covariance estimation granularity We share similar views with QC, it is up to UE implementation. Sub-topic 2-3: Target PDSCH parameters Issue 2-3-1: Transmission rank Support the recommended WF. Issue 2-3-2: MCS We agree with Option2 and take Option1 as the baseline for initial simulation. Issue 2-3-3: Precoding model We prefer Option2. Issue 2-3-4: PRB bundle size OK with Option1. Issue 2-3-5: Performance measurement point Support the recommended WF Issue 2-3-6: HARQ process number Support the recommended WF Sub-topic 2-4: Other parameters for target and interfering PDSCH Issue 2-4-1: SCS Support the recommended WF Issue 2-4-2: Channel bandwidth We prefer Option1 and we can give our compromise to Option2 considering of companies concerns about the application scenario of 50MHz, we think it is necessary to test the typical bandwidth configuration and a larger bandwidth configuration from test coverage perspective. Issue 2-4-3: TDD DL/UL configuration for 30kHz SCS We are both OK with Option1 and Option2. Issue 2-4-4: Number of carriers Ok with Opiton1 Issue 2-4-5: PDCCH and PDSCH allocation Support the recommended WF. Issue 2-4-6: Tx antenna number We support Option1, Option2 is also acceptable for us. Issue 2-4-7: Propagation condition Support Option1, TDLA30-10 can be prioritized over TDLC300-100 for simulation Issue 2-4-8: Antenna correlation We can use Option1 as baseline Issue 2-4-9: PDSCH mapping type Option1 Issue 2-4-10: PRB allocation Option1 Issue 2-4-11: SSB configuration for serving and interfering cells We support Option1+Option1A Issue 2-4-12: Physical cell ID Option1 is fine for us. Issue 2-4-13: TRS/CSI-RS among cells Option1 to have more discuss or further down-selection based on simulation results. Sub-topic 2-5: CQI reporting requirements Issue 2-5-1: Whether to define CQI reporting requirements We support Option1. Issue 2-5-2: Interference covariance estimation granularity for CQI reporting We think it is up to UE implementation, further discussion may be needed. Issue 2-5-3: Interference model for CQI reporting We can start with Option1 and further discuss. Issue 2-5-4: Test metric for CQI reporting Take Option1 as the starting point if we agreed to define the CQI reporting requirements. Sub-topic 2-6: Scenario 2 with non-slot-based transmission Issue 2-6-1: Test parameters We support to discuss scenario 1 first, then come back to Scenario2 after Scenario 1 is stable. Sub-topic 2-7: Release independence Issue 2-7-1: Release independence Support Option1. |
| ZTE | Sub-topic 2-1: Interference model Issue 2-1-1: Sync and async network for FR1 Option 2, sync assumption should be prioritized. Issue 2-1-2: Interference profile Option 1. Issue 2-1-3: Transmission rank of interfering PDSCH Slightly Option 1 preferred. Issue 2-1-4: Precoding of interfering PDSCH Slightly Option 1B preferred. Issue 2-1-5: Modulation order of interfering PDSCH Fine with Option 1. Sub-topic 2-2: DMRS configuration and reference receiver Issue 2-2-1: DMRS configuration Option 2. Issue 2-2-2: Interference covariance estimation granularity Up to receiver implementation, not sure if we need to have it aligned. Sub-topic 2-3: Target PDSCH parameters Issue 2-3-1: Transmission rank Fine with Option 1. Issue 2-3-2: MCS If only one MCS is selected, Option 4 could be a good choice. If two MCS values selected, Option 1. Issue 2-3-3: Precoding model Option 1 slightly preferred. Issue 2-3-4: PRB bundle size Fine with Option 1. Issue 2-3-5: Performance measurement point Ok with WF. Issue 2-3-6: HARQ process number Ok with WF. Sub-topic 2-4: Other parameters for target and interfering PDSCH Issue 2-4-1: SCS Ok with WF. Issue 2-4-2: Channel bandwidth Start with Option 3, and discuss other options once it is completed. Issue 2-4-3: TDD DL/UL configuration for 30kHz SCS Option 1 start with one configuration. Issue 2-4-4: Number of carriers Fine with Option 1. Issue 2-4-5: PDCCH and PDSCH allocation Fine with Option 1. Issue 2-4-6: Tx antenna number Option 2. Issue 2-4-7: Propagation condition Option 2. Issue 2-4-8: Antenna correlation Fine with Option 1. Issue 2-4-9: PDSCH mapping type Fine with Option 1. Issue 2-4-10: PRB allocation Fine with Option 1. Issue 2-4-11: SSB configuration for serving and interfering cells Fine with Option 1. Issue 2-4-12: Physical cell ID Fine with Option 1. Issue 2-4-13: TRS/CSI-RS among cells Option 2 would make more sense. Issue 2-4-14: DMRS and TRS/CSI-RS among cells This issue is partly overlapped with Isse 2-4-13? Sub-topic 2-5: CQI reporting requirements Issue 2-5-1: Whether to define CQI reporting requirements Option 2. It is not sufficiently studied thus we could not conclude on whether or not to define CQI reporting requirements at this very first meeting. Issue 2-5-2: Interference covariance estimation granularity for CQI reporting Up to implementation. Not sure if this should be aligned. Issue 2-5-3: Interference model for CQI reporting Fine with Option 1 as a starting point. Issue 2-5-4: Test metric for CQI reporting Hold until the conclusion is made. Sub-topic 2-6: Scenario 2 with non-slot-based transmission Issue 2-6-1: Test parameters Ok to start with Option 1. Sub-topic 2-7: Release independence Issue 2-7-1: Release independence Fine with Option 1. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #2 | Issue 2-1-1: Sync and async network for FR1 - Proposals - Option 1: (CMCC, China Telecom, Intel) - FDD sync scenario - FDD async scenario - TDD sync scenario - CTC: For the simulation work, sync FDD/TDD can be prioritized over async FDD - Intel: Suggest to prioritize sync case. - Option 2: FDD and TDD sync scenario (E///, Huawei, QC, Apple, MTK, ZTE) - Recommendations for 2nd round: - Further discuss on the GTW session. Issue 2-1-2: Interference profile - Proposals - DIP-based or INR-based: - Option 1: Reuse the DIP based interference profiles from LTE MMSE-IRC receiver (China Telecom, Huawei, E///, CMCC, [MediaTek], [QC], ZTE) - Option 2: INR-based interference profiles from NAICS (Intel) - Intel: Suggest to make system level analysis to find proper interference modelling. - Option 3: Decide based on simulation results (Apple) - Whether to additionally consider HetNet scenario: - Option 1: Yes (CMCC, CTC) - CTC: The DIP values for HetNet need further discussion; one option could be to translate the INRs in NAICS scenario 2 to DIPs, in order to avoid additional system simulation - Option 2: Not to consider HetNet scenario (E///) - Recommendations for 2nd round: - Further discussion on the GTW session. ---------------GTW Discussion ---------------- Intel: DIP used for Rel-11 MMSE-IRC, and INR used for NAICS in Rel-12/Rel-13. INR reflect better definition for interference profile. It should rely on latest available analysis from NAICS. E///: We prefer with same as MMSE-IRC as starting point for simulation assumption. ZTE: The interference profile can be different compared to LTE in NR. At initial satge we reuse LTE as starting point. But other options not excluded. CMCC: We proposed to include HeNet scenario, the interference profile can be reused from NAICS. What’s the next step for interference profile decision ? Intel: Companies can bring initial simulations based on interference files from LTE; meanwhile companies can bring analysis for the interference profiles in NR. E///: Regarding timeline, we can think about more interference profiles especially for high and medium band and HeNet scenario. @CMCC, do you think more SSB configurations from neighbour cells? Apple: From simulation aspect, DIP or INR based may impact performance. What’s the reason for the selection among DIP vs INR? For evaluate profile, what’s the timeline? China Telecomm: We prefer to prioritize DIP based with the consideration of timeline. CMCC: For HetNet, Marco cell transmit less SSB compared to micro cell. Both SSB and data interference can be different compared homogenous scenario. Huawei:We prefer to use DIP as starting point for simulation and open to discuss the methodology and worry about the timelines. Intel: When define SNR and INR in test, we can clear discriminate the power difference among wanted signal and interference signal. ZTE: HeNet can be well control and coordinated, this will impact the interference profiles. E///: In the end, DIP and INR can be worked well, using DIP aligned with LTE MMSE-IRC with same receiver. For HeNet, high band can be different i.e. 3.5GHz , system study maybe be needed. Agreement: Using DIP approach for interference profile definition for intial simulation purpose -Interference profiles from LTE MMSE-IRC can be used for initial simulation -FFS whether HetNet scenario need to be considerd - The interference profile from LTE NAICS can be used for initial simulation purpose under this scenario -Other interference profiles not precluded with the consideration of NR deployment scenario pending on further discussion -Companies are encourage to bring evaluation for the anlysis of performance gain with MMSE-IRC receiver over MMSE receiver under the same inter-cell interference Issue 2-1-3: Transmission rank of interfering PDSCH - Proposals - Option 1: Reuse LTE MMSE-IRC assumption for DMRS-based transmission mode, i.e., random rank with 70% and 30% probability for rank 1 and rank 2 transmission in the interfering cell(s), as the starting point (CMCC, China Telecom, Huawei, DCM, E///, Intel, MTK, ZTE) - Option 2: Reuse LTE NAICS assumption, i.e., random rank with 80% and 20% probability for rank 1 and rank 2 transmission in the interfering cell(s) (Intel, DCM, Apple) - QC: Prefer to keep open. - Recommendations for 2nd round: - Negligible performance difference between the 2 options can be expected. - Can we use option 1 for initial simulation assumption based on majority’s view? Issue 2-1-4: Precoding of interfering PDSCH - Proposals - Option 1: Random precoding (China Telecom, Ericsson) - Option 1A: Random precoding per slot and per PRB bundling granularity, with PRB bundling size of 2 (China Telecom, Ericsson, QC, Intel, Apple, CMCC) - Option 1B: Random precoding per slot and per [smallest] subband as defined in subclause 5.2.1.4 of TS38.214; [single panel type I] codebook (Ericsson, ZTE slightly preferred) - Recommendations for 2nd round: - Random precoding with single panel type I codebook per slot and per PRB bundling granularity, with PRB bundling size of 2. Issue 2-1-5: Modulation order of interfering PDSCH - Proposals - Option 1: Reuse LTE assumption for MMSE-IRC receiver, i.e., 16QAM randomly modulated symbols (China Telecom, E///, Intel, Huawei, DCM, Apple, MTK, CMCC, ZTE) - QC: Prefer to keep open, ok to assume 16QAM but other options not precluded. - Recommendations for 2nd round: - Use option 1 for initial simulation assumption and other options not precluded. Issue 2-2-1: DMRS configuration - Proposals - Option 1: limit scenarios to cases where interference can be rejected with DMRS based covariance estimation. (Apple) - Option 2: for both serving and interfering cells, DMRS Type 1 with single symbol front loaded and 1 additional DMRS, with FDM applied between DMRS and data, i.e., overlapped DMRS between target and interferer (China Telecom, Intel, Huawei, DCM, E///, MTK. CMCC, ZTE) - Huawei: Number of CDM group should be defined to 1 - Option 3: DMRS Type 1 with single symbol front loaded and 1 additional DMRS (E///) - Option 4: Non-overlapping DMRS configuration between target and interferer (Apple) - Recommendations for 2nd round: - More discussion on the GTW session. ---------------GTW discussion-------------- Option 2: DMRS conflicted with DMRS REs from interference cells Apple: We would like to keep both options open. Huawei: Agree with option 2 under the same assumption of CDM group is 1 for all cells. Intel: Fine with option 2. As well as DMRS suffer same interference levels compared to data Res, no matter DMRS conflicted with DMRS or non-conflcited. Better to consder CDM group 1 for all cells Qualcomm: Similar comments as Huawei and Intel. Fine with option 2. ZTE: Support option 2. For option 4, no confliction for DMRS symbols? Apple:Option 1 we proposed aligned with Huawei, same CDM group should be used for all cells.For option , we mean DMRS conflcited with data interference other than DMRS. We are ok to accept option 2 with additional condition as Huawei proposed. China Telecomm: Agree with Huawei, use same CDM group for all cells. E///: We also support option 2. It’s hard to avoid the confliction of DMRS in NW deployment. Agreement: Option 2 with same CDM group configuration equals to 1 for all cells Issue 2-2-2: Interference covariance estimation granularity - Proposals - Option 1: The estimation of interference covariance matrix can be performed at per PRB bundle size and per slot basis (China Telecom, Intel, E///) - Option 2: Up to UE implementation (QC, E///, MTK, CMCC, ZTE) - Recommendations for 2nd round: - More discussion on the GTW session -----GTW discussion--------- Intel: We prefer to open as PRB bundle size vs per PRB. QC: prefer per PRN bindle. ZTE: We think it’s up to implementation. China Telecomm: Without baseline assumption, hard to align the simulation results. E///: Similar view as ZTE. Apple: Share similar view left to UE implementation. Precoding granaruity in target cell. QC: Either per PRB bundle size as baseline, or no baseline. Huawei:no baseline for simulation. To check whether alignment reached or not. Intel: We can open and decide later. ZTE: Provide for information purpose. China Telecomm: We still prefer baseline and accept to further check. E///: Alignment results ? Agreement: Interference covariance estimation granularity up to UE implementation Further check the results and if needed per PRB bundle size or per PRB and per slot basis can be considered as possible options for simulation Issue 2-3-1: Transmission rank - Tentative agreements - Use rank 1 as baseline (CMCC, China Telecom, Intel, E///, Huawei, DCM, QC, Apple, MTK, ZTE) Issue 2-3-2: MCS - Proposals - Option 1: QPSK and 16QAM (Apple, E///, CTC, CMCC, ZTE) - Option 1A: MCS 4 (QPSK, CR=0.3), MCS 13 (16QAM, CR=0.5) (E///, Intel, Apple, CTC) - Option 1B: MCS 5 and 13, the MCSs can be adjusted based on more simulation results (CTC) - Option 2: choose MCS for which the resulting geometry value falls in the range ~[-2.5dB, 0dB] with typical margin considered (China Telecom, E///, CMCC) - Option 3: MCS 5 4 or 13 or 19 (Intel) - Option 4: 16QAM (HW, ZTE) - Option 5: QPSK or 16QAM (QC) - Recommendations for 2nd round: - Further discuss on the GTW session. Issue 2-3-3: Precoding model - Proposals - Option 1: Further discuss whether to use random precoding or follow PMI (China Telecom, ZTE) - Option 2: Single Panel Type I, Random precoder selection (E///, [CTC], Huawei, QC, Intel, Apple, MTK, CMCC) - Recommendations for 2nd round: - Use Single Panel Type I and Random precoder selection for initial simulation, and follow PMI is not precluded. Issue 2-3-4: PRB bundle size - Tentative agreements - Set PRB bundle size as 2 for target PDSCH (E///, CTC, QC, Intel, Apple, CMCC, ZTE) Issue 2-3-5: Performance measurement point - Proposals - Option 1: 70% relative throughput (CMCC, China Telecom, DCM, E///, QC, MTK, ZTE) - Option 1A: SINR (not SNR) at 70% TP (E///, CTC, Huawei, DCM, QC, MTK, CMCC, ZTE) - Option 1B: SNR at 70% TP (Intel) - Intel: similar to NAICS - Recommendations for 2nd round: - Can we use option1 + option 1A based on majority’s view? Issue 2-3-6: HARQ process number - Tentative agreements - 4 for FDD 15kHz SCS and 8 for TDD 30kHz SCS as baseline (CTC, Huawei, DCM, E//, QC, Intel, Apple, MTK, CMCC, ZTE) Issue 2-4-1: SCS - Tentative agreements - FDD 15kHz, TDD 30kHz (Same SCS is used for the target and the interference cells) (CMCC, China Telecom, DCM, Intel, Huawei, E///, QC, Apple, MTK, ZTE). Issue 2-4-2: Channel bandwidth - Proposals - Option 1 (CMCC, DCM) - FDD 15kHz: 10MHz, 50MHz - TDD 30kHz: 40MHz, 100MHz - Option 2 (China Telecom, DCM, CMCC) - FDD 15kHz: 10MHz, 40MHz - TDD 30kHz: 40MHz, 100MHz - Option 3 (DCM, Intel, Huawei, E///, QC, Apple, MTK, ZTE) - FDD 15kHz: 10MHz - TDD 30kHz: 40MHz - Recommendations for 2nd round: - Can we agree option 2 for initial simulation evaluation, and further check if down-selection on CBW is needed. Issue 2-4-3: TDD DL/UL configuration for 30kHz SCS - Tentative agreements - 7D1S2U(S=6D+4G+4U) (CMCC, DCM, Intel, CTC, Huawei, E///, QC, Apple, MTK, ZTE) Issue 2-4-4: Number of carriers - Tentative agreements - In Rel-17, MMSE-IRC receiver performance requirements with interference cell condition is defined only for single carrier scenario (CTC, Huawei, DCM, E///, QC, Intel, Apple, MTK. CMCC, ZTE) Issue 2-4-5: PDCCH and PDSCH allocation - Tentative agreements - For serving and interfering cells (China Telecom, Intel, DCM, Huawei, E///, QC, Apple, MTK, CMCC, ZTE) - Use symbols #0 and #1 of each slot for PDCCH - PDSCH Start symbol 2, Duration 12 Issue 2-4-6: Tx antenna number - Tentative agreements - 2Tx as baseline for serving cell and interfering cells (China Telecom, DCM, Intel, E///, Huawei, QC, Apple, MTK, CMCC, ZTE) Issue 2-4-7: Propagation condition - Proposals - Option 1: Include both TDLA30-10 and TDLC300-100 at this early phase (China Telecom, E///, DCM, [Intel], CMCC) - Option 2: TDLA30-10 (Intel, HW, QC, Apple, ZTE) - Recommendations for 2nd round: - For initial simulation evaluation, can we take both TDLA30-10 and TDLC300-100 and further check if down-selection is needed based on simulation results? Issue 2-4-8: Antenna correlation - Tentative agreements - ULA low as baseline (China Telecom, Intel, Huawei, E///, DCM, QC, Apple, MTK, CMCC, ZTE) Issue 2-4-9: PDSCH mapping type - Tentative agreements - PDSCH mapping type A for serving and interfering PDSCH (China Telecom, DCM, Huawei, E///, QC, Intel, Apple, MTK, CMCC, ZTE) Issue 2-4-10: PRB allocation - Tentative agreements - Full PRB allocation for serving and interfering PDSCH (China Telecom, Huawei, DCM, E///, QC, Intel, Apple, MTK, CMCC, ZTE) Issue 2-4-11: SSB, TRS/CSI_RS configuration for serving and interfering cells - Proposals - Option 1: Aligned SSB configuration for serving and interference cells (CMCC, China Telecom, E///, Huawei, DCM, E///, Intel, ZTE) - Option 1A: (CMCC, China Telecom, Huawei, DCM, Intel) - SSB position in burst: first SSB in Slot#0; SSB periodicity: 20ms. - The slot #0 in every 20 ms is not scheduled for PDSCH transmission - Option 2: Not overlapped SSB configuration for serving and interference cells if it’s not possible to protect TRS (QC) - Apple: We don’t see a reason to aligned SSB configuration between target and interferer. - Recommendations for 2nd round: - More discussion on the GTW session. -------------------GTW Discussion------------------ QC: We didn’t assume interference rejection on SSB, TRS/CSI-RS; we proposed to either SSB or TRS/CSI-RS should be interference free. Intel: TRS collide with data or TRS, not sure what’s the difference. At least we can check the performance difference with different configurations with interference free vs data interference vs TRS/CSI-RS interference. For SSB, we can consider non colliding case, no strong preference. China Telecomm: Share similar view as Intel, not sure the performance impact. Since SSB, TRS can suffer other data interference. E///: For TRS/CSI-RS colliding, we should consider this case. It’s common to configure the same locations over cells under NW. For SSB configuration, we are open for demod since no PDSCH scheduled for SSB slots. Apple: For TRS/CSI-RS, we need to further discuss for different cases interference free vs data interference vs TRS/CSI-RS. For SSB, we can avoid the conliding cases. QC: Can we agree non overlapping for SSB and for TRS/CSI-RS for further evaluation. CMCC: For Homogenous scenario, SSB colliding maybe happened. What’s the performance difference? E///: For SSB transmitted in slot 0, with Homogenous scenario, and for HeNet we can further discuss. Agreements: Configure SSB with different locations and no PDSCH scheduled in SSB slots considering limited number of SSB transmitted -FFS for HeNet scenario if introduced Further evaluate the performance difference among following options for TRS/CSI-RS -TRS/CSI-RS colliding with TRS/CSI-RS interference -TRS/CSI-RS colliding with data interference -TRS/CSI-RS with interference free Issue 2-4-12: Physical cell ID - Proposals - Option 1: Physical cell ID of 0 for the serving cell, and cell ID 1 and 2 for the two interfering cells (China Telecom, Huawei, DCM, E///, Intel, CMCC, ZTE) - QC, Apple: Does PCI matter for demod requirements? - Recommendations for 2nd round: - Use option 1 as a typical configuration if no performance impact can be assured. Issue 2-4-13: TRS/CSI-RS among cells - Proposals - Option 1: Further discuss whether TRS/CSI-RS are collided among cells (China Telecom, CMCC) - Option 2: Neighboring cell(s) schedule CSI reference signals (for both tracking and CSI acquisition) at the same location as the serving cell (E///, ZTE) - E///: CSI-RS configuration for serving and interference cells are aligned with practical deployment. - Option 3: Not overlapped TRS configuration for serving and interference cells (QC) - Recommendations for 2nd round: - More discussion on the GTW session Issue 2-4-14: DMRS and TRS/CSI-RS among cells - Proposals - Option 1: Further discuss whether DMRS and TRS/CSI-RS are collided among cells (China Telecom) - Option 2: TRS from serving cell to not collide with DMRS (QC) - Recommendations for 2nd round: - More discussion on the GTW session Issue 2-5-1: Whether to define CQI reporting requirements - Proposals - Option 1: Yes (China Telecom, Intel, E///, CMCC) - Option 2: Further discuss (Huawei, MediaTek, QC, ZTE) - MediaTek, QC: Enhancement on CQI can be considered only if network can guarantee same interference is experienced by CSI-RS and PDSCH. - Recommendations for 2nd round: - Keep open to the next meeting. Issue 2-5-2: Interference covariance estimation granularity for CQI reporting - Proposals - Option 1: the interference covariance can be estimated and averaged among multiple PRBs, and the exact PRB number for interference covariance averaging needs further discussion (China Telecom, E///) - Option 2: Up to UE implementation (QC, Apple, CMCC, ZTE) - Intel: more analysis is needed to understand the impact of different covariance matrix estimation granularities on reporting statistics and requirements. - Recommendations for 2nd round: - Further discuss on the GTW session. Issue 2-5-3: Interference model for CQI reporting - Proposals - Option 1: As the starting point, model one inter-cell interferer with DIP of -0.41dB and static propagation condition (China Telecom, E///, CMCC, ZTE) - Option 2: Align with assumptions for demodulation requirements (Intel) - QC: For static condition, test metric (2) in Issue 2-5-4 may not work. In LTE, these requirements were defined for fading conditions where this may make more sense. - Recommendations for 2nd round: - Keep open to the next meeting. Issue 2-5-4: Test metric for CQI reporting - Proposals - Option 1: Reuse the LTE test metric as a starting point, which include (China Telecom, E///, Intel, CMCC) - 1) the relative increase of the throughput obtained when the transport format is that indicated by the reported CQI subject to an interference model compared to the case with a white Gaussian noise model, and - 2) when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified DIP, the average BLER for the indicated transport formats shall be greater than or equal to 2%. - Option 2: Further discuss (QC, ZTE) - Recommendations for 2nd round: - Keep open to the next meeting. Issue 2-6-1: Test parameters - Proposals - Option 1 (Intel, ZTE) - SCS/CBW: 15 kHz / 10 MHz for FDD and 30 kHz / 40 MHz for TDD - TDD pattern: 7D1S2U, S = 6D+4G+4U - PDSCH configuration for serving cell - Type A PDSCH mapping with starting symbol 2 and duration 12 - DMRS Type 1 with 1 additional RS - Number of PDSCH DMRS CDM group(s) without data = 1 - RMC: Rank 1, MCS 5 or 13 or 19. - PDSCH configuration for interference cells - Option 1: Type A PDSCH mapping with starting symbol 2 and duration 5 for both cells - Option 2: - Type A PDSCH mapping with starting symbol 2 and duration 5 for cell #1 - Type B PDSCH mapping with starting symbol 7 and duration 7 for cell #2 - Option 3: Type B PDSCH mapping with starting symbol 4 and duration 7 for both cells - Channel model: TDL-A, 30 ns, 10 Hz - Antenna configuration: 2x2 and 2x4 with ULA Low correlation - Receiver assumptions: - Option 1: MMSE-IRC with DMRS based covariance matrix estimation and without time selective interference handling - Option 2: MMSE-IRC with DMRS based covariance matrix estimation and with time selective interference handling - Option 3: MMSE-IRC with DMRS and Data based covariance matrix estimation - CTC, Huawei, DCM, E///, QC, Apple, CMCC: Discuss later. - Recommendations for 2nd round: - Discuss in future meetings according to the WP. Issue 2-7-1: Release independence - Proposals - Option 1: RAN4 discuss whether the UE demodulation and CQI reporting requirements with inter-cell interference is released independent from Rel-15 or not, after RAN4 agree with the detailed simulation assumption (E///, CTC, Huawei, DCM, Intel, MTK, CMCC, ZTE) - Option 2: Further discuss (QC, Intel, Apple) - Recommendations for 2nd round: - Keep open to the next meeting. |

Discussion on 2nd round

R4-2106116	WF on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC

Type: other		For: Approval
					Source: Intel

Abstract:

Decision:		Return to.

Discussion:

Issue 1: Network type

–      Synchronized for FDD and TDD

–      FFS asynchronized for FDD

[Intel] Based on first round, it looks like majority support definition of requirements for synchronized network. Therefore, we would like to check whether it is fine for focus on synchronized network and further discuss asynchronized.

[Huawei]: The WF for this issue is fine for us.

[CMCC]: The WF is OK for us

[Intel]: Taking into account that proponents of asynchronized network are fine with current wording, we assume that we can close this issue for discussion in second round.

[Ericsson]: The WF is OK for us

[Qualcomm]: WF is ok for us.

[Apple]: We are fine with the WF.

Issue 2: Antenna configuration for UE: 2 and 4 RX

[Intel] We have not had any discussion in the first round. Therefore, we would like to check whether we can reuse typical PDSCH assumptions: 2 and 4 Rx.

[CTC]: We support 2Rx and 4 Rx.

We did not raise this issue in the first round because we thought covering 2Rx and 4Rx has been an agreement in the WID:

Rx antenna number: 2Rx and 4Rx

[Huawei]: We agree with CTC’s comments that the antenna number of Rx has been agreed in the WID: 2 and 4Rx. So the agreements for issue2 in WF is fine for us.

[CMCC]：We support reuse typical PDSCH assumptions: 2 and 4 Rx

[Intel] As for number of UE Rx antenna, taking into account that it is already captured in WID, current wording should be fine.

[Ericsson]: We agree to define the requirements with both 2 Rx and 4 Rx.

[Qualcomm] Ok with 2 and 4Rx.

[Apple]: We are fine with 2RX and 4RX.

Issue 3: MCS

Use MCS 4 (QPSK, CR=0.3) and MCS 13 (16QAM, CR=0.5) for initial simulation purpose

Further discuss MCS for requirements definition

Consider MCS corresponding to QPSK, 16QAM and 64QAM modulation formats

[Intel] Based on first round discussion, most of the companies are fine to define requirements for QPSK or 16QAM modulation. Therefore, we suggest to considered existing Rel-15 PDSCH MCS for initial simulations and further discuss MCS for requirements. Also, we would like to check whether we can reduce MCS range for further discussion corresponding to {QPSK, 16QAM, 64QAM} or {QPSK, 16QAM}.

[CTC]: We support covering both QPSK and 16QAM for initial simulation. The current version is ok for us.

[Huawei]: We support to only consider {QPSK, 16QAM}. And ok to use MCS4 and MCS13 for initial simulation purpose.

[CMCC]: We are fine to go with  {QPSK, 16QAM}.

[Intel]: we would like to continue collection of comments for MCS related issue and check the views on the following wording

[Ericsson]: We prefer to use QPSK (MCS4) and 16QAM (MCS13).

[DCM]: We support covering both QPSK(MCS4) and 16QAM(MCS13) for initial simulation.

[Qualcomm]: We are ok to consider both MCS4 and MCS13 for initial simulations. But we would prefer to down-select and define requirements for only one of them.

[Apple]: For initial evaluation QPSK (MCS4) and 16QAM (MCS13) are fine. We also prefer to define requirements with only one.

[Intel] in the last slide we’ve added example of system level assumptions for information based on R4-2106426. Please let us know if this slide is fine for you.

[CTC] For page 9, we prefer not to add this page at this early stage, because we have not reach consensus on whether to do system level simulation yet.

[Intel]: As for slide 9, taking into account that during GTW discussion several companies show interest to check practical NR interference profile, we think that it will be rather beneficial to have at least example of system level assumptions (which were presented in our paper R4-2106426 and companies have time for review) and interested companies can bring their views on system level assumptions or initial simulation results in the next meeting. This slide is marked as “For information” and clarify that this is just example (i.e. it does not request any actions from other companies).

[CTC]: Regarding page 9, in a way, we can understand Intel’s point that the ICI scenario for NR may be different from that of LTE.

However, from our experience in LTE in the past, such system level simulation will need companies to do simulation and align their results, which will cost much more time effort, which is our main concern to it. Based on our concern, currently we still prefer to remove this slide 9 in the WF.

We think removing this page will of course not prevent interested companies from providing system level simulation results for the next meeting (if any).

[Intel]: We think that is very beneficial to have reference/example of system level simulation assumptions for interested companies. Same time, if there is strong concern to have this slide in WF then we can remove this slide and suggest to add the following note in the chairman report: “For information: Example of system level simulation assumptions is presented in the R4-2106426”.

[CTC]: I removed slide 9 (example SL simulation assumption for information) according to our strong concern as expressed. Same time, we can accept including following note in the chairman note:

For information: Interested company have provided example system level simulation assumptions in R4-2106426.

I would like to check whether the above v6 + additional note is acceptable.

[Intel]: Suggested wording for chairman note is also fine for us.

[CTC] For page 7, we prefer to keep the same decision for Interference covariance estimation granularity we made on the GTW session, which should be:

Further check the results and if needed per PRB bundle size or per PRB and per slot basis can be considered as possible options for simulation

[CTC]: Regarding slide 7, maybe I did not made myself clear. Our main point was to remove the wording ‘for information’ to align with the decision in the GTW session.

[Intel]: In GTW session, this sentence was marked in yellow (i.e. this is not agreement) by chairman. Therefore, we add note “For information”. Same time, if is fine for all companies to remove word “For information” then we can keep wording from the v5.

[QC]: Physical cell ID. It implicitly implies that we have agreed to define requirements with 2 interfering cells. We prefer to consider only 1 interfering cell because we don’t understand the motivation behind defining the requirements with 2 interfering cells. From demod processing perspective, UE is not going to use a different algorithm under MMSE-IRC receiver assumption for 1 vs 2 interfering cells.

[Apple]: Physical cell ID: We prefer to use 1 interference cell. We don’t see necessity to have 2 interference cells.

[Intel]: Physical Cell ID. We suggest to define this configuration in generic manner to avoid misunderstanding that 1 or 2 interference cells modelling was already agreed.

[QC]: Channel Bandwidth. We still prefer to consider only 10MHz (FDD) and 40MHz (TDD). As we and other companies have explained, these CBWs cover almost all bands and demod algorithm won’t change with change in CBW. However, proponents have not provided any answer other than trying to test higher CBW. If that is the criteria, many other CBWs can be studied, which is not practical. This discussion had happened in the beginning of Rel-15 for single carrier scenario and in the end, it was decided to focus on CBWs which cover most of the bands. This will unnecessarily increase the test burden without much additional insight. If down-selection is considered and criteria is better gain, most likely higher CBW may show better gains because of larger number of RBs, then we are not going to skip defining requirements for FDD 10MHz and TDD 40MHz because we want to cover most bands. So, I am not sure what further down-selection consideration entails and will prefer not to include higher CBWs.

[Apple]: CBW: We prefer to consider only 10MHz for FDD and 40MHz for TDD like most of the PDSCH demod simulations.

[Huawei]: CBW: We prefer to only define requirements with 10MHz for FDD and 40MHz for TDD.

[Intel]: Channel bandwidth: Based on comments, we’ve kept one CBW per SCS for initial simulation purpose. Same time, we’ve added note that other options are not precluded and clearly captured all available options from this meeting.

[Apple]: SSB : Slot 0 Index 0 for Cell1, Index 1 for Cell2; Periodicity 20ms

[Intel]: SSB position: We suggest to keep open because transmission in different frequency positions can be also consider and more applicable to scenario with multiple interference cells (in case such scenarios will be considered).

[Apple]: TRS/CSI-RS Config: Re-use same parameters as Table 5.2-1 Form 38.101-4 for Cell1; Cell 2 parameters change based on different options

[Intel]: TRS configuration: We are fine with Apple suggestion. We suggest other companies to double check.

[Apple]: Propagation condition: We are okay with both TDLA30-10 and TDLC300-100 for initial evaluation, but prefer to down select for requirements definition.

[Huawei]: Propagation condition: Only consider TDLA30-10 as this is mainly for cell-edge users.

[Intel]: Based on comments from companies, we’ve kept one option for initial simulation purpose and clearly captured other options.

[Apple]: DIP1: -1.73dB

[CTC]: In the first round discussion, companies have different options on the exact DIP value(s) and interference cell number, and we have not reached agreement yet. So, in our understanding, we suggest to FFS on the exact values and cell number:

- Interference profile

  - Using DIP approach for interference profile definition for initial simulation purpose

    - Interference profiles  from LTE MMSE-IRC can be used for initial simulation

      - DIP1: -1.73 dB

      - Further decide the exact DIP value(s) and interference cell number based on simulation results.

[Ericsson]: We have the same comment as China Telecom on DIP configurations. We want to consider at least the following interference profiles from LTE MMSE-IRC for the initial simulation.

| DIP | (DIP1, DIP2) = (-2.23, -8.06) (DIP1) = (-2.23) (DIP1, DIP2) = (-1.73, -8.66) (DIP1) = (-1.73) |
| --- | --- |

[Intel]: DIP: We’ve captured proposal from China Telecom

R4-2106117	WF on CQI reporting requirements for inter-cell interference MMSE-IRC

Type: other		For: Approval
					Source: Ericsson

Abstract:

Decision:		Return to.

Discussion:

[Ericsson]: The draft WF on CQI reporting requirements for inter-cell interference MMSE-IRC has already uploaded as follow. BTW, we plan to keep all the CQI related issues open since this is the 1st meeting.

- MMSE-IRC receiver for intra-cell inter-user interference

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2104607 | CMCC | Proposal 1: the following SCS and bandwidth combination should be considered: - FDD - 10MHz/15kHz - 50MHz/15kHz - TDD - 40MHz/30kHz - 100MHz/30kHz Proposal 2: For TDD 30kHz, use typical 7D1S2U(S=6D+4G+4U) for TDD configuration. Proposal 3: For Tx antenna number, we propose to consider 8 and 16 Tx antenna number. Proposal 4: For MIMO correlation, include XP High and XP Medium in this phase. Proposal 5: - For FDD, the number of HARQ processes set to 4; - For TDD, the number of HARQ processes set to 8; Proposal 6: For propagation channel, TDLA30-10 can be the baseline. Proposal 7: - SSB position in burst: first SSB in Slot#0 - SSB periodicity: 20ms. Proposal 8: For the number of paired UEs, we propose to consider 1, 2, 3 paired UEs. Proposal 9: For Rank configuration, at least include Rank1 and further evaluate Rank2 in this phase. Proposal 10: - Target PDSCH MCS configuration: - QPSK/0.30, 16QAM/0.48 and 64QAM/0.5 for Rank 1 - 64QAM for Rank 2 - Interference PDSCH MCS configuration - 16QAM/0.48 as the baseline Proposal 11: Use target UE’s throughput vs SNR as the test metric for MMSE-IRC receiver demodulation performance requirements, and consider 70% relative throughput for the test points. |
| R4-2104847 | Apple Inc. | Proposal #1: Transmit co-scheduled UE on different CDM group as target UE. Proposal #2: Use number of CDM groups without data on target UE as 2. Observation #1: ZF precoder is effective in nulling inter-user interference when UE reported PMI or actual channel is used rather than random PMI. Observation #2: 1.5 - 3dB performance improvement is observed with QRD orthogonalization over ZF precoding with randomly generated PMI. Proposal #3: Use QRD orthogonalization to generate precoders for MU-MIMO. Proposal #4: The total number of layers transmitted shall be less than or equal to the number of UE RX antenna. Proposal #5: Limit the study and evaluation for MU-MIMO to up to 16QAM for 2x2 and up to 64QAM for 4x4. Proposal #6: For 4x4 limit the layers combinations to [1,1], [1,2], [2,1] on target and co-scheduled UE respectively. |
| R4-2106427 | Intel Corporation | Proposal 1: Consider the following assumptions for MU-MIMO modelling: Explicit modelling of PDSCH+DMRS signal for one interference UE Random or feedback-based precoder selection of serving UE Selection of precoder for interference UE to ensure orthogonality of serving and interference precoders Random PDSCH signal generation for interference UE with 16QAM modulation Proposal 2: Consider the following simulation assumptions for MMSE-IRC requirements for scenario with intra-cell inter-user interference: SCS/CBW: 15 kHz / 10 MHz for FDD and 30 kHz / 40 MHz for TDD Type A PDSCH mapping with starting symbol 2 and duration 12 Serving PDSCH Rank 1 or Rank 2, MCS 5 or 12 or 19 Interference PDSCH Rank is same as for Serving PDSCH DMRS Type 1 with 1 additional RS DMRS AP mapping: Rank 1 – Option 1: Number of CDM groups without data is equal to 1, UE #1 uses AP 1000, UE #2 uses AP 1001. Rank 1 – Option 2: Number of CDM groups without data is equal to 2, UE #1 uses AP 1000, UE #2 uses AP 1002. Rank 2: Number of CDM groups without data is equal to 2, UE #1 uses APs 1000 and 1001, UE #2 uses AP 1002 and 1003. TDD pattern: 7D1S2U, S = 6D+4G+4U Channel model: TDL-A, 30 ns, 10 Hz Antenna configuration: 2x2 or 4x2 and 2x4 or 4x4 Candidate receivers: Option 1: MMSE-IRC processing with serving signal demodulation Option 2: MMSE-IRC processing with joint (serving + interference) signal demodulation Observation #1: MMSE-IRC does not allow to improve MU-MIMO performance for scenarios with QPSK modulation. Observation #2: MMSE-MRC cannot reach maximum throughput for scenarios with 16QAM modulation. Observation #3: Using of MMSE-IRC processing with interference-plus-noise covariance matrix estimation based on active serving DMRS positions leads to significant performance degradation for scenarios with 2 DMRS CDM groups. |
| R4-2104954 | China Telecom | Proposal 1: Cover 8Tx and 16 Tx antenna ports, 2Rx and 4Rx antennas. Proposal 2: Only measure the PDSCH throughput performance of one target UE, and use follow PMI in the PDSCH simulation. Proposal 3: Use random PMI selection for the paired UE(s). Proposal 4: Use Zero Forcing as the BS precoding algorithm if the feasibility can be confirmed by the TE vendor. Proposal 5: For 8Tx, use type I single panel codebook with wideband PMI for all the target and paired UEs, and the same BS precoding matrix is applied in the whole transmitted bandwidth. Proposal 6: For 16Tx, use Rel-15/16 (e)type II codebook for all the target and paired UEs, and use subband PMI if it is feasible for TE to calculate ZF precoding matrix per subband. Proposal 7: Assume low correlation among the co-scheduled UEs. Proposal 8: In the initial simulation, consider different options for the number of co-scheduled UEs an the number of layers per UE: - 2 co-scheduled UEs (1 target UE and 1 paired UEs) and 1 layer per UE. - 2 co-scheduled UEs (1 target UE and 1 paired UEs) and 2 layers per UE. - 4 co-scheduled UEs (1 target UE and 3 paired UEs) and 1 layer per UE. - 4 co-scheduled UEs (1 target UE and 3 paired UEs) and 2 layers per UE. Proposal 9: In both target and interfering PDSCH, assume PDSCH mapping type A with full PRB allocation. Proposal 10: Use Type 1 DMRS with 1 additional DMRS: - Use single symbol front loaded DMRS for cases with no more than 4 layers for all the considered UEs, and use double symbol front loaded DMRS for cases with 5-8 layers for all the considered UEs. - Assume FDM between DMRS and data, i.e., the number of DMRS CDM groups without data as 1, for no more than 2 layers for all the considered UEs; and assume TDM between DMRS and data, i.e., the number of DMRS CDM groups without data as 2, for 3-8 layers for all the considered UEs. - For the i-th UE with 2-layer transmission, DMRS port 2i-2 and 2i-1 (i = 1, 2,...) are used. Proposal 11: The ratio of PDSCH EPRE to DM-RS EPRE is assumed as 0 dB and -3 dB when the number of DM-RS CDM groups without data is 1 and 2 respectively. Proposal 12: Assume 16QAM randomly modulated symbols in the interfering PDSCH. Proposal 13: Cover QPSK MCS 4, 16QAM MCS 13, and 64QAM MCS 19 in the initial simualtions. The following observation and proposals were made w.r.t. the reference receiver: Observation 1: It is unknown whether the precoding matrix in the multiple contiguous PRBs for the co-scheduled UE(s) is the same. Proposal 14: The estimation of interference covariance matrix can be performed at per PRB and per slot basis. Proposal 15: For cases with 2 DMRS CDM groups, the interference should be estimated based on the REs occupied by both of the two DMRS CDM groups. The following proposals were made w.r.t. other PDSCH simulation parameters: Proposal 16: Cover at least 15kHz SCS for FDD and 30kHz SCS for TDD. Proposal 17: Cover 10 MHz and 40 MHz channel bandwidth for FDD 15kHz SCS, 40MHz and 100MHz channel bandwidth for TDD 30kHz SCS. Proposal 18: Cover two TDD configurations for 30kHz SCS: - Configuration 1: 7D1S2U, S = 6D:4G:4U - Configuration 2: DDDSUDDSUU, S1=10G: 2G: 2U, S2 = 10G: 2G: 2U Proposal 19: Re-use the Rel-15 assumptions on HARQ process number, i.e., 4 for FDD 15kHz SCS and 8 for TDD 30kHz SCS. Proposal 20: For the propagation condition, include TDLA30-10 and TDLC300-100 in Phase I, and decide whether down-selection or adjustment is needed later. Use XP high or XP medium for the antenna correlation. Proposal 21: For SSB position, reuse the Rel-15 assumptions , i.e., configure the first SSB in Slot #0 in every 20 slots, and the slot #0 in every 20 slots is not scheduled for PDSCH transmission. Proposal 22: For TRS and ZP CSI-RS, reuse the Rel-15 assumptions for PDSCH demodulation requirement. For NZP CSI-RS, reuse the Rel-15 general assumptions for PDSCH demodulation requirement and extend the configuration for more than 4 ports. |
| R4-2106868 | Ericsson | Proposal 1: Specify intra-cell inter-user interference model as follows: Precoded modulation symbols of an interference signal are randomly mapped onto one of the remaining DMRS antenna port. The update granularity for randomized mapping antenna port is PRB bundling size in frequency domain and slot in time domain. For each slot and each PRB bundling size, a precoding matrix for the number of layers, v, associated to the selected rank shall be selected randomly from [Single Panel Type I]. The selected precoding matrix shall not be identical to the precoding matrix applied for the UE under test.. Proposal 2: For inter-user interference scenario, configure 2 DMRS CDM groups without data, that is, DMRS ports 1000/1001 are assigned for the UE under test and DMRS ports 1002/1003 are assigned for the co-scheduled UE. Proposal 3: For the MMSE-IRC receiver performance evaluation, RAN4 should assume the following parameters as the starting point. Parameters; FDD; TDD. CBW; 10MHz; 40MHz. TDD configuration; N/A; 7DS2U, S=6D+4G+4U Schedule PDSCH in special slots. SCS; 15kHz; 30kHz. PDSCH configuration; Type A, Start symbol 2, Duration 12​; Type A, Start symbol 2, Duration 12​. PRB bundling size; 2; 2. PDSCH DMRS configuration; Type 1, 1+1; Type 1, 1+1. DMRS port assignment; 1000 for UE under test for rank 1 1000/1001 for UE under test for rank 2 1002 for co-scheduled UE for rank 1 1002/1003 for co-scheduled UR for rank 2; 1000 for UE under test for rank 1 1000/1001 for UE under test for rank 2 1002 for co-scheduled UE for rank 1 1002/1003 for co-scheduled UR for rank 2. MCS; MCS 4 (QPSK 0.3) MCS 13 (16QAM 0.48) MCS 19 (64QAM 0.5); MCS 4 (QPSK 0.3) MCS 13 (16QAM 0.48) MCS 19 (64QAM 0.5). Rank; 1 and 2; 1 and 2. PDSCH precoder; Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity; Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity. Antenna configuration; 2x2, 2x4 for rank 1 4x2, 4x4 for rank 2; 2x2, 2x4 for rank 1 4x2, 4x4 for rank 2. Propagation condition; TDLA30-10; TDLA30-10. Metric; SNR to achieve 70% of maximum throughput; SNR to achieve 70% of maximum throughput. Proposal 4: For evaluation RAN4 should compare the performance difference between the case with co-scheduled UE and without co-scheduled UE. |
| R4-2106834 | Huawei, HiSilicon | Observation 1: Intra-cell interference between paired UEs has significant impact on system performance Proposal 1: Define PDSCH demodulation requirement for intra-cell inter-user interference suppressing Observation 2: LTE MU-MIMO test case has verified one single-layer transmission companied by one interfering simultaneous transmission, which indicates two UEs a pair Proposal 2: Take two Users as a pair and network does not need to ensure the perfect orthogonality between these two UEs Observation 3: LTE MU-MIMO test cases use randomly selected precoder for both paired UEs Proposal 3: Reuse LTE precoding method of random selection without identical as a starting point for evaluation, fixed PMI for both UEs or at least one of them is not precluded Proposal 4: Use 4 as the precoding granularity for both UEs Proposal 5: Use a multi-path fading channel with relative high frequency selective characteristic as channel model, take TDLC300 as a start to evaluate Proposal 6: Consider low correlation level for defining requirement Proposal 7: Not to consider any Null-forming method or any other interference suppression or scheduling schemes from BS side Proposal 8: RAN4 discusses and decides whether to introduce network assistance Proposal 9: Consider this draft interference model for further evaluation Proposal 10: PDSCH mapping type: Type A Proposal 11: Consider 2x2, 2x4 for antenna configuration Proposal 12: Use same DMRS pattern and same sequence for both paired UEs Proposal 13: The DUT and the co-scheduled UE can be either in the same CDM group or not, but with same PRG configurations |

Open issues summary

Sub-topic 3-1: Inter-user interference modeling for phase I evaluation

Issue 3-1-1: Paired UE number

- Proposals

  - Option 1: Consider 1, 2, 3 paired UEs. (CMCC)

  - Option 2: Consider both 1 and 3 paired UEs, make down-selections later based on the simulation outcome (CTC)

  - Option 3: Only 1 paired UE (HW)

    - HW: Same with LTE MU-MIMO test cases.

- Recommended WF

  - For initial evaluation, can we take option 2 as a compromise among different options?

Issue 3-1-2: Rank for target and interference PDSCH

- Proposals

  - Option 1: At least cover rank 1 (CMCC)

  - Option 2: Rank 1 or rank 2 for target UE, and the rank for interference UE is same with that for target UE (Intel)

  - Option 3: Cover both rank 1 and rank 2 per UE (CTC, E///)

  - Option 4: Limit the rank combinations to [1,1], [1,2], [2,1] on target and co-scheduled UE for 4x4 (Apple)

    - Apple: The total number of layers transmitted shall be less than or equal to the number of UE RX antenna.

- Recommended WF

  - For initial evaluation, can we include both rank 1 and rank 2 per UE (with the same rank for target and interference UEs)?

Issue 3-1-3: Correlation between the propagation channel of the paired UEs

- Proposals

  - Option 1: Low (CTC, HW)

- Recommended WF

  - For initial evaluation, can we take option 1 as starting point to simplify the simulation setup?

Issue 3-1-4: Antenna configuration

- Proposals

  - Tx antenna number:

    - Option 1: 8Tx and 16Tx (CMCC, CTC)

      - CMCC, CTC: Under MU-MIMO test setup, the number of transmit antenna will be larger than that of SU-MIMO test setup.

    - Option 2: 2Tx or 4Tx (Intel)

    - Option 3: 2Tx for rank 1 and 4Tx for rank 2 (E///)

    - Option 4: 2Tx only (HW)

      - HW: 2Tx was selected for LTE MU-MIMO test cases.

  - Rx antenna number:

    - Option 1: Cover both 2Rx and 4Rx (CMCC, CTC, Intel, E///, HW)

- Recommended WF

  - For Tx antenna number, companies’ views are divergent, encourage more discussion in the first round by taking into account the practical scenario and test complexity

  - For Rx antenna number, agree option 1.

Issue 3-1-5: Codebook type

- Proposals

  - Option 1: (CTC)

    - For 8Tx, use type I SP codebook with wideband PMI reporting for target and paired UEs.

    - For 16Tx, use Rel-15/16 (e)type II codebook for target and paired UEs. Use subband PMI if it is feasible for TE to calculate ZF precoding matrix per subband.

      - CTC: For 8Tx, type I single panel codebook with wideband PMI is a mandatory UE feature without capability.

  - Option 2: For 2Tx and 4Tx, use Type I SP codebook. Type II precoder can also be applied for 4Tx (E///)

- Recommended WF

  - Encourage more discussion in the first round.

Issue 3-1-6: PMI selection and precoding matrix generation

- Proposals

  - Option 1: Random based target UE PMI selection (Apple, Intel, E///, HW)

    - Option 1A: Random selection based precoder generation with QRD orthogonalization processing as below (Apple)

- Choose precoders of the N co-scheduled users randomly and horizontally concatenate them to get the overall precoder matrix P as follows: P=P1⋯PN - Compute the l2-norm squared of every column of P as follows: r=diagPHP where r is a vector of length NL≜i=1NNLi - Use QRD or SVD to get the orthonormal basis of P as follows: Q,R=qr(P) - Keep only the first NL columns of Q and scale every column using its corresponding value in r vector as follows Q=Q:,1:NL×diagr where diagr is a diagonal matrix with the square root of r entries of its diagonal. The purpose of this scaling is to preserve the power of every column such that the Frobenius norm of Pi' is one for i=1,2…,N as in the codebook. - P1' is the first NL1 columns of Q; P2' is the next NL2 columns of Q, and so on., i.e. Pi'=Q:,j=1i-1NLj+1:j=1iNLj, i=1,2,…,N

    - Option 1B: Random PMI selection for the target UE, and select the precoder for the interference UE to ensure orthogonality (Intel)

      - Intel: Feedback-based PMI selection mode is more preferable solution because it close to practical assumptions. However, we can check two PMI selection procedures and analyze pros and cons of each option.

    - Option 1C: Random PMI selection for both target and interference UE, with ensuring the selected PMI matrix shall not be identical to the precoding matrix applied for the UE under test. (E///, HW)

      - HW: 1) LTE MU-MIMO test cases use randomly selected precoder for both paired UEs; 2) Not to consider any Null-forming method or any other interference suppression or scheduling schemes from BS side

  - Option 2: Feedback-based target UE PMI selection (CTC, Intel)

    - Option 2A: If the feasibility can be confirmed by the TE vendor, use ZF precoding based on the reported PMI from the target UE, and the randomly generated PMI from the interference UE(s). (CTC)

    - Option 2B: Feedback-based PMI selection for the target UE, select the precoder for the interference UE to ensure the orthogonality (Intel)

  - Option 3: Fixed precoding matrix for one or both co-scheduled UEs (HW)

- Recommended WF

  - For option 1A and 2A with QRD/ZF processing from Apple/CTC, invite the inputs from TE vendors on the test feasibility.

  - For option 1B and 2B from Intel, it seems more details on how to ensure the orthogonality are needed.

  - Encourage more discussion, aiming to come up with one baseline option for initial evaluation

Issue 3-1-7: PRB bundling size and precoding granularity

- Proposals

  - Option 1: Per 2 PRBs for frequency domain and per slot for time domain (E///)

  - Option 2: 4 for both target and interference UEs (HW)

  - Option 3: (CTC)

    - Wideband for 8Tx for target and paired UEs.

    - For 16Tx, use subband precoding if it is feasible for TE to calculate ZF precoding matrix per subband.

- Recommended WF

  - Encourage more discussion in the first round.

Issue 3-1-8: MCS for interfering PDSCH

- Proposals

  - Option 1: Normal PDSCH with MCS 16QAM/0.48 as the baseline (CMCC)

  - Option 2: Random 16QAM signal generation (Intel, CTC)

- Recommended WF

  - Encourage more discussion in the first round.

Sub-topic 3-2: DMRS configuration for phase I evaluation

Issue 3-2-1: DMRS ports for target and interfering UEs

- Proposals

  - For rank 1 transmission per UE, with 1 target UE and 1 interference UE:

    - Option 1: DMRS port 0 for target UE, DMRS port 1 for the interference UE (China Telecom, Intel)

    - Option 2: DMRS port 0 for target UE, DMRS port 2 for the interference UE, i.e., different CDM groups (Intel, Apple, E///)

  - For rank 1 transmission per UE, with 1 target UE and more than 1 interference UE:

    - Option 1: DMRS port 0 for target UE, DMRS port i for the i-th interference UE (i = 1, 2,...) (China Telecom)

  - For rank 2 transmission per UE:

    - Option 1: DMRS port 0/1 for target UE, DMRS port 2i and 2i+1 for the i-th interference UE (i = 1, 2,...) (Intel, Apple, E///, CTC)

  - When rank [1, 2] or rank [2, 1] transmission for the target and interference UEs:

    - Option 1: DMRS port 0 (and 1) for target UE, port 2 (and 3) for the interference UE, i.e., use different CDM groups for the target and interference UEs (Apple, E///)

- Recommended WF

  - Encourage more discussion in the first round, aiming to come up with one baseline option for initial evaluation purpose

Issue 3-2-2: DMRS type and DMRS additional position

- Proposals

  - Option 1: DMRS Type 1 with 1 additional DMRS (Intel, CTC, E///)

- Recommended WF

  - Use option 1 if no objection during the first round discussion.

Issue 3-2-3: Ratio of PDSCH EPRE to DM-RS EPRE

- Proposals

  - Option 1: 0 dB and -3 dB when the number of DM-RS CDM groups without data is 1 and 2 respectively (CTC)

- Recommended WF

  - Encourage feedback from more companies

Issue 3-2-4: Whether to use the same DMRS pattern and the same sequence for all co-scheduled UEs

- Proposals

  - Option 1: Yes (HW)

    - HW: In real network deployment, same DMRS pattern for MU-MIMO paired UEs has been always used

- Recommended WF

  - Encourage feedback from more companies

Sub-topic 3-3: Reference receiver for phase I evaluation

Issue 3-3-1: Candidate Receivers

- Proposals

  - Option 1: MMSE-IRC processing with serving signal demodulation (Intel)

    - xS=HSHHSHSH+RI+N-1y.

  - Option 2: MMSE-IRC processing with joint (serving + interference) signal demodulation (Intel)

    - xS+I=HS+IHHS+IHS+IH+RN-1y, where HS+I=HSHI.

    - Intel: for MU-MIMO mode, UE has enough information to make channel estimation of interference signal and execute joint (serving + interference) demodulation processing.

- Recommended WF

  - Encourage feedback from more companies

Issue 3-3-2: Interference estimation for cases with 2 DMRS CDM groups

- Proposals

  - Option 1: For cases with 2 DMRS CDM groups, the interference should be estimated based on the REs occupied by both of the two DMRS CDM groups (CTC)

- Recommended WF

  - Encourage feedback from more companies

Issue 3-3-3: Interference estimation granularity

- Proposals

  - Option 1: Per PRB and per slot based interference covariance matrix estimation (CTC)

    - CTC: It is unknown whether the precoding matrix in the multiple contiguous PRBs for the co-scheduled UE(s) is the same.

- Recommended WF

  - Encourage feedback from more companies

Issue 3-3-4: Whether to introduce network assistance to assist the receiver

- Proposals

  - Option 1: RAN4 discusses and decides whether to introduce network assistance (HW)

    - HW: With network assistance, the tested UE can perform interference cancellation with less cost by detection.

- Recommended WF

  - Encourage feedback from more companies.

Sub-topic 3-4: PDSCH parameters for phase I evaluation

Issue 3-4-1: SCS

- Proposals

  - Option 1: Cover both 15kHz SCS for FDD and 30kHz SCS for TDD. (CMCC, CTC, Intel, E///)

- Recommended WF

  - Use option 1 if no objection during the first-round discussion.

Issue 3-4-2: Channel Bandwidth

- Proposals

  - Option 1 (CMCC)

    - For FDD 15kHz SCS: Cover 10MHz and 50MHz CBW.

    - For TDD 30kHz SCS: Cover 40MHz and 100MHz CBW.

      - CMCC: We also support to consider the maximum bandwidth for FDD and TDD, which is 50MHz for FDD and 100MHz for TDD.

  - Option 2: 10 MHz for FDD 15kHz SCS and 40 MHz for TDD 30kHz SCS (Intel, E///)

  - Option 3: (CTC)

    - For FDD 15kHz SCS: Cover 10MHz and 40MHz CBW.

    - For TDD 30kHz SCS: Cover 40MHz and 100MHz CBW.

- Recommended WF

  - Encourage more discussion during the first round.

Issue 3-4-3: TDD Configuration

- Proposals

  - Option 1: 7D1S2U(6D+4G+4U) for 30kHz TDD (CMCC, Intel, E///)

  - Option 2: Cover both of the TDD patterns below for 30kHz TDD (CTC)

    - 7D1S2U, S = 6D:4G:4U

    - DDDSUDDSUU, S1=10G: 2G: 2U, S2 = 10G: 2G: 2U

- Recommended WF

  - Encourage more discussion during the first round.

Issue 3-4-4: MIMO correlation for each UE

- Proposals

  - Option 1: Consider both XP High and XP Medium, and make further down-selection based on evaluation results (CMCC)

  - Option 2: XP High or XP Medium (CTC)

- Recommended WF

  - Encourage feedback from more companies.

Issue 3-4-5: Propagation Condition

- Proposals

  - Option 1: TDLA30-10 as baseline (CMCC, Intel, E///)

    - CMCC: We are also support to evaluate other propagation channel in Phase1.

  - Option 2: Cover both TDLA30-10 and TDLC300-100 in phase I, and decide whether down-selection or adjustment is needed later (CTC, [CMCC])

  - Option 3: TDLC300 channel (HW)

    - HW: When transmitted signal is experiencing large time delay and frequency selective propagation condition, the precoding performance will have an obviously loss compared to flat channel, which is matched with real network scenario.

- Recommended WF

  - Considering the different preferences on TDLA30/TDLC300, can we include both and take option 2 for initial evaluation?

Issue 3-4-6: MCS for Target UE

- Proposals

  - Option 1: (CMCC)

    - QPSK MCS 4, 16QAM MCS 13 and 64QAM MCS 19 for Rank 1

    - 64QAM MCS 19 for Rank 2

  - Option 2: MCS 5 or 12 or 19 (Intel)

  - Option 3: Cover QPSK MCS 4, 16QAM MCS 13, and 64QAM MCS 19 for initial simulation (CTC, E///)

  - Option 4: Up to 16QAM for 2x2 and up to 64QAM for 4x4 (Apple)

- Recommended WF

  - Encourage suggestions on how to down-select a MCS set for initial evaluation.

Issue 3-4-7: PDSCH Mapping Type

- Proposals

  - Option 1: Type A (Intel, CTC, E///, HW)

- Recommended WF

  - Use option 1 if no objection during the first-round discussion.

Issue 3-4-8: PDSCH Resource Allocation

- Proposals

  - Time Domain:

    - Option 1: Starting symbol 2 and duration 12 (Intel, E///)

  - Frequency Domain:

    - Option 1: Full PRB allocation (CTC)

- Recommended WF

  - Use option 1 for both time and frequency domain as baseline

Issue 3-4-9: HARQ Process Number

- Proposals

  - Option 1: 4 for FDD 15kHz SCS and 8 for TDD 30kHz SCS (CMCC, CTC)

    - CMCC: Reuse the typical HARQ configuration in R16 PDSCH demodulation requirements.

- Recommended WF

  - Use option 1 as baseline

Issue 3-4-10: SSB Configuration

- Proposals

  - Option 1: (CMCC, CTC)

    - SSB position in burst: first SSB in Slot#0; SSB periodicity: 20ms.

    - The slot #0 in every 20 ms is not scheduled for PDSCH transmission

      - CMCC, CTC: Reuse the SSB configuration in NR PDSCH demodulation requirement.

- Recommended WF

  - Use option 1 as baseline

Issue 3-4-11: TRS, NZP CSI-RS and ZP CSI-RS Configuration

- Proposals

  - Option 1: (CTC)

    - For TRS and ZP CSI-RS, reuse the Rel-15 assumptions for PDSCH demodulation requirement.

    - For NZP CSI-RS, reuse the Rel-15 general assumptions for PDSCH demodulation requirement and extend the configuration for more than 4 ports

- Recommended WF

  - Encourage feedback from more companies.

Issue 3-4-12: Performance evolution metrics

- Proposals

  - Option 1: Measure the 70% max throughput performance of the target UE (CMCC, CTC, E///)

    - Evaluate the gain of MMSE-IRC over MMSE under the same simulation setup

- Recommended WF

  - Encourage feedback from more companies.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Company A | Sub-topic 3-1: Inter-user interference modeling for phase I evaluation Issue 3-1-1: Paired UE number Issue 3-1-2: Rank for target and interference PDSCH Issue 3-1-3: Correlation between the propagation channel of the paired UEs Issue 3-1-4: Antenna configuration Issue 3-1-5: Codebook type Issue 3-1-6: PMI selection and precoding matrix generation Issue 3-1-7: PRB bundling size and precoding granularity Issue 3-1-8: MCS for interfering PDSCH Sub-topic 3-2: DMRS configuration for phase I evaluation Issue 3-2-1: DMRS ports for target and interfering UEs Issue 3-2-2: DMRS type and DMRS additional position Issue 3-2-3: Ratio of PDSCH EPRE to DM-RS EPRE Issue 3-2-4: Whether to use the same DMRS pattern and the same sequence for all co-scheduled UEs Sub-topic 3-3: Reference receiver for phase I evaluation Issue 3-3-1: Candidate Receivers Issue 3-3-2: Interference estimation for cases with 2 DMRS CDM groups Issue 3-3-3: Interference estimation granularity Issue 3-3-4: Whether to introduce network assistance to assist the receiver Sub-topic 3-4: PDSCH parameters for phase I evaluation Issue 3-4-1: SCS Issue 3-4-2: Channel Bandwidth Issue 3-4-3: TDD Configuration Issue 3-4-4: MIMO correlation for each UE Issue 3-4-5: Propagation Condition Issue 3-4-6: MCS for Target UE Issue 3-4-7: PDSCH Mapping Type Issue 3-4-8: PDSCH Resource Allocation Issue 3-4-9: HARQ Process Number Issue 3-4-10: SSB Configuration Issue 3-4-11: TRS, NZP CSI-RS and ZP CSI-RS Configuration Issue 3-4-12: Performance evolution metrics |
| China Telecom | Sub-topic 3-1: Inter-user interference modelling for phase I evaluation Issue 3-1-1: Paired UE number Support the recommended WF. More co-scheduled UE number should be considered for the phase I evaluation because it is a very typical scenario for MU-MIMO usage. Issue 3-1-2: Rank for target and interference PDSCH Support the recommended WF. MMSE-IRC receiver performance in MU-MIMO with both 1 layer and 2-layer transmission should be evaluated in phase I, because 2-layer transmission is also a typical scenario as UE mandatory 4Rx. Issue 3-1-3: Correlation between the propagation channel of the paired UEs Support the recommended WF. In our understanding, since the channel of the paired UE is only reflected by its selected PMI, we would like to check if we can combine this issue with issue 3-1-6? Issue 3-1-4: Antenna configuration For Tx antenna number, we support option 1 because it is typical that larger BS Tx number including 8Tx and 16Tx will be used for MU-MIMO scenario. For Rx antenna number, support the recommended WF. Issue 3-1-5: Codebook type Option 1. Both Type I and Type II codebook should be considered in the phase I evaluation. It is common understanding that Type II is designed for MU-MIMO scenario. Issue 3-1-6: PMI selection and precoding matrix generation In practical, ZF/QRD/orthogonal precoding based on the reported PMI will always be used for MU-MIMO. Therefore, it is our view that the benefit of introducing MMSE-IRC receiver for MU-MIMO scenario need to be shown under more practical simulation assumptions, rather than introducing extra interference due to random precoding. Moreover, random PMI selection for the target UE will cause decreased performance (especially for 8Tx, 16Tx). So, we support option 2A from practical MU-MIMO implementation perspective. However, as pointed out in the recommended WF, the test feasibility for option 1A/2A is pending TE vendors’ feedback, and more details are needed for option 1B/2B. To assist the simulation evaluation work for the next meeting, we suggest to firstly evaluate the option with clear test feasibility, i.e., New option 2C: Feedback-based PMI selection for target UE, and random PMI selection for interference UE, with ensuring the selected PMI matrix shall not be identical to the precoding matrix applied for the UE under test and other options are not precluded. Issue 3-1-7: PRB bundling size and precoding granularity We are fine with using smaller PRB bundling size and precoding granularity. But we need to check with TE vendors about the test feasibility if we are using ZF/QRD/orthogonal precoding per 2/4 PRBs. Issue 3-1-8: MCS for interfering PDSCH Since there will be no performance difference for the target UE between the 2 options, both options are ok for us. Slightly prefer option 2 for simulation simplification consideration. Sub-topic 3-2: DMRS configuration for phase I evaluation Issue 3-2-1: DMRS ports for target and interfering UEs In our understanding, using separate DMRS CDM groups for the target UE and interference UE will improve the channel estimation accuracy by using DMRS power boosting. And better interference estimation can also be expected without the inter user interference on the DMRS. As a result, when rank [1, 1] or rank [2,2] for the target and interference UEs, we are also ok with separate CDM groups. However, for the other rank configurations, including rank [1, 2] or rank [2, 1] or rank 1/2 transmission per UE with more than 1 interference UE scenarios, we do not think separate DMRS CDM groups for the target UE and interference UE should be assumed, because such assumption is not a fair DMRS port assignation for all the co-scheduled UE, which will exaggerate the performance improvement for the target UE. Issue 3-2-2: DMRS type and DMRS additional position Support the recommended WF. Issue 3-2-3: Ratio of PDSCH EPRE to DM-RS EPRE Support option 1. Issue 3-2-4: Whether to use the same DMRS pattern and the same sequence for all co-scheduled UEs Need more clarification on ‘DMRS pattern’, we are ok with using the same ‘DMRS type’ and ‘DMRS additional position’ for all co-scheduled UEs. We are ok with using the same sequence for all co-scheduled UEs Sub-topic 3-3: Reference receiver for phase I evaluation Issue 3-3-1: Candidate Receivers Support option 1, and option 2 is also ok for us if the interference channel can be obtained without extra RRC message. Issue 3-3-2: Interference estimation for cases with 2 DMRS CDM groups Option 1 Issue 3-3-3: Interference estimation granularity Option 1 Issue 3-3-4: Whether to introduce network assistance to assist the receiver We need more clarification on what detailed information is needed. Sub-topic 3-4: PDSCH parameters for phase I evaluation Issue 3-4-1: SCS Support the recommended WF. Issue 3-4-2: Channel Bandwidth Support option 3. In addition to FDD 15kHz 10MHz and TDD 30kHz 40MHz, a larger CBW should be considered for FDD and TDD respectively. The difference between option 1 and option 3 is to include 40MHz or 50MHz for FDD 15kHz, and we still prefer 40MHz since 50MHz may not be implemented in many bands/UEs. Issue 3-4-3: TDD Configuration We will not insist on option 2, and option 1 can be fine for us. Issue 3-4-4: MIMO correlation for each UE In our understanding, option 1/2 are of the same meaning. We suggest to include both correlation levels for initial evaluation simulation, and make down-selection based on simulation results, i.e., option 1 is also ok for us. Issue 3-4-5: Propagation Condition Support the recommended WF. Issue 3-4-6: MCS for Target UE Support option 3, for phase I, each modulation order QPSK, 16QAM and 64QAM need to be evaluated. Issue 3-4-7: PDSCH Mapping Type Support the recommended WF. Issue 3-4-8: PDSCH Resource Allocation Support the recommended WF. Issue 3-4-9: HARQ Process Number Support the recommended WF. Issue 3-4-10: SSB Configuration Support the recommended WF. Issue 3-4-11: TRS, NZP CSI-RS and ZP CSI-RS Configuration option 1 Issue 3-4-12: Performance evolution metrics Option 1, it is important that clear target UE performance improvement of MMSE-IRC over MMSE can be shown under MU-MIMO scenario, because in our understanding, MMSE receiver should be the baseline in our evaluation. The metric that target UE performance degradation from SU-MIMO to MU-MIMO does not show the overall network performance improvement under MU-MIMO, and it is hard to tell how much degradation is acceptable. |
| Huawei, HiSilicon | Sub-topic 3-1: Inter-user interference modeling for phase I evaluation Issue 3-1-1: Paired UE number We prefer option 3, which is 1 target UE + 1 interference UE. With more number of interfering UEs exist, combined interference will tend to be white noise. Issue 3-1-2: Rank for target and interference PDSCH We support the recommended WF Layer combination for target UE and interference UE can be: 1+1 for 2Rx, 2+2 for 4Rx Cross polarization is usually used by the base station, and each paired UE will be equally treated in real network. Thus, we prefer to select same number of layer for paired UEs in testing. Issue 3-1-3: Correlation between the propagation channel of the paired UEs Support the recommended WF. Do not add too much correlation on propagation channel so that the correlation of total equivalent channel sustain normal level. Because in real network scenario, if two users has high correlation, then they will not be paired by network. Issue 3-1-4: Antenna configuration Option 3 is fine for us. In our contribution, 2Tx are proposed for rank 1 configuration. We can also accept 4Tx for rank 2 configurations. Issue 3-1-5: Codebook type We think in phase I, type I single panel and type II codebook both can be evaluated before any down selection. Issue 3-1-6: PMI selection and precoding matrix generation We support option 1C. Random selection is easy to be implemented in testing and the equivalent channel correlation between paired UEs can be ensured to a low level by configuring channel and antenna correlation to low. Why we don't prefer feedback based precoding method: In real network scenario, feedback based precoding method refers to SRS measurement on uplink channel or PMI reporting, each of them will bring measuring errors into the calculation of precoder because of time delay or other factors. These errors will impact the observation of MMSE-IRC receiver performance. Why we don’t prefer ZF, QRD or other BS precoding schemes: Here we intend to measure the performance of UE receiver of MMSE-IRC by defining some requirements. Based on the ‘control variates’ method, we are supposed to reduce the factors that might have impact on demodulation performance as much as possible. To purely test the MMSE-IRC performance under some objective conditions, we need to remove network’s behaviour. Besides, performing ZF and QRD in testing will bring much more complexity to the test environment building for TE. We need to see if they are positive on performing any BS precoding schemes. Issue 3-1-7: PRB bundling size and precoding granularity We prefer to consider 4 or 2 PRB for bundling size and precoding granularity. Wideband is not preferred since it cannot simulate the real network scenario for MU-MIMO. Issue 3-1-8: MCS for interfering PDSCH Sub-topic 3-2: DMRS configuration for phase I evaluation Issue 3-2-1: DMRS ports for target and interfering UEs For rank 1, we are open to consider target UE and interference UE to be either in the same CDM group or not. If they are in the same CDM group, then the interference PRG can be determined. The corresponding DMRS configuration is : - DMRS port 0 for target UE, DMRS port 1 for the interference UE - DMRS port 0 for target UE, DMRS port 2 for the interference UE , which can be further down select after evaluation. For rank 2, we prefer to consider target UE and interference UE to be in the different CDM group. The corresponding DMRS port configuration is: DMRS port 0, 1 for target UE, DMRS port 2, 3 for the interference UE Issue 3-2-2: DMRS type and DMRS additional position We support the recommended WF. Issue 3-2-3: Ratio of PDSCH EPRE to DM-RS EPRE The Ratio of PDSCH EPRE to DMRS EPRE can be further determined after we have agreements on the configuration of CDM group. Issue 3-2-4: Whether to use the same DMRS pattern and the same sequence for all co-scheduled UEs We support option 1. Paired UEs should using the same DMRS Type and DMRS additional position. Sub-topic 3-3: Reference receiver for phase I evaluation Issue 3-3-1: Candidate Receivers We support option 1. MMSE-IRC processing with serving signal demodulation is the baseline implementation. Besides, if we consider random precoding method for interference UE, we don't have the real channel information from it. Whether the interference (fake) UE is able to give feedback of its channel information needs to be confirmed by TE. Issue 3-3-2: Interference estimation for cases with 2 DMRS CDM groups We are ok with option 1. Issue 3-3-3: Interference estimation granularity We prefer to consider the same granularity as that of precoding. Issue 3-3-4: Whether to introduce network assistance to assist the receiver RAN4 can discuss whether to introduce the network assistance. Here are some expected pro’s and con’s: Pro’s: Such network assistance information usually contains information of co-scheduled UEs like resource allocation, modulation schemes or channel coding schemes, etc. With these parameters and configurations informed, the tested UE can perform interference cancellation with less cost by detection. Con’s: The potential impact on the other WG or spec. needs to be taken into account. Sub-topic 3-4: PDSCH parameters for phase I evaluation Issue 3-4-1: SCS OK with recommended WF. Issue 3-4-2: Channel Bandwidth We support to consider the maximum bandwidth for FDD and TDD, which is 50MHz for FDD and 100MHz for TDD, since under full bandwidth, it is more obvious to see the benefit by using MMSE-IRC. Issue 3-4-3: TDD Configuration OK with option 1. Issue 3-4-4: MIMO correlation for each UE We prefer to consider low MIMO correlation for each UE. For example: XP low, or ULA low. Issue 3-4-5: Propagation Condition We support option 3. When transmitted signal is experiencing large time delay and frequency selective propagation condition, the precoding performance will have an obviously loss compared to flat channel, which is matched with real network scenario. Issue 3-4-6: MCS for Target UE Option 3 is fine for us. Prefer to further down select from MCS 4, 13 and 19 in phase I. Issue 3-4-7: PDSCH Mapping Type OK with recommended WF. Issue 3-4-8: PDSCH Resource Allocation OK with recommended WF. Issue 3-4-9: HARQ Process Number OK with recommended WF. Issue 3-4-10: SSB Configuration OK with recommended WF. Issue 3-4-11: TRS, NZP CSI-RS and ZP CSI-RS Configuration OK with TRS and ZP configuration of option 1. For NZP, only extend the configuration if more than 4 Tx ports are introduced. Issue 3-4-12: Performance evolution metrics OK with option 1. |
| Ericsson | Sub-topic 3-1: Inter-user interference modeling for phase I evaluation Issue 3-1-1: Paired UE number Option 3 for the initial evaluation. It also depends on the outcome of Issue 3-1-4. Issue 3-1-2: Rank for target and interference PDSCH We support the recommended WF. It is also important to make sure co-scheduled UE(s) use different DMRS CDM group from the UE under test. Issue 3-1-3: Correlation between the propagation channel of the paired UEs Support the recommended WF. Issue 3-1-4: Antenna configuration For the evaluation, we prefer the simple setup, i.e., Option 3 as a start point, but we’re open with option 1. The most important thing is whether we show the performance gain with Rel-15 MMSE-IRC receiver compared with non-co-scheduled UE(s) scenario. Issue 3-1-5: Codebook type Option 2. It depends on discussion on issue 3-1-4. Issue 3-1-6: PMI selection and precoding matrix generation Option 1C. Any NW implementation assumption shall be precluded in UE Demod test, since it could be a risk of different test results depending on the test equipment. We suggest not to have any precoding algorithm assumption in the UE demodulation requirements. Issue 3-1-7: PRB bundling size and precoding granularity It is our understanding it is the PRB bunding size for both UE under test and co-scheduled UE(s). We prefer to configure 2 PRBs, same assumption as the existing UE demodulation requirements. Issue 3-1-8: MCS for interfering PDSCH Option 2. Sub-topic 3-2: DMRS configuration for phase I evaluation Issue 3-2-1: DMRS ports for target and interfering UEs It depends on the conclusion of number of so-scheduled UE(s) and their rank(s), but it is also important to make sure co-scheduled UE(s) use different DMRS CDM group from the UE under test. Issue 3-2-2: DMRS type and DMRS additional position Support the recommended WF. Issue 3-2-3: Ratio of PDSCH EPRE to DM-RS EPRE This configuration should follow RAN1 spec. Issue 3-2-4: Whether to use the same DMRS pattern and the same sequence for all co-scheduled UEs To Hauwei, we are not sure if we understand the proposal correctly. Does this option propose the DMRS sequence generation specified in TS38.211 7.4.1.1.1? If so, it depends on the parameters like scramblingID or cell ID. Huawei: Correct. What we are proposing is to configure - Same DMRS type - Same DMRS additional position - Same scrambling ID - Same cell ID (of course, since they are inter-cell paired UEs) for paired UEs in defining requirement. Sub-topic 3-3: Reference receiver for phase I evaluation Issue 3-3-1: Candidate Receivers It is up to UE implementation, as far as it does not require any UE capability. But we prefer to assume option 1 to define the minimum requirements. Issue 3-3-2: Interference estimation for cases with 2 DMRS CDM groups It is up to UE implementation, but option 1 is fine. Issue 3-3-3: Interference estimation granularity It is up to UE implementation, but option 1 is fine. Issue 3-3-4: Whether to introduce network assistance to assist the receiver We prefer to define the requirements based on Rel-15 MMSE-IRC receiver with the DMRS-based interference covariance estimation. We don’t want to consider the network assisted signalling. Sub-topic 3-4: PDSCH parameters for phase I evaluation Issue 3-4-1: SCS Support the recommended WF. Issue 3-4-2: Channel Bandwidth Option 2. Same comment as Issue 2-4-2, In our understanding, we set 10MHz for FDD SCS=15kHz and 40MHz for TDD SCS=30kHz, considering the supported bands. For example, we are wondering how many FDD bands support CBW of 50MHz. Issue 3-4-3: TDD Configuration Option 1. We would like to prioritize 7D1S2U. But DDDSUDDSUU is also fine, depending on the number of test cases. Issue 3-4-4: MIMO correlation for each UE Options 1 and 2 are fine for evaluation. But we should not preclude low correlation also. But should we discuss it together with Issue 3-1-3? Issue 3-4-5: Propagation Condition Support the recommended WF. Issue 3-4-6: MCS for Target UE Option 3. Issue 3-4-7: PDSCH Mapping Type Support the recommended WF. Issue 3-4-8: PDSCH Resource Allocation Support the recommended WF. Issue 3-4-9: HARQ Process Number Support the recommended WF. Issue 3-4-10: SSB Configuration Support the recommended WF. Issue 3-4-11: TRS, NZP CSI-RS and ZP CSI-RS Configuration For 2Tx/4Tx scenarios, we are fine to reuse Rel-15 assumption for TRS/ZP-CSI-RS/NZP-CSI-RS configurations. Issue 3-4-12: Performance evolution metrics Since Rel-15 NR baseline receiver is MMSE-IRC, we are wondering it is reasonable to study the gain of MMSE-IRC over MMSE. Probably we should compare the performance difference between the case with co-scheduled UE and without co-scheduled UE? |
| Qualcomm | Sub-topic 3-1: Inter-user interference modeling for phase I evaluation Issue 3-1-1: Paired UE number Prefer Option 3. As per our understanding of RAN1 spec, there is no restriction on having same scrambling id for both CDM groups. UEs in different CDM groups can have different scrambling id. Therefore, in practice, target UE can assume same scrambling id only for the UEs in its own CDM group. Hence, we can’t have more than one paired UE. We encourage companies to double check this with their RAN1 colleagues. Issue 3-1-2: Rank for target and interference PDSCH Prefer rank1 for both target and co-scheduled UE based on comment in Issue 3-1-1. Issue 3-1-3: Correlation between the propagation channel of the paired UEs Ok with recommended WF. Issue 3-1-4: Antenna configuration With only 2 DMRS ports and random precoding, 2Tx or 4Tx should be enough. For following PMI, higher number of Tx will be useful. So, it should be discussed together with Issue 3-1-6. Issue 3-1-5: Codebook type It is related to Issue 3-1-6. With random PMI, single panel Type I codebook should be sufficient. Issue 3-1-6: PMI selection and precoding matrix generation We prefer to choose random PMI for target UE and then choose PMI matrix for co-scheduled UE from single panel type I codebook such that it is orthogonal. We think our proposal is similar to Option 1B, but we don’t support any feedback based PMI since that will mix the objective of the fixed MCS test with CSI reporting and it will be hard to tell what caused the UE to fail the test. Issue 3-1-7: PRB bundling size and precoding granularity Ok with Option 1 or Option 2. Issue 3-1-8: MCS for interfering PDSCH No preference. For joint decoding receiver, it has to be Option 1. Sub-topic 3-2: DMRS configuration for phase I evaluation Issue 3-2-1: DMRS ports for target and interfering UEs We prefer to consider only the 1st scenario with Option 1 as commented in Issue 3-1-1. Issue 3-2-2: DMRS type and DMRS additional position Ok with Option 1. Issue 3-2-3: Ratio of PDSCH EPRE to DM-RS EPRE This is true by RAN1 spec. Issue 3-2-4: Whether to use the same DMRS pattern and the same sequence for all co-scheduled UEs As per RAN1 spec, it can only be assumed for UEs within same CDM group. Spec allows for different scrambling id for different CDM groups. So, it’s not true for UEs in different CDM groups. Sub-topic 3-3: Reference receiver for phase I evaluation Issue 3-3-1: Candidate Receivers Prefer Option 1. Issue 3-3-2: Interference estimation for cases with 2 DMRS CDM groups We prefer not to consider this scenario as commented in Issue 3-1-1. Issue 3-3-3: Interference estimation granularity This is up to UE implementation, so RAN4 should not enforce any particular granularity. For simulation assumptions, RAN4 can assume granularity of PRB bundling size similar to other test cases. Issue 3-3-4: Whether to introduce network assistance to assist the receiver Resource allocation, modulation order etc. can keep changing very quickly if co-scheduled UE is moving. Can network assistance keep up with that? Sub-topic 3-4: PDSCH parameters for phase I evaluation Issue 3-4-1: SCS Ok with Option 1. Issue 3-4-2: Channel Bandwidth Prefer Option 2. Issue 3-4-3: TDD Configuration Ok with Option 1. Issue 3-4-4: MIMO correlation for each UE Ok with Option 1. Issue 3-4-5: Propagation Condition Prefer Option 1 since channel estimation is only based on PRB bundling size. So, frequency selectivity doesn’t matter that much for this test purpose. Issue 3-4-6: MCS for Target UE If we can only have Rank1 for target UE, it may not be practical to go beyond 16QAM. Issue 3-4-7: PDSCH Mapping Type Ok with Option 1. Issue 3-4-8: PDSCH Resource Allocation Ok with recommended WF. Issue 3-4-9: HARQ Process Number Ok with recommended WF. Issue 3-4-10: SSB Configuration Ok with recommended WF. Issue 3-4-11: TRS, NZP CSI-RS and ZP CSI-RS Configuration Ok with Option 1. Issue 3-4-12: Performance evolution metrics Ok with Option 1. |
| Intel | Issue 3-1-1: Paired UE number Based on our understanding, testing of scenario with 1 serving UE and 1 interference UE will be sufficient from coverage point of view. Also, such scenario can be handled by 2 Rx and 4 Rx UEs. Same time, handling of interference from higher number of UEs can be applied only by 4 Rx UE in case we assume MMSE-IRC processing. Therefore, we support Option 3. Issue 3-1-2: Rank for target and interference PDSCH We think that requirements should be defined at least for Rank 1 per UE, because it allows to define requirements for 2 Rx and 4 Rx UEs. Same time, 4 Rx UE is required to handle higher rank conditions with MMSE-IRC processing. Issue 3-1-3: Correlation between the propagation channel of the paired UEs Option 1 is fine for us. Issue 3-1-4: Antenna configuration As for Tx configuration, based on our understanding, using of configuration with high number of Tx antennas allows to achieve better spatial orthogonality for multiplexing of multiple UEs and is more practical assumption. Same time, using one or another configuration depends on agreement on PMI modelling. Using of random PMI modelling for scenarios with high Tx antennas will lead to very poor performance. Therefore, if follow PMI approach will be considered then we are fine to use 4, 8 or 16 Tx configuration. For random PMI approach it is better to use 2 or 4 Tx configuration. Issue 3-1-5: Codebook type To avoid connection with optional features, we suggest to consider Type I SP codebooks. Issue 3-1-6: PMI selection and precoding matrix generation Clarification for our options 1B and 2B. For example, for Rank 1 case, first we select precoder for serving UE (random or feedback based) WS then we select precoder for interference to ensure that WSH*WI=0. In result, based on our understanding, we will get precoder corresponding to Rank 2 case (first layer is serving and second is interference). Probably we can check the performance for different case to understand more suitable configuration for requirements definition. Issue 3-1-7: PRB bundling size and precoding granularity We can consider 2 PRB bundling size and, depending PMI selection approach, 2 PRB (for small Tx number) or wideband (for large Tx number) precoding granularity. Issue 3-1-8: MCS for interfering PDSCH Based on our understanding, both options will be the same from performance point of view. Same time, option 2 is simpler in terms of generation process during the test. Therefore, we support Option 2. Issue 3-2-1: DMRS ports for target and interfering UEs As for scenario with rank 1 transmission per UE, with 1 target UE and 1 interference UE, probably, it is better to make down selection. We prefer Option 2. For other scenarios, Option 1 is fine for us. Issue 3-2-3: Ratio of PDSCH EPRE to DM-RS EPRE Support Option 1. Based on our understanding, it is the only possible configuration for NR system. Issue 3-2-4: Whether to use the same DMRS pattern and the same sequence for all co-scheduled UEs Support Option 1. Issue 3-3-1: Candidate Receivers We can check performance difference for both receiver algorithms to understand whether Option 2 can bring significant improvement. In case significant improvement is observed we can discuss whether to define dedicated requirements for such receiver. Issue 3-3-2: Interference estimation for cases with 2 DMRS CDM groups Support Option 1. Based on analysis from our paper, if UE uses only DMRS from one CDM group then we will observe significant performance degradation. Issue 3-3-3: Interference estimation granularity Based on our understanding, we can consider per PRB bundling estimation granularity (i.e. at least 2 PRBs) for MU-MIMO case. Because we assume that, PDSCH allocation for co-scheduled UEs will be aligned and precoder granularity also potentially will be aligned. Issue 3-3-4: Whether to introduce network assistance to assist the receiver Need more time to think about this. Issue 3-4-2: Channel Bandwidth Based on our understanding, single CBW per SCS is sufficient from test coverage point of view. Receive processing will be the same for different CBWs. Performance for different CBS is already verified in Rel-15 and Rel-16. Therefore, we support Option 2. Issue 3-4-3: TDD Configuration Testing of one typical UL/DL configuration is sufficient. Therefore, we support Option 1. Issue 3-4-4: MIMO correlation for each UE It depends on Tx antenna configuration assumption. ULA Low can be considered for 2 and 4 Tx cases. XP High or XP Medium can be considered for 8 and 16 Tx cases. Issue 3-4-5: Propagation Condition Option 2 is fine for us. Issue 3-4-6: MCS for Target UE There is a typo in our proposal for QPSK MCS. Support Option 3. As for down selection, based on our analysis in this meeting, for scenario with MCS 4, we can not verify whether UE uses MMSE-MRC or MMSE-IRC receiver for demodulation. Therefore, as potential down selection, we can consider only MCS 13 and MCS 19. Issue 3-4-11: TRS, NZP CSI-RS and ZP CSI-RS Configuration Option 1 is fine for us. Issue 3-4-12: Performance evolution metrics Option 1 is fine for us. |
| Keysight Technologies | Sub-topic 3-1: Inter-user interference modeling for phase I evaluation Issue 3-1-6: PMI selection and precoding matrix generation From TE feasibility point of view: - Option 1A: more complex and does not 100% solves the issue. It’s feasible, but less predictable, with problems when the QRD is not guaranteeing orthogonality. And by far, the most complex to implement from TE point of view. - Option 1B: It seems a better approach than option 1A. It’s much simpler and easier to guarantee orthogonality in all cases. - Option 1C: It’s the easiest way of doing it from TE point of view. This is how it was done in LTE - Option 2A: Same comment as 1A. If the random precoder is too similar to the PMI reported one, then orthogonality is hard to get. This is basically as complex as option 1A with the addition of adding PMI reports and derive the PMI from the reports. - Option 2B: Same as 1B with the complexity of adding the PMI reports. - Option 3: This is the easiest possibility. Hence from TE point of view, the preferences are in this order 3, 1C, 1B, 2B |
| Apple | Sub-topic 3-1: Inter-user interference modeling for phase I evaluation Issue 3-1-1: Paired UE number We support option 3. Only 1 paired UE along with target UE. With 2 RX, we can only have max 2 layers transmission. Issue 3-1-2: Rank for target and interference PDSCH For 2RX we can only have 1+1 combination. With 4x4, 2+2 shows performance degradation, as shown in our paper. We should limit such scenarios or at least further study the different combinations. Issue 3-1-3: Correlation between the propagation channel of the paired UEs We support the recommended WF. Issue 3-1-4: Antenna configuration We think 2, 4TX should be sufficient for performance evaluation. Also related to PMI selection issue. We shouldn’t use large number of TX with random PMI and the performance will be degraded. Issue 3-1-5: Codebook type We support to use SP Type I codebook. Type II precoder requires additional UE capability and also random precoder generation for Type II cannot be well aligned between companies based on the discussions in Rel-16 Demod Enhancements and eMIMO WIs. Issue 3-1-6: PMI selection and precoding matrix generation We support to use random PMI and also prefer to avoid combining requirements for PDSCH demod and CSI feedback. We would like to understand feasibility of QRD orthogonalization from TE vendors. Based on our results and analysis, it provides better interference suppression compared to ZF precoding. It would be impractical not to employ any nulling or interference suppression techniques at gNB side for MU transmission. When the rank of target and paired UE are the same we can pick a pair of precoders randomly and they would be orthogonal, but not when the rank are different. Issue 3-1-7: PRB bundling size and precoding granularity We can go with option 1 as baseline assumption. Issue 3-1-8: MCS for interfering PDSCH The difference between options is unclear – normal vs random PDSCH. Okay with 16QAM as baseline. Sub-topic 3-2: DMRS configuration for phase I evaluation Issue 3-2-1: DMRS ports for target and interfering UEs We propose to limit to 1 paired UE. For all combinations the target and paired UE should be on different CDM grps. Issue 3-2-2: DMRS type and DMRS additional position We support the recommended WF. Issue 3-2-3: Ratio of PDSCH EPRE to DM-RS EPRE We support option 1. This need not explicity specified in our understanding. Issue 3-2-4: Whether to use the same DMRS pattern and the same sequence for all co-scheduled UEs We’re not sure what same DMRS pattern and sequence means. We should have the same DMRS type and position, but different scr ID can be used in our understanding. Sub-topic 3-3: Reference receiver for phase I evaluation Issue 3-3-1: Candidate Receivers We prefer option 1. We would like to understand the motivation to consider joint signal demodulation. Would we specify requirements for both? Issue 3-3-2: Interference estimation for cases with 2 DMRS CDM groups We think this is UE implementation specific and cannot be specified as simulation assumption. Issue 3-3-3: Interference estimation granularity We think this is UE implementation specific and cannot be specified as simulation assumption. Issue 3-3-4: Whether to introduce network assistance to assist the receiver Network assistance could be very useful for MU interference mitigation. Considering that paired UE might not always be on the reserved CDM group. We can further discuss details of such assistance information. Sub-topic 3-4: PDSCH parameters for phase I evaluation Issue 3-4-1: SCS We support the recommended WF. Issue 3-4-2: Channel Bandwidth We support option 2 and don’t see the need to introduce different CBW. Issue 3-4-3: TDD Configuration We support option 1. Issue 3-4-4: MIMO correlation for each UE We would like to understand why only medium and high correlation are considered and not Low correlation. For 2x2 with 1 layer per UE, high correlation would largely degrade performance. Issue 3-4-5: Propagation Condition We support option 1 as baseline. Issue 3-4-6: MCS for Target UE For initial evaluation we are okay with option 3. But we prefer to limit 64QAM to 4x4 only. Issue 3-4-7: PDSCH Mapping Type We support the recommended WF. Issue 3-4-8: PDSCH Resource Allocation We support the recommended WF. Issue 3-4-9: HARQ Process Number We support the recommended WF. Issue 3-4-10: SSB Configuration We support the recommended WF. Issue 3-4-11: TRS, NZP CSI-RS and ZP CSI-RS Configuration We support the recommended WF. Issue 3-4-12: Performance evolution metrics Okay with performance metric as SNR @70% max TP. Not sure if we should also evaluate against MMSE receiver. Perhaps performance against SU-MIMO to understand degradation introduced due to co-scheduled UE would be a better criteria. We should further downselect scenarios/ parameters that results in less degradation compared to SU-MIMO to achieve better system performance. |
| Anritsu | Issue 3-1-6: PMI selection and precoding matrix generation Unfortunately we came to a conclusion that we need more time to study the feasibility of this MU-MIMO test by our equipment. There are some fundamental questions with this test such as: - Does this interference test need to be carried out by conducted method, or OTA, or both? If it needs to be carried out by the conducted method, then we need to consider a way to evaluate interference between multiple carriers under a condition that each (or all) carrier is delivered to UE via cables. To see influence between carriers, we imagine that we perhaps need to create all the DL 8 or 16 carriers by either of the precoding methods (random or feedback-based), combine them all, and again split them to transmit to UE antenna connectors. (Sorry this is just my image and there might be more smart way to achieve it….) If the test need to be carried out by OTA, then we need to think about the number and placement of probes in the test chamber - What is the hardware configurations to achieve this test by our system? - This depends on the outcome above. |
| Rohde & Schwarz | Issue 3-1-6: PMI selection and precoding matrix generation Some of the options seem to be very complex when it comes to TE implementation. We are currently reviewing the options internally, but cannot make a final judgement right now, which options may be feasible to implement or not. Therefor we would suggest to not conclude on this issue right now and have more time to perform a more in-depth analysis. |
| MediaTek | Sub-topic 3-1: Inter-user interference modeling for phase I evaluation Issue 3-1-1: Paired UE number Prefer Option 3. Issue 3-1-2: Rank for target and interference PDSCH Support the recommended WF. Issue 3-1-3: Correlation between the propagation channel of the paired UEs Support the recommended WF. Sub-topic 3-2: DMRS configuration for phase I evaluation Issue 3-2-1: DMRS ports for target and interfering UEs For the scenario with rank 1 transmission per UE, with 1 target UE and 1 interference UE, we supprot Option 2. |
| CMCC | Sub-topic 3-1: Inter-user interference modeling for phase I evaluation Issue 3-1-1: Paired UE number We prefer Option1 can take Option2 as a compromise considering of simulation efforts. Issue 3-1-2: Rank for target and interference PDSCH Support the recommended WF Issue 3-1-3: Correlation between the propagation channel of the paired UEs Support the recommended WF Issue 3-1-4: Antenna configuration It is related to Issue 3-3-1 and Issue3-3-6. For Tx antenna number, if there are 3 paired UEs and support Rank 2, or we use following PMI, the number of transmit antenna should be larger than 8. However, if we only consider 1 paired UE and random PMI finally, we also support Option2 and Option3. For Rx antenna number, agree option 1. Issue 3-1-5: Codebook type This issue is based on issue 3-1-4; If we consider 8Tx or 16Tx, then we can take Option1 as the starting point If we consider 2Tx and 4Tx, then we prefer to use TypeI codebook. Issue 3-1-6: PMI selection and precoding matrix generation Option 1 is fine for us. The specific PMI selecting methodology can be further discussed. Issue 3-1-7: PRB bundling size and precoding granularity This issue is based on issue 3-1-4; If we consider 8Tx or 16Tx, then we can take Option3 as the starting point If we consider 2Tx and 4Tx, then we prefer to use Option1. Issue 3-1-8: MCS for interfering PDSCH Option2 is also OK for us that reuse LTE methodology. Sub-topic 3-2: DMRS configuration for phase I evaluation Issue 3-2-2: DMRS type and DMRS additional position Support Opion1. Sub-topic 3-3: Reference receiver for phase I evaluation Issue 3-3-1: Candidate Receivers Prefer option 1. Issue 3-3-2: Interference estimation for cases with 2 DMRS CDM groups Option1 makes sense, however, it may decide by UE implementation. Issue 3-3-3: Interference estimation granularity It may decide by UE implementation. Issue 3-3-4: Whether to introduce network assistance to assist the receiver Need further discussion Sub-topic 3-4: PDSCH parameters for phase I evaluation Issue 3-4-1: SCS Option1 Issue 3-4-2: Channel Bandwidth We support Option1 to consider both typical configuration and maximum bandwidth configuration. Issue 3-4-3: TDD Configuration Both Option1 and Option2 is OK for us. Issue 3-4-4: MIMO correlation for each UE Option1 Issue 3-4-5: Propagation Condition Support recommended WF Issue 3-4-6: MCS for Target UE Support Option1 Issue 3-4-7: PDSCH Mapping Type Option 1 Issue 3-4-8: PDSCH Resource Allocation Support the recommended WF Issue 3-4-9: HARQ Process Number Support the recommended WF Issue 3-4-10: SSB Configuration Support the recommended WF Issue 3-4-11: TRS, NZP CSI-RS and ZP CSI-RS Configuration Option1 can be the starting point. Issue 3-4-12: Performance evolution metrics Support Option1 |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #3 | Issue 3-1-1: Paired UE number - Proposals - Option 1: Consider 1, 2, 3 paired UEs. (CMCC) - Option 2: Consider both 1 and 3 paired UEs, make down-selections later based on the simulation outcome (CTC, CMCC) - CTC: More co-scheduled UE number should be considered for the phase I evaluation because it is a very typical scenario for MU-MIMO usage. - Option 3: Only 1 paired UE (HW, E///, QC, Intel, Apple, MTK) - QC: in practice, target UE can assume same scrambling id only for the UEs in its own CDM group. Hence, we can’t have more than one paired UE. - Apple: With 2 RX, we can only have max 2 layers transmission. - Recommendations for 2nd round: - Keep open to the next meeting. Issue 3-1-2: Rank for target and interference PDSCH - Proposals - Option 1: At least cover rank 1 (CMCC, Intel) - Option 2: Rank 1 or rank 2 for target UE, and the rank for interference UE is same with that for target UE (Intel) - Option 3: Cover both rank 1 and rank 2 per UE (CTC, E///, Huawei, [Intel], MTK, CMCC) - Option 3A: Rank 1+1 for 2Rx, rank 2+2 for 4Rx (Huawei, Apple) - Option 4: Limit the rank combinations to [1,1], [1,2], [2,1] on target and co-scheduled UE for 4x4 (Apple) - Option 5: Rank 1 only for target UE and interference UE (QC) - Option 6: Rank 1+1 for 2Rx, further study 2+2 with 4Rx (Apple) - Recommendations for 2nd round: - Can we use option 3A for initial simulation evaluation and see if down-selection is needed based on simulation results? Issue 3-1-3: Correlation between the propagation channel of the paired UEs - Tentative agreements: - Ensure Low correlation between the co-scheduled UEs. (CTC, HW, E///, QC, Intel, Apple, MTK, CMCC) Issue 3-1-4: Antenna configuration - Proposals - Tx antenna number: - Option 1: 8Tx and 16Tx (CMCC, CTC, [E///], [Intel]) - Option 2: 2Tx or 4Tx (Intel, [CMCC]) - Option 3: 2Tx for rank 1 and 4Tx for rank 2 (E///, HW, [CMCC]) - Option 4: 2Tx only (HW) - Option 5: 2 or 4 Tx if random PMI is used, and 4, 8 or 16 Tx if follow PMI is used (Intel) - Option 6: 2Tx and 4Tx (Apple) - Rx antenna number: - Option 1: Cover both 2Rx and 4Rx (CMCC, CTC, Intel, E///, HW) - Recommendations for 2nd round: - For Tx antenna number, combined with Issue 3-1-6, include both of the following configurations for the initial simulation evaluation, and make down-selection based on simulation results: - 2Tx and 4 Tx with random PMI for the target UE; - 8Tx and 16 Tx with following PMI for the target UE; - For Rx antenna number, cover both 2Rx and 4Rx (CMCC, CTC, Intel, E///, HW) -----------------GTW discussion ------------ Apple: For 2Tx and 4Tx cases with random PMI as starting point. With following PMI, we want to avoid to enable PMI feedback for demodulation requirements. Huawei: Similar view as Apple, starting with 2Tx and 4Tx over random PMI as starting point. QC: similar view as Huawei.. China Telecomm: For MU-MIMO, larger Tx ports with following PMI is more reasonable. Samsung: Similar view as Huawei and Apple. Intel: Fine with 2Tx and 4Tx as starting points. For MU-MIMO, larger Tx points should be considered. Agreement: Using 2Tx and 4Tx with random PMI for target UE as starting point for initial simulation Other options not excluded - Interested companies can bring analysis with 8Tx and 16Tx cases with following PMI for target UE. Issue 3-1-5: Codebook type - Proposals - Option 1: (CTC, CMCC) - For 8Tx, use type I SP codebook with wideband PMI reporting for target and paired UEs. - For 16Tx, use Rel-15/16 (e)type II codebook for target and paired UEs. Use subband PMI if it is feasible for TE to calculate ZF precoding matrix per subband. - Option 2: For 2Tx and 4Tx, use Type I SP codebook. Type II precoder can also be applied for 4Tx (E///) - Option 3: Cover both Type I SP and Type II for phase I (CTC, HW) - Option 4: Type I SP only with random PMI (QC) - Option 5: Type I SP only (Intel, Apple) - Option 6: Type I codebook for 2Tx and 4Tx (CMCC) - Recommendations for 2nd round: - Discuss after antenna configuration and PMI generation are agreed. Issue 3-1-6: PMI selection and precoding matrix generation - Proposals - Option 1: Random based target UE PMI selection (Apple, Intel, E///, HW, QC, CMCC) - Option 1A: Random selection based precoder generation with QRD orthogonalization processing as below (Apple) - Option 1B: Random PMI selection for the target UE, and select the precoder for the interference UE to ensure orthogonality (Intel, QC) - Option 1C: Random PMI selection for both target and interference UE, with ensuring the selected PMI matrix shall not be identical to the precoding matrix applied for the UE under test. (E///, HW) - Option 2: Feedback-based target UE PMI selection (CTC, Intel) - Option 2A: If the feasibility can be confirmed by the TE vendor, use ZF precoding based on the reported PMI from the target UE, and the randomly generated PMI from the interference UE(s). (CTC) - Option 2B: Feedback-based PMI selection for the target UE, select the precoder for the interference UE to ensure the orthogonality (Intel) - Option 2C: Feedback-based PMI selection for target UE, and random PMI selection for interference UE, with ensuring the selected PMI matrix shall not be identical to the precoding matrix applied for the UE under test (CTC) - Option 3: Fixed precoding matrix for one or both co-scheduled UEs (HW) - TE vendors’ feedback on the feasibility of the above options: - Keysight: 1A and 2A are of less feasibility. The preferences are in this order 3, 1C, 1B, 2B. - Anritsu, R&S: Need more time to study. - Recommendations for 2nd round: - Further discuss on the GTW session. Issue 3-1-7: PRB bundling size and precoding granularity - Proposals - Option 1: Per 2 PRBs for frequency domain and per slot for time domain (E///, CTC, HW, QC, Apple) - Option 2: 4 for both target and interference UEs (HW, CTC, QC) - Option 3: (CTC) - Wideband for 8Tx for target and paired UEs. - For 16Tx, use subband precoding if it is feasible for TE to calculate ZF precoding matrix per subband. - Option 4: 2 PRB for smaller Tx and wideband for large Tx (Intel, [CMCC]) - Recommendations for 2nd round: - Keep open to the next meeting. Issue 3-1-8: MCS for interfering PDSCH - Tentative agreements: - Random 16QAM signal generation (Intel, CTC, E///, Apple, CMCC) Issue 3-2-1: DMRS ports for target and interfering UEs - Proposals - For rank 1 transmission per UE, with 1 target UE and 1 interference UE: - Option 1: DMRS port 0 for target UE, DMRS port 1 for the interference UE (China Telecom, Intel, HW, QC) - Option 2: DMRS port 0 for target UE, DMRS port 2 for the interference UE, i.e., different CDM groups (Intel, Apple, E///, CTC, HW, MTK) - For rank 1 transmission per UE, with 1 target UE and more than 1 interference UE: - Option 1: DMRS port 0 for target UE, DMRS port i for the i-th interference UE (i = 1, 2,...) (China Telecom, Intel) - For rank 2 transmission per UE: - Option 1: DMRS port 0/1 for target UE, DMRS port 2i and 2i+1 for the i-th interference UE (i = 1, 2,...) (Intel, Apple, E///, CTC, HW) - When rank [1, 2] or rank [2, 1] transmission for the target and interference UEs: - Option 1: DMRS port 0 (and 1) for target UE, port 2 (and 3) for the interference UE, i.e., use different CDM groups for the target and interference UEs (Apple, E///, Intel) - Option 2: DMRS port 0 (and 1) for target UE, DMRS port i (i+1) for the i-th interference UE (i = 1, 2,...) (CTC) - Recommendations for 2nd round: - For rank 1 transmission per UE, with 1 target UE and 1 interference UE, can we use option 2 based on majority’s view? - For rank 2 transmission per UE, with 1 target UE and 1 interference UE, use option 1. - Keep open for the other scenarios pending decision on the rank and paired UE number. Issue 3-2-2: DMRS type and DMRS additional position - Tentative agreements: - DMRS Type 1 with 1 additional DMRS (Intel, CTC, E///, HW, QC, Apple, CMCC) Issue 3-2-3: Ratio of PDSCH EPRE to DM-RS EPRE - Proposals - Option 1: 0 dB and -3 dB when the number of DM-RS CDM groups without data is 1 and 2 respectively (CTC, [E///], QC, Intel, Apple) - Option 2: Further discuss (HW) - Recommendations for 2nd round: - Can we agree with option 1 since this is the only approach aligned with RAN1 spec. Issue 3-2-4: Whether to use the same following DMRS configuration for all co-scheduled UEs - Proposals - Option 1: Use the same following DMRS configuration for all co-scheduled UEs (HW, [CTC], Intel) - Same DMRS type - Same DMRS additional position - Same scrambling ID - Same cell ID - Option 2: Different scrambling id for different CDM groups (QC, Apple) - Recommendations for 2nd round: - More discussion in the second round. Issue 3-3-1: Candidate Receivers - Proposals - Option 1: MMSE-IRC processing with serving signal demodulation (Intel, CTC, HW, E///, QC, Apple, CMCC) - xS=HSHHSHSH+RI+N-1y. - Option 2: MMSE-IRC processing with joint (serving + interference) signal demodulation (Intel, [CTC]) - xS+I=HS+IHHS+IHS+IH+RN-1y, where HS+I=HSHI. - Recommendations for 2nd round: - Prioritize option 1 for initial simulation and other options are not precluded. Issue 3-3-2: Interference estimation for cases with 2 DMRS CDM groups - Proposals - Option 1: For cases with 2 DMRS CDM groups, the interference should be estimated based on the REs occupied by both of the two DMRS CDM groups (CTC, HW, E///, Intel, CMCC) - Option 2: Not to consider this scenario (QC) - Option 3: Up to UE implementation and cannot be specified as simulation assumption (Apple) - Recommendations for 2nd round: - Further discuss based on the agreement in Issue 3-2-1. Issue 3-3-3: Interference estimation granularity - Proposals - Option 1: Per PRB and per slot based interference covariance matrix estimation (CTC, E///) - Option 2: Same with the PRB bundling size (HW, [QC], Intel) - Option 3: Up to UE implementation (Apple, CMCC) - Recommendations for 2nd round: - Further discuss on the GTW session. Issue 3-3-4: Whether to introduce network assistance to assist the receiver - Proposals - Option 1: RAN4 discusses and decides whether to introduce network assistance (HW, Apple) - Option 2: Not to consider network assistance (E///) - Recommendations for 2nd round: - Further discuss in the second round. Issue 3-4-1: SCS - Tentative agreements - Cover both 15kHz SCS for FDD and 30kHz SCS for TDD. (CMCC, CTC, Intel, E///, HW, QC, Apple) Issue 3-4-2: Channel Bandwidth - Proposals - Option 1 (CMCC) - For FDD 15kHz SCS: Cover 10MHz and 50MHz CBW. - For TDD 30kHz SCS: Cover 40MHz and 100MHz CBW. - Option 2: 10 MHz for FDD 15kHz SCS and 40 MHz for TDD 30kHz SCS (Intel, E///, QC, Apple) - Option 3: (CTC) - For FDD 15kHz SCS: Cover 10MHz and 40MHz CBW. - For TDD 30kHz SCS: Cover 40MHz and 100MHz CBW. - Option 4: 50 MHz for FDD 15kHz SCS and 1000 MHz for TDD 30kHz SCS (HW) - Recommendations for 2nd round: - Keep open to the next meeting. Issue 3-4-3: TDD Configuration - Tentative agreements: - 7D1S2U(6D+4G+4U) for 30kHz TDD (CMCC, Intel, E///, CTC, HW, QC, Apple, CMCC) Issue 3-4-4: MIMO correlation for each UE - Proposals - Option 1: Consider both XP High and XP Medium, and make further down-selection based on evaluation results (CMCC, CTC, QC) - Option 2: XP High or XP Medium (CTC) - Option 3: XP Low or ULA Low (HW) - Option 4: Cover High, Medium and Low correlation for evaluation (E///, [Apple]) - Option 5: ULA Low can be considered for 2 and 4 Tx cases. XP High or XP Medium can be considered for 8 and 16 Tx cases. (Intel) - Recommendations for 2nd round: - Can we use option 1 to reduce the simulation work? Issue 3-4-5: Propagation Condition - Proposals - Option 1: TDLA30-10 as baseline (CMCC, Intel, E///, QC, Apple) - Option 2: Cover both TDLA30-10 and TDLC300-100 in phase I, and decide whether down-selection or adjustment is needed later (CTC, CMCC, E///, Intel) - Option 3: TDLC300 channel (HW) - Recommendations for 2nd round: - Further discuss on the GTW session. Issue 3-4-6: MCS for Target UE - Proposals - Option 1: (CMCC) - QPSK MCS 4, 16QAM MCS 13 and 64QAM MCS 19 for Rank 1 - 64QAM MCS 19 for Rank 2 - Option 2: MCS 5 or 12 or 19 (Intel) - Option 3: Cover QPSK MCS 4, 16QAM MCS 13, and 64QAM MCS 19 for initial simulation (CTC, E///, HW, Intel, Apple) - Option 4: Up to 16QAM for 2x2 and up to 64QAM for 4x4 (Apple) - Option 5: Lower than 64QAM for rank 1 (QC) - Recommendations for 2nd round: - Can we use option 3 for initial simulation, and see if down-selection is needed based on simulation results? Issue 3-4-7: PDSCH Mapping Type - Tentative agreements: - Type A (Intel, CTC, E///, HW, Apple, CMCC) Issue 3-4-8: PDSCH Resource Allocation - Tentative agreements: (CTC, HW, E///, QC, Apple, CMCC) - Time Domain: - Starting symbol 2 and duration 12 as baseline - Frequency Domain: - Full PRB allocation as baseline Issue 3-4-9: HARQ Process Number - Tentative agreements - 4 for FDD 15kHz SCS and 8 for TDD 30kHz SCS (CMCC, CTC, HW, E///, QC, Apple, CMCC) Issue 3-4-10: SSB Configuration - Tentative agreements (CMCC, CTC, HW, E///, QC, Apple) - SSB position in burst: first SSB in Slot#0; SSB periodicity: 20ms. - The slot #0 in every 20 ms is not scheduled for PDSCH transmission Issue 3-4-11: TRS, NZP CSI-RS and ZP CSI-RS Configuration - Tentative agreements - For TRS and ZP CSI-RS, reuse the Rel-15 assumptions for PDSCH demodulation requirement. (CTC, HW, E///, QC, Intel, Apple, CMCC) - For NZP CSI-RS, - Reuse the Rel-15 general assumptions for PDSCH demodulation requirement if 2Tx or 4Tx is used. (CTC, HW, E///, QC, Intel, Apple, CMCC) - The configuration should be extended if 8Tx or 16Tx is used. (CTC, HW, QC, Intel, Apple, CMCC) Issue 3-4-12: Performance evolution metrics - Proposals - Option 1: Measure the 70% max throughput performance of the target UE (CMCC, CTC, E///, HW, QC, Intel, Apple) - Option 1A: Evaluate the gain of MMSE-IRC over MMSE under the same simulation setup (CTC, HW, QC, Intel, CMCC) - Option 1B: Performance difference between the case with and without co-scheduled UE (E///, Apple) - Recommendations for 2nd round: - Further discuss in the GTW session. --------------GTW discussion---------- Agreement: Option 1A as baseline, Interested companies also encouraged to bring analysis for option 1B. |

Discussion on 2nd round

R4-2106118	WF on MMSE-IRC receiver for intra-cell inter-user interference

Type: other		For: Approval
					Source: Huawei

Abstract:

Decision:		Return to.

Discussion:

Huawei: In the WF,

- All tentative agreements(either made by the moderator or by the session chair in the GTW) have been marked as green.

- Some suggested WF has been given and marked as yellow

- All the other issues that left no highlight color are encouraged to be discussed in the second round

Companies please check and to see:

- if each page of WF is correctly reflect the discussion and tentative agreements in the summary

- if there is anything else missed

- if each yellow part is acceptable

And finally, companies are highly encouraged to give their comments toward all the open issues.

Issue 3-1-1: Paired UE number

Huawei: We prefer option 3, to first consider the scenario of 1 target UE + 1 interference UE, considering the limit time for simulations before the May meeting.

CTC: Based on the decision we made on yesterday’s GTW session towards antenna configuration, we still prefer option 2 because options for 8Tx and 16Tx are not precluded (it is typical that there will be more co-scheduled UEs with larger Tx number).

So we prefer to just keep the options here for the next meeting, and no additional decisions in the WF. BTW, we think our first round agreement on covering both 2Rx and 4Rx is missing in page 4.

Ericsson: It depends on the Tx antenna configurations. For 2Tx/4Tx, we are fine with option 3 (Only 1 paired UE).

Intel: We think that from simulation and testing point of view Option 3 should be sufficient, because receive processing will be the same for different scenarios and can be verified.

Qualcomm: We prefer Option 3.

Apple: We prefer option 3. We cannot have more than 1 paired user for 2x2.

CMCC: We prefer Option2.

Issue 3-1-2: Rank for target and interference PDSCH

Huawei: We prefer option 3, to cover both rank 1 and rank 2. If we only have rank 1, then 1 Rx is enough, there is no need to consider 4Rx.

CTC: Support option 3.

Ericsson: We support Option 3 (Cover both rank 1 and rank 2 per UE).

Intel: Support Option 3.

Qualcomm: We support Option 5. RAN1 spec doesn’t mandate the scrambling id for both CDM groups to be same. So if target UE has ports on only 1 CDM group, it has no way of knowing whether other CDM group has same scrambling id or not. Without knowing this, UE can’t do anything about other CDM group. Even if RAN4 makes assumption of same scrambling id on both CDM groups, UEs have to be designed based on RAN1 spec.

Also, 38.211 Section 7.4.1.1.2 states that UE can only assume QCL for ports within the same CDM group. So, UE can’t assume same delay, Doppler etc for other CDM group. Below is a snippet from 38.211:

Unless specified otherwise, the UE may assume that the PDSCH DM-RS within the same CDM group are quasi co-located with respect to Doppler shift, Doppler spread, average delay, delay spread, and spatial Rx (when applicable). The UE may assume that DMRS ports associated with a TCI state as described in clause 5.1.6.2 of [6, TS 38.214] of a PDSCH are QCL with QCL Type A, Type D (when applicable) and average gain.

Therefore, we propose to evaluate only Rank1 so that both target UE and co-scheduled UE can be on the same CDM group. To proponents of Rank2, please suggest how we can cover Rank2 with no restriction on scrambling id and no QCL info in RAN1.

If RAN4 really wants to cover Rank2 for target UE, we should consider 2 front-loaded symbols where each CDM group can have up to 4 ports. In that case, we can have Rank 2 on both target and co-scheduled UE while keeping all of them within same CDM group.

Apple: For 2x2 Rank [1,1] with target and co-scheduled UE on different CDM grp - port 0 and port 2. For 4x4, With [1,2] ports {0,[2,3]}; [2,1] ports {[0,1],2}; [2,2] ports {[0,1],[2,3]}. We don’t understand Qualcomm’s comment on both target and co-schduled UE have to be in same CDM grp. The gNB can transmit to 2 UEs on different CDM gaps. RAN1 has no restriction on that. The UE is only configured 1 CDM grp.

CMCC: We support Option3.

Huawei: We agree with Apple that there is no restriction on having co-scheduled UE in same CDM group. Besides, we are discussing requirements for MMSE-IRC, we don’t think target UE needs to know the QCL of other UEs. Does Qualcomm mean to consider joint demodulation with estimating interference UE’s channel?

Ericsson2: Regarding the comments by Qualcomm, we want to check with our RAN1 colleagues. Meanwhile, can we keep open for the ranks of target and interference PDSCH? Probably we can add the following sentence in the WF:

-  Companies to check further the possibility to schedule interfering UE(s) on the different CDM group.

Apple: We have added note on slide #3 in this version:

•Option 2A: [1+1], [2+2] for target UE and interference UE

Option 2B: [1+1], [2+1] for target UE and interference UE

Option 2C: [1+1], [1+2] for target UE and interference UE

Note: Rank 2 only for 4RX case

Issue 3-1-5: Codebook type

Huawei: Option 2 and 3 are our first preference. But we can decide this after antenna configuration and PMI generation are agreed.

CTC: Prefer to keep all options here for this meeting. And we are ok with using option 3 for initial simulation.

Ericsson: It depends on the Tx antenna configurations. For 2Tx/4Tx, we are fine with options 2/3.

Intel: We think that Type I SP should be considered for these requirements (Option 5), because this is mandatory feature and we think that it is rather important to ensure correct MU-MIMO processing for all UEs.

Qualcomm: We prefer Option 4 since it was already agreed to start with Random PMI. With random PMI, we don’t see how Type I vs Type II random precoding will make a difference.

Apple: SP Type I codebook only with random PMI.

CMCC: Option2 is also Ok for us. If we are the only proponent for Option6, it can be removed since the Option6 is already included in Option2.

Ericsson2: For 2 antenna port case, we are fine to use type-I, because type-II does not supports 2 antenna ports. For 4 antenna ports case, even with the random PMI, the precoding matrix is different between type-I and type-II. We would like to evaluate the performance.

CTC: For the codebook type, we proposed to cover both Type I and Type II PMI for initial simulation evaluation because Type II is mainly designed for MU-MIMO scenario.

In the second round, we preferred to keep all options for this meeting and option 3 can be prioritized for initial simulation. And it seems we are not the only proponent of covering both Type I and Type II PMI.

However, if you would like to narrow down the number of options, I think at least an option 2 : ‘Cover both Type I SP and Type II for phase I’ should be added.

Huawei: For the agreed scenario of 2Tx and 4Tx with random PMI, we think  there is no difference with Type I SP and Type II. But anyway we are open for considering Type II codebook. Please check the following wording:

- Codebook Type

  - Option 1: Type I Single Panel only with random PMI

  - Option 2: Cover Type I Single Panel and Type II codebook for phase I

    - Option 2A:  For 2Tx and 4Tx, use Type I SP codebook. Type II precoder can also be applied for 4Tx

CTC: Currently we think Type II codebook should at least be evaluated in this MU-MIMO scenario. And options for 8T 16T have not been excluded.

Anyway, thank you very much for addressing our concern, the wording is fine for us.

Intel: In case wording from HW below will be captured in WF, we suggest to remove “with random PMI” for option 1 to avoid using of SP Type I for follow PMI case.

Issue 3-1-6: PMI selection and precoding matrix generation

Huawei: We support option 1C.

ZF or other network precoding schemes bring more testing complexity for TE and the procedure is now unclear for TE so that they need further analysis which is time consuming.

Besides, network precoding scheme will significantly affect the performance, so that people can’t tell the exact receiver performance, which is violating the ‘control variate’ principal.

Same time, we think there is no need to ensure a perfectly orthogonality. There is no perfect orthogonality in real network scenario, otherwise there will be no or at least reduce the meaning of MMSE-IRC receiver.

In real network scenario, MMSE-IRC receiver is expected to deal with the residual interference after the precoding of base station. While, if we create a PMI for interference UE only for ensuring it to be perfectly orthogonal to the precoder of target UE, then there will be no inter-user interference, especially if we are assuming low correlation and flat propagation condition.

CTC: Based on TE vendor’s feedback, option 1B/1C/2B/2C can all be tested so we prefer to keep the 4 options here for the first meeting. And currently we still prefer option 1B/2B with orthogonal precoding which is more practical.

Ericsson: Option 1C, but we suggest to wait for the TE vendors’ input regarding the feasibility of options.

R&S: I have updated Rohde & Schwarz views on “Inter-user interference modeling for phase I evaluation” in the WF, however more checking on some options is still needed.

Intel: One additional option for random case, we can randomly select precoder from codebooks corresponding to number of MIMO layers equal to total number of MU MIMO layers (i.e. serving Rank + interference Rank) and take several columns from this precoder for serving UE signal and remaining columns for interference UE signal. At current stage, we think that we can keep all options and interested companies can check performance difference and whether IRC processing can be verified.

Qualcomm: Prefer Option 1B. In practice, NW will try to choose precoders for each UE orthogonal to each other. At the same time, we don’t want to increase the TE complexity in implementing ZF or QRD procedure. So, TE can randomly choose PMI matrix from, for example, single panel type I codebook and then pick another PMI matrix orthogonal to target UE from the same codebook. This will ensure orthogonality while keeping TE complexity low.

Apple: Our preference is 1A and prefer to keep it open. We are also fine with option 1B.

CMCC: We prefer Option1C. Option1B is aslo acceptable.

Intel: During 2nd round discussion, we’ve suggest additional option for random case. Could we include it in the WF or, at least, capture that other options are not precluded?

Ericsson2: For 2Tx case (rank1 for target UE and rank1 for interfering UE), we are fine with 1B, random PMI selection for the target UE from Type-I codebook, and select the precoder for the interference UE from Type-I codebook to ensure orthogonality, e.g., if target UE use [1; 1]/sqrt(2), set [1; -1]/sqrt(2) for interfering UE.

For 4Tx case, we need more time to check, but our preference is to choose precoders of both target UE and interfering UE(s) from type-I or type-II codebook.

Issue 3-1-7: PRB bundling size and precoding granularity

Huawei: We prefer option 2.

CTC: We are ok with either option 1 and option 2 (per slot precoding granularity in time domain should be added in option 2).

Ericsson: Option 1 (2 for both target and interference UEs)

Intel: Support Option 1 as more typical configuration for PDSCH tests.

Qualcomm: Prefer Option 1 as 2PRBs is default value.

Apple: Option 1; Same as PDSCH demod requirements assumption.

Issue 3-2-1: DMRS ports for target and interfering UEs

Huawei: We are ok with different CDM groups for target UE and interference UE and the corresponding DMRS port configurations.

CTC: Firstly, options for DMRS port assignment for more than 1 paired UE is missing in page 8.

Secondly, we still prefer option 2 for rank [1, 2] or rank [2, 1] transmission scenario, which is also missing in page 8. As we commented in the first round, option 1 is not a fair port assignment for the interference UE.

Ericsson: We support DMRS port 0 for target UE, DMRS port 2 for the interference UE, i.e., different CDM groups.

Clarification for ‘rank’ in Page 8. Does this rank means the rank for UE under test? If so, ‘With rank [1,2] or rank [2,1] transmission for the target UE and interference UE’ should be ‘With rank [2,2] or rank [2,1] transmission for the target UE and interference UE’?

Anyway, we prefer to use different CDM groups for the target and interference UEs.

Intel: We are fine to keep only scenario with different CDM groups for Rank 1 case.

Qualcomm: As explained in Issue 3-1-2, we need to have co-scheduled UE in the same CDM group as target UE. So, we can only have Rank1 transmission with DMRS port 0 for target UE and port 1 for interference UE. We encourage companies to please check RAN1 spec and suggest how we can have rank2 transmission with 1 front-loaded symbol without assuming same scrambling id and QCL info. We can have Rank2 on target UE if we assume 2 front loaded symbols as that can have 4 ports per CDM group. So, we don’t agree with anything under rank2 transmission on slide 8 and slide 9. We prefer 1st option for rank1 transmission on slide 8.

Apple: For 2x2 Rank [1,1] with target and co-scheduled UE on different CDM grp - port 0 and port 2. For 4x4, With [1,2] ports {0,[2,3]}; [2,1] ports {[0,1],2. Also we should configure target UE with 2 CDM gaps

We don’t understand Qualcomm’s comment on both target and co-schduled UE have to be in same CDM grp. The gNB can transmit to 2 UEs on different CDM grps. RAN1 has no restriction on that. The UE is only configured 1 CDM grp.

Ericsson2: Same comment as Issue 3-1-2.

Issue 3-2-4: Whether to use the same following DMRS configuration for all co-scheduled UEs

Huawei: From our point of view, using same scrambling ID for paired UEs is usually configured in the real network. And if the traffic becomes heavier, which implies more UEs got paired, then different scrambling ID might be considered. Thus, based on the real network scenario configurations, we suggest to use same scrambling ID for defining requirements albeit there is no restriction on that for UEs in different CDM groups.

Ericsson: We want to keep open.

Intel: Based on our understanding at least we should consider scenario with Same DMRS type and Same DMRS additional position to ensure reliable channel estimation in MU-MIMO scenarios. As for scrambling ID, based on our understanding it should not have big impact of performance. Therefore, we can consider same or different.

Qualcomm: To clarify our position, we don’t want to define tests with different scrambling ids. At the same time, UE can only assume same scrambling id for its own CDM group. So, we prefer to define tests for only those scenarios where target UE and co-scheduled UE are in the same CDM group.

Apple: We need to further check.

Huawei: Based on our understanding, there is no restriction on having same(different) scrambling IDs for co-scheduled UE either they are in the same CDM group or not. For simulation and results alignment purpose, we propose to use same scrambling ID for co-scheduled UEs.

Issue 3-3-2: Interference estimation for cases with 2 DMRS CDM groups

Huawei: OK with option 1.

CTC: Support option 1 as simulation assumption, which is reasonable to ensure the MMSE-IRC performance under this scenario.

Ericsson: Up to UE implementation.

Intel: Based on our analysis for this meeting, we think that if UE does not use assumption from Option 1 then it will lead to significant performance degradation. Same time, we are fine not to specify the details of Rx processing.

Qualcomm: As explained in Issue 3-1-2, we encourage companies to provide views on how UE will decode other CDM group if it cant assume the same scrambling id and QCL info. Therefore, we prefer Option 2.

Apple: We prefer option 3, up to UE implementation.  To Qualcomm, why does UE have to decode other CMD grp? Are we considering joint estimation?

Issue 3-3-3: Interference estimation granularity

Huawei: Support option 2.

CTC: Prefer to keep the options here to the next meeting.

Ericsson: Up to UE implementation.

Intel: We can reuse agreement for inter-cell interference.

Qualcomm: Prefer Option 2 or Option 3.

Apple: We prefer option 3, up to UE implementation.

Issue 3-3-4: Whether to introduce network assistance to assist the receiver

Huawei: Need more analysis and can be decided further.

Ericsson: We can discuss it further, but we don’t think it is necessary.

Intel: Can be further discussed.

Qualcomm: We are open to discuss it. Can Huawei clarify how network can keep up with providing latest information on modulation order, allocation etc. for each co-scheduled UE to each UE? Is there any other information that Huawei is think about in network assistance?

CMCC: We prefer not introduce the network assistance. Maybe proponent could provide more clairfication about the necessity about network assistance.

Apple: We prefer to keep this open and discuss further.

Issue 3-4-2: Channel Bandwidth

Huawei: We prefer option 4, to consider the maximum bandwidth for FDD and TDD, which is 50MHz for FDD and 100MHz for TDD, since under full bandwidth, it is more obvious to see the benefit by using MMSE-IRC. But since many companies are in favor of small CBW, we signaled readiness to accept option 1 to at least consider both 10MHz and 50MHz CBW for 15kHz SCS, and 40MHz and 100MHz CBW for 30kHz SCS.

CTC: We still prefer option 3.

Ericsson: Option 2. It is important to cover almost all FR1 RF bands.

Intel: Based on our understanding, RX processing and MMSE-IRC performance benefits will be the same for different CBWs. Therefore, we think that one CBW per SCS is sufficient. Same time, we are fine to check several CBW for initial simulation purpose (i.e. similar to inter-cell interference assumptions).

Qualcomm: Prefer Option 2. Same as existing test cases.

Apple: We prefer option 2. Same as existing test cases.

CMCC: We prefer Option1 and Option3 is also acceptable for us.

Intel:

we have comment related to slide 12. Could you explicitly add potential option for receiver

- Candidate Receiver

  - Prioritize MMSE-IRC processing with serving signal demodulation for initial simulation

    - xS=HSHHSHSH+RI+N-1y

  - Other options are not precluded

    - Option 1: MMSE-IRC processing with joint (serving + interference) signal demodulation

      - xS+I=HS+IHHS+IHS+IH+RN-1y, where HS+I=HSHI.

MIMO correlation for each UE: Could we also add ULA low? If 2 Tx case will be considered then ULA Low is typical correlation model.

Qualcomm: Additional comments below:

- On slide 16, MIMO correlation is not clear. Is it XP High and XP Medium? With low correlation, we may not see much benefit for MU-MIMO because it will be harder to have orthogonal channel between UEs.

- Propagation condition and MCS: We will prefer to have a down-selection based on simulation results rather than deciding later. Otherwise, there will be too many test cases.

Apple: Additional comments below.

Slide #6: Option 3 is missing

Slide #16:

MIMO correlation: Only consider low correlation. Medium and high will not give good performance with co-scheduled UE in MU-MIMO

Propagation condition: We already have many options to consider for other parameters. We prefer to only consider TDLA30-10. Some companies are suggesting follow PMI, for that high Doppler will not work anyway.

MCS: Rank1: QPSK, 16QAM; Rank 2: 16QAM, 64QAM

- Recommendations for Tdocs

  - 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC | Intel |  |
| WF on CQI reporting requirements for inter-cell interference MMSE-IRC | Ericsson |  |
| WF on MMSE-IRC receiver for intra-cell inter-user interference | Huawei |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2104951 | Work plan for Further enhancement on NR demodulation performance WI | China Telecom | Revised |  |
| R4-2104952 | TR skeleton (V0.0.1) for Inter-user interference suppression for NR Multiple-User Multiple-Input Multiple-Output (MU-MIMO) | China Telecom | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

  - 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2106116 | WF on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC | Intel | Agreeable Add following note in the chairman note: For information: Interested company have provided example system level simulation assumptions in R4-2106426. |  |
| R4-2106117 | WF on CQI reporting requirements for inter-cell interference MMSE-IRC | Ericsson | Agreeable |  |
| R4-2106118 | WF on MMSE-IRC receiver for intra-cell inter-user interference | Huawei | Agreeable |  |
| R4-2106119 | Work plan for Further enhancement on NR demodulation performance WI | China Telecom | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents