3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2106340

Online Meeting, Apr 12 – 20, 2021

Agenda item:	5.5.2.2.2.3

Source: 	Qualcomm Incorporated

Title: 	On UE Rx-Tx measurement accuracy requirements

Document for:	Discussion

# Introduction

UE Rx-Tx measurement accuracy requirements were further discussed in RAN4#98-e and agreements and open issues were captured in a WF [1]. In this paper, we discuss the following remaining questions:

- Applicability of accuracy requirements in case of NTA_offset change

- Applicability of accuracy requirements under TA adjustment

- Applicability of accuracy requirements in case of non-HO serving cell change

- Group delay calibration margin

# Applicability of accuracy requirements in case of NTA_offset change

This issue has been discussed in the last three RAN4 meetings with little progress. The WF from RAN4#98-e lists the following options [1]:

- Option 1: Clarify in section 10.1.25.2 in TS 38.133: “UE Rx-Tx time difference accuracy requirements shall not apply if NTA_offset defined in Table 7.1.2-2 in 38.133 changes during the UE Rx-Tx measurement period.”

- Option 2: Capture in the specification that UE Rx-Tx accuracy requirements do not apply in case the UE UL timing changes during the measurement period

We prefer to address the various scenarios corresponding to different types of timing changes separately. We support option 1.

Proposal 1: Clarify in section 10.1.25.2 in TS 38.133: “UE Rx-Tx time difference accuracy requirements shall not apply if NTA_offset defined in Table 7.1.2-2 in 38.133 changes during the UE Rx-Tx measurement period.”

# Applicability of accuracy requirements under TA adjustment

Regarding this question, the following options were listed in the WF [1]:

- Option 1. UE Rx-Tx measurement accuracy requirements shall not apply if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment or based on network-configured TA

- Option 2:

  - UE Rx-Tx measurement accuracy requirements shall not apply if the uplink transmission timing changes during the UE Rx-Tx measurement period due to network-configured TA command.

  - UE Rx-Tx measurement accuracy requirements shall apply if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment

- Option 3: Capture in the specification that UE Rx-Tx accuracy requirements do not apply in case the UE UL timing changes during the measurement period

There is already a prior agreement in RAN4 [3] related to this question: “UE Rx-Tx time difference accuracy requirements do not apply under TA change during the measurement period.” In our view this agreement applies to TA command from the network. Measurement accuracy requirements should be applicable under autonomous timing adjustments.

Proposal 2a: UE Rx-Tx measurement accuracy requirements shall not apply if the uplink transmission timing changes during the UE Rx-Tx measurement period due to network-configured TA command.

Proposal 2b: UE Rx-Tx measurement accuracy requirements shall apply if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment.

# Applicability of accuracy requirements in case of non-HO serving cell change

In RAN4#98-e, companies started discussing the question of applicability of UE Rx-Tx measurement accuracy requirements when a cell change other than HO occurs during the measurement period. One option was captured in the WF for consideration [1]:

- Option 1: The UE shall continue and complete a UE Rx-Tx measurement while meeting UE Rx-Tx measurement accuracy requirements in clause 10.1.23, when a serving cell change (including SCell change, addition, release, activation, or deactivation, or PSCell change, addition, or release) occurs during the measurement, provided the cell change does not impact the configuration of the SRS used for the measurement

The question above is also being considered in the discussion of UE Rx-Tx measurement (period) requirements. Given that these questions are tightly coupled, RAN4 may decide first if the measurement requirements do apply and, if so, whether the accuracy requirements apply as well.

Proposal 3: UE Rx-Tx measurement accuracy requirements in the case of serving cell changes other than HO that do not impact the configuration of SRS for positioning are FFS.

# Group delay calibration margin

RAN4 decided to add a group delay calibration error margin to RSTD accuracy requirements [2]. In our view, RAN4 should do the same for UE Rx-Tx.

Proposal 4: RAN4 will add a non-zero group delay calibration margin to the UE Rx-Tx accuracy requirements in FR1 and FR2.

Based on our observations, we think the group delay calibration margin should scale inversely with PRS bandwidth. i.e. since the receiver front-end should be configured to the desired BW during calibration, the calibration accuracy itself will be limited by the signal bandwidth.

Observation 1: The group delay calibration margin should scale inversely with PRS bandwidth.

Proposal 5: For UE Rx-Tx measurements in FR1, add a group delay calibration margin of 4 ns * (100/BW), where BW is the PRS bandwidth in MHz, to the accuracy requirements.

# Conclusions

Proposal 1: Clarify in section 10.1.25.2 in TS 38.133: “UE Rx-Tx time difference accuracy requirements shall not apply if NTA_offset defined in Table 7.1.2-2 in 38.133 changes during the UE Rx-Tx measurement period.”

Proposal 2a: UE Rx-Tx measurement accuracy requirements shall not apply if the uplink transmission timing changes during the UE Rx-Tx measurement period due to network-configured TA command.

Proposal 2b: UE Rx-Tx measurement accuracy requirements shall apply if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment.

Proposal 3: UE Rx-Tx measurement accuracy requirements in the case of serving cell changes other than HO that do not impact the configuration of SRS for positioning are FFS.

Proposal 4: RAN4 will add a non-zero group delay calibration margin to the UE Rx-Tx accuracy requirements in FR1 and FR2. FFS the exact values of the margins for FR1 and FR2.

Observation 1: The group delay calibration margin should scale inversely with PRS bandwidth.

Proposal 5: For UE Rx-Tx measurements in FR1, add a group delay calibration margin of 4 ns * (100/BW), where BW is the PRS bandwidth in MHz, to the accuracy requirements.

# References

[1] R4-2103586, WF on NR Positioning Performance Requirements

[2] R4-2017371, WF on NR Positioning Performance Requirements