3GPP TSG-RAN WG4 Meeting # 98-bis-e                                                        R4-2106380

# Electronic Meeting, Apr. 12-20, 2021

Source:	ZTE Corporation

Title:	TP for TR 37.717-11-21: DC_1A-3A-7A-20A_n38A

Agenda Item:	7.6.2

Document for:	Approval

- Introduction

This contribution provides a text proposal on DC_1A-3A-7A-20A_n38A  for TR37.717-11-21[1] .

- Reference

- TR 37.717-11-21  Dual Connectivity (DC) of x bands (x=1, 2, 3, 4) LTE inter-band CA (x DL/1 UL) and 2 bands NR Inter-band CA  (2 DL/1 UL) band combinations, v0.3.0

- Text Proposal

----- Start of TP -----

    - 5.x.2	DC_1-3-7-20_n38

      - 5.x.2.1	Configuration for EN-DC

Table 5.x.2.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-3A-7A-20A_n38A | 1Ax 3Ax 20Ax |
| NOTE x: UL carrier shall be supported in Band 1, Band 3 or band 20 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. |  |

      - 5.x.2.2	∆TIB and ∆RIB values

Table 5.x.2.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-3A-7A-20A_n38A | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 20 | 0.5 |

Table 5.x.2.2.-2: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1A-3A-7A-20A_n38A | 1 | 0 |
|  | 3 | 0 |
|  | 20 | 0 |

      - 5.x.2.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

----- End of TP -----