3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2106468

Online, Apr 12-20, 2021

Source: 	Qualcomm Incorporated

Title: 	Discussion on PDSCH Demodulation Requirements for 47 GHz band

Agenda item:	7.27.4.2

Document for:	Discussion

# Introduction

During the last RAN4 meeting companies agreed to verify whether the existing UE demodulation performance requirements can be extended to the 47GHz band according to the WID in [1].

This paper presents the simulation results based on the simulation assumptions in the WF [2].

# Simulation Setup and Results

The simulation results listed in this section compare demodulation performances for different carrier frequencies, to inform the ongoing discussion in RAN4 on whether the limit of applicability of the current UE demodulation requirements for FR2 (40GHz) can be extended to 48.2GHz, with the goal to include the 47 GHz band (n262).

The simulation assumptions and the tests presented here are in line with the agreements in the WF [1], and the performance is compared between carrier frequency 39GHz and 48GHz. The Phase Noise model used in the simulation is based on UE implementation.

All references in the test number and table columns refer to TS 38.101-4.

| Test Number | Table | Rank | MCS | Fc[GHz](Band) | Alignment SNR[dB] (@70% of Average Peak Throughput) |
| --- | --- | --- | --- | --- | --- |
| Test 1-3 | 7.2.2.2.1-3 | 1 | 18 | 39 (n259) | 9.25dB |
| “ | “ | “ | “ | 48 (n262) | 9.25dB |
| Test 2-6 | 7.2.2.2.1-4 | 2 | 17 | 39 (n259) | 15dB |
| “ | “ | “ | “ | 48 (n262) | 15dB |

Observation 1: No significant difference in Demodulation performance is seen when comparing results for 47GHz (band n262) with 39 GHz for tests 1-3 and 2-6 in 38.101-4.

Proposal 1: For UE Demodulation performance in FR2 up to 64QAM (MCS 18), the existing UE performance requirements in 38.101-4 can be extended up to 48.2GHz including the 47GHz band.

Proposal 2: If UE Demodulation performance in FR2 for 256QAM needs to be extended, a dedicated additional evaluation of the performances is required.

# Conclusions

Observation 1: No significant difference in Demodulation performance is seen when comparing results for 47GHz (band n262) with 39 GHz for tests 1-3 and 2-6 in 38.101-4.

Proposal 1: For UE Demodulation performance in FR2 up to 64QAM (MCS 18), the existing UE performance requirements in 38.101-4 can be extended up to 48.2GHz including the 47GHz band.

Proposal 2: If the range of applicability of UE Demodulation performance in FR2 for 256QAM needs to be extended, a dedicated additional evaluation of the performances is required.

# References

[1] RP-201232, “New WID on introduction of NR 47 GHz band”, 3GPP TSG-RAN Meeting # 95-e, Online, June 29 – July 3, 2020

[2] R4-2103875, “Way forward on UE/BS demodulation on NR 47GHz band”, 3GPP TSG-RAN4 Meeting #98-e	, Electronic Meeting, 25 January – 5 February 2021