3GPP TSG-RAN WG4 Meeting # 98-bis-e 	R4-2106489

# Electronic Meeting, Apr. 12-20, 2021

Source:	Huawei, HiSilicon, CMCC, China Unicom

Title:	RX EVM for support of 1024QAM

Agenda item:	8.15.3

Document for:	Approval

1 Introduction

In RAN#89 e-meeting, a new work item, i.e. introduction of DL 1024-QAM for NR FR1 was approved [1]. The objective is to specify downlink 1024-QAM for NR PDSCH operation in FR1, together with related procedures, signaling and necessary RF requirements. The RAN4 objectives are listed below.

- Specify corresponding UE and BS RF core requirements [RAN4]

  - UE and BS RF core requirements are specified for stationary wireless scenarios with up to 2 layer DL MIMO

  - The cell size(s) and type of stationary wireless scenarios for which UE and BS RF core requirements are defined will be studied and decided by RAN4.

In this contribution, we provide some discussion on UE RX EVM requirements for DL 1024-QAM.

2 Discussion

In RAN1 discussion, for link-level simulation, 0 and 3% was adopted for RX EVM, while 0 and 2% was adopted for TX EVM [1]. In LTE 1024-QAM discussion, RAN 4 agreed that 1.5% to 2% EVM could be assumed for RX EVM[2].

[RAN4]: RAN4 recommends that following RX EVM could be assumed in the evaluation:

- 2%-4% for typical UEs

- 1.5% to 2% for high end UEs

Support of such RX EVM values may result in the increased UE power consumption, chip size and cost. The related enhancements could be potentially applicable to at least very high capability devices listed examples in the WID.

For NR1024-QAM, the UE requirements are specified for stationary scenarios, hence we think 1.5% to 2% EVM can be reused for link-level simulation.

Proposal 1: 1.5% to 2% RX EVM could be assumed in the evaluation for link-level simulation.

3 Conclusion

In this contribution, we provide discussion on UE RX EVM for the support of 1024-QAM.

Proposal 1: 1.5% to 2% RX EVM could be assumed in the evaluation for link-level simulation.

Reference

- “RAN1 Chairman’s Notes”, RAN1#103-e, October 26th – November 13th, 2020.

- RP-202866, “Revised WID on Introduction of DL 1024-QAM for NR FR1”, Nokia, RAN#90e, December 7-11, 2020.

- R4-1706112, Reply LS regarding 1024QAM, RAN4

Appendix A

Table A.1. System-level evaluation assumptions

| Scenario | Urban Macro |
| --- | --- |
| Layout | 7 sites, 3 sector per site |
| Inter-BS distance | 500m |
| Carrier frequency | 4GHz |
| Simulation bandwidth | 100 MHz |
| Numerology | 30KHz |
| Duplex configuration | TDD: DDDSU |
| Channel model | 3D UMa |
| BS total Tx power | 49dBm |
| BS antenna configuration | (M,N,P,Mg,Ng) = (8,8,2,1,1) (dH,dV) = (0.5, 0.7)λ |
| BS antenna height | 25m |
| BS antenna element gain + connector loss | 6.4 dBi |
| UE Tx power | Maximum 23dBm |
| UE antenna configuration | 2Tx4Rx |
| UE antenna gain | 0dBi |
| UE antenna element gain pattern | Omnidirectional |
| UE receiver noise figure | 9dB |
| UE distribution | 10 UEs per sector; 80% indoor+20% outdoor; 100% outdoor |
| UE receiver | MMSE-IRC |
| Channel estimation | Real |
| Traffic model | 1) Full buffer; 2) FTP model with packet size 2.5Mbytes |
| MIMO scheme | SU-MIMO with rank adaptation and MU-MIMO |
| CSI scheme | Reciprocity based; 2T SRS |