3GPP TSG-RAN WG4 Meeting # 98bis-e                            R4-2106678

Electronic Meeting, 12– 20 April, 2021

Source: 	Huawei, HiSilicon

Title: 	Discussion on MSD due to cross band isolation and counter intermodulation

Agenda Item:	7.31.2.2

Document for:	Approval

Introduction

The WID [1] Introduction of bandwidth combination set 4 (BCS4) for NR was approved in RAN #90e meeting. The objectives related to MSD requirements are listed below:

- Ensure that all required analysis including MSD, etc. be performed for BCS4 for every existing band combination configuration (up to 3 bands)

It need to be studied further whether MPR/A-MPR is needed to be included in this WI.

In last RAN4 meeting, companies discussed this issue, but there were no conclusions on MSD due to cross-band isolation referring to the WF [2]. In this paper, we’d like to further discuss this issue.

Discussion

The definition for this two kinds of MSD

For MSD due to cross band isolation, it’s clear defined that Rx de-sensing in one band will be caused by another band due to lack of isolation in the band filters referring to 38.101-1-f50. It means that the mainly reason which may cause the MSD is the lack of isolation in the band filters. All of band combinations’ exceptions are specified based on this logic in current specifications. For example, band n78 interferes band n41 Rx due to the bad performance of band n78 filter in frequency range band n41. For CA_n3-n41, band n41 interferes band n3 Rx due to the bad performance of band n41 filter in frequency range band n3 Rx.

Observation 1: The definition about MSD due to cross band isolation is that Rx de-sensing in one band will be caused by another band due to lack of isolation in the band filters.

As some larger channel bandwidths were introduced into the specification, the Rx victim band may fall into the second adjacent channel or the first adjacent channel of aggressor band. That means the counter IM interferences may fall into Rx victim bands based the analysis [4] and [5]. Meanwhile, the frequency relation between aggressive band and victim band can be found and it will help network configure RB allocation efficiently to avoid the MSD due to counter inter-modulation.

Observation 2: For MSD due to counter intermodulation, the frequency relation between aggressive band and victim band can be found and it is very helpful for network to configure RB allocation efficiently in order to avoid the MSD due to counter inter-modulation.

The discussion on the Highest CBW/Lcrb specified for the aggressor band

In last meeting, companies provided some general guideline for cross-band isolation MSD and UL configuration specifications [3]. The Highest CBW/Lcrb specified for the aggressor band was proposed for MSD due to cross band isolation. In my opinion, there are some disadvantages for this configuration,

1) In RAN4, UL RB limitation is allowed for all the definition of REFSENS, so it may not be suitable to specify the Highest CBW/Lcrb specified for the aggressor band.

2) In current specification, some of exceptions such as CA_n41-n66 and CA_n41-n78 are not specified based on the highest CBW/Lcrb. If RAN4 adopt this general principle, all the exceptions need to be reviewed.

3) The network can’t get a guideline on how to achieve better performance using RB/UL BW configuration.

Observation 3: there are some disadvantages for the Highest CBW/Lcrb specified for the aggressor band as below.

1) In RAN4, UL RB limitation is allowed for all the definition of REFSENS, so it may not be suitable to always specify the Highest CBW/Lcrb specified for the aggressor band.

2) In current specification, some of exceptions such as CA_n41-n66 and CA_n41-n78 are not specified based on the highest CBW/Lcrb. If RAN4 adopt this general principle, all the exceptions need to be reviewed.

3) The network can’t get a guideline on how to achieve better performance using RB/UL BW configuration.

For SCS, SCS should be the smallest SCS that is compatible with the UL CBW.

For Carrier Frequency, the UL and DL carrier frequencies should be configured to minimize the gap separating the DL victim carrier to the UL carrier frequency.

For DL configuration, the fully allocated DL configuration can be used for DL victim band. However, we can specify the MSD requirements using BW-agnostic method.

Proposal 1: For DL configuration, the fully allocated DL configuration can be used for DL victim band using BW-agnostic method. SCS should be the smallest SCS that is compatible with the UL CBW. The UL and DL carrier frequencies should be configured to minimize the gap separating the DL victim carrier to the UL carrier frequency.

Discussion on MSD due to counter inter-modulation

In last meeting, companies argued that introducing a new kind of MSD may cause the test workload. However, there aren’t too many band combinations which need the MSD due to counter inter-modulation interference. However, it can help network schedule UE efficiently using the specific frequency relations. It’s very important to distinguish whether Rx victim band fall into 1st/2nd adjacent channel or not when RAN4 specify the MSD.

Observation 4: Since there aren’t too many band combinations which need the MSD due to CIM interference, it will not cause much test efforts.

Observation 5: It’s very important to distinguish whether Rx victim band fall into 1st/2nd adjacent channel or not when RAN4 specify the MSD.

Proposal 2: It’s proposed to introduce MSD due to CIM interference for inter-band CA.

Table 1: Reference sensitivity exceptions (MSD) due to counter intermodulation interference for CA

| UL band | DL band | SCS of UL band (kHz) | LCRB of UL band | Applicable UL BW(MHz) | MSD value of DL band (dB) | CIM order |
| --- | --- | --- | --- | --- | --- | --- |
| n1 | n3 | 15 | 25 | ≥ 25 | 4.5 | CIM5 |
| n1 | n3 | 15 | 25 | 50 | 17 | CIM3 |
| NOTE 1: For CIM5, the MSD exceptions are applicable to the case that CIM5 of UL band falls into the DL channels. (The frequency of CIM5 can be expressed as fCIM5=FC_UL-5(fUL-FC_UL), where FC_UL is the centre frequency of UL channel and fUL is the allocated transmission frequency of UL band). NOTE 2: For CIM3, the MSD exceptions are applicable to the case that CIM3 of UL band falls into the DL channels. (The frequency of CIM3 can be expressed as fCIM3=FC_UL-3(fUL-FC_UL), where FC_UL is the centre frequency of UL channel and fUL is the allocated transmission frequency of UL band). |  |  |  |  |  |  |

Conclusion

Based on the analysis and discussion, the proposal and observations are shown below.

Observation 1: The definition about MSD due to cross band isolation is that Rx de-sensing in one band will be caused by another band due to lack of isolation in the band filters.

Observation 2: For MSD due to counter intermodulation, the frequency relation between aggressive band and victim band can be found and it is very helpful for network to configure RB allocation efficiently in order to avoid the MSD due to counter inter-modulation.

Observation 3: there are some disadvantages for the Highest CBW/Lcrb specified for the aggressor band as below.

1) In RAN4, UL RB limitation is allowed for all the definition of REFSENS, so it may not be suitable to always specify the Highest CBW/Lcrb specified for the aggressor band.

2) In current specification, some of exceptions such as CA_n41-n66 and CA_n41-n78 are not specified based on the highest CBW/Lcrb. If RAN4 adopt this general principle, all the exceptions need to be reviewed.

3) The network can’t get a guideline on how to achieve better performance using RB/UL BW configuration.

Proposal 1: For MSD due to cross band isolation, the fully allocated DL configuration can be used for DL victim band using BW-agnostic method. SCS should be the smallest SCS that is compatible with the UL CBW. The UL and DL carrier frequencies should be configured to minimize the gap separating the DL victim carrier to the UL carrier frequency.

Observation 4: Since there aren’t too many band combinations which need the MSD due to CIM interference, it will not cause much test efforts.

Observation 5: It’s very important to distinguish whether Rx victim band fall into 1st/2nd adjacent channel or not when RAN4 specify the MSD.

Proposal 2: It’s proposed to introduce MSD due to CIM interference for inter-band CA.

Table 1: Reference sensitivity exceptions (MSD) due to counter intermodulation interference for CA

| UL band | DL band | SCS of UL band (kHz) | LCRB of UL band | Applicable UL BW(MHz) | MSD value of DL band (dB) | CIM order |
| --- | --- | --- | --- | --- | --- | --- |
| n1 | n3 | 15 | 25 | ≥ 25 | 4.5 | CIM5 |
| n1 | n3 | 15 | 25 | 50 | 17 | CIM3 |
| NOTE 1: For CIM5, the MSD exceptions are applicable to the case that CIM5 of UL band falls into the DL channels. (The frequency of CIM5 can be expressed as fCIM5=FC_UL-5(fUL-FC_UL), where FC_UL is the centre frequency of UL channel and fUL is the allocated transmission frequency of UL band). NOTE 2: For CIM3, the MSD exceptions are applicable to the case that CIM3 of UL band falls into the DL channels. (The frequency of CIM3 can be expressed as fCIM3=FC_UL-3(fUL-FC_UL), where FC_UL is the centre frequency of UL channel and fUL is the allocated transmission frequency of UL band). |  |  |  |  |  |  |

- References

[1]	RP-202832, “New WID: Introduction of bandwidth combination set 4 (BCS4) for NR”, Ericsson, T-Mobile USA

[2] R4-2103271, “WF on NR BCS4 RAN4 #98e”, Ericsson

[3] R4-2102928, “Cross-band MSD for ENDC and NR-CA BCS4”, Skyworks Solutions, Inc.

[4] R4-2015552, “Consideration on Cross band isolation impact with larger BW”, Huawei, HiSilicon

[5] R4-2009628, “ENDC crossband noise impact with large NR BW”, Qualcomm Incorporated