3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2106763

Electronic Meeting, 12 April – 20 April 2021

Source:	Ericsson, Telstra

Title:	TP to TR 37.717.11-21: Addition of DC configurations for DC_3-7_n3-n78

Agenda item:	9.7.2

Document for:	Approval

# Background

This text proposal for TR 37.717-11-21 to add DC_3A-7A_n3A-n78A and DC_3A-7C_n3A-n78A as defined in WID [1].

# Text Proposal

##### ---Start of changes---

## 7.x	DC_3-7_n3-n78

### 7.x.1	Operating bands for DC

Table 7.x.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7_n3-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.x.2	Channel bandwidths per operating band for DC

Table 7.x.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_3A-7A_n3A-n78A | DC_3A_n3A4 DC_7A_n3A DC_3A_n78A DC_7A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-7C_n3A-n78A | DC_3A_n3A4 DC_7A_n3A DC_7C_n3A DC_3A_n78A DC_7C_n78A DC_7A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 200 |
|  |  | 7 | See CA_7C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| NOTE 4: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.x.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.x.4	∆TIB and ∆RIB values

For DC_3-7_n3-n78, the TIB,c and RIB,c values are derived from CA_n3-n7-n78 and given in the tables below.

Table 7.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7_n3-n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |

Table 7.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7_n3-n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |

### 7.x.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

##### ---End of changes---

# Reference

[1]	RP-202292, Revised WID on DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) in Rel-17, LG Electronics