3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2106832

# Electronic Meeting, 12th - 20th Apr, 2021

Title: 	Discussion on NR UE demodulation for 35MHz and 45MHz bandwidth

Source: 	Huawei, HiSilicon

Agenda item:	7.25.6

Document for:	Discussion

Background

At the RAN#91-e meeting, the revised WID [1] was approved. In this contribution, we share our views about the demodulation requirements for NR UE 35MHz and 45MHz bandwidth.

Discussion

Here we analysis the impact on demodulation requirements for 35MHz and 45MHz bandwidth.

CA requirements

In Rel-16, PDSCH CA requirements are defined based on the single carrier requirements for different channel bandwidth combination. For new channel bandwidth 35MHz and 45MHz, it is reasonable to define the corresponding requirements.

Define PDSCH CA requirements for 35MHz and 45MHz bandwidth with the following assumption.

| Parameter | Value |
| --- | --- |
| MCS | 13 |
| Propagation condition | TDLA30-10 |
| Antenna configuration | 2x2 ULA Low, 2x4 ULA Low |
| Bandwidth | 35MHz, 45MHz |
| Test metric | 70% maximum throughput |
| Others | Same as Table 5.2A-1, 5.2A-3 and 5.2A-3 of TS 38.101-4 |

SDR test

For the SDR test, the tested bandwidth combination is selected to achieve the largest data rate calculated among all bandwidth combinations, so there are no extra requirements need to be defined. The only one thing to be modified is to add the corresponding bandwidth to the parameters configuration, i.e. Table 5.5A-4 of TS38.101-4.

Add support of 35MHz and 45MHz bandwidth for SDR tests in Table 5.5A-4 of TS 38.101-4.

Proposals

In this contribution, we discuss on demodulation performance for NR FR1 35MHz and 45MHz bandwidth. Our observations and proposals are:

- Define PDSCH CA requirements for 35MHz and 45MHz bandwidth with the following assumption.

| Parameter | Value |
| --- | --- |
| MCS | 13 |
| Propagation condition | TDLA30-10 |
| Antenna configuration | 2x2 ULA Low, 2x4 ULA Low |
| Bandwidth | 35MHz, 45MHz |
| Test metric | 70% maximum throughput |
| Others | Same as Table 5.2A-1 and 5.2A-2 of TS 38.101-4 |

Add support of 35MHz and 45MHz bandwidth for SDR tests.

Reference

RP-210722, Introduction of channel bandwidths 35MHz and 45MHz for NR FR1, RAN#91-e, Huawei, HiSilicon