3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2106949

Electronic Meeting, 12th – 20th April, 2021

Title: 	Link level simulation results for gNB TOA measurement

Source: 	Huawei, HiSilicon

Agenda item:	5.5.2.3.3

Document for:	Discussion

Introduction

RRM accuracy requirements for gNB Rx-Tx measurements have been discussed in RAN4#98-e, and the conclusions are captured in the WF [1]. Based on [1] the baseline Rx-Tx accuracy and the dependence on SRS parameters will be decided based on link level simulation results.

In this paper we will provide our simulation results on gNB TOA measurement performance.

Discussion

Simulation assumption

The simulation assumption is same as in [2].

- the SINR side conditions are +3dB and -13dB

- the number of samples is 1

Simulation results

In Tables 1-3, we show the TOA estimation accuracy for 2 different SNR level with different SCS, where SNR1=3dB and SNR2=-13dB. The results are for TOA error in Tc, which is derived as the 90% value on the CDF curve for absolute TOA error.

Table 1: Link level simulation results for TOA estimation with 15kHz (Unit: Tc)

| RB | Comb and Symbol | Channel |  |
| --- | --- | --- | --- |
|  |  | AWGN |  |
|  |  | SNR1 = 3dB | SNR2 = -13dB |
| 24 | 2 + 2 | 115 | 179 |
|  | 4 + 4 | 116 | 178 |
|  | 8 + 8 | 115 | 154 |
|  | 8 + 12 | 116 | 122 |
| 52 | 2 + 2 | 58 | 58 |
|  | 4 + 4 | 57 | 58 |
|  | 8 + 8 | 58 | 58 |
|  | 8 + 12 | 58 | 58 |
| 104 | 2 + 2 | 29 | 29 |
|  | 4 + 4 | 28 | 29 |
|  | 8 + 8 | 29 | 29 |
|  | 8 + 12 | 28 | 28 |
| 264 | 2 + 2 | 14 | 15 |
|  | 4 + 4 | 14 | 15 |
|  | 8 + 8 | 14 | 14 |
|  | 8 + 12 | 15 | 14 |

Table 2: Link level simulation results for TOA estimation with 30kHz (Unit: Tc)

| RB | Comb and Symbol | Channel |  |
| --- | --- | --- | --- |
|  |  | AWGN |  |
|  |  | SNR1 = 3dB | SNR2 = -13dB |
| 48 | 2 + 2 | 29 | 29 |
|  | 4 + 4 | 29 | 29 |
|  | 8 + 8 | 29 | 30 |
|  | 8 + 12 | 29 | 29 |
| 132 | 2 + 2 | 14 | 14 |
|  | 4 + 4 | 14 | 14 |
|  | 8 + 8 | 15 | 14 |
|  | 8 + 12 | 14 | 15 |
| 272 | 2 + 2 | 7 | 7 |
|  | 4 + 4 | 7 | 7 |
|  | 8 + 8 | 7 | 7 |
|  | 8 + 12 | 7 | 7 |

Table 3: Link level simulation results for TOA estimation with 120kHz (Unit: Tc)

| RB | Comb and Symbol | Channel |  |
| --- | --- | --- | --- |
|  |  | AWGN |  |
|  |  | SNR1 = 3dB | SNR2 = -13dB |
| 32 | 2 + 2 | 15 | 14 |
|  | 4 + 4 | 14 | 15 |
|  | 8 + 8 | 14 | 14 |
|  | 8 + 12 | 14 | 14 |
| 64 | 2 + 2 | 7 | 7 |
|  | 4 + 4 | 7 | 7 |
|  | 8 + 8 | 7 | 7 |
|  | 8 + 12 | 7 | 7 |
| 132 | 2 + 2 | 4 | 4 |
|  | 4 + 4 | 4 | 4 |
|  | 8 + 8 | 4 | 4 |
|  | 8 + 12 | 4 | 4 |

From the above tables, we can observe that

Observation 1: The performance is almost not dependent on SNR conditions.

For most of the cases the TOA estimation error is quite similar for +3dB and -13dB side condition. Only in cases of small PRB number, i.e. 24, there is some difference.

Observation 2: The performance is almost not dependent on comb and symbol size.

For most of the cases the TOA estimation error is quite similar for all combinations of comb+symbol. Only in cases of small PRB number, i.e. 24, there is some difference under -13dB SINR.

Observation 3: The accuracy improves in proportion with BW in Hz due to better resolution.

This observation is aligned with the expectation and also aligned with the observations from UE simulations.

Conclusions

In this paper we provided our simulation results for gNB Rx-Tx measurement performance.

Observation 1: The performance is almost not dependent on SNR conditions.

Observation 2: The performance is almost not dependent on comb and symbol size.

Observation 3: The accuracy improves in proportion with BW in Hz due to better resolution.

Reference

- R4-2103587, WF on gNB positioning measurement requirements, Ericsson

- R4-2012142, Link simulation assumptions for deriving positioning SRS configurations, Nokia