3GPP TSG-RAN WG4 Meeting # 98-bis-e 	R4-217089

# Electronic Meeting, Apr. 12-20, 2021

Agenda Item:	5.1.2.9

Source:	MediaTek Inc.

Title:	Discussion on UE Transmit Timing for NR-U

Document for:		Discussion

- Introduction

In this paper, our views are provided for the reference cell definition for UE transmit timing in NR-U, with DRX in use or measurement gap.

- Reference Cell Definition

The reference cell definition for the case with DRX, MGRP was discussed in Topic # 9 in the Moderator’s summary in last meeting [3] and the WF RAN4 97e [2] as below.

UE Transmit Timing

- Reference Cell Definition

  - In the requirements of clause 7.1.2, the term reference cell on a carrier frequency subject to CCA is not available at the UE refers to when at least one SSB is configured by gNB, but the first two successive candidate SSB positions for the same SSB index within the discovery burst transmission window are not available at the UE due to DL CCA failures at gNB during the last 160 ms; otherwise the reference cell on the carrier frequency subject to CCA is considered as available at the UE

- DRX case: FFS, unless this can also be solved in RAN4#97-e

- FFS: when there is no available serving SSB outside gap

In NR-U, the availability of serving SSB is not guaranteed, due to LBT. Thus, for the reference cell measurements without gap required, the serving SSB within gap will be not helpful for UE timing synchronization with the serving cell, because UE may not say on the serving carrier within gap. For this case, the availability of the reference cell should be based on “SSB outside gap”, as illustrated in Figure 1.

Figure 1. the reference cell measurements without gap required

Proposal 1: The availability of the reference NR-U cell should be based on “serving SSB outside gap”.

In case of DRX in use, the availability of the reference cell should be consistent to the approach in ILDE mode, i.e., the availability of a reference cell should be based on the DRX cycles with at least one SMTC/DRS window where there are no SSBs available.

Proposal 2: If DRX is configured, the availability of the reference NR-U cell is based on DRX cycles.

- Summary

In this paper, the reference cell definition for UE transmit timing in NR-U requirements have been discussed. We have the following proposals.

Proposal 1: The availability of the reference NR-U cell should be based on “serving SSB outside gap”.

Proposal 2: If DRX is configured, the availability of the reference NR-U cell is based on DRX cycles.

References

[1] 38.133, V16.6.0.

[2] R4-2017080, WF on NR-U RRM Core Requirements, Ericsson.

[3] R4-2103687, Email discussion summary for [98e][205] NR_unlic_RRM_1, Ericsson.