3GPP TSG-RAN WG4 #98-bis-e	R4-2107091

Electronic Meeting, April 12th – April 20th, 2021

Agenda item:	5.1.4.2

Source:	MediaTek Inc.

Title:	Discussion on PDSCH requirements for NR-U

Document for:	Discussion

- Introduction

In RAN4#98-e, UE demodulation requirements for NR-U were discussed and a way forward was agreed [1]. In this contribution, we provide our views on the remaining open issues.

- Requirement definition according to UE capability of supporting CSI-validation features - FFS: Option 1: One generic LBT modelling for all test cases - Option 1a: no applicable test cases for UE which does not support CSI-validation features - Option 1c: for UE which does not support CSI-validation feature set LBT failure probability as 0. Separate requirements will be introduced. - FFS: Option 2: Different test set-up/[performance requirements] for UE with different UE capability - Test set1: LBT modelling which applied for UE support CSI validation feature - Test set2: Test set-up for UE which does not support CSI-validation feature - FFS how to enable TRS always on considering LBT failure - Use a Random DL duration within the COT, with equal probability between: - Option 1: {2,3,5,6} Slots; - Option 1 is chosen as baseline assumption for the simulation results to be submitted to the next meeting. - Option 2: {2,4,6,7} Slots; - To be verified that Option 2 does not cause issues with Japan regulation; - Allocate PDSCH in the last slot in the Downlink portion of the COT in Symbols: - Option 1: {6,9,12,14} - Option 1 is chosen as baseline assumption for the simulation results to be submitted to the next meeting. - Option 2: {5-14} - Define the same probability of LBT Failure for Scenario C and Scenario A. - FFS The probability value is [0.50, 0.25].

- Discussion

Different test set-up/requirements according to UE capability of supporting CSI-validation features

For NR-U operation, gNB should perform LBT to determine whether it can transmit PDSCH and SSB/CSI-RS. Also, in RAN4#97e [2], it was agreed to model LBT failure as part of the downlink transmission model and apply the downlink transmission model to all DL signals in unlicensed carrier (including SSB and TRS transmission). For UE to perform channel tracking, it should aware of the presence or absence of the P/SP-CSI-RS. According to LS from RAN1 [3], it is not mandatory for UE to determine the presence or absence of the P/SP-CSI-RS. Considering the real deployment scenario, we think UE should support some optional capability for CSI-validation. As it was agreed to define test cases for UE with capability csi-RS-ValidationWith-DCI, we think it is not necessary to define some unrealistic/artificial test cases, e.g., set LBT failure probability as 0 or enable TRS always on considering LBT failure.

Proposal 1: Do not define test cases for UE which does not support CSI-validation features.

Random DL duration and symbol length of the last slot in the COT

The downlink transmission model is illustrated in figure 1. For the random DL duration, considering the regional regulation, we support to randomly choose one with equal probability from the set {2, 3, 5, 6}. For the symbol length of the last slot in the COT, to reduce the number of possibilities, it can be randomly chosen with equal probability from the set {6, 9, 12, 14}.

Figure 1. Illustration for the downlink transmission model

Proposal 2: COT duration is randomly chosen with equal probability from the set {2, 3, 5, 6} slots.

Proposal 3: Symbol length for the last slot in the COT is randomly chosen with equal probability from the set {6, 9, 12, 14} symbols.

LBT failure probability

It was agreed in RAN4#98e to apply the same LBT probability for both scenario A and scenario C. The remaining issue is to determine the value between 0.5 and 0.25. Based on the simulation assumption in Annex, we provide a simulation result in figure 2 for different LBT probability, considering different slot length and symbols length in the last slot. It can be shown that there is small difference between LBT probability 0.25 and 0.5. To reduce testing time, LBT failure probability 0.25 could be accepted.

Figure 2. Normalized throughput versus SNR for different LBT probability

Proposal 4: Define LBT failure probability as 0.25.

Frequency occupation for the CSI-RS for tracking

For operation with shared spectrum channel access, the maximum bandwidth of the CSI-RS resource for tracking is 48 RBs [4].

if carrier Ngridsize,μ=52, NBWP,isize=52, μ=0 and the carrier is configured in paired spectrum, the bandwidth of the CSI-RS resource, as given by the higher layer parameter freqBand configured by CSI-RS-ResourceMapping, is X resource blocks, where X ≥ 28 resources if the UE indicates trs-AddBW-Set1 for the trs-AdditionalBandwidth capability and X ≥ 32 if the UE indicates trs-AddBW-Set2 for the AdditionalBandwidth capability; in these cases, if the UE is configured with CSI-RS comprising X<52 resource blocks, the UE does not expect that the total number of PRBs allocated for DL transmissions but not overlapped with the PRBs carrying CSI-RS for tracking is more than 4, where all CSI-RS resource configurations shall span the same set of resource blocks; otherwise, the bandwidth of the CSI-RS resource, as given by the higher layer parameter freqBand configured by CSI-RS-ResourceMapping, is the minimum of 52 and NBWP,isize resource blocks, or is equal to NBWP,isize resource blocks. For operation with shared spectrum channel access, freqBand configured by CSI-RS-ResourceMapping, is the minimum of 48 and NBWP,isize resource blocks, or is equal to NBWP,isize resource blocks.

As the number of RB for the smallest bandwidth for test cases is 51, the number of RB for CSR-RS should be 48.

Proposal 5: The RB number of CSI-RS for tracking should be 48.

- Conclusion

The proposals to define the test cases for NR-U PDSCH are summarized as below:

Proposal 1: Do not define test cases for UE which does not support CSI-validation features.

Proposal 2: COT duration is randomly chosen with equal probability from the set {2, 3, 5, 6} slots.

Proposal 3: Symbol length for the last slot in the COT is randomly chosen with equal probability from the set {6, 9, 12, 14} symbols.

Proposal 4: Define LBT failure probability as 0.25.

Proposal 5: The RB number of CSI-RS for tracking should be 48.

- Reference

- R4-2103987, “Way Forward on NR-U UE demodulation requirements”, Qualcomm

- R4-2017685, “Way Forward on NR-U UE demodulation requirements”, Qualcomm

- R1-2006195, “LS on UE behavior for P/SP-CSI-RS reception in NR-U”, MediaTek

- 3GPP TS38.214, “NR; Physical layer procedure for data”

- Annex

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Common serving cell parameters | Physical Cell ID |  |  | 0 |
|  | SSB position in burst |  |  | First SSB in Slot #0 |
|  | SSB periodicity |  | ms | 20 |
|  | QSSB |  |  | 8 |
|  | DRS Transmission Window Duration |  | ms | 1 |
|  | DRS Transmission Window Periodicity |  | ms | 20 |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-R,S resource 1,2,3,4 |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 |
|  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |
|  | CDM Type |  |  | 'No CDM’ for CSI-RS resource 1,2,3,4 |
|  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS periodicity |  | Slots | 40 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset |  | Slots | 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = 48 |
|  | QCL info |  |  | TCI state #0 |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 2 |
|  | Length (L) |  |  | According to DL Transmission Model |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |
| RB Allocation |  |  |  | Full BW |
| MIMO Layers |  |  |  | 2 |
| Number of HARQ Processes |  |  |  | 8 |
| DL Transmission Model | Maximum COT Duration |  | ms | 4 |
|  | Idle Time after COT |  | ms | 1 |
|  | DL Transmission Model Period/ Fixed Frame Period |  | ms | 5 |
|  | Probability of LBT Failure pLBT |  |  | 0.25 or 0.5 |
|  | Duration of the Downlink portion of the COT |  | Slots | {2,3,5,6} |
|  | PDSCH Allocation in the last Slot of the Downlink portion of the COT |  | Symbols | {6,9,12,14} |
| TCI state #0 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| Maximum number of HARQ transmission |  |  |  | 4 |
| Symbols for all unused REs |  |  |  | OCNG Annex A.5 |