3GPP TSG-RAN WG4 Meeting # 98-bis-e 	R4-2107121

# Electronic Meeting, Apr. 12-20, 2021

Agenda Item:	8.8.2.2

Source: 		Qualcomm Incorporated

Title: 	Simulation results for NTN co-existence calibration

Document for:	Discussion

# Introduction

In RAN4#98e meeting, NTN co-existence simulation was discussed and RAN4 agreed to start the simulation alignment in Q2 2021 [1]. The details of simulation scenarios and assumptions are captured in [2] [3]. In this paper, we provide our calibration results for single NTN system.

# 2. Discussion

## 2.1 Simulation scenarios

In TR 38.821, there are 30 cases for the calibration of single NTN system in the DL and UL directions.

The simulated scenarios for the calibration, have been done for the handheld as terminal and at the S-band and are summarized as below:

Table 1: List of calibration study cases

| Case | Satellite orbit | Satellite parameter set | Central beam elevation | Terminal | Frequency Band | Frequency/ Polarization Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| 4 | GEO | Set 1 | 45 deg | Handheld | S-band | Option 1 |
| 5 | GEO | Set 1 | 45 deg | Handheld | S-band | Option 2 |
| 9 | LEO-600 | Set 1 | 90 deg | Handheld | S-band | Option 1 |
| 10 | LEO-600 | Set 1 | 90 deg | Handheld | S-band | Option 2 |
| 14 | LEO-1200 | Set 1 | 90 deg | Handheld | S-band | Option 1 |
| 15 | LEO-1200 | Set 1 | 90 deg | Handheld | S-band | Option 2 |
| 19 | GEO | Set 2 | 45 deg | Handheld | S-band | Option 1 |
| 20 | GEO | Set 2 | 45 deg | Handheld | S-band | Option 2 |
| 24 | LEO-600 | Set 2 | 90 deg | Handheld | S-band | Option 1 |
| 25 | LEO-600 | Set 2 | 90 deg | Handheld | S-band | Option 2 |
| 29 | LEO-1200 | Set 2 | 90 deg | Handheld | S-band | Option 1 |
| 30 | LEO-1200 | Set 2 | 90 deg | Handheld | S-band | Option 2 |

## 2.2. Network layout model

Four additional tiers of beams are considered in the simulation surrounding the 19 inner beams layout (In total, there are 127 beams).

FRF1 and FRF3 considered in the system level simulations as shown in the next figure.

Figure 1:  Frequency reuse factor (FRF1 and FRF3)

| FRF1 | FRF3 |
| --- | --- |

Figure 2 shows the satellite with 90-degree elevation angle and the beams footprint on earth.

Figure 2: cell layout for single NTN operator with satellite at 90-degree elevation angle

## 2.3. Simulation parameters

Table 2 Set-1 satellite parameters for co-existence study

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture | 2GHz | 22 m | 2 m | 2 m |
| Satellite EIRP density |  | 59 dBW/MHz | 40 dBW/MHz | 34 dBW/MHz |
| Satellite Tx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| 3dB beamwidth |  | 0.4011 deg | 4.4127 deg | 4.4127 deg |
| Satellite beam diameter |  | 250 km | 90 km | 50 km |

Table 3 Set-2 satellite parameters for co-existence study

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture | 2GHz | 12 m | 1 m | 1 m |
| Satellite EIRP density |  | 53.5 dBW/MHz | 34 dBW/MHz | 28 dBW/MHz |
| Satellite Tx max Gain |  | 45.5 dBi | 24 dBi | 24 dBi |
| 3dB beamwidth |  | 0.7353 deg | 8.8320 deg | 8.8320 deg |
| Satellite beam diameter |  | 450 km | 190 km | 90 km |

Table 4 UE characteristics

| Characteristics | Handheld |
| --- | --- |
| Frequency band | 2 GHz |
| Polarisation | Linear: +/-45°X-pol |
| Rx Antenna gain | 0 dBi per element |
| Antenna temperature | 290 K |
| Noise figure | 7 dB |
| Tx transmit power | 200 mW (23 dBm) |
| Tx antenna gain | 0 dBi per element |

To align with the simulation results listed in TR 38.811, 10 UEs per are beam considered for the system level simulations. UEs placed in the 19 inner beams only considered for the metrics statistic (e.g., coupling loss, and SINR).

## 2.4. Satellite antenna pattern modelling

Satellite antenna pattern modelling can be referred to section 6.1.1.1 in TR 38.821.

## 2.5. Propagation model

Propagation model between NTN and UE can be referred to section 6.6 in TR 38.811.

Base-line conditions:

Clear Sky

Line of sight (LOS)

Rural deployment with free space path loss model and shadow fading based on elevation angle according to TR 38.811 table 6.6.2-3.

## 2.6. Transmission power control model

For downlink scenario, no power control scheme is applied.

For uplink scenario, TPC model specified in Section 9.1 TR 36.942 is NTN with following parameters.

Where, Pmax = 23dBm, CLx-ile and γ are set as following:

- CLx-ile = 88 + 10*log10 (200/X) + 11 – Y

where X is UL transmission BW (MHz) and Y is the BS/Satellite noise figure (It is assumed to be same as UE noise figure, i.e., 7dB. This should be further discussed.)

- γ = 1.

## 2.7 Simulation results (CL and SINR)

- DL results

- Case 4 GEO - Set 1

- Case 5 GEO - Set 1

- Case 9 LEO 600 - Set 1

- Case 10 LEO 600 - Set 1

- Case 14 LEO 1200 - Set 1

- Case 15 LEO 1200 - Set 1

- Case 19 GEO - Set 2

- Case 20 GEO - Set 2

- Case 24 LEO 600 - Set 2

- Case 25 LEO 600 - Set 2

- Case 29 LEO 1200 - Set 2

- Case 30 LEO 1200 - Set 2

- UL results

- Case 4 GEO - Set 1

- Case 5 GEO - Set 1

- Case 9 LEO 600 - Set 1

- Case 10 LEO 600 - Set 1

- Case 14 LEO 1200 - Set 1

- Case 15 LEO 1200 - Set 1

- Case 19 GEO - Set 2

- Case 20 GEO - Set 2

- Case 24 LEO 600 - Set 2

- Case 25 LEO 600 - Set 2

- Case 29 LEO 1200 - Set 2

- Case 30 LEO 1200 - Set 2

- UE TX Power Distribution

The UL CLx-ile calculated based on the equation in 2.6 with UL transmission BW 0.36 MHz for 1 UE as mentioned in TR 38.821 and the satellite noise figure assumed to be same as UE 7dB with  γ = 1. Then CLx-ile will be 119.4.

By looking to CL curves in the UL for any scenario, the min value around 122 dB, hence all the UEs will transmit with full Tx power 23 dBm in all the scenarios.

Observation: Further discussion is needed for CLx-ile and the noise figure for the satellite to align the UE Tx power in the UL.

# 3.	Conclusion

In this paper, we provided both DL and UL simulation results for single NTN system for calibration. Based on the simulation results, we have the following observation:

Observation: Further discussion is needed for CLx-ile and the noise figure for the satellite to align the UE Tx power in the UL.

# 4.	References

- R4-2103949, Email discussion summary for [98e][311] NTN_Solutions_Part2, Thales

- R4-2103878, WF for NTN co-existence study, Samsung

- R4-2103998, Simulation assumption for NTN co-existence study, CATT